- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Flicker Mitigation in Clause 6.9.6 |  |
| Date Submitted | 2010-07-09 |  |
| Source | Joachim W. Walewski (Siemens AG, Germany) | Voice: +49-89-636-45850 Fax: E-mail: joachim.walewski curly a siemens.com |
| Re: | Response to LB #50; DCN |  |
| Abstract | This contribution proposes new text for Clause 6.9.6 |  |
| Purpose | [To resolve the LB comments related to 5.5.3 and 5.5.4] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Blue: new text (changes tracked toward sub-editor versions of D2 from 21st June 2010)

Pink: Editorial instructions

Crossed out: deleted

Orange: Labels (hot links) in framemaker

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

6.9.6.4 PHY type independent dimming

Light dimming is generally a MAC layer function; however, it does require that the PHY periodically adjust the light source. The details are described in 7.3.10

Figure 48—Lighting Dimming Model

Adjustment time is inserted into either the idle pattern or into the data frame, as shown below, to adjust the average intensity of the light.

Move Figure 49 to subclause 6.9.6.5 (see below).

Figure 49—Flicker compensation

6.9.6.5 Flicker mitigation

Flicker mitigation applies to both data transmission and idle periods. First, idle periods are discussed and how to avoid inter-frame flicker between both periods and during the idle period itself.

For OOK (PHY I-III) visibility patterns as described in Subclause 6.9.6.1 are used to ensure light emission by the VLC transmitters that has the same average brightness of adjacent MFTPs as during data transmission, unless the dimmer setting is changed above the MAC sublayer. These patterns shall be sent out of band. . It is preferable that the idle patterns do not conflict with the code words generated by the line code for the data.

For VPPM (PHY I-II) no particular visibility patterns are needed. Here, flicker between data transmission states is avoided by sending freely chosen  visibility patterns at the same dimmer setting of adjacent MFTPs as during data transmission, unless the dimmer setting is changed above the MAC sublayer. . It is preferable that the idle patterns do not conflict with the code words generated by the line code for the data.

For CSK (PHY III) freely chosen visibility patterns shall sent at the same amplitude-dimming during adjacent MFTPS as during data transmission, unless the dimmer setting is changed above the MAC sublayer.  . It is preferable that the idle patterns do not conflict with the code words generated by the line code for the data.

During data transmission with VPPM flicker shall be avoided by sending freely chosen visibility patterns and modulating consecutive data frames with the duty-cycles specified according to the current dimmer setting. For particularities of VPPM dimming see Subclause 6.9.6.2.

When using OOK (PHY I-III) flicker can be avoided by the following procedures.

An adjustment time is inserted into either the idle pattern or into the data frame, as shown in Figure 49, in order to adjust the average intensity of the light. Idle patterns are modulated out of band.

Figure 49—Flicker mitigation

Use original file in current D2 version, not the above bitmap.

To mitigate inter-frame flicker, after each data frame one sends an idle pattern, whose average

brightness is equal to that of the data frame; that is, the adjustment pattern is chosen so, that the emitted optical energy of the data and the adjacent idle frame together are independent of the content of the data frame and equal from one transmission cycle to the next.  The idle pattern has the same duty cycle as the modulated frame, but the pulse repetition rate may be set lower so as not to cause "in-band" modulation interference with any VLC modulation. For example, one can send a constant signal.  Any types of idle pattern can be used for compensation. It is preferable that the idle patterns do not conflict with the code words generated by the line code for the data.