- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Flicker Mitigation in Clause 6.9.6 |  |
| Date Submitted | 2010-07-09 |  |
| Source | Joachim W. Walewski (Siemens AG, Germany) | Voice: +49-89-636-45850 Fax: E-mail: joachim.walewski curly a siemens.com |
| Re: | Response to LB #50; DCN |  |
| Abstract | This contribution proposes new text for Clause 6.9.6 |  |
| Purpose | [To resolve the LB comments related to 5.5.3 and 5.5.4] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Blue: new text (changes tracked toward sub-editor versions of D2 from 21st June 2010)

Pink: Editorial instructions

Crossed out: deleted

Orange: Labels (hot links) in framemaker

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

6.9.6.5 Flicker mitigation

Flicker mitigation can be divided into intra-frame and inter-frame flicker (see subclause 5.5.3.1). Inter-frame flicker in OOK is avoided by aid of line coding (see subclause 6.1.3), while VPPM inherently does not cause any inter-frame flicker. Inter-frame flicker is avoided in PHY III due to line coding and the high line rates at which this modulation is used (see subclause 6.1.3).

Inter-frame flicker mitigation applies to both data transmission and idle periods.

While idling, OOK (PHY I-II) visibility patterns as described in Subclause 6.9.6.1 are used to ensure light emission by the VLC transmitters that has the same average brightness over adjacent MFTPs as during data transmission, unless the dimmer setting is changed above the MAC sublayer. These patterns can be modulated out of band (see subclause 5.5.3.1.2). . It is preferable that the idle patterns do not conflict with the code words generated by the line code for the data.

During data transmission with OOK, an adjustment time can be inserted into either the idle pattern or into the data frame, as shown in Figure 49, in order to adjust the average intensity of the light. Idle patterns can be modulated out of band.

Figure 49—Flicker mitigation

Figure 49 moved here from subclause 6.9.6.4. Use original file, not the above bitmap. Figure will be revised in response to CIDs 110 and ???.

To mitigate inter-frame flicker, after each data frame one sends an idle pattern, whose average

brightness is equal to that of the data frame; that is, the adjustment pattern is chosen so, that the emitted optical energy of the data and the adjacent idle frame together are independent of the content of the data frame and equal from one transmission cycle to the next.  The idle pattern has the same duty cycle as the modulated frame, but the pulse repetition rate may be set lower so as not to cause "in-band" modulation interference with any VLC modulation. For example, one can send a constant signal.  Any types of idle pattern can be used for compensation. It is preferable that the idle patterns do not conflict with code words generated by the line code for the data.

For VPPM (PHY I-II) flicker between data frames is avoided by sending freely chosen  visibility patterns at the same dimmer setting during adjacent MFTPs as during data transmission, unless the dimmer setting is changed above the MAC sublayer. It is preferable that the idle patterns do not conflict with code words generated by the line code for the data.

During data transmission with VPPM flicker shall be avoided by modulating consecutive data frames with the duty-cycles specified according to the current dimmer setting. For particularities of VPPM dimming see Subclause 6.9.6.2.

In order to avoid flicker when using CSK (PHY III), data frames or freely chosen visibility patterns are sent at the same amplitude-dimming during adjacent MFTPS, unless the dimmer setting is changed above the MAC sublayer.  . It is preferable that the idle patterns do not conflict with the code words generated by the line code for the data.