- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [9th July, 2010] |  |
| Source | [553-696 subcommittee: Sang-Kyu Lim, Dae Ho Kim, Il Soon Jang, You Jin Kim, Tae-Gyu Kang, Joachim Walewski, Richard D. Roberts, Sridhar Rajagopal, Taehan Bae, Jae Seung Son, Clint Chaplin, Eun Tae Won] [ETRI, Siemens, Intel, Samsung Electronics] | Voice: [ +82-42-860-1573] Fax: [+82-42-860-5218] E-mail: [sklim@etri.re.kr] |
| Re: | [Response to LB #50 comments] |  |
| Abstract | [This document describes the current status of new text for clause 6.9.6 in D2 draft.] |  |
| Purpose | [To resolve the LB comments related to 6.9.6] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

6.9.6 Dimming and flicker mitigation

This provides normative text related to dimming and flicker mitigation support.

6.9.6.1 Dimming on idle time

Basically, the dimming on idle time is achieved by the technologies described in 5.5.3.2. The technologies for the dimming on idle time are designed to avoid flicker.

6.9.6.1.1 Idle pattern and adjustment time dimming

An in-band or out-of-band idle pattern whose duty cycle variation results in brightness variation may be optionally inserted between the data frames for light dimming. The adjustment time (which means "ON" and "OFF" time of a light source) can be also inserted into either the idle pattern or into the data frame (if using the dimmed OOK mode) to reduce or increase the average brightness of a light source.

6.9.6.1.2 Visibility pattern dimming

The visibility pattern is an in-band idle pattern and is sent as part of the payload of the visibility frame as defined in 7.5.  A set of 11 base low resolution visibility patterns with 10% step size for dimming are defined in Figure 46. Any set of 11 base low resolution visibility patterns of any length can be used as long as there is no conflict between the visibility pattern and a valid RLL code. The low resolution patterns are used to develop high resolution visibility patterns by averaging them across time to generate the required high resolution pattern. For example, if visibility patterns are available at 10% resolution, then a 25% visibility pattern can be attained for example, by alternately sending a 20% visibility pattern followed by a 30% visibility pattern. This method guarantees all visibility patterns will retain the same properties as the base low resolution visibility patterns. The high resolution visibility pattern shall be provided by using the low resolution patterns using the algorithm specified in Figure 47. The visibility patterns are repeated to satisfy the frame length as mentioned in the PHY header.

Figure 46 – Visibility Patterns (example for 8B10B code)

Figure 47—Generating high resolution for dimming using visibility patterns and VPPM

6.9.6.2 Dimming on data transmission time

The dimming technologies on data transmission time depend on the PHY modulation schemes and are designed to avoid flicker.

6.9.6.2.1 CSK-mode dimming

The CSK-mode dimming is described in 5.5.3.1.3.  In CSK, total average power of multiple light sources is constant. For dimming control, the instantaneous power per light source is changed to keep the same average intensity. CSK keeps the center color of the color constellation with required intensity.  A color stabilization scheme for illuminators is also provided in 6.9.6.4.

6.9.6.2.2 OOK-mode dimming

The OOK-mode dimming is described in 5.5.3.1.4.  The OOK-mode dimming is supported by using the dimmed OOK bit field set in the PHY header as explained in Table 23 in 6.4.1.3.

6.9.6.2.3 VPPM-mode dimming

The VPPM-mode dimming is described in 5.5.3.1.5.  The VPPM PHY shall have basic dimming level support at 10% duty cycle resolution. To support higher resolution for dimming from 0 to 100% in steps of 0.1%, the VPPM PHY shall use the algorithm as provided in Figure 47. For example, for supporting 25% dimming, the VPPM PHY shall alternately send 20% and 30% duty cycle symbols.

6.9.6.3 Flicker mitigation

Flicker mitigation can be divided into intra-frame mitigation and inter-frame flicker mitigation (see  5.5.3.2).

Intra-frame flicker mitigation refers to mitigation flicker within the transmission of a data frame. Intra-frame flicker in OOK is avoided by the use of the dimmed OOK mode as in 6.9.6.2.1 and RLL coding (see  6.1.3). VPPM inherently does not cause any inter-frame flicker and also uses a RLL code. Inter-frame flicker is avoided in CSK by ensuring constant average power across multiple light sources along with scrambling and the high optical clock rates (MHz) at which this modulation is used (see  6.1.3).

Inter-frame flicker mitigation applies to both data transmission (RX mode) and idle periods. While idling, visibility patterns or idle patterns as described in  6.9.6.1 may be used to ensure light emission by the VLC transmitters have the same average brightness over adjacent MFTPs as during data transmission. These patterns can be modulated in-band or out-of-band as in 6.9.6.1.1.

When the dimmer setting is changed above the MAC sublayer, the MAC and PHY layers adjust the data transmission and idle time transmission to adjust to the new dimmer settings.

Table 1 : Flicker mitigation for various modulation modes

| Flicker mitigation | Data transmission (Intra-frame flicker) | Idle or RX periods (Inter-frame flicker) |
| --- | --- | --- |
| OOK modulation | Dimmed OOK mode, RLL code | Idle/visibility patterns |
| VPPM modulation | VPPM guarantees no intra-frame flicker, RLL code |  |
| CSK modulation | Constant average power across multiple light sources, scrambler, high optical clock rates (MHz) |  |