DSME comment resolution

CID 17 and 50	1

CID 20	2

CID 30	2

CID 79	4

CID 97	5

CID 99	6

CID 111	7

CID 1610 and 984	7

CID 1612	11

Proposed Resolution for Comments Related to Group ACK Mechanism	17

CID 258	22

CID 260 & CID 1049	24

CID 500	25

CID 997	26

CID 568, 1064, 1207	26

CID 568, 1064, 1207, 1366, 409, 410, 412, 418, 426, 428, 429, 430, 431, 432, 433, 434, 627, 672, 677, 678, 1224, 1613, 1614, 1615, 1616, 1617	27

CID 975, 976, 1085 and 1471	29

# CID 17 and 50

Insert in subclause 7.1.17 after Table 78:

Table 78.ccc—Additional MAC enumerations description

| Enumeration | Value | Description |
| --- | --- | --- |
| BAD_CHANNEL | 0xd0 | The channel probe has detected the bad channel condition. |
| MAC_DECISION | 0xd1 | The allocation of DSME-GTS shall be determined by the MLME of the devices. |

# CID 20

Figure 39.a ―Message sequence chart for DSME-GTS allocation/deallocation/reallocation

# CID 30

Figure 39.d Message sequence chart for link status report

# CID 79

Figure 65.aa―SAB sub-block in Channel Adaptation Mode

# CID 97

Insert in subclause 7.5.10.8 after line 21:

Table 78.ddd—Allocation Counter Table (macACT) Description

| Field | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Direction | Integer | 0-1 | The direction of the allocated DSME-GTS. 0 : Transmission (TX), 1: Reception (RX) |
| Source/Destination Address | Device address | 0x0000–0xfffd | The 16-bit short address of the device that is the source (if RX) or the destination of the allocated DSME-GTS. |
| Counter | Integer | 0x0000–0xffff | An idle counter, in other word, the number of beacon intervals since the allocated DSME-GTS was used. |
| LinkQuality | Integer | 0x0000-0xffff | The link quality of the allocated DSME-GTS. |

# CID 99

Device 2

Device 1

The number of superframes in a multi

-

superframe

: N

= 2

(MO

–

SO

)

Multi

-

superframe

Duration:

MD =

aBaseSuperframeDuration

*2

MO

symbols

CAP

Superframe

Beacon slot

(

Beacon

)

Beacon Interval

CAP

Multi

-

Superframe

Multi

-

Superframe

Beacon slot

(

Beacon

)

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

Example:    BO

= 6, SO = 3, MO = 5

- Figure 0.a—DSME-based multi-superframe structure

Device 2

Device 1

CAP

Superframe

Beacon slot

(

Beacon

)

Beacon Interval

CAP

Multi

-

Superframe

Multi

-

Superframe

Beacon slot

(

Beacon

)

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

CAP

Example:    BO

= 6, SO = 3, MO = 5

CAP

CAP

EGTS Slots

…

Channels

0

1

2

2  4

1  3

5  2

0

1

2

3

4

5

6

Figure 0.g—Channel Adaptation Mode

# CID 111

# CID 1610 and 984

There are 5 modifications in the draft which need to be done to resolve comment CID 984 and 1610.

1. Replace the whole section of 7.1.20.4.4 with the following text:

7.1.20.4.4 MLME-DSME-INFO.indication

7.1.20.4.4.1 General

The MLME-DSME-INFO.indication primitive is used to indicate the reception of a DSME-GTS information request.

7.1.20.4.4.2 Semantics

The semantics of the MLME-DSME-INFO.indication primitive are as follows:

MLME-DSME-INFO.indication     (

DeviceAddress,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 78.?? specifies the parameters for the MLME-DSME-INFO.indication primitive.

Table 78.?? – MLME-DSME-INFO.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64-bit IEEE address. | The address of the device requesting DSME-GTS information. |

7.1.20.4.4.3 When generated

The MLME-DSME-INFO.indication primitive is generated by the MLME of the Destination device and issued to its next higher layer to indicate the reception of a DSME-GTS information request command.

7.1.20.4.4.4 Appropriate usage

When the next higher layer of a Destination device receives the MLME-DSME-INFO.indication primitive, the Destination device issues the MLME-DSME-INFO.response primitive to its MLME.

7.1.20.4.5 MLME-DSME-INFO.response

7.1.20.4.5.1 General

The MLME-DSME-INFO.response primitive is used to initiate a response to a MLME-DSME-INFO.indication primitive.

7.1.20.4.5.2 Semantics

The semantics of the MLME-DSME-INFO.response primitive are as follows:

MLME-DSME-INFO.response     (

DeviceAddress,

Timestamp,

DSMECharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table 78.?? specifies the parameters for the MLME-DSME-INFO.response primitive.

Table 78.?? — MLME-DSME-INFO.response parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64-bit IEEE address. | The address of the device requesting DSME-GTS information. |
| Timestamp | Integer | 0x000000–0xffffff | The time, in symbols, at which the DSME information reply command (see7.3.12.6) was transmitted. The symbol boundary is described by macSyncSymbolOffset (see Table 86 in 7.4.1). This is a 24-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. |
| DSMECharacteristics | DSME Characteristics | See 7.3.12.4.3 | The characteristics of the DSME-GTS between Source device and Destination device. This value is correct only when DSME Characteristics Type subfield is 110. Otherwise, it indicates that Source device’s request for DSME-GTS information is fail. |

7.1.20.4.5.3 Appropriate usage

The MLME-DSME-INFO.response primitive is generated by the next higher layer of a Destination device and issued to its MLME in order to respond to the MLME-DSME-INFO.indication primitive.

7.1.20.4.5.4 Effect on receipt

When the MLME of a Destination device receives the MLME-DSME-INFO.response primitive, it generates a DSME-GTS information reply command.

7.1.20.4.6 DSME information sequence chart

Figure 39.c illustrates the sequence of messages necessary for successful DSME-GTS information request. Figure 39.c depicts the messages flow for the case in which the Source device requests the timestamp and the DSME-GTS parameters from the Destination device.

Figure 39.c - Message sequence chart for DSME-GTS information request

2.

Insert ‘SecurityLevel, KeyIdMode, KeySource, KeyIndex’ between Line 33 and 34 in Page 57 Section 7.1.20.4.3.2.

3.

Change ‘two (e.g., 16-bit short addressing)’ (L4&L19 P112) to ‘three (i.e., 64-bit extended addressing)’.

4.

Insert ‘DSME Characteristics field is correct only when DSME Characteristics Type subfield is 110. Otherwise, it indicates that Source device’s request for DSME-GTS information is fail.’ at the end of ‘7.3.12.6 DSME information reply command’ (P112).

5.

Delete ‘the DSME information reply command frame indicating a failure or’ (L34 P168).

# CID 1612

Insert a new subclause “7.1.20.x MLME-DSME-SUPERFRAME” in the clause “7.1.20 DSME-specific MAC sublayer service specification”:

7.1.20.x.1 General

These primitives define how a device is instructed to request the parameters of superframe structure stored by Connection Device and notified of those parameters.

7.1.20.x.2 MLME-DSME-SUPERFRAME.request

7.1.20.x.2.1 General

The MLME-DSME-SUPERFRAME.request primitive allows the device to request the parameters of superframe structure stored by Connection Device.

7.1.20.x.2.2 Semantics

The semantics of the MLME-DSME-SUPERFRAME.request primitive are as follows:

MLME- DSME-SUPERFRAME.request   (

DstAddrMode

DstAddr,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table x.1 specifies the parameters for the MLME-DSME-SUPERFRAME.request primitive.

Table x.1 – MLME-DSME-SUPERFRAME.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DstAddrMode | Integer | 0x02–0x03 | The addressing mode of the Connection Device to which the request is intended. This parameter can take one of the following values: 0x02 = 16-bit short address, 0x03 = 64-bit extended address. |
| DstAddr | Device address | As specified by the DstAddrMode parameter | The address of the Connection Device to which the request is intended. |

7.1.20.x.2.3 Appropriate usage

The MLME-DSME-SUPERFRAME.request primitive is generated by the next higher layer of a device and issued to its MLME to request the parameters of superframe structure stored by Connection Device.

7.1.20.x.2.4 Effect on receipt

On receipt of the MLME-DSME-SUPERFRAME.request primitive, the MLME of the device generates a DSME superframe request command.

7.1.20.x.3 MLME-DSME-SUPERFRAME.indication

7.1.20.x.3.1 General

The MLME-DSME-SUPERFRAME.indication primitive is used to indicate the reception of a DSME superframe request command.

7.1.20.x.3.2 Semantics

The semantics of the MLME-DSME-SUPERFRAME.indication primitive are as follows:

MLME- DSME-SUPERFRAME.indication   (

DeviceAddress,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table x.2 specifies the parameters for the MLME-DSME-SUPERFRAME.indication primitive.

Table x.2 – MLME-DSME-SUPERFRAME.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64-bit IEEE address. | The address of the device requesting the parameters of superframe structure stored by Connection Device. |

7.1.20.x.3.3 When generated

The MLME-DSME-SUPERFRAME.indication primitive is generated by the MLME of the Connection Device and issued to its next higher layer to indicate the reception of a DSME superframe request command.

7.1.20.x.3.4 Appropriate usage

When the next higher layer of the Connection Device receives the MLME-DSME-SUPERFRAME.indication primitive, the Connection Device determines whether it has the parameters. The next higher layer of the Connection Device then issues the MLME-DSME-SUPERFRAME.response primitive to its MLME.

7.1.20.x.4 MLME-DSME-SUPERFRAME.response

7.1.20.x.4.1 General

The MLME-DSME-SUPERFRAME.response primitive is used to initiate a response to an MLME-DSME-SUPERFRAME.indication primitive.

7.1.20.x.4.2 Semantics

The semantics of the MLME-DSME-SUPERFRAME.response primitive are as follows:

MLME- DSME-SUPERFRAME.response   (

DeviceAddress,

BeaconOrder,

SuperframeOrder,

Multi-superframeOrder,

status,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table x.3 specifies the parameters for the MLME-DSME-SUPERFRAME.response primitive.

Table x.3 – MLME-DSME-SUPERFRAME.response parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64 bit IEEE address. | The address of the device requesting the parameters of superframe structure stored by Connection Device. |
| BeaconOrder | Integer | 0-15 | The parameters of superframe structure stored by Connection Device. Also see 7.5.10.1.1 for the explanation. |
| SuperframeOrder | Integer | 0-15 |  |
| Multi-superframeOrder | Integer | 0-15 |  |

7.1.20.x.4.3 Appropriate usage

The MLME-DSME-SUPERFRAME.response primitive is generated by the next higher layer of the Connection Device and issued to its MLME in order to respond to the MLME-DSME-SUPERFRAME.indication primitive.

7.1.20.x.4.4 Effect on receipt

On receipt of the MLME-DSME-SUPERFRAME.response primitive, the MLME of the Connection Device generates a DSME superframe response command.

7.1.20.x.5 MLME-DSME-SUPERFRAME.confirm

7.1.20.x.5.1 General

The MLME-DSME-SUPERFRAME.confirm primitive is used to inform the next higher layer of the initiating device whether its request is successful.

7.1.20.x.5.2 Semantics

The semantics of the MLME-DSME-SUPERFRAME.confirm primitive are as follows:

MLME- DSME-SUPERFRAME.confirm   (

BeaconOrder,

SuperframeOrder,

Multi-superframeOrder,

status,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

Table x.4 specifies the parameters for the MLME-DSME-SUPERFRAME.confirm primitive.

Table x.4 – MLME-DSME-SUPERFRAME.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BeaconOrder | Integer | 0-15 | The parameters of superframe structure stored by Connection Device. Also see 7.5.10.1.1 for the explanation. |
| SuperframeOrder | Integer | 0-15 |  |
| Multi-superframeOrder | Integer | 0-15 |  |

7.1.20.x.5.3 When generated

The MLME-DSME-SUPERFRAME.confirm primitive is generated by the initiating MLME and issued to its next higher layer in response to MLME-DSME-SUPERFRAME.request primitive.

7.1.20.x.5.4 Appropriate usage

On receipt of the MLME-DSME-SUPERFRAME.confirm primitive, the next higher layer of the initiating device is notified of the result of its request of the parameters of superframe structure stored by Connection Device.

Insert a new subclause “7.3.12.x DSME-Superframe request command” in the clause “7.3.12 DSME-commands”:

The superframe request command allows a device to request the parameters of superframe structure stored by Connection Device.

The superframe request command shall be formatted as illustrated in Figure x.1.

| octets: | 1 |
| --- | --- |
| MHR fields | Command Frame Identifier (see Table 82) |

Figure x.1 – Superframe request command format

Both the Destination Addressing Mode subfield and the Source Addressing Mode subfield of the Frame Control field shall each be set to three (i.e., 64-bit extended addressing). The Destination Address field shall be the address of the Connection Device.

Insert a new subclause “7.3.12.y DSME-Superframe response command” in the clause “7.3.12 DSME-commands”:

The superframe response command allows the Connection Device to reply the results of the request for the parameters of superframe structure.

The superframe response command shall be formatted as illustrated in Figure y.1.

| bits: | 8 | 4 | 4 | 4 |
| --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier (see Table 82) | Beacon Order | Superframe Order | Multi-superframe Order |

Figure y.1 – Superframe response command format

The Destination Addressing Mode and Source Addressing Mode subfields of the Frame Control field shall each be set to three (i.e., 64-bit extended addressing). The Destination Address field shall contain the extended address of the requesting device.

The Beacon Order field shall contain the value of BO stored by the Connection Device. The Superframe Order field shall contain the value of SO stored by the Connection Device. The Multi-superframe Order field shall contain the value of MO stored by the Connection Device. If Beacon Order field value equal to 15, Superframe Order field and Multi-superframe Order field shall be ignored.

(Please also update Table 82 for command frame identifiers of DSME-Superframe request command and ‘DSME-Superframe response command’.)

# Proposed Resolution for Comments Related to Group ACK Mechanism

IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Proposed Resolution for Comments Related to Group ACK Mechanism.doc |  |
| Date Submitted | [6 July, 2010] |  |
| Source | [Ghulam Bhatti] [MERL] [201 Broadway, Suit 8, Cambridge, MA 02139] | Voice: [ 617-621-7513 ] Fax: [ 617-621-7550 ] E-mail: [ gbhatti@merl.com] |
| Re: |  |  |
| Abstract | [This document proposes changes in the TG4 draft specification in order to resolve GACK related comments. ] |  |
| Purpose | [Submit to the P802.15.4e Working Group] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Proposed Comment Resolution (Related to GACK Mechanism)

Info for editors:

- Currently Group ACK mechanism is not described in a systematic way. Bits and pieces are scattered around. Specifically, its use in LL mode and DSME mode is confusing. A better description that follows IEEE style can be devised as follows.

- The frame structure for GACK has been moved into section 7.2.2.3. This section already has description of different frame structures including ACK farme. Since GACK frame follows the same general structure as for ACK frame (as specified in Figure 53 on page 72 of R7 specification), we only need to describe MAC payload for the GACK frame. The structure for the MHR for GACK frame is shared with a normal ACK frame. We need to add a new section 7.2.2.3.3.

- Section 7.2.3.1.4.5 is then updated. That section describes how GACK is used in short frame format of LL mode.

- Section 7.2.4.2.5 is modified too. That section describes how GACK frame will be used in DSME mode.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++

*Add the following sub-section*

7.2.2.3.3 Group Acknowledgement (GACK) frame

7.2.2.3.3.1 General

A Group ACK frame is meant for reducing the communication overhead in a network. Instead of transmitting an ACK frame for individual data frames received in each GTS, a coordinator device, if using the GACK mechanism,  shall acknowledge the reception of multiple data frames by transmitting only a single GACK frame. A Group ACK frame shall follow the same general structure as the acknowledgement frame, as defined in Figure 53 in section 7.2.2.3. However, the GACK frame shall have its own MAC payload sub-field. The structure of MAC payload in a GACK frame is described in the following sub-section. It may, however, be mentioned that some of these fields may not be used in the short frame structure used for LL mode.

7.2.2.3.3.2 Group ACK frame control field

Specifically, Ack. Control field for a GACK frame shall have the following format.

Ack. Identifier

Payload Size

bits: 0-2

Reserved

3-4

Bitmap Size

7

5-6

Figure XXX – Group ACK control field

Ack Identifier: As described in section 7.2.2.3.2.2.2, a value of 0x02 for ACK Identifier subfield of Acknowledgement Control Field distinguishes a GACK frame from a normal individual ACK frame.

Payload Size: This field shall specify the length of the payload of a GACK frame. It is measured in number of octets.

Bitmap Size: This field shall specify the size of bitmap in terms of octets. The maximum number of bits in the bitmap could be thirty-two.

7.2.2.3.3.3 Group ACK frame payload field

The acknowledgement frame payload field for GACK specifies the information that is needed while constructing a group acknowledgement frame. A GACK frame is used for acknowledging a successful reception of multiple GTS frames by a coordinator. In a normal operational mode, the receiver node individually acknowledges each successfully received GTS frame. However, repeated switching over between Rx and Tx modes by the receiver is costly and time consuming. The concept of group acknowledgement allows the receiver to receive all scheduled GTS frames, while withholding individual acknowledgements, and then transmit a single ACK frame to acknowledge all successfully received frames.  Moreover, a GACK frame can also specify additional timeslots for the failed GTS transmissions to be retransmitted in the extended CFP. That is, a group acknowledgement not only indicates the reception status for a group of GTS data frames but it also specifies new slot allocations, if any, for retransmission of failed GTS transmissions.

The structure of the Acknowledgement Payload field of the group acknowledgement frame is shown in Figure 53.t.

Ack. Control

GACK Index

GACK Device List

GACKBitmap

PAN ID

Octets:

GTS Directions

2

1|2

Variable

Variable

Variable

1

Figure 53.t— MAC payload field GACK frame

Ack. Control: as specified in Section 7.2.2.3.2.2.2.

PAN ID: This field shall identify the PAN of the transmitting coordinator.

GACK Bitmap: This shall be a bitmap of size as specified by ‘Bitmap Size’ field in Ack. Control as described in Section 7.2.2.3.3.2. The bitmap shall indicate the state of transmissions during the CFP or ECFP of the current superframe. A bit set to ‘1’ shall indicate the fact that the coordinator received the data frame successfully in the corresponding GTS. A ‘0’, on the other hand, shall mean that the coordinator failed in receiving a data frame in the corresponding time slot.  . It is important to note that this field may have some unused bits that were actually padded to make the bitmap size a multiple of eight.

GACK Device List: This list identifies the sensor nodes that are being allocated the time slots in ECFP

portion of the superframe. This field shall be non-existent in the GACK frame in LL mode.

GACK Index: It is a list that specifies the start of each GTS for the allocated nodes in the same order as in

DSME device list. This field is applicable only in those systems that allow a GTS to consist of multiple

time slots. This field shall be non-existent in a system operating in LL mode or that does not allow frequency channel hopping.

GTS Directions: This list specifies the direction of transmission (uplink or downlink) for each GTS.

This is applicable only in the systems that allow the coordinator to transmit a frame to its sensor nodes by

using a GTS. This field shall be non-existent in the GACK frame in LL mode.

*Add the following text at the start of line 14, page 81.*

“The general structure of a GACK frame is specified in section 7.2.2.3.3.  However, short frame structure used in LL mode shall use a shortened structure for the GACK frame. Specifically, a GACK frame in LL mode shall use the same acknowledgement frame MHR field as described above in section 7.2.3.1.4.”

*Replace text from line 24, page 87 through line 16, page 88 with following text*

“structure of a GACK frame is specified in section 7.2.2.3.3. The beacon frame transmitted by a DSME coordinator shall indicate if the GACK mechanism is being used for acknowledging the reception of data frames. Specifically, by setting appropriate value to GACK Flag (bit 32 in Figure 53.p) in a DSME beacon frame, as described in Section 7.2.4.2.2.10, shall indicate if the coordinator will transmit individual ACK frames or a GACK frame for acknowledging the received data frames.”

*Add the following text at line 20, page 88*

“Some or all devices with failed transmissions, as indicated by the bitmap, can potentially be allocated another time slot in the extended CFP (ECFP) portion of the superframe. GACK Device List field in the MAC payload shall contain a list of all such devices. If the list has at least one device in it, the ECFP shall immediately follow the GACK frame. Otherwise, ECFP portion shall be non-existent. If ECFP is not non-existent, then another GACK frame shall follow to inform the transmitting nodes of the status of reception for their retransmissions.”

*Remore Figure 53.t*

# CID 258

(WJEONG)

The following changes are required to resolve the comment:

1. Change the name of subfield “DefaultSequenceFlag” in Figure53.q in Page 86 to “SequenceID”.

2. The paragraph at line 15-17 in Page 86, is changed to:

The SequenceID is 1 octet in length and indicate the ID of channel hopping sequence in use. SequenceID of 0 indicates that a default hopping sequence shall be used. SequenceID of 1 indicates that a hopping sequence generated by PAN coordinator shall be used. The other SequenceID values denote the sequence set by NHL shall be used. A device shall request a channel hopping sequence to its coordinator when it associates to a PAN, if SequencID is 1.

3. Replace the first element of DCHDescriptor DefaultSequenceFlag In Table 78.rr in page 54 with the following:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SequenceID | Integer | 0x0-0xF | Indicate the ID of channel hopping sequence in use: 0x0 : a default hopping seuqnce 0x1: a hopping sequence generated by PAN coordinator, 0x2 – 0xF: a hopping sequence set by NHL. If SequencID is 1, a device shall request a channel hopping sequence to its coordinator using ChannelSequenceRequest parameter when it associates to a PAN. |

4. Replace the last row of the Table in page 13 with the following:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelSequenceRequest | Integer | 0-1 | If SequenceID is ‘1’, this parameter shall be ‘1’, and a device shall request channel hopping sequence to its coordinator. Otherwise, this parameter shall be ‘0’, and a device shall not request channel hopping sequence. |

# CID 260 & CID 1049

(WJEONG)

Replace the table at line 19 in page 16 and the last element of DSME-PANDescriptor in Table 78.zz in page 63 with the following table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelHoppingSpecification | Set of Octets | - | Describes channel hopping information (See Figure 53.q.) |

# CID 500

(WJEONG)

Replace the paragraph at line 21 to line 22 in page 86 with the following one:

The ChannelOffset Bitmap Length subfield is 2 octet in length and shall specify the length of ChannelOffset Bitmap subfield in bits.

# CID 997

(WJEONG)

Replace the equation at line 5 and paragraph at line 6 to line 7 in page 160 with the following one:

C(i) = macChannelHoppingSequence [(i + macChannelOffset + BSN) % sequence length]

where macChannelHoppingSeuqnce[j] represents the (j)th channel number in macChannelHoppingSequence PIB, macChannelOffset is the channel offset value and phyChannelsSupportedLength is the number of channels in the PHY used, e.g. phyMaxSUNChannelsSupported elements for SUN-PHY or the number of channels available in phyChannelsSupported for IEEE802.15.4-2006.)

# CID 568, 1064, 1207

1 CID # 568 : Replace "Table 72A" with "Table 78.rr" in page 129

2. CID # 1064 : Replace the element of “SuperframeStartBank” in Table 78.qq in page 53 with the following table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SDIndex | Integer | 2(BO-SO) | Specifies the allocating SD indexing number for beacon transmitting frame. This parameter is ignored if either the BeaconOrder parameter has a value of 15 |

3. CID # 1207 : Replace Figure 65.bb in page111 with the following figure

# CID 568, 1064, 1207, 1366, 409, 410, 412, 418, 426, 428, 429, 430, 431, 432, 433, 434, 627, 672, 677, 678, 1224, 1613, 1614, 1615, 1616, 1617

1 CID # 568 : Replace "Table 72A" with "Table 78.rr" in page 129

2. CID # 1064 : Replace the element of “SuperframeStartBank” in Table 78.qq in page 53 with the following table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SDIndex | Integer | 2(BO-SO) | Specifies the allocating SD indexing number for beacon transmitting frame. This parameter is ignored if either the BeaconOrder parameter has a value of 15 |

3. CID # 1207 : Replace Figure 65.bb in page111 with the following figure

4. CID # 1366 :

4.1 Replace Figure 53.r in page86 with the following figure

| bits: 0 | 1-4 | 5-7 | 8-55 |
| --- | --- | --- | --- |
| Deferred Beacon Flag | Deferred Beacon Time | Reserved | Beacon Timestamp |

4.2 Replace text from line 6 through line 7, page 87 with following text

The Beacon Timestamp subfield is 6 octets in length and shall specify the time of beacon transmission in unit of 1us for time synchronization.

5. CID # 409, 410, 412, 418, 426, 428, 429, 430, 431, 432, 433, 434, 627, 672, 677, 678, 1224, 1613, 1614, 1615, 1616, 1617 :

Delete the element of “MLME-DSME-START”, “MCPS-DSME-DATA” “MLME-DSME-BEACON-NOTIFY” “MCPS-DSME-SCAN” in Table 46.c in page 13

# CID 975, 976, 1085 and 1471

Delete the row '100 Robust DSME Allocation' from Table 84.b page 109 subclause 7.3.12.4.3

Delete subclause 7.5.10.11 Robust DSME allocation

Insert the following paragraph between line 41 and 42 on page 163 subclause 7.5.10.5:

The Destination device can also allocate a DSME-GTS based on the knowledge of current channel condition, i.e. when channel condition is bad, different slots of a DSME-GTS may use different channels. If the Destination device decides to allocate the DSME-GTS slots, it shall configure the ABT Specification subfield of DSME allocation based on the Destination device’s knowledge of channel quality. If DSME-GTS allocation with different slots in different channels approved at the Source device successfully, the Source device and the Destination device shall exchange Data frames according to GTS slots and channels specified in ABT.

Change the paragraph at line 23 to line 30 on page 158 in subclause 7.5.10.1.3 with the following paragraph:

In Channel Adaptation mode (i.e. ChannelDiversityMode is set to ‘0’), the Destination device may allocate DSME-GTS slots in single channel to a Source device. The Destination device may also allocate DSME-GTS slots in different channels to a Source device based on the knowledge of current channel quality. If DSME-GTS allocation with different slots in different channels approved at the Source device successfully, the Source device and the Destination device shall exchange Data frames according to GTS slots and channels specified in ABT.