- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [2nd August, 2010] |  |
| Source | [Il Soon Jang , Sang-Kyu Lim, Dae Ho Kim, You Jin Kim, Tae-Gyu Kang] [ETRI] [Daejeon, Korea] | Voice: [ +82-42-860-5424] Fax: [+82-42-860-5218] E-mail: [isjang@etri.re.kr] |
| Re: | [Response to LB #50 comments] |  |
| Abstract | [The text for MLME Primitives on color frame is described as per the results of comment resolution] |  |
| Purpose | [To reflect the results of the comment resolution] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Blue: new text

Black: deleted text

Pink: Editorial instructions

Orange: modified text on AM 2 meeting, 16th July, 2010.

Red: modified text on AM 1 meeting, 2nd August, 2010 (based on DCN 10/630/r1)

This document is based on DCN 10/348/r0 which describes the comment resolution on CIDs 177, 247, 635, 645, 660, 798, 799, 800, 792, 793, 794, 796, 797, 785, 786, 787, 788, 790, and 791.

6.2.1.1 has been modified as per DCN 10/348/r0. This subclause connects with the resolution of the CID 247.

6.2.1.1 PD-DATA.request

The PD-DATA.request primitive requests the transfer of an MPDU (i.e. PSDU) from the MAC sublayer to the local PHY entity.

The semantics of the PD-DATA.request primitive is are as follows:

PD-DATA.request	(

psduLength,

psdu,

bandplanID

)

Table 7 specifies the parameters for the PD-DATA.request primitive.

| Table 7 - PD-DATA.request parameters |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Valid range | Description |
| psduLength | Unsigned Integer | <=aMaxPHYPacketSize | The number of octets in the PSDU to be transmitted by the PHY entity. |
| psdu | Set of octets |  | The set of octets forming the PSDU to be transmitted by the PHY entity. |
| bandplanID | integer | bandplanID | The color band of psdu |

The new subclause of 7.1.17 has been added as per DCN 10/348/r0. This subclause connects with the resolution of the CID 635.

7.1.17 Primitives for requesting color frame settings from DME

Figure <xx> shows the message sequence chart for color packet frame function. The Originator higher layer uses the MLME-COLOR primitives for color packet frame function.

Figure <xx> Message sequence chart for color packet  frame function

7.1.17.1 MLME-COLOR.request

The MLME-COLOR.request primitive requests that the MLME performs the color packet frame function as defined in 7.6.10.

7.1.17.1.1 Semantics of the service primitive

The semantics of the MLME-COLOR.request primitive are as follows;

MLME-COLOR.request	(

macCFType macCFConfig,

macCFColor,

macCFEnable,

macCFMsg

)

Table <xx1>, Table <xx2>, and Table <xx3> specify the parameters for the MLME-COLOR.request primitive, the parameters of color packet frame configuration message format for MAC procedures indication, and  the parameters of color packet frame configuration message format for channel quality indication ACK frame, respectively.

Table <xx1> MLME-COLOR.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| macCFType macCFConfig | Integer | 0-2 | 0 : Configuration for MAC procedures indication 1: Configuration for channel quality indication 2: Configuration for ‘Originator next higher layer’ upper layer |
| macCFColor | Integer | bandplanID | bandplanID |
| macCFEnable | BOOL | TRUE, FALSE | Color frame enable/disable |
| macCFMsg | CF MSG Format |  | When macCFConfig macCFType is 2, this field is not used. |

Rick Wrote: I don’t understand the primitives in Table xx2.  How are these used?  For example, the description says the “Color frame is transmitted using macDuringASOCColor bandplan ID between MLME-ASSOCIATE.request and MLME-ASSOCIATE.confirm”.  To be honest, I just don’t understand what is intended here.  Where did macDuringASSOCColor come from?  This parameter (I’m assuming it is a parameter) belongs to what primitive?  Or is this a primitive?  I’m at a loss what to do because I don’t understand!

Table <xx2> Color frame packet configuration message format for MAC procedures indication

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| macDuringASSOCColor | Integer | bandplanID | Color frame is transmitted using macDuringASSOCColor bandplanID between MLME‐ASSOCIATE.request and MLME‐ASSOCIATE.confirm. |
| macDuringDISASSOCColor | Integer | bandplanID | Color frame is transmitted using macDuringDISASSOCColor bandplanID between MLME‐DISASSOCIATE.request and MLME‐DISASSOCIATE.confirm,. |
| macDuringSYNCColor | Integer | bandplanID | Color frame is transmitted using macDuringSYNCColor bandplanID between MLME‐SYNC.request and MLME‐SYNC.confirm. |
| macDuringSCANColor | Integer | bandplanID | Color frame is transmitted using macDuringSCANColor bandplanID between MLME‐SCAN.request and MLME‐SCAN.confirm. |

Rick wrote: Same problem as before.  Is this a primitive or a parameter?  If it is a parameter then what primitive does it belong to?  We need an explanation of what is intended and then perhaps we can fix this up!

Table <xx3> Color frame packet configuration message format for channel quality ACK Frame

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| macACKFrameColor | Integer | bandplanID | Use macACKFrameColor for ACK Frame which denotes ACK |
| macNACKFrameColor | Integer | bandplanID | Use macNACKFrameColor for ACK Frame which denotes NACK |

Table <xx3> Color frame configuration message format for channel quality indication

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| macACKColor | Integer | bandplanID | Use macACKColor for Color Frame which denotes ACK |
| macNACKColor | Integer | bandplanID | Use macNACKColor for Color Frame which denotes NACK |

7.1.17.1.2 Appropriate Usage

The MLME-COLOR.request primitive is generated by the MLME and issued to its MLME whenever the color packet usage algorithm requires the required color packet value.

The MLME-COLOR.request primitive is received by the Originator MLME whenever color frame function is desired.

7.1.17.1.3 Effect on receipt

If the transmitter is enabled on the receipt of the MLME-COLOR.request primitive, MLME will cause the MAC to perform the required color packet function. MLME will issue MLME-COLOR.confirm primitive with the status of SUCCESS after a color frame is transmitted.

If color frame transmission is supported upon the receipt of the MLME-COLOR. request primitive, a color frame shall be transmitted and the Originator MLME MAC will issue a MLME-COLOR.confirm primitive and  the status of SUCCESS, UNSUPPORTED_ATTRIBUTE or INVALID_PARAMETER will be returned.

7.1.17.2 MLME-COLOR.confirm

MLME-COLOR.confirm primitive reports the results of a color frame request.

7.1.17.2.1 Semantics of the service primitive

The semantics of the MLME-COLOR.confirm primitive are as follows;

MLME-COLOR.confirm 		(

status

)

Table <xx> specifies the parameters for the MLME-COLOR.confirm primitive.

Table <xx> MLME-COLOR.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, UNSUPPORTED_ATTRIBUTE, or INVALID_PARAMETER | The result of the request to perform a MLME-COLOR.request. |

7.1.17.2.2 When generated

The MLME-COLOR.confirm primitive is generated by the MLME and issued to its MLME in response to a MLME-COLOR.request primitive. The MLME-COLOR.confirm primitive will return a status of SUCCESS, indicating a successful color packet transmission. If the PIBAttribute parameter specifies an attribute that is not found in the database, as shown in Table <xx>, the MLME will issue the MLME-COLOR.confirm primitive with a status of UNSUPPORTED_ATTRIBUTE. If the PIBAttibuteValue parameter specifies a value that is out of the valid range for the given attribute, the MLME will issue the MLME-SET.confirm primitive with a status of INVALID_PARAMETER.

The MLME-COLOR.confirm primitive is generated by the Originator MLME and issued by Originator next higher layer in response to a MLME-COLOR.request primitive. The MLME-COLOR.confirm primitive will return a status of SUCCESS, indicating a successful color frame transmission. If the parameters are not found in the valid range, the Originator MLME will issue the MLME-COLOR.confirm primitive with a status of INVALID_PARAMETER.

The MLME-COLOR.confirm primitive is issued to its MLME in response to a MLME-COLOR.request primitive. The MLME-COLOR.confirm primitive shall return a status of SUCCESS either indicating a successful color frame transmission, or a status of UNSUPPORTED_ATTRIBUTE if color frame is not supported, or a status of INVALID_PARAMETER.

7.1.17.2.3 Appropriate Usage

On receipt of the MLME-COLOR.confirm primitive, the Originator next higher layer is notified of the results of the color configuration for color frame transmission from Originator MLME. originator of MAC

The new subclause of 5.6.4.6 has been added as per DCN 10/348/r0. This subclause connects with the resolution of the CID 177.

5.6.4.6 Color frame

Figure <XX> shows the structure of the color frame, which originates from the MAC sublayer.  MAC payload contains the color pattern or data. color frame payload.  MAC payload is prefixed with an MHR and suffixed with an MFR. The MHR contains the MAC Frame Control field and the MFR contains a 16-bit FCS. The MHR, MFR and MAC payload together form the color frame, (i.e., MPDU). A color pattern consists of random value. Data field format and values are dependent on application which wants to send some data to a peer device.

The MPDU is then passed to the PHY as the PSDU, which becomes the PHY payload. The PHY payload is prefixed with an SHR, containing the Preamble Sequence field, and a PHR containing the length of the PHY payload in octets. The preamble sequence enables the receiver to achieve symbol synchronization. The SHR, PHR, and PHY payload together form the PHY packet, (i.e., PPDU).

The color frame is used for the color frame packet function.

Basically the default purpose of the color frame usage is to provide some intuitive information to user. The color frame intuitively provides information, such as the current of MAC procedures indication, channel quality indication, link adaptation or application-dependent information, to the user via the various colors of the color frame. In addition to the intuitive information for user, it is possible to send some information through the color of the color frame or some data through the payload of color frame. But a user may not use these supplementary functions. A user can know the intuitive information by seeing the color of the color frame. A peer device which receives the color frame as well as a user can obtain another information through the color of the color frame.

Rick’s wrote

The color frame intuitively provides information to the user on device status and channel quality via the various illumination colors. In addition, a VLC device can use the color frame to transfer status information messages to a peer MAC layer.

Figure <xx> Schematic view of the color frame

The subclause 5.6.3 has been modified as per DCN 10/348/r0. This subclause connects with the resolution of the CID 177.

5.6.3 	Frame structure

The frame structures have been designed to keep the complexity to a minimum while at the same time making them sufficiently robust for transmission on a noisy channel. Each successive protocol layer adds to the structure with layer-specific headers and footers. This standard defines five six frame structures, which are introduced in 5.6.3.1 through 5.6.3.5 5.6.3.6:

- A beacon frame, used by a coordinator to transmit beacons

- A data frame, used for all transfers of data

- An acknowledgment frame, used for confirming successful frame reception

- A MAC command frame, used for handling all MAC peer entity control transfers

- A visibility/dimming frame, available to maintain the proper light intensity between data packets

- A color frame, used for providing information such as device status and channel quality to users intuitively

The subclause 7.2.1.1.1 has been modified as per DCN 10/348/r0. This subclause connects with the resolution of the CID 645.

7.2.1.1.1 Frame type subfield

The Frame Type subfield is 3 bits in length and shall be set to one of the nonreserved values listed in Table 67.

| Table 67-Values of the Frame Type subfield |  |
| --- | --- |
| Frame type value b2 b1 b0 | Description |
| 000 | Beacon |
| 001 | Data |
| 010 | Acknowledgment |
| 011 | MAC command |
| 100 | Visibility |
| 101 | Color |
| 101110–111 | Reserved |

The subclause 7.2.2.4 has been added as per DCN 10/348/r0. This subclause connects with the resolution of the CID 660.

7.2.2.4 Color frame format

The MAC color frame shall be formatted as illustrated in Figure <XX1>. The MHR for a color packet frame shall contain the Frame Control field and the MFR for a color frame shall contain the frame check sequence.

Figure <XX1> - Color frame format

In the Frame Control field, the Frame Type subfield shall contain the value that indicates a color frame as shown in Figure <XX2>.

The Color Packet Indicator field(CPI) shall contain whether this frame include color frame payload.

The Color Frame Payload Indicator (CFPI) field shall indicate whether this frame includes data or not.

Rick Wrote: in regards to the CPI, how does this indicate if a color frame payload is included?  It is a 3 bit field so how are the 3 bits set to indicate what?  Also, I don’t understand what is meant by “color frame payload”?  Isn’t there always a payload; otherwise, what is the point of sending the color frame?  But perhaps I’m not understanding how this works.  Can you explain a bit more and then perhaps we can fix the text.

The Peer Device Information Indicator (PDII PDDI) field shall indicate contain whether this frame includes “another information for a peer device” or not so that a peer device receiving the color frame can interpret the color of the color frame, in addition to the instinctive information for a user. the information for transferring to the peer device such as channel quality.

Rick wrote: so how is this done?  What is meant by “such as channel quality”?  What else can be send and how do we encode the information.  The PDII is a 4 bit field so we have value from 0 to 15.  How are these values mapped to channel quality and what else can these bits indicate?

Figure <XX2> Frame Control Field

The subclause 7.3.18 has been added as per DCN 10/348/r0. This subclause connects with the resolution of the CID 660.

7.3.18 Color frame command

Color frame command is used to enable or disable the color frame function of a peer device. Color frame command  format is shown in Figure <XX2>.

| Octets: 7 | 1 | N 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (see Table 79) | Color frame transmission enable/disable flag |

Figure <XX2> Color frame command format

Rick wrote: what is a color frame transmission?  What is the bit pattern or data content that defines a color frame transmission?  Is this the same as the visibility frame?  In addition, the text in 7.3.18 says the “Color frame command is used to enable or disable the color frame of the peer device”.  What does this mean and how does the frame do this (the frame shown in Figure XX2)?

The clause 7.3 has been modified as per DCN 10/348/r0. This clause connects with the resolution of the CID 660.

7.3 MAC command frames

The command frames defined by the MAC sublayer are listed in Table 79. A coordinator shall be capable of transmitting and receiving all command frame types, with the exception of the GTS request command, while the requirements for a device are indicated by an “X” in the table. MAC commands shall only be transmitted in the CAP for beacon-enabled WPANs or at any time for nonbeacon-enabled WPANs.

How the MLME shall construct the individual commands for transmission is detailed in 7.3.1 through 7.3.13. MAC command reception shall abide by the procedure described in 7.6.6.2.

| - MAC command frames |  |  |  |  |
| --- | --- | --- | --- | --- |
| Command frame identifier | Command name | Device |  | Subclause |
|  |  | Tx | Rx |  |
| 0x01 | Association request | X |  | Error: Reference source not found |
| 0x02 | Association response |  | X | Error: Reference source not found |
| 0x03 | Disassociation notification | X | X | Error: Reference source not found |
| 0x04 | Data request | X |  | Error: Reference source not found |
| 0x05 | WPAN ID conflict notification | X |  | Error: Reference source not found |
| 0x06 | Beacon request |  |  | Error: Reference source not found |
| 0x07 | Coordinator realignment |  | X | Error: Reference source not found |
| 0x08 | GTS request |  |  | Error: Reference source not found |
| 0x09 | Blinking notification |  |  | Error: Reference source not found |
| 0x0a | Dimming notification |  |  | Error: Reference source not found |
| 0x0b | Fast link recovery |  |  | Error: Reference source not found |
| 0x0c | Mobility notification |  |  | Error: Reference source not found |
| 0x0d | Information element exchange |  |  | Error: Reference source not found |
| 0x0e | GTS Response |  |  | Error: Reference source not found |
| 0x0f | Clock rate change notification |  |  | Error: Reference source not found |
| 0x10 | Multiple channel assignment |  |  | Error: Reference source not found |
| 0x11 | Channel hopping |  |  | Error: Reference source not found |
| 0x12 | VLC cell and mobility |  |  | Error: Reference source not found |
| 0x13 | Visible frame transmission |  |  | Error: Reference source not found |
| 0x14 | Color frame transmission |  |  | 7.3.18 |
| 0x140x15–0xff | Reserved |  |  | — |

The subclause 7.6.10 has been modified as per DCN 10/348/r0. This subclause connects with the resolutions of  CIDs 785, 786, 787, 788, 790, 791, 792, 793, 794, 795, 796, 797, 798, 799, and 800.

7.6.10 Supplemental color frame scheme information  The title has been changed as per the resolution of CID 786.

A color frame is a frame that intuitively intutively provides information such as device status and channel quality such as the current of MAC procedures indication, channel quality indication, link adaptation or application-dependent information, to the user via the various colors of the color frame. to the user, via the various colors used in the VLC application. A VLC device can use the color frame to transfer the status information messages to a peer device via the MAC service access point. In addition to the intuitive information for user, it is possible to send some information through the color of the color frame or some data through the payload of color frame. But a user may not use these supplementary functions. A user can know the intuitive information by seeing the color of the color frame. A peer device which receives the color frame as well as a user can obtain another information through the color of the color frame.

When the color frame is used, the device needs to check the current state and then display the corresponding color frame state information to the user. In addition, messages can be transferred to a peer device. Various states of a device and the corresponding colors are described in Table 88(CID 785) and the colors mapped for each state of devices are based on the bandplan ID.

| Table 88-Connection State and Color Band Choice for Indication |  |
| --- | --- |
| State | Color bandplan band ID chosen |
| Connection request | Color “A” |
| Device information exchange | Color “B” |
| Data transfer | Color “C”~Color”E” |
| Disconnection | Color “F” |
| Misalignment | Color “G” |

Figure 118 Step1 – Color frame usage for connection procedure

The basic communication procedures for color frame usage in VLC consist consists of three MAC states: idle state, connection state, and data exchange state. In the basic communication procedures, the first step begins when device 1 transmits the "Connection Request Frame" to device 2. It finishes when device 1 receives the "Connection Response Frame" from device 2.  Device 1 can transmit color 'A' frame to device 2 while device 1 sends "Connection Request Frame" to device 2 and device 1 receives "Connection Response Frame" from device 2. The user can know intuitively the current communication state of device 1 through color 'A'.

Figure 119 Step2 – A color frame usage for exchange information procedure

The second step is device 1 shifts from the connection state to the data exchange state. Device 1 can transmit the color 'B' frame to device 2 during the state transition so that the user can intuitively know the current communication state through the use of color 'B'.

Figure 120 Step 3 – A color frame usage for transfer data procedure

The third step has device 2 receiving data frames from device 1. Device 1 continues in the data exchange state while device 2 receives a data frame from device 1. Device 1 can transmit the color 'C' frame to device 2 during the data exchange state. Likewise, device 1 can receive a data frame from device 2 and device 2 continues in the data exchange state while device 1 receives this data frame. Device 2 can transmit color 'C' or another color frame to device 1 during the data exchange state. In other words, color frame can be applied in both directions. The color frame will be applied only to path on which the application data is not transferred because the other path, the one on which the application data is transferred, is too busy to send the color frame.

Figure 121 Step 4 – A color frame usage for transfer disconnection procedure

In the last step, device 1 shifts from the data exchange state to the idle state. Device 1 can transmit a color 'F' frame to device 2 during the state transition. The user can know the current step of the communication procedure through the color frame during these communication procedures.

Figure 122 Color frame in device state diagram (CID 787)

Figure 122 Color frame in device 1 state diagram

This slide Figure 122(CID 788, 790) shows the device 1 state diagram in previous basic communication MAC state transition(CID 791) procedures . Each color frame is transferred during each state transition.

Figure 123 Color frame in device 2 state diagram

Error: Reference source not found and Error: Reference source not found show the communication state diagrams describing the previous basic communication procedures. Each color frame, according to the bandplan, can be transmitted during each state transition in Error: Reference source not found and Error: Reference source not found. In the case of an asymmetric data transfer, device 2 may not transmit color frames to device 1 during the transmission of the data frame.

Figure 124 Color frame usage in file transfer application(CID 792, 793, 794, 796, 797)

Figure 124 Color frame usage in file transfer application

Figure 125 Color frame usage for informing data transmission quality to user

Figure 124 and Figure 125 show other usages for color frames in VLC applications. Error: Reference source not found describes how the user can know the remaining or transferred file size through the color frame in VLC file transfer applications. Device 1 transfers some files to Device 2. When the remained file size, which is not transferred to Device 2, is more than L bytes, Device 2 transfers the color frame with color ‘A’. When remained file size, which is not transferred to Device 2, is less than L bytes and more than M bytes, it transfers the color frame with color ‘B’. When remained file sizes is less than N bytes, it transfers color frame with color ‘C’ (CID 792, 793, 794, 796, 797). Figure 125 describes how the user can know the quality of the data transmission or the communication quality, such as frame error rate(FER) packet error rate (PER), through the color frame. The FER PER statistics need to be averaged over a long frame in order to choose the color frame. This information can help provide misalignment indication to the user. Different colors can be used to indicate different states of misalignment. For example, green, blue, and red color frames can be used to visualize low, middle, and high FER PER values, respectively.

| - An example of a Color Status Table in Coarse Link Adaptation |  |  |
| --- | --- | --- |
| Color of Color Frame | Data Transmission Quality | Action Item |
| Color “A” | Current FERPER < FERPER #1 | Increase the data rate |
| Color “B” | FERPER #1<= FERPER <= FERPER #2 | Hold the current data rate |
| Color “C” | Current FERPER => FERPER #2 | Decrease the data rate |

Error: Reference source not found describes an example of color frame usage scheme (CID 795) to visualize the link adaptation status between two devices. A device can transmit the link adaptation information to a peer device through the color frame. User can also know the FERPER status based on Error: Reference source not found. The device receiving the color frame can change the data rate based on a color frame table, such as Error: Reference source not found because the device, as well as the user, can also detect the transmitted color frame. In table 89, color ‘A’ means the increase of data rate, color ‘B’ means that keep the current data rate, and color ‘C’ means the decrease of the data rate for peer device.(CID 795).

7.6.10.1 Color packet structure(CID 798, 799, 800)

The color packet header can include the indicators as follows. There can be an indicator indicating that the transmitted packet is just a color packet; that is, there is no data messages. It is also possible to have an indicator indicating whether the color packet include some messages for a peer device or not.

The Color Packet is defined in Color packet

| Color Packet Header | Color Packet Pattern |
| --- | --- |
| - Color packet |  |

.