- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [15th July, 2010] |  |
| Source | [Il Soon Jang , Sang-Kyu Lim, Dae Ho Kim, You Jin Kim, Tae-Gyu Kang] [ETRI] [Daejeon, Korea] | Voice: [ +82-42-860-5424] Fax: [+82-42-860-5218] E-mail: [isjang@etri.re.kr] |
| Re: | [Response to LB #50 comments] |  |
| Abstract | [The text of 7.1.16 is modified as per the results of comment resolution] |  |
| Purpose | [To reflect the results of the comment resolution] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Blue: new text

Black: deleted text

Pink: Editorial instructions

Orange: modified text on AM 1 meeting, 16th July, 2010.

This document is based on DCN 10/491/r0 which describes the comment resolution on CIDs 625, 634, 632, 631, 626, 627, 632a

- Primitives for requesting dimmer settings from the DME

MLME-SAP dimmer primitives defines how to request dimmer data from a DME.

All devices shall provide an interface for these dimmer primitives.

Figure 60 Message sequence chart for performing the dimming function (CID 632)

- MLME-DIMMER.request

The MLME-DIMMER.request primitive requests that the MLME perform the dimmer function as defined in Error: Reference source not found.

The semantics of the PLME-DIMMER.request primitive are as follows:

MLME-DIMMER.request	(

PIBAttribute

PIBDimAttributeValue

)

MLME-DIMMER.request parameters specifies the parameters for the MLME-DIMMER.request primitive.

| - MLME-DIMMER.request parameters |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Valid range | Description |
| PIBAttribute | Enumeration | Table 84 Table 85(CID 625) | The identifier of the PIB attribute to set. |
| PIBAttributeValue | Various | Attribute specific | The value of the indicated PIB attribute to set. |

- When generated

The MLME-DIMMER.request primitive is generated by the MLME and issued to its MLME whenever the dimmer algorithm requires the required dimming value.

- Effect on receipt Appropriate Usage(CID 634)

If the transmitter is enabled on receipt of the MLME-DIMMER.request primitive, the MLME will cause the MAC to perform the required dimming function. When the MAC has completed the required dimming, the MLME will issue the MLME-DIMMER.confirm primitive with the status of SUCCESS.

- MLME-DIMMER.confirm

The MLME-DIMMER.confirm primitive reports the results of a dimming request.

The semantics of the MLME-DIMMER.confirm primitive are as follows:

MLME-DIMMER.confirm	(

status

)

MLME-DIMMER.confirm parameters specifies the parameters for the MLME-DIMMER.confirm primitive.

| - MLME-DIMMER.confirm parameters |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Valid range | Description |
| status | Enumeration | SUCCESS, UNSUPPORTED_ATTRIBUTE, INVALID_PARAMETER | The result of the request to perform a CCA. |

- When generated

The MLME-DIMMER.confirm primitive is generated by the MLME and issued to its MLME in response to a MLME-DIMMER.request primitive. The MLME-DIMMER.confirm primitive will return a status of SUCCESS, indicating a successful dimming. The reasons for these status values are fully described in Effect on receipt.

- Effect on receipt Appropriate Usage(CID 634)

On receipt of the MLME-DIMMER.confirm primitive, the MLME is notified of the results of the dimming. If the DIMMER attempt was successful, the status parameter is set to SUCCESS. Otherwise, the status parameter will indicate the error.