IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Yet Another Frame Proposal |  |
| Date Submitted | 12 July 2010 |  |
| Source | Jonathan Simon Dust Networks 30695 Huntwood Ave Hayward, CA 94544 | Voice: 510-400-2936 Fax: E-mail: jsimon@dustnetworks.com |
| Re: | [Overhead reduction] |  |
| Abstract | [Describes a frame type that could be used generically to represent a wide variety of frame options] |  |
| Purpose | [To propose an approach to be included in the next draft] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Proposal:

The desired set of options for fields across all optional modes is:

| Octets: 1/2 | 0/1 | 0/2 | 0/1/2/8 | 0/2 | 0/1/2/8 | 0/5/6/10/14 | variable | variable | 0/2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dest PAN | Dest Addr | Src pan | Src Addr | Aux Sec | IE’s | Payload | FCS |

We define a new frame type using one of the currently reserved types (e.g. 101). The frame control is self-describing, and contains 1 or 2 octets.  The intent is to replace the multipurpose frame in 15-10-0532-01.

| Bit: 0-2 | 3 | 4-5 | 6-7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Type | long FC | Src mode | Dst mode | PAN ID | Security enabled | Aux sec header | Frame pending | Ack requested | IEs | Seq # | Version |
| 101 | 0 = no 1 = yes | 00 = none 01 = 1octet 10 = 2octet 11 = 8octet | 00 = none 01 = 1octet 10 = 2octet 11 = 8octet | 0 = none 1 = 2 in MHR | 0 = off 1 = on and no FCS | 0 = none, and frame counter must be reconstructed if security is on 1- in MHR | 0 = no 1 = yes | 0 = no 1 = yes | 0 = none 1 = present | 0 = absent 1 = present in MHR | 0 (version is 0 if 1 octet FC) |

The base frame control field is 1 octet.  It allows for configuring whether a sequence number is included, and allows for configuring 0/1/2/8 octet addresses.  Frames that require security, want to specify ack request or frame pending, want to carry a PAN ID, or specify IE’s need to use the 2 octet frame control.  Fields occur in the MHR in the order of their selector bits when present.