IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Yet Another Frame Proposal |  |
| Date Submitted | 12 July 2010 |  |
| Source | Jonathan Simon Dust Networks 30695 Huntwood Ave Hayward, CA 94544 | Voice: 510-400-2936 Fax: E-mail: jsimon@dustnetworks.com |
| Re: | [Overhead reduction] |  |
| Abstract | [Describes a frame type that could be used generically to represent a wide variety of frame options] |  |
| Purpose | [To propose an approach to be included in the next draft] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Proposal:

The desired set of options for fields across all optional modes is:

| Octets: 1/2 | 0/1 | 0/2 | 0/1/2/8 | 0/2 | 0/1/2/8 | 0/5/6/10/14 | variable | variable | 0/2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dest PAN | Dest Addr | Src pan | Src Addr | Aux Sec | IE’s | Payload | FCS |

We define a new frame type using one of the currently reserved types (e.g. 101). The frame control is self-describing, and contains 1 or 2 octets.  The intent is to replace the multipurpose frame in 15-10-0532-01.  It generally trades a slightly longer frame while only using one frame type.

| Bit: 0-2 | 3 | 4-5 | 6-7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Type | long FC | Src mode | Dst mode | PAN ID | Security enabled | Aux sec header | Frame pending | Ack requested | IEs | Seq # | Version |
| 101 | 0 = no 1 = yes | 00 = none 01 = 1octet 10 = 2octet 11 = 8octet | 00 = none 01 = 1octet 10 = 2octet 11 = 8octet | 0 = none 1 = 2 in MHR | 0 = off 1 = on and no FCS | 0 = none, and frame counter must be reconstructed if security is on 1- in MHR | 0 = no 1 = yes | 0 = no 1 = yes | 0 = none 1 = present | 0 = absent 1 = present in MHR | 0 (version is 0 if 1 octet FC) |

The base frame control field is 1 octet.  It allows for configuring whether a sequence number is included, and allows for configuring 0/1/2/8 octet addresses.  Frames that require security, want to specify ack request or frame pending, want to carry a PAN ID, or specify IE’s need to use the 2 octet frame control. IE’s are supported without versioning a legacy frame. Short frame control headers must contain a sequence number. Fields occur in the MHR in the order of their selector bits when present.

General Low Latency Frame (4 subframe types supported, optional security and ack):

In 532:

| Octets: 1 | 0/1 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- |
| Frame Control | Seq # | Aux Sec | payload | FCS |

In 603, without security or ack (unless ack is always on by definition) – 1 octet longer than 532.

| Octets: 1 | 1 | variable | 2 |
| --- | --- | --- | --- |
| Frame Control | Seq # | payload | FCS |

In 603, with security, ack, and optional IE describing subframe type –1 to 3 octets longer than 532. Seq # becomes optional:

| Octets: 2 | 0/1 | 0/5/6/10/14 | 3 octets | variable | 0 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Aux Sec | IE (2 bits for subtype, 6 bits leftover) | payload | FCS |

Multipurpose Frame Blink without source or PAN in 532:

In 532:

| Octets: 1 | 1 | 2 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dst address | Aux Sec | payload | FCS |

In 603 with security & short address – 1 octet longer if using seq#, same if not. No security case same as 532, since we can use 1 octet FC. Option of long dst address.

| Octets: 1/2 | 0/1 | 2/8 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dst address | Aux Sec | payload | FCS |

Multipurpose Frame Blink with source:

In 532:

| Octets: 1 | 1 | 2/8 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Src address | Aux Sec | payload | FCS |

In 603 with security – 1 octet longer if seq #, same if not.  No security case same as 532, since we can use 1 octet FC.

| Octets: 1/2 | 0/1 | 2/8 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Src address | Aux Sec | payload | FCS |

Multipurpose Frame Blink with PAN

In 532:

| Octets: 1 | 1 | 2 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dst PAN | Aux Sec | payload | FCS |

In 603 with security – 1 octet longer if seq #, same if not.  No security same as 532, since we can use 1 octet FC.

| Octets: 1/2 | 0/1 | 2 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dst PAN | Aux Sec | payload | FCS |

Multipurpose Frame Blink with source and PAN

In 532:

| Octets: 1 | 1 | 2 | 2/8 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dst PAN | Src address | Aux Sec | payload | FCS |

In 603 with security – 1 octet longer if seq #, same if not.  No security same as 532, since we can use 1 octet FC.

| Octets: 1/2 | 0/1 | 2 | 2/8 | 0/5/6/10/14 | variable | 0/2 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dst PAN | Src address | Aux Sec | payload | FCS |

Multipurpose Frame CSL wakeup

In 532 or 603:

| Octets: 1 | 1 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Seq # | Dst PAN | Dst address | payload | FCS |

Multipurpose Frame TSCH DATA1/ DATA2

In 532 (2 separate subframe types):

| Octets: 1 | 0/2 | variable |
| --- | --- | --- |
| Frame Control | Src address | payload |

In 603 – 1 octet longer (same generic frame type), possibility of long src address:

| Octets: 2 | 0/2/8 | variable |
| --- | --- | --- |
| Frame Control | Src address | payload |