- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [4th August, 2010] |  |
| Source | [Il Soon Jang , Sang-Kyu Lim, Dae Ho Kim, You Jin Kim, Tae-Gyu Kang, *Sridhar Rajagopal] [ETRI, *Samsung] [Daejeon, Korea] | Voice: [ +82-42-860-5424] Fax: [+82-42-860-5218] E-mail: [isjang@etri.re.kr] |
| Re: | [Response to LB #50 comments] |  |
| Abstract | [This document describes the final text for color function support] |  |
| Purpose | [To complete the text for color function support] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

This document is based on DCN 10/348/r0 which describes the comment resolution on CIDs 177, 247, 635, 645, 660, 798, 799, 800, 792, 793, 794, 796, 797, 785, 786, 787, 788, 790, and 791.

Agreements on color frame

- Merge visibility frame and color frame definition in Clause 3 and rename to CVD frame.

- Add CVD to list of acronyms.

- Global search and replace of visibility frame to CVD frame  Sridhar

- Merge 4.5.4.5 and 4.5.4.6 (information about visibility frame and color frame) in latest version that the editor has.

- PPDUlinkQuality should be referenced for changing the color status indication

3. Definitions

CVD frame: A  frame  used for color, visibility and dimming support. The CVD frame visually provides information such as communication status and channel quality to the user, via the various colors.  The CVD frame is also sent during idle or receive modes of operation for continuous visibility and dimming support. During the CVD frame transmission, the device is still emitting light while not communicating, and it is thus able to fulfill its lighting function. The payload of the frame consists of visibility patterns of appropriate intensity and color.

4. Acronyms and abbreviations

CVD  Frame 	color-visibility dimming frame

5.1 Introduction

a)Star, peer-to-peer or broadcast operation

b)Allocated 16-bit short or 64-bit extended addresses

c)Scheduled or random access with collision avoidance transmission

d)Fully acknowledged protocol for transfer reliability

e)Color quality indication (CQI)

f)Dimming support

g)Visibility support

h)Color function support

5.5.4 Frame structure

The frame structures have been designed to keep the complexity to a minimum while at the same time making them sufficiently robust for transmission on a noisy channel. Each successive protocol layer adds to the structure with layer-specific headers and footers. This standard defines five frame structures, which are defined in 5.5.4.1 through 5.5.4.5

a)A beacon frame, used by a coordinator to transmit beacons

b)A data frame, used for all transfers of data

c)An acknowledgment frame, used for confirming successful frame reception

d)A MAC command frame, used for handling all MAC peer entity control transfers

e)A CVD frame, used to maintain the proper light intensity between data packets, support dimming and for visually providing information such as communication status and channel quality to the user

5.5.4.5  CVD  frame

Add in this subclause:

A user can perceive intuitively information such as the current step of communication procedure, the data transmission quality, the transferred file size or remaining file size. The CVD frame can also be used to inform link status such as misalignment between the two devices, light direction or sending data status.

7.1.1.1 MCPS-DATA.request

The MCPS-DATA.request primitive requests the transfer of a data SPDU (i.e. MSDU) from a local SSCS entity to a single peer SSCS entity.

The semantics of the MCPS-DATA.request primitive are as follows:

MCPS-DATA.request(

SrcAddrMode,

DstAddrMode,

DstWPANId,

DstAddr,

msduLength,

msdu,

msduHandle,

TxOptions,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndexDataRate

ColorReceived,

ColorNotReceived

)

Table 30 specifies the parameters for the MCPS-DATA.request primitive.

Add below table to table 30

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ColorReceived | BOOLEAN | TRUE/FALSE | ColorReceived shall be set as TRUE, if CVD frame is sent when data frame is successfully received. |
| ColorNotReceived | BOOLEAN | TRUE/FALSE | ColorNotReceived shall be set as TRUE, if CVD frame is sent when data frame is not received. |

7.1.3.1 MLME-ASSOCIATE.request

The MLME-ASSOCIATE.request primitive allows a device to request an association with a coordinator.

The semantics of the MLME-ASSOCIATE.request primitive are as follows:

MLME-ASSOCIATE.request(

LogicalChannel,

CoordAddrMode,

CoordWPANId,

CoordAddress,

CapabilityInformation,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

ColorAssoc)

Table 37 specifies the parameters for the MLME-ASSOCIATE.request primitive.

Table 37—MLME-ASSOCIATE.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ColorAssoc | BOOLEAN | TRUE/FALSE | ColorAssoc shall be set as TRUE, if CVD frame is sent during association. |

7.1.4.1 MLME-DISASSOCIATE.request

The MLME-DISASSOCIATE.request primitive is used by an associated device to notify the coordinator of its intent to leave the VLC WPAN. It is also used by the coordinator to instruct an associated device to leave the VLC WPAN.

The semantics of the MLME-DISASSOCIATE.request primitive are as follows:

MLME-DISASSOCIATE.request(

DeviceAddrMode,

DeviceWPANId,

DeviceAddress,

DisassociateReason,

TxIndirect,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

ColorDisAssoc)

Table 41 specifies the parameters for the MLME-DISASSOCIATE.request primitive.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ColorDisAssoc | BOOLEAN | TRUE/FALSE | ColorDisAssoc shall be set as TRUE, if CVD frame is sent during disassociation. |

7.1.10.1 MLME-SCAN.request

The MLME-SCAN.request primitive is used to initiate a channel scan over a given list of channels. A device can use a channel scan to measure the energy on the channel, search for the coordinator with which it associated, or search for all coordinators transmitting beacon frames within the POS of the scanning device.

The semantics of the MLME-SCAN.request primitive are as follows:

MLME-SCAN.request(

ScanType,

ScanChannels,

ScanDuration,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex,

ColorScan)

Table 55 specifies the parameters for the MLME-SCAN.request primitive.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ColorScan | BOOLEAN | TRUE/FALSE | ColorScan shall be set as TRUE, if CVD frame is sent during SCAN. |

7.2.1.1.2 Frame type subfield

The Frame Type subfield is 3 bits in length and shall be set to one of the nonreserved values listed in Table 68.

| Table 68-Values of the Frame Type subfield |  |
| --- | --- |
| Frame type value b2 b1 b0 | Description |
| 000 | Beacon |
| 001 | Data |
| 010 | Acknowledgment |
| 011 | MAC command |
| 100 | CVD |
| 101-111 | Reserved |

7.2.3.2.1 Capability Information Field

The Capability Information field is illustrated in Table 75.

The Power Source subfield is 1 bit in length and shall be set to one if the device is receiving power from the alternating current mains. Otherwise, the Power Source subfield shall be set to zero.

The Battery information subfield, shown in Table 76, is 2 bits in length. It is set to reserved (11) if the power source is set to 1.

| Table 75 - Capability Information Field |  |  |
| --- | --- | --- |
|  | Bit | Function |
| MAC layer capabilities | 0 | Power source |
|  | 1-2 | Battery information |
|  | 3 | Security capability |
|  | 4 | Co-ordinator capability |
|  | 5 | Traffic support |
|  | 6-7 | Topology support |
|  | 8-9 | Device type |
|  | 10 | Beacon support |
|  | 11 | Dimming support in MAC |
|  | 12 | Continuous visibility transmission (for infrastructure) |
|  | 13 | CVD support |
|  | 14-15 | Reserved |
| PHY layer capabilities | 16 | PHY I support |
|  | 17 | PHY II support |
|  | 18 | Alternate PHY (CSK) support |
|  | 19 | Dimming support in PHY (VPM) |
|  | 20-22 | Max supported TX clock |
|  | 23-25 | Max supported RX clock |
|  | 26 | Explicit clock notification request |
|  | 27-31 | Reserved |
| Band capabilities | 8*n | Aggregate channels |
|  | 8*n | Guard channels |
|  | 32-39 | Bands used for CSK (any 3 bits of the bits set to 1 can be used) |
| Physical device capabilities | 40-42 | Number of optical source types |
|  | 43-45 | Multiple direction support |
|  | 46-55 | Number of LEDs per optical source type |

7.4.2 MAC PIB attributes

Add below table to Table 86

| Name | Identifier | Type | Valid range | Description |
| --- | --- | --- | --- | --- |
| macDuringASSOCColor | 0x5f | Unsigned integer | 0~6 | CVD frame is requsted using macDuringASSOCColor bandplanID between MLME‐ASSOCIATE.request and MLME‐ASSOCIATE.confirm.if CVD frame is sent |
| macDuringDISASSOCColor | 0x61 | Unsigned integer | 0~6 | CVD frame is transmitted using macDuringDISASSOCColor bandplanID between MLME‐DISASSOCIATE.request and MLME‐DISASSOCIATE.confirm if CVD frame is sent |
| macDuringSCANColor | 0x62 | Unsigned integer | 0~6 | CVD frame is transmitted using macDuringSCANColor bandplanID between MLME‐SCAN.request and MLME‐SCAN.confirm if CVD frame is sent |
| macColorReceived | 0x63 | Unsigned integer | 0~6 | Use macColorReceived for CVD Frame which denotes ‘received’ |
| macColorNotReceived | 0x64 | Unsigned integer | 0~6 | Use macColorNotReceived for CVD Frame which denotes ‘not received’ |
| macCFAppColor | 0x65 | Unsigned integer | 0~6 | Color of CVD frame for Application-dependent information |

7.6.12 Color function support

update Table 88

| Table 88-Connection State and Color Band Choice for Indication |  |  |
| --- | --- | --- |
| State | Color chosen | Bandplan ID |
| Connection request | Color “A” | 0 |
| Device information exchange | Color “B” | 1 |
| Data transfer | Color “C”~Color”E” | 2,3,4 |
| Disconnection | Color “F” | 5 |

Update all figures and text to use CVD frame  in this clause

Update the last clause and Table 89 as follows;

Figure 124 and Figure 125 show other usages for CVD frames in VLC applications. Error: Reference source not found describes how the user can know the remaining or transferred file size through theeCVD frame in VLC file transfer applications. Device 1 transfers some files to Device 2. When the remained file size, which is not transferred to Device 2, is more than L bytes, Device 2 transfers theeCVD frame with color ‘A’. When remained file size, which is not transferred to Device 2, is less than L bytes and more than M bytes, it transfers theeCVD frame with color ‘B’. When remained file sizes is less than N bytes, it transferseCVD frame with color ‘C’. Figure 125 and table 89 describs how the user can know the quality of the data transmission or the communication quality, such as frame error rate(FER), through theeCVD frame. The FER statistics need to be averaged over a long frame in order to choose theeCVD frame. This information can help provide misalignment indication to the user. Different colors can be used to indicate different states of misalignment. For example, green, blue, and redeCVD frames can be used to visualize low, middle, and high FER values, respectively.

| - Color status table in channel quality |  |  |
| --- | --- | --- |
| Color of CVD frame | Data Transmission Quality | Action Item |
| Color “A” | Current FER < FER #1 | low data rate |
| Color “B” | FER #1<= FER <= FER #2 | medium data rate |
| Color “C” | Current FER => FER #2 | high data rate |