IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PSC Study Group Draft PAR |  |
| Date Submitted | December 10, 2010 |  |
| Source | [Soo-Young Chang; Peter Murray] [CSUS; Self-Employed] | E-mail: [sychang@ecs.csus.edu] |
| Re: |  |  |
| Abstract | PAR draft for the PSC Study Group. |  |
| Purpose | To develop the PAR for a standard within the 802.15 WG. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

PAR FORM

Submitter Email: petermurr@mac.com

PAR Status: Unapproved PAR, PAR for a New IEEE Standard

Type of Project: New IEEE Standard

PAR Request Date: 20-Jan-2011

PAR Approval Date:

PAR Expiration Date:

1.1 Project Number: P802.15.8

1.2 Type of Document: Standard

1.3 Life Cycle: Full Use

2.1 Title: Standard for Information Technology - Telecommunications and Information Exchange Between Systems - Local and Metropolitan Area Networks - Specific Requirements - Part 15.8: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for short-range wireless Personal Space Communicatios (PSC).

3.1 Working Group: Wireless Personal Area Network (WPAN) Working Group (C/LM/WG802.15)

Contact Information for Working Group Chair

Name: Robert F Heile

Email Address: bheile@ieee.org

Phone: 781-929-4832

Contact Information for Working Group Vice-Chair

None

3.2 Sponsoring Society and Committee: IEEE Computer Society/LAN/MAN Standards Committee (C/LM)

Contact Information for Sponsor Chair

Name: Paul Nikolich

Email Address: p.nikolich@ieee.org

Phone: 857.205.0050

Contact Information for Standards Representative

None

4.1 Type of Ballot: Individual

4.2 Expected Date of submission of draft to the IEEE-SA for Initial Sponsor Ballot: 07/2013

4.3 Projected Completion Date for Submittal to RevCom: 02/2014

5.1 Approximate number of people expected to be actively involved in the development of this project: 60

5.2 Scope: This standard defines the PHY and MAC specifications optimized for personal space communications, for dynamically scalable data rates from 10 kbps to 54 Mbps, principally operating in a range of 30 meters or less with a capability of extension through relays in the 2.4GHz and 60GHz unlicensed bands. It provides mechanisms to coexist with other IEEE 802 standard technologies. It supports features including dynamic grouping with up to five concurrent n-to-m communications, locating of devices with accuracy of 10cm circular error probable (CEP), less than 20ms latency, fast synchronization with  less than 500usec lock time for fast association of devices, soft handover for mobile devices, dynamic topology configuration, and enhanced security. (More information regarding this project is provided in Section 8.1.)

5.3 Is the completion of this standard dependent upon the completion of another standard: No

5.4 Purpose: This standard provides a wireless communication means to a fixed and mobile personal space involving various heterogeneous devices associated with a single individual to be controlled and managed in a personally tailored fashion. It uses dynmic mixture and scalability of data rates and device association to support simultaneous multiple streaming of various types of user specific information such as low rate control data, voice, and video with a single technology.

5.5 Need for the Project:

To connect diverse heterogeneous and ubiquitous personal devices with no technical commonalities for personally tailored environment and entertainment management (refer to 8.1 Applications and service examples of PSC), it is beneficial to have a unified solution with one technology, not by combining multiple technologies into a combo chip. There are standards that could serve parts of the PSC, but no single standard supports combinations of all features uniquely optimized for this connectivity adaptable to these personal space services and applications: simultaneous use of multiple channels, dynamic scalability of data rates, fast device synchronization and association, relays and handover for range extension, and configurability of topologies. Therefore there is a need for a new standard that is optimized for and serves the PSC. (More information regarding this project is provided in Section 8.1.)

5.6 Stakeholders for the Standard: The stakeholders include:

- Telecom industry

- Mobile device manufacturers

- Game device manufacturers and content providers

- Consumer electronics industry

6.1.a Is the Sponsor aware of any copyright permissions needed for this project?: No

6.1.b Is the Sponsor aware of possible registration activity related to this project?: No

7.1 Are there other standards or projects with a similar scope?: No.

7.2 Joint Development: No.

8.1 Additional Explanatory Notes (Item Number and Explanation):

Definition of Personal Space Communications (PSC): note for 2.1 and 5.2

A personal space is defined as a physical and virtual space which a person regards as psychologically the person’s. For this standard, the personal space is defined as a physical space where environment and deives are controlled by a person. Personal space communication is connectivity between the individual and the devices and among devices associated with this individual without human intervention in the personal space for exchange of information and management of the environment.

Applications and service examples of PSC: note for 5.5

Applications based on the proposed standard for PSC include multiple peer-to-peer communications, group games requiring low latency, conferencing, multi-lingual simultaneous interpretation systems, personal broadcasting, stereo wireless karaoke, wireless tour guide, wireless audio, drive-in shop operations audio, mobile Voice over Internet Protocal (VoIP), internet radio, mobile Internet Protocol TV (IPTV), graphic remote control, wireless Private Branch Exchange (PBX), and convergence of such applications. As an example, if we are standing under an art piece in a museum, we can use our PSC featured devices to listen to the information about the display on any of channels of our preferred languages. At the same time, the PSC devices allow us to talk to the tour guide for specific questions.

All of the above applications can be accomplished with one type of devices. An important part of the standard is the low latency, and wireless speakers and karaoke are good examples of this. The lip synchronization between the video on the TV and the sound from the speakers can be made to be undetectable. Quick association of devices around the user is another important aspect of this standard. Not like today’s lengthy and cumbersome procedure to get connected to a wireless network and authorized by the router or a server, a PSC device is instantly able to see devices in the space with a limited access for identification and association purposes. Once a PSC user sees what devices are around, the user can choose and associate with any of available devices which are ready to be connected and provide services. This feature instantly enables the user to enjoy numerous services such as point of sale contents, audio streaming to an audio system, video streaming to a TV, and an instant group game through group association.