IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title |  |  |
| Date Submitted | [08 September, 2010] |  |
| Source | [Jaeseung Son, Atyua Yokoi] [Samsung Electronics] [Suwon, Korea] | Voice:[82-31-279-7293] Fax: [ ] E-mail:[taehan.bae@samsung.com] |
| Re: | [] |  |
| Abstract | [Update of 5.8.4.1.2] |  |
| Purpose | [5.8.4.1.2 in D2 is updated.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

(Current D2)

5.8.4.1.2 Design rule for 8-CSK

8-CSK symbol points are defined by the design rule in Figure 40. Points I, J and K show the center of the 3 color bands on xy color coordinates. P0 to P7 are 8 symbol points of 8-CSK. P0, P4 and P7 are 3 vertices of the triangle IJK. P1 and P2 are points that divide side JK and side JI in the ratio 1:2. Point c and b are midpoints of the side JK and side JI. Line ba and line cd meet at right angles with line KI. P3 is a point that divides line ba in the ratio 1:2. P5 is a point that divides line cd in the ratio 1:2. P6 is a midpoint of the line KI.

(Corrected)

5.8.4.1.2 Design rule for 8-CSK

8-CSK symbol points are defined by the design rule in Figure 40. Points I, J and K show the center of the 3 color bands on xy color coordinates. P0 to P7 are 8 symbol points of 8-CSK. P0, P4 and P7 are 3 vertices of the triangle IJK. P1 and P2 are points that divide side JK and side JI in the ratio 1:2. Point b and c are midpoints of the line JI and line JK. P6 is a midpoint of the line KI. P3 is a point that divides line AB in the ratio 1:2. P5 is a point that divides line DC in the ratio 1:2.