IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | OFDM Frame Example |  |
| Date Submitted | 9 September, 2010 |  |
| Source | [Emmanuel Monnerie] [Landis+Gyr] | Voice: [ +1 678 641 1986 ] Fax: [ ] E-mail: [ Emmanuel . monnerie @ landisgyr . com ] |
| Re: | Comment Resolution for TG4g Draft D1 |  |
| Abstract | This document presents an example of OFDM Frame, built from the payload all the way to the time domain samples. It is similar to the 802.11-2007 Annex G. |  |
| Purpose | Proposed resolution for Comments ID 1780 through 1783 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Annex X

(informative)

An example of encoding a frame for OFDM PHY

X.1 Introduction

The purpose of this annex is to show an example of encoding a frame for the OFDM PHY, as described in

Clause 6.12b. This example covers all the encoding details defined by this standard.

The encoding illustration goes through the following stages:

- Generating the short training sequence section of the preamble;

- Generating the long preamble sequence section of the preamble;

- Generating the OFDM Header and the corresponding HCS;

- The PSDU from the MAC should already contain an FCS, which is assumed to be four bytes here.  Six tail bits and PAD bits are appended to form the data field.

- Scrambling the data field;

- Reseting the six tail bits back to zeros;

- Encoding the data with a convolutional encoder and puncturing;

- Interleaving the data field;

- Mapping into complex symbols;

- Frequency Spreading;

- Concatenating the preamble, the OFDM header and the data field;

- Pilot, guard and DC tone insertion;

- Transforming from frequency to time domain using the IFFT and adding a circular prefix;

- Concatenating the OFDM symbols into a single, time-domain signal.

In the description of time domain waveforms, a complex baseband signal at 666.666 ksample/s is used. This example uses the 400 kbps data rate (QPSK  ½ rate modulation), which corresponds to OFDM Option 2 and MCS level 4, and a message of 72 octets. The OFDM Header uses the 50 kbps data rate (BPSK 1/2 rate coded with 4x frequency repetition), which corresponds to MCS level 0.

X.2 The Message

The message being encoded is the same one than the one used in 802.11-2007 annex G. It consists of the first 72 characters of the well-known “Ode to Joy” by F. Schiller:

Joy, bright spark of divinity,

Daughter of Elysium,

Fire-insired we tread

Thy sanctuary.

Thy magic power re-unites

All that custom has divided,

All men become brothers

Under the sway of thy gentle wings…

The message is converted to ASCII and a CRC32 is added as defined in clause 7.2.1.9;

Note: the MAC header will not be included. It is assumed to be part of the message in this annex.

The resulting 76 octets PSDU is shown in Table X.1.

Table X.1 – The message

| Octet # | Value (Hex) | Octet # | Value (Hex) | Octet # | Value (Hex) | Octet # | Value (Hex) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4A | 21 | 20 | 41 | 6F | 61 | 69 |
| 2 | 6F | 22 | 64 | 42 | 66 | 62 | 72 |
| 3 | 79 | 23 | 69 | 43 | 20 | 63 | 65 |
| 4 | 2C | 24 | 76 | 44 | 45 | 64 | 64 |
| 5 | 20 | 25 | 69 | 45 | 6C | 65 | 20 |
| 6 | 62 | 26 | 6E | 46 | 79 | 66 | 77 |
| 7 | 72 | 27 | 69 | 47 | 73 | 67 | 65 |
| 8 | 69 | 28 | 74 | 48 | 69 | 68 | 20 |
| 9 | 67 | 29 | 79 | 49 | 75 | 69 | 74 |
| 10 | 68 | 30 | 2C | 50 | 6D | 70 | 72 |
| 11 | 74 | 31 | 0A | 51 | 2C | 71 | 65 |
| 12 | 20 | 32 | 44 | 52 | 0A | 72 | 61 |
| 13 | 73 | 33 | 61 | 53 | 46 | 73 | 42 |
| 14 | 70 | 34 | 75 | 54 | 69 | 74 | E2 |
| 15 | 61 | 35 | 67 | 55 | 72 | 75 | 94 |
| 16 | 72 | 36 | 68 | 56 | 65 | 76 | C0 |
| 17 | 6B | 37 | 74 | 57 | 2D |  |  |
| 18 | 20 | 38 | 65 | 58 | 69 |  |  |
| 19 | 6F | 39 | 72 | 59 | 6E |  |  |
| 20 | 66 | 40 | 20 | 60 | 73 |  |  |

X.3 Generation of the OFDM Header

X.3.1 HCS and PAD bits insertion

In this example the payload data has a size of 76 octets and it will be encoded using QPSK modulation, ½ rate coding (Rate field value = 4). The scrambler index will be the first one (Scrambler ID = 0).

The corresponding OFDM Header, including the HCS is represented in table X.2

Table X.2 – OFDM Header

| Field Name | Bit # | Bit Value |  | Field Name | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- |
| Rate | 1 | 0 |  | Scrambler | 20 | 0 |
|  | 2 | 0 |  |  | 21 | 0 |
|  | 3 | 1 |  | RFU | 22 | 0 |
|  | 4 | 0 |  | HCS | 23 | 1 |
|  | 5 | 0 |  |  | 24 | 1 |
| RFU | 6 | 0 |  |  | 25 | 0 |
| Length | 7 | 0 |  |  | 26 | 1 |
|  | 8 | 0 |  |  | 27 | 0 |
|  | 9 | 0 |  |  | 28 | 0 |
|  | 10 | 0 |  |  | 29 | 0 |
|  | 11 | 1 |  |  | 30 | 1 |
|  | 12 | 0 |  | Tail | 31 | 0 |
|  | 13 | 0 |  |  | 32 | 0 |
|  | 14 | 1 |  |  | 33 | 0 |
|  | 15 | 1 |  |  | 34 | 0 |
|  | 16 | 0 |  |  | 35 | 0 |
|  | 17 | 0 |  |  | 36 | 0 |
| RFU | 18 | 0 |  |  |  |  |
|  | 19 | 0 |  |  |  |  |

In this configuration, no extra PAD bit is necessary. The size of the header will fill up exactly 6 OFDM symbols.

X.3.2 Convolutional Encoding

After convolutional encoding of the OFDM header, the size is now doubled and the corresponding bits are represented in table X.3. No puncturing is applied in this configuration.

Table X.3 – OFDM Header after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 19 | 0 | 37 | 1 | 55 | 0 |
| 2 | 0 | 20 | 0 | 38 | 0 | 56 | 1 |
| 3 | 0 | 21 | 1 | 39 | 0 | 57 | 1 |
| 4 | 0 | 22 | 1 | 40 | 1 | 58 | 0 |
| 5 | 1 | 23 | 0 | 41 | 1 | 59 | 0 |
| 6 | 1 | 24 | 1 | 42 | 1 | 60 | 0 |
| 7 | 0 | 25 | 1 | 43 | 0 | 61 | 1 |
| 8 | 1 | 26 | 1 | 44 | 0 | 62 | 1 |
| 9 | 1 | 27 | 0 | 45 | 1 | 63 | 0 |
| 10 | 1 | 28 | 0 | 46 | 1 | 64 | 0 |
| 11 | 1 | 29 | 1 | 47 | 1 | 65 | 1 |
| 12 | 1 | 30 | 0 | 48 | 0 | 66 | 1 |
| 13 | 0 | 31 | 0 | 49 | 1 | 67 | 0 |
| 14 | 0 | 32 | 0 | 50 | 0 | 68 | 0 |
| 15 | 1 | 33 | 1 | 51 | 1 | 69 | 1 |
| 16 | 0 | 34 | 1 | 52 | 1 | 70 | 0 |
| 17 | 1 | 35 | 1 | 53 | 1 | 71 | 1 |
| 18 | 1 | 36 | 1 | 54 | 0 | 72 | 1 |

X.3.3 Interleaving

In this configuration, no bit interleaving is necessary for the OFDM header. The formula defined in 6.12b.2.4 will lead to an interleaving that does not change the position of the bits.

X.3.4 Bit mapping

The 72 bits are split into 6 OFDM symbols. The bit mapping for the OFDM header in this example is BPSK, therefore Q is always 0. The I value is mapped as defined in figure 65.l.

Table X.4 – Bit mapping for the OFDM Header

|  | Symbol 0 |  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 2 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 3 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 4 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 5 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 6 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 7 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 8 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 9 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 10 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 11 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 12 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |

X.3.5 Frequency Spreading

In this example, a frequency spreading of 4 is applied to the OFDM header. The original 12 bits in each symbol (Bin # 25 through 36 in table X.5) are duplicated within the same symbol. The resulting symbols have 48 data bit each. The duplicated bits have a phase rotation as defined in 6.12b.2.5.

Table X.5 –OFDM Header in the frequency domain

|  | Symbol 0 |  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 2 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 3 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 |
| 4 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 6 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 |
| 7 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 8 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 9 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 10 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 11 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 12 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 13 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 14 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 15 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 16 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 17 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 18 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 19 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 20 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 21 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 22 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 23 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 24 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 25 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 26 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 27 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 28 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 29 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 30 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 31 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 32 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 33 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 34 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 35 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 36 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 37 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 38 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 39 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 40 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 41 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 42 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 43 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 44 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 45 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 46 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 47 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 48 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |

X.4 Generation of the data Symbols

X.4.1 PAD Insertion and Data Scrambling

The original 76 octets of data, as defined in table X.0, are concatenated with 6 zero tail bits and 10 pad bits (as calculated by the formulas in 6.12b.2.9). The 6 tail bits are forced to 0 after scrambling. The resulting 624 bits are represented in table X.6 (First and last 48 bits only).

Table X.6 – First and last 48 bits after pad insertion and scrambling

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 577 | 1 | 601 | 0 |
| 2 | 0 | 26 | 0 | 578 | 0 | 602 | 1 |
| 3 | 1 | 27 | 0 | 579 | 1 | 603 | 1 |
| 4 | 0 | 28 | 0 | 580 | 0 | 604 | 1 |
| 5 | 1 | 29 | 0 | 581 | 1 | 605 | 1 |
| 6 | 1 | 30 | 0 | 582 | 0 | 606 | 1 |
| 7 | 0 | 31 | 1 | 583 | 0 | 607 | 1 |
| 8 | 0 | 32 | 1 | 584 | 0 | 608 | 0 |
| 9 | 0 | 33 | 0 | 585 | 1 | 609 | 0 |
| 10 | 1 | 34 | 0 | 586 | 0 | 610 | 0 |
| 11 | 1 | 35 | 0 | 587 | 1 | 611 | 0 |
| 12 | 1 | 36 | 0 | 588 | 1 | 612 | 0 |
| 13 | 1 | 37 | 0 | 589 | 0 | 613 | 0 |
| 14 | 1 | 38 | 1 | 590 | 0 | 614 | 0 |
| 15 | 0 | 39 | 0 | 591 | 1 | 615 | 0 |
| 16 | 1 | 40 | 1 | 592 | 0 | 616 | 0 |
| 17 | 0 | 41 | 1 | 593 | 1 | 617 | 0 |
| 18 | 1 | 42 | 0 | 594 | 0 | 618 | 0 |
| 19 | 0 | 43 | 1 | 595 | 1 | 619 | 0 |
| 20 | 1 | 44 | 0 | 596 | 1 | 620 | 1 |
| 21 | 0 | 45 | 1 | 597 | 1 | 621 | 1 |
| 22 | 0 | 46 | 1 | 598 | 1 | 622 | 0 |
| 23 | 1 | 47 | 0 | 599 | 1 | 623 | 1 |
| 24 | 0 | 48 | 0 | 600 | 0 | 624 | 1 |

X.4.2 Convolutional Encoding and puncturing

After convolutional encoding of the payload, the size is now doubled and the corresponding bits are represented in table X.7. No puncturing is applied in this configuration.

Table X.7 – First and last 48 bits after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 1201 | 0 | 1225 | 1 |
| 2 | 1 | 26 | 0 | 1202 | 1 | 1226 | 1 |
| 3 | 0 | 27 | 1 | 1203 | 0 | 1227 | 0 |
| 4 | 1 | 28 | 0 | 1204 | 1 | 1228 | 0 |
| 5 | 0 | 29 | 1 | 1205 | 1 | 1229 | 0 |
| 6 | 0 | 30 | 1 | 1206 | 1 | 1230 | 0 |
| 7 | 1 | 31 | 1 | 1207 | 0 | 1231 | 0 |
| 8 | 0 | 32 | 0 | 1208 | 0 | 1232 | 0 |
| 9 | 0 | 33 | 1 | 1209 | 0 | 1233 | 0 |
| 10 | 0 | 34 | 1 | 1210 | 1 | 1234 | 0 |
| 11 | 1 | 35 | 0 | 1211 | 1 | 1235 | 0 |
| 12 | 1 | 36 | 1 | 1212 | 0 | 1236 | 0 |
| 13 | 0 | 37 | 1 | 1213 | 0 | 1237 | 0 |
| 14 | 1 | 38 | 1 | 1214 | 0 | 1238 | 0 |
| 15 | 1 | 39 | 1 | 1215 | 0 | 1239 | 1 |
| 16 | 0 | 40 | 1 | 1216 | 0 | 1240 | 1 |
| 17 | 0 | 41 | 0 | 1217 | 0 | 1241 | 1 |
| 18 | 0 | 42 | 0 | 1218 | 1 | 1242 | 0 |
| 19 | 0 | 43 | 0 | 1219 | 1 | 1243 | 1 |
| 20 | 1 | 44 | 0 | 1220 | 0 | 1244 | 0 |
| 21 | 1 | 45 | 1 | 1221 | 0 | 1245 | 1 |
| 22 | 1 | 46 | 0 | 1222 | 1 | 1246 | 1 |
| 23 | 1 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 0 | 48 | 0 | 1224 | 1 | 1248 | 1 |

X.4.3 Interleaving

A 2-step interleaver is applied to the data, as defined in 6.12b.2.4. The resulting data (first and last 48 bits only) is represented in table X.8.

Table X.8 – First and last 48 bits after interleaving

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 1201 | 1 | 1225 | 0 |
| 2 | 0 | 26 | 0 | 1202 | 1 | 1226 | 1 |
| 3 | 1 | 27 | 0 | 1203 | 1 | 1227 | 1 |
| 4 | 1 | 28 | 1 | 1204 | 0 | 1228 | 1 |
| 5 | 1 | 29 | 1 | 1205 | 0 | 1229 | 1 |
| 6 | 0 | 30 | 0 | 1206 | 1 | 1230 | 1 |
| 7 | 0 | 31 | 0 | 1207 | 0 | 1231 | 0 |
| 8 | 0 | 32 | 1 | 1208 | 1 | 1232 | 1 |
| 9 | 1 | 33 | 0 | 1209 | 0 | 1233 | 0 |
| 10 | 1 | 34 | 0 | 1210 | 1 | 1234 | 0 |
| 11 | 0 | 35 | 1 | 1211 | 1 | 1235 | 0 |
| 12 | 1 | 36 | 0 | 1212 | 0 | 1236 | 1 |
| 13 | 0 | 37 | 1 | 1213 | 0 | 1237 | 1 |
| 14 | 0 | 38 | 1 | 1214 | 0 | 1238 | 0 |
| 15 | 1 | 39 | 0 | 1215 | 0 | 1239 | 0 |
| 16 | 1 | 40 | 1 | 1216 | 0 | 1240 | 0 |
| 17 | 0 | 41 | 0 | 1217 | 0 | 1241 | 0 |
| 18 | 1 | 42 | 0 | 1218 | 1 | 1242 | 1 |
| 19 | 1 | 43 | 1 | 1219 | 0 | 1243 | 1 |
| 20 | 1 | 44 | 0 | 1220 | 0 | 1244 | 1 |
| 21 | 0 | 45 | 1 | 1221 | 0 | 1245 | 0 |
| 22 | 1 | 46 | 0 | 1222 | 0 | 1246 | 1 |
| 23 | 0 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 0 | 48 | 0 | 1224 | 1 | 1248 | 1 |

X.4.4 Bit mapping

The bit mapping for the OFDM header in this example is QPSK (2 data bit per vector). The I and Q vectors are mapped as defined in figure 65.l. The resulting data is represented in table X.9 (first two and last two symbols).   [Rounding can be done to 3 decimal places for all numbers instead of 3 and 4 decimal places.]

Table X.9 – Bit mapping for the OFDM payload

| Vector # | I | Q | Vector # | I | Q | Vector # | I | Q | Vector # | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.7071 | -0.707 | 48 | -0.707 | -0.707 | 529 | -0.707 | -0.707 | 577 | 0.7071 | -0.707 |
| 2 | 0.7071 | 0.7071 | 49 | -0.707 | -0.707 | 530 | -0.707 | 0.7071 | 578 | 0.7071 | -0.707 |
| 3 | 0.7071 | -0.707 | 50 | -0.707 | -0.707 | 531 | 0.7071 | -0.707 | 579 | -0.707 | -0.707 |
| 4 | -0.707 | -0.707 | 51 | -0.707 | 0.7071 | 532 | -0.707 | 0.7071 | 580 | 0.7071 | -0.707 |
| 5 | 0.7071 | 0.7071 | 52 | -0.707 | -0.707 | 533 | -0.707 | -0.707 | 581 | 0.7071 | -0.707 |
| 6 | -0.707 | 0.7071 | 53 | -0.707 | -0.707 | 534 | 0.7071 | -0.707 | 582 | -0.707 | -0.707 |
| 7 | -0.707 | -0.707 | 54 | -0.707 | 0.7071 | 535 | 0.7071 | -0.707 | 583 | 0.7071 | -0.707 |
| 8 | 0.7071 | 0.7071 | 55 | 0.7071 | -0.707 | 536 | -0.707 | 0.7071 | 584 | 0.7071 | -0.707 |
| 9 | -0.707 | 0.7071 | 56 | -0.707 | 0.7071 | 537 | 0.7071 | 0.7071 | 585 | 0.7071 | -0.707 |
| 10 | 0.7071 | 0.7071 | 57 | -0.707 | -0.707 | 538 | 0.7071 | 0.7071 | 586 | 0.7071 | 0.7071 |
| 11 | -0.707 | 0.7071 | 58 | 0.7071 | 0.7071 | 539 | 0.7071 | -0.707 | 587 | -0.707 | -0.707 |
| 12 | -0.707 | -0.707 | 59 | 0.7071 | 0.7071 | 540 | 0.7071 | -0.707 | 588 | -0.707 | 0.7071 |
| 13 | 0.7071 | -0.707 | 60 | 0.7071 | -0.707 | 541 | 0.7071 | 0.7071 | 589 | 0.7071 | 0.7071 |
| 14 | -0.707 | 0.7071 | 61 | 0.7071 | 0.7071 | 542 | -0.707 | 0.7071 | 590 | -0.707 | 0.7071 |
| 15 | 0.7071 | -0.707 | 62 | -0.707 | 0.7071 | 543 | 0.7071 | -0.707 | 591 | 0.7071 | -0.707 |
| 16 | -0.707 | 0.7071 | 63 | 0.7071 | 0.7071 | 544 | -0.707 | 0.7071 | 592 | -0.707 | 0.7071 |
| 17 | -0.707 | -0.707 | 64 | 0.7071 | 0.7071 | 545 | -0.707 | 0.7071 | 593 | 0.7071 | -0.707 |
| 18 | 0.7071 | -0.707 | 65 | 0.7071 | 0.7071 | 546 | -0.707 | 0.7071 | 594 | 0.7071 | 0.7071 |
| 19 | 0.7071 | 0.7071 | 66 | -0.707 | 0.7071 | 547 | 0.7071 | 0.7071 | 595 | 0.7071 | -0.707 |
| 20 | -0.707 | 0.7071 | 67 | -0.707 | -0.707 | 548 | 0.7071 | 0.7071 | 596 | -0.707 | 0.7071 |
| 21 | -0.707 | -0.707 | 68 | 0.7071 | -0.707 | 549 | -0.707 | -0.707 | 597 | 0.7071 | -0.707 |
| 22 | 0.7071 | -0.707 | 69 | 0.7071 | 0.7071 | 550 | -0.707 | -0.707 | 598 | 0.7071 | -0.707 |
| 23 | 0.7071 | -0.707 | 70 | 0.7071 | 0.7071 | 551 | 0.7071 | 0.7071 | 599 | 0.7071 | 0.7071 |
| 24 | 0.7071 | -0.707 | 71 | 0.7071 | -0.707 | 552 | -0.707 | -0.707 | 600 | -0.707 | -0.707 |
| 25 | 0.7071 | -0.707 | 72 | -0.707 | -0.707 | 553 | -0.707 | 0.7071 | 601 | 0.7071 | 0.7071 |
| 26 | 0.7071 | -0.707 | 73 | 0.7071 | -0.707 | 554 | -0.707 | 0.7071 | 602 | 0.7071 | -0.707 |
| 27 | -0.707 | -0.707 | 74 | 0.7071 | 0.7071 | 555 | -0.707 | 0.7071 | 603 | -0.707 | 0.7071 |
| 28 | -0.707 | -0.707 | 75 | -0.707 | 0.7071 | 556 | -0.707 | -0.707 | 604 | -0.707 | 0.7071 |
| 29 | -0.707 | 0.7071 | 76 | 0.7071 | -0.707 | 557 | -0.707 | 0.7071 | 605 | -0.707 | 0.7071 |
| 30 | -0.707 | -0.707 | 77 | -0.707 | -0.707 | 558 | -0.707 | -0.707 | 606 | 0.7071 | -0.707 |
| 31 | 0.7071 | -0.707 | 78 | 0.7071 | -0.707 | 559 | -0.707 | -0.707 | 607 | -0.707 | -0.707 |
| 32 | -0.707 | -0.707 | 79 | 0.7071 | 0.7071 | 560 | -0.707 | -0.707 | 608 | -0.707 | -0.707 |
| 33 | -0.707 | 0.7071 | 80 | -0.707 | 0.7071 | 561 | 0.7071 | 0.7071 | 609 | -0.707 | 0.7071 |
| 34 | 0.7071 | 0.7071 | 81 | -0.707 | -0.707 | 562 | -0.707 | 0.7071 | 610 | -0.707 | -0.707 |
| 35 | 0.7071 | 0.7071 | 82 | 0.7071 | 0.7071 | 563 | 0.7071 | 0.7071 | 611 | -0.707 | -0.707 |
| 36 | -0.707 | -0.707 | 83 | -0.707 | 0.7071 | 564 | 0.7071 | -0.707 | 612 | -0.707 | 0.7071 |
| 37 | -0.707 | 0.7071 | 84 | 0.7071 | -0.707 | 565 | 0.7071 | -0.707 | 613 | -0.707 | 0.7071 |
| 38 | 0.7071 | -0.707 | 85 | 0.7071 | 0.7071 | 566 | 0.7071 | -0.707 | 614 | 0.7071 | 0.7071 |
| 39 | 0.7071 | 0.7071 | 86 | 0.7071 | 0.7071 | 567 | -0.707 | 0.7071 | 615 | 0.7071 | 0.7071 |
| 40 | 0.7071 | 0.7071 | 87 | -0.707 | -0.707 | 568 | -0.707 | -0.707 | 616 | -0.707 | 0.7071 |
| 41 | 0.7071 | 0.7071 | 88 | -0.707 | -0.707 | 569 | 0.7071 | 0.7071 | 617 | -0.707 | -0.707 |
| 42 | -0.707 | 0.7071 | 89 | 0.7071 | -0.707 | 570 | 0.7071 | -0.707 | 618 | -0.707 | 0.7071 |
| 43 | 0.7071 | 0.7071 | 90 | 0.7071 | -0.707 | 571 | -0.707 | 0.7071 | 619 | 0.7071 | -0.707 |
| 44 | 0.7071 | 0.7071 | 91 | -0.707 | 0.7071 | 572 | 0.7071 | 0.7071 | 620 | -0.707 | -0.707 |
| 45 | 0.7071 | -0.707 | 92 | -0.707 | 0.7071 | 573 | 0.7071 | 0.7071 | 621 | -0.707 | 0.7071 |
| 46 | 0.7071 | -0.707 | 93 | 0.7071 | -0.707 | 574 | -0.707 | -0.707 | 622 | 0.7071 | 0.7071 |
| 47 | -0.707 | -0.707 | 94 | 0.7071 | 0.7071 | 575 | -0.707 | 0.7071 | 623 | -0.707 | 0.7071 |
| 48 | -0.707 | -0.707 | 95 | -0.707 | -0.707 | 576 | 0.7071 | 0.7071 | 624 | -0.707 | 0.7071 |

X.4.5 Frequency Spreading

In this example, no frequency spreading is applied to the OFDM payload. The vectors from table X.9 are mapped directly into the frequency domain. The next paragraphs are illustrating the mapping in the frequency domain, taking into account the pilot tones, the DC tone and the guard tones.

X.5 Conversion from frequency domain to time domain

X.5.1 Pilot, DC and Guard tone insertion

The following steps are applied to both the OFDM header and the OFDM payload. Before going to the next steps, table X.5 and X.9 should be appended, resulting in 19 symbols of 48 bins in the frequency domain. The 48 bins are mapped in the frequency domain by inserting pilot tones, guard tones and a DC tone, as defined in 6.12b.2.6.

Table X.10 Complete Packet in the frequency domain (first and last three symbols)

| Subcarrier | Symbol 0 | Symbol 1 | Symbol 2 | Symbol 16 | Symbol 17 | Symbol 18 |
| --- | --- | --- | --- | --- | --- | --- |
| -32 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -31 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -30 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -29 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -28 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -27 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -26 | 0 - 1i | 0 - 1i | 0 + 1i | -0.707 + 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -25 | 0 + 1i | 0 + 1i | 0 - 1i | 0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -24 | 0 - 1i | 0 + 1i | 0 - 1i | 0.707 - 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| -23 | 0 + 1i | 0 + 1i | 0 + 1i | -0.707 - 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -22 | 0 + 1i | 0 + 1i | 0 + 1i | 0.707 - 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -21 | 0 - 1i | 0 - 1i | 0 + 1i | -0.707 - 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| -20 | -1 + 0i | 1 + 0i | -1 + 0i | -0.707 - 0.707i | 0.707 - 0.707i | 1 + 0i |
| -19 | 0 - 1i | 0 - 1i | 0 - 1i | 0.707 - 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -18 | 0 - 1i | 0 + 1i | 0 + 1i | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| -17 | 0 + 1i | 0 + 1i | 0 + 1i | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| -16 | 0 - 1i | 0 - 1i | 0 - 1i | 0.707 + 0.707i | 0.707 - 0.707i | 0.707 + 0.707i |
| -15 | 0 + 1i | 0 - 1i | 0 + 1i | 1 + 0i | 1 + 0i | -0.707 - 0.707i |
| -14 | 0 - 1i | 0 - 1i | 0 - 1i | -0.707 + 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| -13 | 1 + 0i | 1 + 0i | -1 + 0i | 0.707 - 0.707i | 0.707 + 0.707i | 0.707 + 0.707i |
| -12 | 0 - 1i | 0 - 1i | 0 + 1i | 0.707 + 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -11 | -1 + 0i | 1 + 0i | -1 + 0i | -0.707 + 0.707i | 0.707 - 0.707i | 0.707 - 0.707i |
| -10 | 0 + 1i | 0 + 1i | 0 + 1i | -0.707 + 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -9 | -1 + 0i | -1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -8 | 0 + 1i | 0 + 1i | 0 - 1i | -0.707 + 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| -7 | -1 + 0i | -1 + 0i | -1 + 0i | 0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| -6 | -1 + 0i | 1 + 0i | 1 + 0i | 0.707 + 0.707i | 0.707 + 0.707i | -1 + 0i |
| -5 | 0 - 1i | 0 + 1i | 0 + 1i | -0.707 + 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| -4 | -1 + 0i | -1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -3 | 0 + 1i | 0 + 1i | 0 + 1i | -0.707 + 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| -2 | 1 + 0i | -1 + 0i | 1 + 0i | -1 + 0i | 1 + 0i | 0.707 + 0.707i |
| -1 | 0 - 1i | 0 - 1i | 0 - 1i | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 0 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 1 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| 2 | -1 + 0i | -1 + 0i | 1 + 0i | 0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| 3 | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| 4 | -1 + 0i | -1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 5 | 1 + 0i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| 6 | -1 + 0i | 1 + 0i | 1 + 0i | 0.707 - 0.707i | -0.707 - 0.707i | -1 + 0i |
| 7 | 1 + 0i | 1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| 8 | -1 + 0i | -1 + 0i | -1 + 0i | 0.707 + 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 9 | 1 + 0i | -1 + 0i | -1 + 0i | 0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 10 | 1 + 0i | 1 + 0i | 1 + 0i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| 11 | 1 + 0i | 1 + 0i | 1 + 0i | 1 + 0i | -1 + 0i | -0.707 - 0.707i |
| 12 | 1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 13 | 1 + 0i | 1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| 14 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| 15 | 0 - 1i | 0 - 1i | 0 + 1i | -0.707 + 0.707i | 0.707 - 0.707i | 0.707 + 0.707i |
| 16 | 1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| 17 | 0 + 1i | 0 + 1i | 0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 18 | 1 + 0i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 19 | 0 + 1i | 0 + 1i | 0 - 1i | -0.707 + 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| 20 | -1 + 0i | 1 + 0i | 1 + 0i | -0.707 + 0.707i | -0.707 + 0.707i | 1 + 0i |
| 21 | 1 + 0i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| 22 | 0 - 1i | 0 + 1i | 0 + 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 23 | 1 + 0i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 24 | 0 + 1i | 0 + 1i | 0 + 1i | 1 + 0i | -1 + 0i | 0.707 + 0.707i |
| 25 | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| 26 | 0 - 1i | 0 - 1i | 0 - 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| 27 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 28 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 29 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 30 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 31 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |

Note: in table X.12, the pilot tones are represented in bold and the DC/Guard Tones are represented in italic.

X.5.2 Time domain OFDM Header and Payload

The data from table X12 is converted by an IFFT of size 64. Most IFFTs require a reordering of the data. Typically, the order of the frequencies within each symbol should be as follows:

0, 1, 2, … 31, -32, -31, …, -1

After the IFFT, each symbol is extended by a cyclic prefix of 16 samples. Each OFDM symbol has now a size of 80 samples (64 +16).

The resulting data in the time domain has 760 samples (19*80) and is represented in paragraph X.7 (complete packet).

X.6 Generation of the preamble

X.6.1 Generation of the STF

As defined in clause 6.3.4a.1, the STF consists of 4 symbols. Each symbol consists of 64 samples defined by the IFFT of a 64-sample frequency domain pattern, preceded by an 16-sample cyclic prefix (1/4 symbol duration). The frequency domain pattern produces a repeating 16-sample pattern in the time domain. The resulting STF is equivalent to 20 repetitions of the following 16-sample time domain pattern:

Table X.11 – STF time domain pattern

| Sample # | I | Q |
| --- | --- | --- |
| 1 | 0.081315 | 0 |
| 2 | 0.137361 | -0.08132 |
| 3 | 0.057498 | 0.16263 |
| 4 | -0.11913 | 0.081315 |
| 5 | 0 | 0 |
| 6 | 0.119133 | -0.08132 |
| 7 | -0.0575 | -0.16263 |
| 8 | -0.13736 | 0.081315 |
| 9 | -0.08132 | 0 |
| 10 | -0.13736 | -0.08132 |
| 11 | -0.0575 | 0.16263 |
| 12 | 0.119133 | 0.081315 |
| 13 | 0 | 0 |
| 14 | -0.11913 | -0.08132 |
| 15 | 0.057498 | -0.16263 |
| 16 | 0.137361 | 0.081315 |

Note: the last of the 20 patterns is inverted.

X.6.2 Generation of the LTF

As defined in clause 6.3.4a.2, the LTF consists of the following time domain pattern:

Table X.12 – LTF in time domain

| Sample # | I | Q | Sample # | I | Q |
| --- | --- | --- | --- | --- | --- |
| 33 | -0.09375 | 0 | 65 | 0.15625 | 0 |
| 34 | 0.063363 | 0.067436 | 66 | -0.0402 | 0.064633 |
| 35 | 0.018375 | -0.02225 | 67 | -0.07051 | 0.054104 |
| 36 | -0.06927 | -0.05257 | 68 | 0.107429 | -0.06847 |
| 37 | 0.035218 | 0.113468 | 69 | 0.001393 | 0.055726 |
| 38 | -0.06657 | -0.04397 | 70 | -0.12195 | 0.057032 |
| 39 | -0.03957 | -0.12822 | 71 | -0.14879 | -0.04658 |
| 40 | -0.06738 | -0.02588 | 72 | -0.09701 | 0.009262 |
| 41 | -0.07544 | -0.14174 | 73 | 0.012944 | 0.079235 |
| 42 | 0.101412 | -0.08029 | 74 | 0.035889 | 0.152064 |
| 43 | -0.07777 | 0.042168 | 75 | 0.052738 | 0.084803 |
| 44 | -0.05816 | 0.047901 | 76 | 0.110454 | 0.053547 |
| 45 | 0.147524 | -0.05236 | 77 | 0.065864 | -0.02844 |
| 46 | 0.000185 | -0.14496 | 78 | -0.0789 | -0.06447 |
| 47 | 0.057794 | -0.03685 | 79 | -0.04227 | 0.120476 |
| 48 | 0.137476 | 0.06034 | 80 | 0.04324 | -0.01666 |
| 49 | 0.03125 | 0.15625 | 81 | 0.03125 | -0.15625 |
| 50 | 0.04324 | 0.016659 | 82 | 0.137476 | -0.06034 |
| 51 | -0.04227 | -0.12048 | 83 | 0.057794 | 0.036845 |
| 52 | -0.0789 | 0.064469 | 84 | 0.000185 | 0.144958 |
| 53 | 0.065864 | 0.028444 | 85 | 0.147524 | 0.052362 |
| 54 | 0.110454 | -0.05355 | 86 | -0.05816 | -0.0479 |
| 55 | 0.052738 | -0.0848 | 87 | -0.07777 | -0.04217 |
| 56 | 0.035889 | -0.15206 | 88 | 0.101412 | 0.08029 |
| 57 | 0.012944 | -0.07924 | 89 | -0.07544 | 0.141735 |
| 58 | -0.09701 | -0.00926 | 90 | -0.06738 | 0.025875 |
| 59 | -0.14879 | 0.046583 | 91 | -0.03957 | 0.128223 |
| 60 | -0.12195 | -0.05703 | 92 | -0.06657 | 0.043973 |
| 61 | 0.001393 | -0.05573 | 93 | 0.035218 | -0.11347 |
| 62 | 0.107429 | 0.06847 | 94 | -0.06927 | 0.052568 |
| 63 | -0.07051 | -0.0541 | 95 | 0.018375 | 0.02225 |
| 64 | -0.0402 | -0.06463 | 96 | 0.063363 | -0.06744 |

The samples 1 through 32 are part of the cyclic prefix for the LTF. They are simply a copy of samples 65 through 96. The samples 97 through 160 are a repetition of samples 33 through 96.

X.7 The entire packet

The complete packet in the time domain is represented in table X.13. The STF is from sample 1 to 320. The LFT is from sample 321 to 480. The OFDM header and payload is from sample 481 to 1000.

Table X.13 Complete Packet

| Sample # | I | Q | Sample # | I | Q | Sample # | I | Q | Sample # | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0813 | 0 | 501 | -0.0127 | -0.0739 | 1001 | -0.0339 | -0.0911 | 1501 | 0.046 | -0.0778 |
| 2 | 0.1374 | -0.0813 | 502 | 0.0108 | -0.1578 | 1002 | -0.07 | -0.1182 | 1502 | 0.1049 | -0.0428 |
| 3 | 0.0575 | 0.1626 | 503 | 0.0809 | -0.0891 | 1003 | 0.0561 | 0.1135 | 1503 | 0.1462 | -0.1202 |
| 4 | -0.1191 | 0.0813 | 504 | -0.023 | -0.0002 | 1004 | 0.0012 | 0.0636 | 1504 | 0.0769 | -0.1058 |
| 5 | 0 | 0 | 505 | -0.035 | 0.0313 | 1005 | -0.0656 | 0.0777 | 1505 | 0.0442 | -0.0092 |
| 6 | 0.1191 | -0.0813 | 506 | 0.0811 | -0.0856 | 1006 | -0.0061 | 0.0399 | 1506 | -0.0081 | 0.03 |
| 7 | -0.0575 | -0.1626 | 507 | 0.0321 | -0.1944 | 1007 | -0.0274 | -0.0894 | 1507 | -0.0309 | -0.0338 |
| 8 | -0.1374 | 0.0813 | 508 | -0.0063 | -0.0954 | 1008 | -0.0022 | 0.0141 | 1508 | -0.036 | 0.0494 |
| 9 | -0.0813 | 0 | 509 | -0.0013 | -0.0445 | 1009 | -0.0442 | 0.0442 | 1509 | 0.0184 | 0.0439 |
| 10 | -0.1374 | -0.0813 | 510 | -0.0623 | -0.0018 | 1010 | -0.04 | 0.099 | 1510 | 0.1178 | -0.0108 |
| 11 | -0.0575 | 0.1626 | 511 | -0.0229 | -0.0042 | 1011 | 0.1117 | -0.007 | 1511 | 0.012 | 0.1872 |
| 12 | 0.1191 | 0.0813 | 512 | 0.0317 | -0.0312 | 1012 | 0.102 | -0.172 | 1512 | -0.073 | 0.0811 |
| 13 | 0 | 0 | 513 | -0.0938 | 0 | 1013 | 0.0115 | 0.0533 | 1513 | -0.0377 | -0.0598 |
| 14 | -0.1191 | -0.0813 | 514 | -0.1313 | -0.0105 | 1014 | -0.0324 | 0.1434 | 1514 | 0.0236 | 0.0885 |
| 15 | 0.0575 | -0.1626 | 515 | -0.0297 | 0.0413 | 1015 | 0.0087 | 0.0105 | 1515 | 0.0264 | 0.0399 |
| 16 | 0.1374 | 0.0813 | 516 | -0.0726 | 0.0698 | 1016 | 0.0319 | 0.0463 | 1516 | -0.027 | -0.0259 |
| 17 | 0.0813 | 0 | 517 | -0.009 | 0.0449 | 1017 | -0.0248 | 0.1002 | 1517 | 0.0482 | -0.0124 |
| 18 | 0.1374 | -0.0813 | 518 | 0.066 | -0.0058 | 1018 | 0.0006 | -0.0948 | 1518 | 0.0006 | -0.0509 |
| 19 | 0.0575 | 0.1626 | 519 | -0.1183 | 0.0691 | 1019 | -0.1335 | -0.0949 | 1519 | -0.0328 | -0.0642 |
| 20 | -0.1191 | 0.0813 | 520 | -0.0509 | 0.1226 | 1020 | -0.1895 | 0.151 | 1520 | 0.0838 | -0.0336 |
| 21 | 0 | 0 | 521 | 0.0846 | -0.0884 | 1021 | 0.0443 | 0.0291 | 1521 | 0.0975 | 0.0442 |
| 22 | 0.1191 | -0.0813 | 522 | 0.0198 | 0.0185 | 1022 | 0.0242 | -0.0092 | 1522 | -0.0014 | -0.0005 |
| 23 | -0.0575 | -0.1626 | 523 | 0.1578 | 0.069 | 1023 | 0.0272 | 0.0261 | 1523 | 0.0432 | -0.0778 |
| 24 | -0.1374 | 0.0813 | 524 | 0.1679 | -0.1456 | 1024 | 0.1513 | -0.0939 | 1524 | 0.0785 | 0.0789 |
| 25 | -0.0813 | 0 | 525 | -0.0252 | -0.0276 | 1025 | 0.035 | 0.0092 | 1525 | -0.0384 | -0.0388 |
| 26 | -0.1374 | -0.0813 | 526 | -0.0517 | 0.0511 | 1026 | -0.0001 | -0.0251 | 1526 | -0.0144 | -0.0665 |
| 27 | -0.0575 | 0.1626 | 527 | -0.0072 | 0.1602 | 1027 | -0.077 | -0.1073 | 1527 | -0.0833 | 0.1151 |
| 28 | 0.1191 | 0.0813 | 528 | 0.0053 | 0.1347 | 1028 | -0.2123 | -0.0922 | 1528 | 0.0285 | -0.0178 |
| 29 | 0 | 0 | 529 | -0.0938 | -0.0938 | 1029 | -0.0705 | 0.0299 | 1529 | 0.0884 | -0.0884 |
| 30 | -0.1191 | -0.0813 | 530 | -0.0704 | 0.0089 | 1030 | 0.0202 | 0.2039 | 1530 | -0.0866 | -0.0186 |
| 31 | 0.0575 | -0.1626 | 531 | 0.0387 | -0.019 | 1031 | -0.0444 | -0.0132 | 1531 | -0.0191 | 0.0176 |
| 32 | 0.1374 | 0.0813 | 532 | 0.0305 | -0.1033 | 1032 | -0.0458 | -0.0719 | 1532 | 0.0027 | 0.1144 |
| 33 | 0.0813 | 0 | 533 | 0.1506 | -0.064 | 1033 | 0.0469 | 0.0339 | 1533 | -0.0452 | 0.028 |
| 34 | 0.1374 | -0.0813 | 534 | 0.0134 | 0.0104 | 1034 | 0.1251 | 0.0444 | 1534 | -0.1013 | -0.0636 |
| 35 | 0.0575 | 0.1626 | 535 | -0.1074 | 0.1347 | 1035 | 0.0742 | 0.0846 | 1535 | -0.104 | 0.0275 |
| 36 | -0.1191 | 0.0813 | 536 | 0.2336 | 0.0013 | 1036 | 0.0284 | -0.0775 | 1536 | -0.0237 | 0.013 |
| 37 | 0 | 0 | 537 | 0.0975 | 0.0313 | 1037 | 0.0161 | 0.0678 | 1537 | -0.0975 | -0.0442 |
| 38 | 0.1191 | -0.0813 | 538 | -0.1635 | 0.0647 | 1038 | 0.0182 | 0.1366 | 1538 | -0.0113 | -0.0183 |
| 39 | -0.0575 | -0.1626 | 539 | -0.0001 | -0.0878 | 1039 | 0.0388 | -0.1515 | 1539 | 0.061 | -0.0032 |
| 40 | -0.1374 | 0.0813 | 540 | -0.006 | -0.0244 | 1040 | 0.0253 | -0.1164 | 1540 | -0.0158 | 0.0043 |
| 41 | -0.0813 | 0 | 541 | 0.0325 | 0.0132 | 1041 | -0.221 | 0.1768 | 1541 | -0.003 | -0.0394 |
| 42 | -0.1374 | -0.0813 | 542 | 0.0068 | 0.0881 | 1042 | -0.159 | -0.0987 | 1542 | 0.0495 | -0.1416 |
| 43 | -0.0575 | 0.1626 | 543 | -0.0951 | 0.0258 | 1043 | 0.0729 | -0.1008 | 1543 | 0.0775 | -0.019 |
| 44 | 0.1191 | 0.0813 | 544 | 0.07 | -0.0522 | 1044 | 0.1031 | 0.0404 | 1544 | -0.1018 | 0.1572 |
| 45 | 0 | 0 | 545 | 0.0313 | 0.0625 | 1045 | 0.0041 | -0.1319 | 1545 | -0.0975 | 0.0092 |
| 46 | -0.1191 | -0.0813 | 546 | 0.0353 | -0.0245 | 1046 | 0.0918 | -0.1753 | 1546 | -0.0186 | 0.0059 |
| 47 | 0.0575 | -0.1626 | 547 | 0.1127 | 0 | 1047 | 0.0677 | -0.0315 | 1547 | 0.01 | 0.1558 |
| 48 | 0.1374 | 0.0813 | 548 | -0.0812 | -0.0568 | 1048 | 0.009 | -0.0116 | 1548 | 0.2762 | 0.064 |
| 49 | 0.0813 | 0 | 549 | 0.0586 | -0.0945 | 1049 | -0.069 | -0.0065 | 1549 | 0.1263 | 0.0708 |
| 50 | 0.1374 | -0.0813 | 550 | 0.1806 | 0.1341 | 1050 | -0.0625 | 0.0031 | 1550 | 0.0774 | 0.0459 |
| 51 | 0.0575 | 0.1626 | 551 | -0.1053 | 0.0103 | 1051 | 0.0386 | 0.0462 | 1551 | 0.2325 | -0.0613 |
| 52 | -0.1191 | 0.0813 | 552 | -0.1111 | -0.0162 | 1052 | 0.0071 | 0.1276 | 1552 | -0.1041 | 0.0311 |
| 53 | 0 | 0 | 553 | 0.0404 | 0.0884 | 1053 | 0.0876 | -0.0362 | 1553 | -0.0792 | 0 |
| 54 | 0.1191 | -0.0813 | 554 | 0.0132 | 0.0311 | 1054 | -0.0073 | -0.1248 | 1554 | 0.1272 | -0.0289 |
| 55 | -0.0575 | -0.1626 | 555 | 0.0236 | 0.1247 | 1055 | -0.0828 | 0.046 | 1555 | 0.0535 | -0.0004 |
| 56 | -0.1374 | 0.0813 | 556 | 0.104 | 0.0807 | 1056 | 0.0868 | 0.0151 | 1556 | -0.0384 | -0.0712 |
| 57 | -0.0813 | 0 | 557 | 0.0565 | -0.0037 | 1057 | 0.0846 | 0.0221 | 1557 | -0.0553 | 0.0259 |
| 58 | -0.1374 | -0.0813 | 558 | -0.1119 | 0.0068 | 1058 | 0.1566 | 0.0421 | 1558 | 0.086 | 0.0146 |
| 59 | -0.0575 | 0.1626 | 559 | -0.1248 | -0.0569 | 1059 | 0.0341 | -0.1434 | 1559 | -0.0683 | -0.0955 |
| 60 | 0.1191 | 0.0813 | 560 | -0.0305 | -0.107 | 1060 | -0.1056 | -0.0634 | 1560 | -0.0744 | -0.044 |
| 61 | 0 | 0 | 561 | -0.0625 | 0.0312 | 1061 | 0.0887 | -0.0194 | 1561 | 0.1196 | -0.0129 |
| 62 | -0.1191 | -0.0813 | 562 | -0.0761 | 0.0156 | 1062 | 0.0499 | -0.0776 | 1562 | -0.0221 | 0.1205 |
| 63 | 0.0575 | -0.1626 | 563 | -0.0127 | 0.07 | 1063 | -0.059 | 0.0857 | 1563 | 0.082 | 0.1402 |
| 64 | 0.1374 | 0.0813 | 564 | 0.0955 | -0.0471 | 1064 | -0.0846 | 0.0044 | 1564 | 0.0902 | 0.0216 |
| 65 | 0.0813 | 0 | 565 | 0.0767 | -0.0757 | 1065 | 0.0027 | -0.0652 | 1565 | -0.124 | 0.0787 |
| 66 | 0.1374 | -0.0813 | 566 | 0.1259 | 0.1346 | 1066 | -0.0127 | 0.0055 | 1566 | -0.1075 | 0.0142 |
| 67 | 0.0575 | 0.1626 | 567 | 0.1617 | -0.0361 | 1067 | -0.1993 | -0.09 | 1567 | -0.1252 | 0.0065 |
| 68 | -0.1191 | 0.0813 | 568 | -0.0688 | -0.0941 | 1068 | -0.0546 | 0.0464 | 1568 | 0.0418 | 0.1054 |
| 69 | 0 | 0 | 569 | -0.1288 | 0.0442 | 1069 | -0.0018 | 0.1197 | 1569 | 0.1859 | -0.0442 |
| 70 | 0.1191 | -0.0813 | 570 | -0.001 | 0.0009 | 1070 | 0.0267 | -0.0454 | 1570 | -0.0284 | -0.0753 |
| 71 | -0.0575 | -0.1626 | 571 | 0.0179 | 0.0537 | 1071 | 0.0479 | 0.0005 | 1571 | -0.1136 | 0.0265 |
| 72 | -0.1374 | 0.0813 | 572 | -0.0176 | -0.051 | 1072 | -0.1386 | 0.0537 | 1572 | -0.0184 | -0.0208 |
| 73 | -0.0813 | 0 | 573 | -0.0819 | -0.1868 | 1073 | 0.0442 | 0 | 1573 | -0.01 | -0.0802 |
| 74 | -0.1374 | -0.0813 | 574 | -0.0835 | -0.0116 | 1074 | 0.0943 | -0.037 | 1574 | 0.0239 | -0.0236 |
| 75 | -0.0575 | 0.1626 | 575 | -0.1054 | 0.0304 | 1075 | 0.023 | 0.0643 | 1575 | 0.0299 | 0.1062 |
| 76 | 0.1191 | 0.0813 | 576 | -0.0548 | -0.0512 | 1076 | 0.0812 | 0.1195 | 1576 | -0.0892 | 0.0226 |
| 77 | 0 | 0 | 577 | 0.0625 | 0.0625 | 1077 | -0.0536 | -0.0502 | 1577 | -0.0038 | -0.1288 |
| 78 | -0.1191 | -0.0813 | 578 | -0.0895 | 0.0245 | 1078 | 0.0377 | 0.0523 | 1578 | 0.0779 | 0 |
| 79 | 0.0575 | -0.1626 | 579 | -0.0546 | -0.0068 | 1079 | -0.0117 | 0.1135 | 1579 | -0.0288 | 0.1315 |
| 80 | 0.1374 | 0.0813 | 580 | 0.1191 | 0.1562 | 1080 | -0.1328 | -0.09 | 1580 | -0.0365 | 0.0222 |
| 81 | 0.0813 | 0 | 581 | -0.0694 | -0.0349 | 1081 | 0.056 | 0.0377 | 1581 | -0.0638 | -0.1333 |
| 82 | 0.1374 | -0.0813 | 582 | -0.0744 | -0.1239 | 1082 | 0.0156 | 0.0821 | 1582 | -0.0503 | -0.1178 |
| 83 | 0.0575 | 0.1626 | 583 | 0.0336 | 0.0064 | 1083 | -0.0632 | -0.001 | 1583 | -0.0475 | -0.116 |
| 84 | -0.1191 | 0.0813 | 584 | -0.0983 | -0.025 | 1084 | -0.0736 | 0.0986 | 1584 | -0.0155 | -0.1272 |
| 85 | 0 | 0 | 585 | -0.0792 | 0.0496 | 1085 | -0.0617 | -0.0263 | 1585 | 0.0975 | 0.0442 |
| 86 | 0.1191 | -0.0813 | 586 | 0.0212 | -0.1277 | 1086 | 0.0517 | -0.0815 | 1586 | -0.0014 | -0.0005 |
| 87 | -0.0575 | -0.1626 | 587 | 0.0417 | -0.1349 | 1087 | 0.081 | 0.0504 | 1587 | 0.0432 | -0.0778 |
| 88 | -0.1374 | 0.0813 | 588 | 0.0603 | 0.1072 | 1088 | 0.0947 | 0.0291 | 1588 | 0.0785 | 0.0789 |
| 89 | -0.0813 | 0 | 589 | 0.0507 | -0.0919 | 1089 | 0.0221 | 0.0221 | 1589 | -0.0384 | -0.0388 |
| 90 | -0.1374 | -0.0813 | 590 | -0.006 | -0.061 | 1090 | -0.0477 | 0.0641 | 1590 | -0.0144 | -0.0665 |
| 91 | -0.0575 | 0.1626 | 591 | -0.0786 | 0.0717 | 1091 | 0.0759 | 0.0732 | 1591 | -0.0833 | 0.1151 |
| 92 | 0.1191 | 0.0813 | 592 | -0.0533 | 0.023 | 1092 | 0.1705 | -0.0117 | 1592 | 0.0285 | -0.0178 |
| 93 | 0 | 0 | 593 | -0.0625 | -0.0313 | 1093 | 0.0309 | -0.0194 | 1593 | 0.0884 | -0.0884 |
| 94 | -0.1191 | -0.0813 | 594 | -0.0995 | -0.1803 | 1094 | -0.0468 | 0.121 | 1594 | -0.0866 | -0.0186 |
| 95 | 0.0575 | -0.1626 | 595 | 0.0276 | -0.0152 | 1095 | 0.0472 | 0.064 | 1595 | -0.0191 | 0.0176 |
| 96 | 0.1374 | 0.0813 | 596 | 0.1086 | 0.0599 | 1096 | -0.0824 | 0.0259 | 1596 | 0.0027 | 0.1144 |
| 97 | 0.0813 | 0 | 597 | 0.0429 | -0.018 | 1097 | -0.0598 | 0.0781 | 1597 | -0.0452 | 0.028 |
| 98 | 0.1374 | -0.0813 | 598 | -0.0353 | 0.0114 | 1098 | 0.1038 | -0.0245 | 1598 | -0.1013 | -0.0636 |
| 99 | 0.0575 | 0.1626 | 599 | 0.005 | -0.0202 | 1099 | -0.0703 | -0.0619 | 1599 | -0.104 | 0.0275 |
| 100 | -0.1191 | 0.0813 | 600 | 0.0629 | 0.0905 | 1100 | -0.0916 | -0.0922 | 1600 | -0.0237 | 0.013 |
| 101 | 0 | 0 | 601 | 0.0038 | -0.0442 | 1101 | 0.046 | -0.1014 | 1601 | -0.0663 | -0.0221 |
| 102 | 0.1191 | -0.0813 | 602 | 0.1153 | -0.0523 | 1102 | 0.0615 | -0.0589 | 1602 | 0.0146 | 0.0452 |
| 103 | -0.0575 | -0.1626 | 603 | 0.1499 | 0.1604 | 1103 | -0.0019 | -0.1152 | 1603 | 0.0792 | -0.0696 |
| 104 | -0.1374 | 0.0813 | 604 | -0.0928 | -0.0473 | 1104 | -0.1423 | 0.0617 | 1604 | 0.2029 | -0.0467 |
| 105 | -0.0813 | 0 | 605 | -0.0819 | -0.0136 | 1105 | -0.221 | 0.1768 | 1605 | 0.0622 | 0.0658 |
| 106 | -0.1374 | -0.0813 | 606 | 0.0368 | 0.0885 | 1106 | -0.159 | -0.0987 | 1606 | -0.0001 | 0.0173 |
| 107 | -0.0575 | 0.1626 | 607 | 0.0104 | 0.0116 | 1107 | 0.0729 | -0.1008 | 1607 | -0.03 | -0.0333 |
| 108 | 0.1191 | 0.0813 | 608 | -0.053 | 0.0958 | 1108 | 0.1031 | 0.0404 | 1608 | 0.0089 | -0.1029 |
| 109 | 0 | 0 | 609 | -0.125 | 0 | 1109 | 0.0041 | -0.1319 | 1609 | 0.2042 | -0.0313 |
| 110 | -0.1191 | -0.0813 | 610 | -0.0424 | -0.0923 | 1110 | 0.0918 | -0.1753 | 1610 | 0.0236 | 0.083 |
| 111 | 0.0575 | -0.1626 | 611 | -0.0121 | -0.0481 | 1111 | 0.0677 | -0.0315 | 1611 | -0.0405 | 0.019 |
| 112 | 0.1374 | 0.0813 | 612 | -0.023 | -0.0248 | 1112 | 0.009 | -0.0116 | 1612 | 0.0501 | 0.045 |
| 113 | 0.0813 | 0 | 613 | 0.0123 | -0.0588 | 1113 | -0.069 | -0.0065 | 1613 | -0.0366 | -0.0293 |
| 114 | 0.1374 | -0.0813 | 614 | -0.1801 | -0.0301 | 1114 | -0.0625 | 0.0031 | 1614 | -0.0689 | -0.1275 |
| 115 | 0.0575 | 0.1626 | 615 | -0.1119 | 0.0865 | 1115 | 0.0386 | 0.0462 | 1615 | -0.0814 | 0.0672 |
| 116 | -0.1191 | 0.0813 | 616 | 0.2078 | 0.0573 | 1116 | 0.0071 | 0.1276 | 1616 | 0.106 | 0.0744 |
| 117 | 0 | 0 | 617 | 0.1417 | 0.1379 | 1117 | 0.0876 | -0.0362 | 1617 | 0.0221 | -0.0663 |
| 118 | 0.1191 | -0.0813 | 618 | 0.0471 | 0.1098 | 1118 | -0.0073 | -0.1248 | 1618 | -0.1383 | 0.0201 |
| 119 | -0.0575 | -0.1626 | 619 | 0.0923 | -0.0792 | 1119 | -0.0828 | 0.046 | 1619 | 0.0757 | 0.1015 |
| 120 | -0.1374 | 0.0813 | 620 | 0.0517 | -0.0281 | 1120 | 0.0868 | 0.0151 | 1620 | 0.0075 | 0.0598 |
| 121 | -0.0813 | 0 | 621 | 0.0507 | -0.0202 | 1121 | 0.0792 | 0.1547 | 1621 | 0.0529 | -0.0182 |
| 122 | -0.1374 | -0.0813 | 622 | 0.0916 | 0.0438 | 1122 | -0.1099 | 0.1996 | 1622 | 0.0669 | -0.0997 |
| 123 | -0.0575 | 0.1626 | 623 | 0.0851 | 0.0998 | 1123 | -0.0471 | 0.1936 | 1623 | -0.1409 | -0.1089 |
| 124 | 0.1191 | 0.0813 | 624 | 0.0057 | 0.0287 | 1124 | -0.0845 | -0.0462 | 1624 | 0.0488 | 0.0111 |
| 125 | 0 | 0 | 625 | -0.0625 | 0.0312 | 1125 | -0.06 | -0.0287 | 1625 | 0.0884 | 0.0038 |
| 126 | -0.1191 | -0.0813 | 626 | -0.0761 | 0.0156 | 1126 | -0.0443 | 0.0334 | 1626 | -0.0409 | -0.1157 |
| 127 | 0.0575 | -0.1626 | 627 | -0.0127 | 0.07 | 1127 | -0.0236 | -0.1059 | 1627 | -0.1166 | 0.0292 |
| 128 | 0.1374 | 0.0813 | 628 | 0.0955 | -0.0471 | 1128 | 0.0514 | -0.0693 | 1628 | -0.1288 | 0.1025 |
| 129 | 0.0813 | 0 | 629 | 0.0767 | -0.0757 | 1129 | -0.0221 | 0.0625 | 1629 | -0.0321 | 0.036 |
| 130 | 0.1374 | -0.0813 | 630 | 0.1259 | 0.1346 | 1130 | 0.0208 | 0.0794 | 1630 | -0.1377 | 0.0855 |
| 131 | 0.0575 | 0.1626 | 631 | 0.1617 | -0.0361 | 1131 | 0.0203 | 0.063 | 1631 | 0.0068 | -0.0864 |
| 132 | -0.1191 | 0.0813 | 632 | -0.0688 | -0.0941 | 1132 | -0.0092 | -0.0064 | 1632 | 0.1698 | -0.1521 |
| 133 | 0 | 0 | 633 | -0.1288 | 0.0442 | 1133 | -0.0017 | -0.1128 | 1633 | -0.0663 | 0.0221 |
| 134 | 0.1191 | -0.0813 | 634 | -0.001 | 0.0009 | 1134 | -0.0134 | 0.0834 | 1634 | -0.1288 | -0.005 |
| 135 | -0.0575 | -0.1626 | 635 | 0.0179 | 0.0537 | 1135 | 0.1179 | 0.2091 | 1635 | 0.0079 | 0.0387 |
| 136 | -0.1374 | 0.0813 | 636 | -0.0176 | -0.051 | 1136 | 0.0276 | -0.0576 | 1636 | 0.0709 | 0.0776 |
| 137 | -0.0813 | 0 | 637 | -0.0819 | -0.1868 | 1137 | -0.0092 | -0.0663 | 1637 | -0.0363 | 0.015 |
| 138 | -0.1374 | -0.0813 | 638 | -0.0835 | -0.0116 | 1138 | 0.134 | 0.0386 | 1638 | -0.0821 | 0.0263 |
| 139 | -0.0575 | 0.1626 | 639 | -0.1054 | 0.0304 | 1139 | -0.0063 | -0.02 | 1639 | -0.061 | -0.0921 |
| 140 | 0.1191 | 0.0813 | 640 | -0.0548 | -0.0512 | 1140 | -0.0102 | 0.0107 | 1640 | -0.0519 | -0.0788 |
| 141 | 0 | 0 | 641 | -0.0938 | -0.0938 | 1141 | 0.1183 | 0.0454 | 1641 | 0.048 | 0.0442 |
| 142 | -0.1191 | -0.0813 | 642 | -0.161 | -0.0163 | 1142 | 0.0381 | 0.0009 | 1642 | 0.0127 | -0.0391 |
| 143 | 0.0575 | -0.1626 | 643 | 0.069 | -0.0489 | 1143 | -0.0954 | -0.0602 | 1643 | 0.1369 | -0.1024 |
| 144 | 0.1374 | 0.0813 | 644 | -0.0259 | -0.1291 | 1144 | -0.1256 | 0.0745 | 1644 | 0.1462 | -0.102 |
| 145 | 0.0813 | 0 | 645 | 0.0923 | 0.0598 | 1145 | -0.0183 | 0.1029 | 1645 | -0.1013 | -0.0462 |
| 146 | 0.1374 | -0.0813 | 646 | 0.1863 | 0.0736 | 1146 | 0.0207 | -0.0517 | 1646 | 0.0215 | -0.0693 |
| 147 | 0.0575 | 0.1626 | 647 | 0.0582 | -0.1068 | 1147 | 0.0076 | 0.0157 | 1647 | -0.0626 | -0.1302 |
| 148 | -0.1191 | 0.0813 | 648 | 0.1209 | 0.0543 | 1148 | 0.0224 | 0.006 | 1648 | -0.1778 | 0.0273 |
| 149 | 0 | 0 | 649 | -0.0625 | 0.0038 | 1149 | -0.0453 | -0.0435 | 1649 | 0.0404 | 0.0221 |
| 150 | 0.1191 | -0.0813 | 650 | -0.1089 | -0.1864 | 1150 | -0.0842 | -0.0831 | 1650 | -0.0096 | -0.0082 |
| 151 | -0.0575 | -0.1626 | 651 | 0.0573 | -0.0155 | 1151 | 0.0451 | -0.0659 | 1651 | -0.0562 | 0.0986 |
| 152 | -0.1374 | 0.0813 | 652 | 0.0481 | 0.014 | 1152 | 0.1347 | 0.0624 | 1652 | -0.0045 | -0.0916 |
| 153 | -0.0813 | 0 | 653 | 0.0739 | 0.013 | 1153 | 0.035 | -0.1105 | 1653 | 0.0797 | -0.1068 |
| 154 | -0.1374 | -0.0813 | 654 | 0.0347 | 0.114 | 1154 | -0.0174 | -0.0107 | 1654 | 0.1204 | 0.1194 |
| 155 | -0.0575 | 0.1626 | 655 | -0.0525 | -0.0826 | 1155 | -0.0471 | 0.0816 | 1655 | -0.0515 | 0.0651 |
| 156 | 0.1191 | 0.0813 | 656 | -0.0258 | -0.1721 | 1156 | -0.0334 | -0.167 | 1656 | 0.0097 | 0.056 |
| 157 | 0 | 0 | 657 | 0.0625 | 0.0938 | 1157 | 0.0287 | 0.06 | 1657 | 0.1196 | 0.1158 |
| 158 | -0.1191 | -0.0813 | 658 | 0.0189 | 0.1665 | 1158 | -0.1003 | 0.0383 | 1658 | 0.0015 | 0.0564 |
| 159 | 0.0575 | -0.1626 | 659 | -0.0406 | -0.0316 | 1159 | -0.074 | -0.0646 | 1659 | 0.0019 | 0.0984 |
| 160 | 0.1374 | 0.0813 | 660 | 0.0854 | -0.0668 | 1160 | 0.0401 | 0.1804 | 1660 | 0.046 | 0.117 |
| 161 | 0.0813 | 0 | 661 | 0.1192 | -0.0286 | 1161 | -0.0533 | -0.0754 | 1661 | -0.0251 | -0.0048 |
| 162 | 0.1374 | -0.0813 | 662 | 0.0439 | -0.0482 | 1162 | -0.0314 | -0.1943 | 1662 | -0.0084 | 0.0114 |
| 163 | 0.0575 | 0.1626 | 663 | 0.0157 | 0.0419 | 1163 | 0.0297 | -0.0474 | 1663 | -0.0213 | 0.1051 |
| 164 | -0.1191 | 0.0813 | 664 | 0.0396 | 0.0266 | 1164 | 0.0058 | -0.096 | 1664 | -0.15 | 0.0033 |
| 165 | 0 | 0 | 665 | 0.0054 | -0.0533 | 1165 | -0.0425 | -0.0381 | 1665 | -0.0663 | -0.0221 |
| 166 | 0.1191 | -0.0813 | 666 | -0.0544 | -0.0866 | 1166 | -0.1165 | -0.0016 | 1666 | 0.0146 | 0.0452 |
| 167 | -0.0575 | -0.1626 | 667 | 0.0017 | -0.1572 | 1167 | -0.0952 | 0.0627 | 1667 | 0.0792 | -0.0696 |
| 168 | -0.1374 | 0.0813 | 668 | 0.0312 | 0.0304 | 1168 | 0.0971 | 0.0815 | 1668 | 0.2029 | -0.0467 |
| 169 | -0.0813 | 0 | 669 | 0.0945 | 0.129 | 1169 | 0.2301 | -0.0221 | 1669 | 0.0622 | 0.0658 |
| 170 | -0.1374 | -0.0813 | 670 | 0.0307 | -0.0302 | 1170 | 0.0713 | -0.0121 | 1670 | -0.0001 | 0.0173 |
| 171 | -0.0575 | 0.1626 | 671 | -0.0938 | 0.0727 | 1171 | -0.0063 | 0.0023 | 1671 | -0.03 | -0.0333 |
| 172 | 0.1191 | 0.0813 | 672 | -0.0358 | 0.0817 | 1172 | 0.0074 | -0.0484 | 1672 | 0.0089 | -0.1029 |
| 173 | 0 | 0 | 673 | -0.1563 | -0.0313 | 1173 | 0.0197 | -0.1208 | 1673 | 0.2042 | -0.0313 |
| 174 | -0.1191 | -0.0813 | 674 | -0.1669 | 0.0442 | 1174 | 0.1439 | -0.0769 | 1674 | 0.0236 | 0.083 |
| 175 | 0.0575 | -0.1626 | 675 | -0.0724 | 0.0117 | 1175 | -0.0021 | 0.0249 | 1675 | -0.0405 | 0.019 |
| 176 | 0.1374 | 0.0813 | 676 | -0.1284 | 0.0136 | 1176 | -0.0564 | -0.0502 | 1676 | 0.0501 | 0.045 |
| 177 | 0.0813 | 0 | 677 | -0.0611 | 0.0598 | 1177 | 0.0754 | -0.1342 | 1677 | -0.0366 | -0.0293 |
| 178 | 0.1374 | -0.0813 | 678 | -0.0494 | 0.0489 | 1178 | 0.0003 | -0.0122 | 1678 | -0.0689 | -0.1275 |
| 179 | 0.0575 | 0.1626 | 679 | 0.0423 | 0.0947 | 1179 | -0.0393 | 0.1014 | 1679 | -0.0814 | 0.0672 |
| 180 | -0.1191 | 0.0813 | 680 | 0.0604 | -0.0514 | 1180 | -0.1117 | 0.0089 | 1680 | 0.106 | 0.0744 |
| 181 | 0 | 0 | 681 | -0.0625 | -0.1288 | 1181 | 0.0194 | -0.0266 | 1681 | 0.1013 | 0.0884 |
| 182 | 0.1191 | -0.0813 | 682 | 0.0442 | 0.105 | 1182 | 0.0882 | -0.031 | 1682 | -0.1275 | -0.0086 |
| 183 | -0.0575 | -0.1626 | 683 | 0.0038 | 0.1582 | 1183 | -0.1379 | -0.0368 | 1683 | -0.1993 | 0.0024 |
| 184 | -0.1374 | 0.0813 | 684 | 0.0017 | -0.0426 | 1184 | 0.0444 | 0.1167 | 1684 | -0.1107 | -0.0003 |
| 185 | -0.0813 | 0 | 685 | 0.064 | -0.0885 | 1185 | 0.0792 | 0.1547 | 1685 | -0.0458 | -0.0087 |
| 186 | -0.1374 | -0.0813 | 686 | -0.0767 | 0.0797 | 1186 | -0.1099 | 0.1996 | 1686 | -0.2114 | 0.1141 |
| 187 | -0.0575 | 0.1626 | 687 | -0.0337 | 0.0803 | 1187 | -0.0471 | 0.1936 | 1687 | -0.1255 | 0.11 |
| 188 | 0.1191 | 0.0813 | 688 | 0.0965 | -0.0507 | 1188 | -0.0845 | -0.0462 | 1688 | 0.0341 | 0.1002 |
| 189 | 0 | 0 | 689 | 0.125 | -0.0313 | 1189 | -0.06 | -0.0287 | 1689 | 0.0156 | 0.0339 |
| 190 | -0.1191 | -0.0813 | 690 | 0.0359 | -0.0231 | 1190 | -0.0443 | 0.0334 | 1690 | 0.0896 | -0.1278 |
| 191 | 0.0575 | -0.1626 | 691 | -0.0444 | -0.0196 | 1191 | -0.0236 | -0.1059 | 1691 | 0.0002 | -0.0237 |
| 192 | 0.1374 | 0.0813 | 692 | -0.0866 | -0.0097 | 1192 | 0.0514 | -0.0693 | 1692 | -0.0855 | 0.097 |
| 193 | 0.0813 | 0 | 693 | -0.0879 | -0.0286 | 1193 | -0.0221 | 0.0625 | 1693 | 0.0138 | 0.0262 |
| 194 | 0.1374 | -0.0813 | 694 | 0.0151 | 0.0626 | 1194 | 0.0208 | 0.0794 | 1694 | -0.0635 | -0.0366 |
| 195 | 0.0575 | 0.1626 | 695 | -0.0644 | 0.0952 | 1195 | 0.0203 | 0.063 | 1695 | -0.0722 | 0.0319 |
| 196 | -0.1191 | 0.0813 | 696 | 0.0794 | 0.0573 | 1196 | -0.0092 | -0.0064 | 1696 | 0.1615 | -0.0119 |
| 197 | 0 | 0 | 697 | 0.1821 | -0.0092 | 1197 | -0.0017 | -0.1128 | 1697 | 0.1859 | -0.1105 |
| 198 | 0.1191 | -0.0813 | 698 | -0.0864 | -0.055 | 1198 | -0.0134 | 0.0834 | 1698 | -0.0709 | 0.0412 |
| 199 | -0.0575 | -0.1626 | 699 | 0.0256 | 0.1028 | 1199 | 0.1179 | 0.2091 | 1699 | -0.0671 | 0.0498 |
| 200 | -0.1374 | 0.0813 | 700 | 0.0226 | 0.0652 | 1200 | 0.0276 | -0.0576 | 1700 | 0.1431 | 0.033 |
| 201 | -0.0813 | 0 | 701 | -0.0449 | -0.116 | 1201 | 0 | 0.0442 | 1701 | 0.0175 | 0.082 |
| 202 | -0.1374 | -0.0813 | 702 | 0.0439 | 0.0014 | 1202 | -0.0429 | 0.0426 | 1702 | -0.0857 | -0.0497 |
| 203 | -0.0575 | 0.1626 | 703 | -0.1219 | 0.0546 | 1203 | -0.013 | -0.1395 | 1703 | 0.0132 | -0.0534 |
| 204 | 0.1191 | 0.0813 | 704 | -0.0334 | -0.0708 | 1204 | 0.002 | -0.0285 | 1704 | -0.0695 | -0.0286 |
| 205 | 0 | 0 | 705 | -0.0938 | -0.0938 | 1205 | -0.0159 | 0.0794 | 1705 | -0.0819 | 0.0377 |
| 206 | -0.1191 | -0.0813 | 706 | -0.161 | -0.0163 | 1206 | -0.1359 | -0.1164 | 1706 | 0.0108 | 0.0362 |
| 207 | 0.0575 | -0.1626 | 707 | 0.069 | -0.0489 | 1207 | -0.2065 | -0.0342 | 1707 | -0.0244 | -0.0771 |
| 208 | 0.1374 | 0.0813 | 708 | -0.0259 | -0.1291 | 1208 | 0.071 | -0.0711 | 1708 | -0.0246 | 0.0598 |
| 209 | 0.0813 | 0 | 709 | 0.0923 | 0.0598 | 1209 | 0.0975 | -0.0884 | 1709 | -0.0013 | 0.0978 |
| 210 | 0.1374 | -0.0813 | 710 | 0.1863 | 0.0736 | 1210 | -0.1331 | 0.0771 | 1710 | -0.0328 | 0.1158 |
| 211 | 0.0575 | 0.1626 | 711 | 0.0582 | -0.1068 | 1211 | -0.0323 | -0.0123 | 1711 | -0.0501 | 0.1188 |
| 212 | -0.1191 | 0.0813 | 712 | 0.1209 | 0.0543 | 1212 | 0.0644 | 0.0348 | 1712 | -0.022 | -0.0125 |
| 213 | 0 | 0 | 713 | -0.0625 | 0.0038 | 1213 | -0.0185 | 0.0096 | 1713 | 0.0129 | 0 |
| 214 | 0.1191 | -0.0813 | 714 | -0.1089 | -0.1864 | 1214 | -0.0369 | -0.0069 | 1714 | -0.0048 | -0.092 |
| 215 | -0.0575 | -0.1626 | 715 | 0.0573 | -0.0155 | 1215 | -0.0148 | 0.0971 | 1715 | -0.032 | -0.0437 |
| 216 | -0.1374 | 0.0813 | 716 | 0.0481 | 0.014 | 1216 | -0.0724 | -0.098 | 1716 | 0.0001 | 0.008 |
| 217 | -0.0813 | 0 | 717 | 0.0739 | 0.013 | 1217 | -0.1326 | -0.0442 | 1717 | 0.0458 | -0.0904 |
| 218 | -0.1374 | -0.0813 | 718 | 0.0347 | 0.114 | 1218 | -0.0656 | 0.1306 | 1718 | -0.0081 | 0.062 |
| 219 | -0.0575 | 0.1626 | 719 | -0.0525 | -0.0826 | 1219 | 0.0122 | 0.0866 | 1719 | -0.0368 | 0.0059 |
| 220 | 0.1191 | 0.0813 | 720 | -0.0258 | -0.1721 | 1220 | 0.095 | 0.1166 | 1720 | 0.0094 | -0.0942 |
| 221 | 0 | 0 | 721 | -0.125 | -0.0625 | 1221 | 0.0635 | 0.085 | 1721 | -0.0469 | -0.0469 |
| 222 | -0.1191 | -0.0813 | 722 | -0.0973 | -0.0148 | 1222 | -0.0917 | -0.1169 | 1722 | -0.0295 | -0.0235 |
| 223 | 0.0575 | -0.1626 | 723 | -0.0012 | -0.1213 | 1223 | -0.0346 | -0.2255 | 1723 | 0.0476 | 0.0932 |
| 224 | 0.1374 | 0.0813 | 724 | -0.0521 | -0.0377 | 1224 | 0.0464 | 0.0217 | 1724 | 0.0302 | 0.0284 |
| 225 | 0.0813 | 0 | 725 | 0.0925 | -0.0725 | 1225 | 0.0846 | 0.0701 | 1725 | 0.0616 | -0.0316 |
| 226 | 0.1374 | -0.0813 | 726 | 0.0805 | -0.0201 | 1226 | 0.1167 | -0.1172 | 1726 | 0.0323 | -0.0458 |
| 227 | 0.0575 | 0.1626 | 727 | 0.0505 | 0.1825 | 1227 | 0.0126 | -0.0209 | 1727 | 0.0202 | -0.0168 |
| 228 | -0.1191 | 0.0813 | 728 | 0.044 | 0.0901 | 1228 | 0.0848 | -0.0049 | 1728 | 0.0459 | -0.0284 |
| 229 | 0 | 0 | 729 | -0.056 | 0.069 | 1229 | 0.1378 | 0.0299 | 1729 | 0.035 | -0.1989 |
| 230 | 0.1191 | -0.0813 | 730 | 0.1138 | -0.0365 | 1230 | 0.0582 | 0.0903 | 1730 | 0.1877 | 0.049 |
| 231 | -0.0575 | -0.1626 | 731 | 0.0318 | -0.0235 | 1231 | 0.0183 | -0.0527 | 1731 | 0.0667 | 0.0722 |
| 232 | -0.1374 | 0.0813 | 732 | -0.1948 | 0.141 | 1232 | -0.1921 | -0.053 | 1732 | -0.1273 | -0.1041 |
| 233 | -0.0813 | 0 | 733 | 0.0403 | -0.0197 | 1233 | -0.1326 | 0.0442 | 1733 | 0.0892 | 0.1497 |
| 234 | -0.1374 | -0.0813 | 734 | 0.0709 | 0.0478 | 1234 | 0.0446 | 0.0318 | 1734 | 0.0285 | -0.0006 |
| 235 | -0.0575 | 0.1626 | 735 | -0.1022 | 0.0099 | 1235 | -0.065 | -0.07 | 1735 | -0.046 | -0.1433 |
| 236 | 0.1191 | 0.0813 | 736 | -0.0409 | -0.1372 | 1236 | 0.0263 | -0.0009 | 1736 | 0.0864 | -0.0295 |
| 237 | 0 | 0 | 737 | -0.0312 | 0.0938 | 1237 | 0.0418 | 0.0456 | 1737 | -0.0819 | -0.069 |
| 238 | -0.1191 | -0.0813 | 738 | -0.0316 | 0.1402 | 1238 | 0.0006 | -0.0325 | 1738 | -0.0189 | 0.0371 |
| 239 | 0.0575 | -0.1626 | 739 | 0.0377 | 0.1154 | 1239 | 0.0468 | 0.1148 | 1739 | 0.12 | -0.0366 |
| 240 | 0.1374 | 0.0813 | 740 | 0.0415 | 0.0577 | 1240 | -0.1431 | 0.0578 | 1740 | 0.0131 | -0.0852 |
| 241 | 0.0813 | 0 | 741 | 0.0566 | -0.0335 | 1241 | 0.035 | -0.0259 | 1741 | 0.0325 | 0.0401 |
| 242 | 0.1374 | -0.0813 | 742 | 0.1118 | -0.0427 | 1242 | 0.2033 | 0.0658 | 1742 | -0.0494 | -0.0708 |
| 243 | 0.0575 | 0.1626 | 743 | 0.0485 | -0.1134 | 1243 | -0.0936 | -0.0367 | 1743 | 0.032 | -0.0897 |
| 244 | -0.1191 | 0.0813 | 744 | -0.0546 | -0.0267 | 1244 | -0.0256 | -0.0429 | 1744 | 0.2692 | 0.0683 |
| 245 | 0 | 0 | 745 | 0.0156 | -0.0469 | 1245 | 0.1069 | -0.0721 | 1745 | 0.1013 | 0.0884 |
| 246 | 0.1191 | -0.0813 | 746 | 0.0543 | -0.0765 | 1246 | 0.0137 | -0.0576 | 1746 | -0.1275 | -0.0086 |
| 247 | -0.0575 | -0.1626 | 747 | 0.0451 | -0.0471 | 1247 | 0.0185 | 0.0195 | 1747 | -0.1993 | 0.0024 |
| 248 | -0.1374 | 0.0813 | 748 | 0.1338 | -0.0916 | 1248 | 0.0357 | -0.0471 | 1748 | -0.1107 | -0.0003 |
| 249 | -0.0813 | 0 | 749 | 0.1066 | 0.0945 | 1249 | 0.0884 | 0.0442 | 1749 | -0.0458 | -0.0087 |
| 250 | -0.1374 | -0.0813 | 750 | 0.0151 | 0.0869 | 1250 | 0.011 | 0.0208 | 1750 | -0.2114 | 0.1141 |
| 251 | -0.0575 | 0.1626 | 751 | -0.0301 | 0.0474 | 1251 | -0.0225 | -0.0173 | 1751 | -0.1255 | 0.11 |
| 252 | 0.1191 | 0.0813 | 752 | -0.0377 | 0.0928 | 1252 | 0.1121 | 0.0429 | 1752 | 0.0341 | 0.1002 |
| 253 | 0 | 0 | 753 | -0.0313 | -0.0938 | 1253 | 0.0874 | 0.0034 | 1753 | 0.0156 | 0.0339 |
| 254 | -0.1191 | -0.0813 | 754 | -0.1254 | -0.0276 | 1254 | 0.1166 | 0.04 | 1754 | 0.0896 | -0.1278 |
| 255 | 0.0575 | -0.1626 | 755 | -0.0286 | 0.0982 | 1255 | 0.1059 | -0.0684 | 1755 | 0.0002 | -0.0237 |
| 256 | 0.1374 | 0.0813 | 756 | 0.1427 | 0.0911 | 1256 | -0.0463 | -0.0032 | 1756 | -0.0855 | 0.097 |
| 257 | 0.0813 | 0 | 757 | -0.03 | 0.0359 | 1257 | -0.0404 | 0.1326 | 1757 | 0.0138 | 0.0262 |
| 258 | 0.1374 | -0.0813 | 758 | -0.087 | -0.0243 | 1258 | -0.0089 | -0.1265 | 1758 | -0.0635 | -0.0366 |
| 259 | 0.0575 | 0.1626 | 759 | 0.0714 | 0.0577 | 1259 | 0.0248 | -0.1435 | 1759 | -0.0722 | 0.0319 |
| 260 | -0.1191 | 0.0813 | 760 | -0.0012 | -0.0129 | 1260 | 0.016 | 0.0079 | 1760 | 0.1615 | -0.0119 |
| 261 | 0 | 0 | 761 | -0.1002 | 0.0248 | 1261 | -0.0494 | -0.004 | 1761 | -0.0792 | 0.0221 |
| 262 | 0.1191 | -0.0813 | 762 | -0.0033 | 0.1083 | 1262 | -0.0496 | 0.0749 | 1762 | -0.0747 | -0.0249 |
| 263 | -0.0575 | -0.1626 | 763 | 0.0845 | 0.0176 | 1263 | -0.1103 | 0.1495 | 1763 | -0.097 | 0.005 |
| 264 | -0.1374 | 0.0813 | 764 | 0.1243 | 0.0573 | 1264 | -0.0742 | 0.068 | 1764 | -0.0252 | 0.0435 |
| 265 | -0.0813 | 0 | 765 | 0.1289 | 0.0014 | 1265 | 0 | 0.0442 | 1765 | 0.0543 | 0.0673 |
| 266 | -0.1374 | -0.0813 | 766 | -0.0426 | -0.0249 | 1266 | -0.0429 | 0.0426 | 1766 | 0.1257 | -0.053 |
| 267 | -0.0575 | 0.1626 | 767 | -0.0921 | -0.0246 | 1267 | -0.013 | -0.1395 | 1767 | 0.1154 | 0.0111 |
| 268 | 0.1191 | 0.0813 | 768 | 0.0478 | -0.0988 | 1268 | 0.002 | -0.0285 | 1768 | -0.0188 | -0.0811 |
| 269 | 0 | 0 | 769 | -0.0625 | -0.0625 | 1269 | -0.0159 | 0.0794 | 1769 | 0.035 | -0.1821 |
| 270 | -0.1191 | -0.0813 | 770 | -0.099 | -0.064 | 1270 | -0.1359 | -0.1164 | 1770 | 0.032 | -0.0642 |
| 271 | 0.0575 | -0.1626 | 771 | -0.0338 | 0.0586 | 1271 | -0.2065 | -0.0342 | 1771 | -0.0357 | -0.0469 |
| 272 | 0.1374 | 0.0813 | 772 | -0.1659 | 0.011 | 1272 | 0.071 | -0.0711 | 1772 | 0.051 | 0.1044 |
| 273 | 0.0813 | 0 | 773 | 0.0059 | -0.1799 | 1273 | 0.0975 | -0.0884 | 1773 | -0.0197 | -0.0308 |
| 274 | 0.1374 | -0.0813 | 774 | 0.1605 | 0.0055 | 1274 | -0.1331 | 0.0771 | 1774 | -0.0962 | -0.2398 |
| 275 | 0.0575 | 0.1626 | 775 | -0.0561 | 0.0607 | 1275 | -0.0323 | -0.0123 | 1775 | -0.0639 | 0.0173 |
| 276 | -0.1191 | 0.0813 | 776 | -0.0699 | -0.0572 | 1276 | 0.0644 | 0.0348 | 1776 | -0.073 | 0.0773 |
| 277 | 0 | 0 | 777 | 0.0156 | -0.0469 | 1277 | -0.0185 | 0.0096 | 1777 | -0.0792 | -0.0221 |
| 278 | 0.1191 | -0.0813 | 778 | 0.0115 | -0.0291 | 1278 | -0.0369 | -0.0069 | 1778 | -0.0001 | 0.0257 |
| 279 | -0.0575 | -0.1626 | 779 | -0.0105 | 0.0271 | 1279 | -0.0148 | 0.0971 | 1779 | 0.1836 | 0.0914 |
| 280 | -0.1374 | 0.0813 | 780 | -0.0295 | -0.0521 | 1280 | -0.0724 | -0.098 | 1780 | 0.0123 | 0.0458 |
| 281 | -0.0813 | 0 | 781 | -0.0258 | -0.2012 | 1281 | 0.0625 | 0.0221 | 1781 | -0.1672 | -0.0501 |
| 282 | -0.1374 | -0.0813 | 782 | -0.1324 | -0.0281 | 1282 | 0.0933 | -0.1316 | 1782 | 0.1101 | -0.0543 |
| 283 | -0.0575 | 0.1626 | 783 | -0.0149 | 0.1548 | 1283 | 0.1292 | -0.1184 | 1783 | 0.0551 | -0.0644 |
| 284 | 0.1191 | 0.0813 | 784 | 0.1125 | -0.0268 | 1284 | 0.0373 | 0.1174 | 1784 | -0.0453 | 0.0428 |
| 285 | 0 | 0 | 785 | -0.125 | -0.0625 | 1285 | 0.0033 | 0.0306 | 1785 | 0.1379 | 0.0975 |
| 286 | -0.1191 | -0.0813 | 786 | -0.0973 | -0.0148 | 1286 | -0.0811 | 0.0904 | 1786 | 0.0228 | 0.057 |
| 287 | 0.0575 | -0.1626 | 787 | -0.0012 | -0.1213 | 1287 | 0.0266 | 0.1431 | 1787 | -0.0546 | 0.0019 |
| 288 | 0.1374 | 0.0813 | 788 | -0.0521 | -0.0377 | 1288 | 0.0502 | -0.0151 | 1788 | -0.0622 | -0.1735 |
| 289 | -0.0813 | 0 | 789 | 0.0925 | -0.0725 | 1289 | -0.1574 | -0.0598 | 1789 | -0.0628 | -0.0714 |
| 290 | -0.1374 | 0.0813 | 790 | 0.0805 | -0.0201 | 1290 | 0.0205 | -0.1286 | 1790 | 0.0502 | 0.0021 |
| 291 | -0.0575 | -0.1626 | 791 | 0.0505 | 0.1825 | 1291 | 0.0698 | -0.0074 | 1791 | -0.0636 | -0.0812 |
| 292 | 0.1191 | -0.0813 | 792 | 0.044 | 0.0901 | 1292 | 0.0204 | 0.0433 | 1792 | -0.0489 | 0.0245 |
| 293 | 0 | 0 | 793 | -0.056 | 0.069 | 1293 | 0.0826 | -0.0352 | 1793 | 0.0533 | 0.0221 |
| 294 | -0.1191 | 0.0813 | 794 | 0.1138 | -0.0365 | 1294 | -0.0935 | -0.0238 | 1794 | -0.0251 | 0.0001 |
| 295 | 0.0575 | 0.1626 | 795 | 0.0318 | -0.0235 | 1295 | -0.0093 | -0.1117 | 1795 | -0.0225 | -0.0685 |
| 296 | 0.1374 | -0.0813 | 796 | -0.1948 | 0.141 | 1296 | 0.0804 | 0.0315 | 1796 | -0.0761 | -0.0587 |
| 297 | 0.0813 | 0 | 797 | 0.0403 | -0.0197 | 1297 | -0.0663 | 0.0442 | 1797 | 0.0135 | 0.0265 |
| 298 | 0.1374 | 0.0813 | 798 | 0.0709 | 0.0478 | 1298 | -0.0141 | -0.1006 | 1798 | 0.0718 | -0.1778 |
| 299 | 0.0575 | -0.1626 | 799 | -0.1022 | 0.0099 | 1299 | 0.0249 | 0.0322 | 1799 | -0.075 | -0.1425 |
| 300 | -0.1191 | -0.0813 | 800 | -0.0409 | -0.1372 | 1300 | 0.0215 | -0.0105 | 1800 | 0.1065 | 0.0271 |
| 301 | 0 | 0 | 801 | -0.1563 | -0.0625 | 1301 | 0.0833 | 0.0425 | 1801 | 0.1288 | 0.0183 |
| 302 | 0.1191 | 0.0813 | 802 | -0.0243 | -0.0548 | 1302 | 0.0662 | 0.1177 | 1802 | -0.0132 | 0.05 |
| 303 | -0.0575 | 0.1626 | 803 | 0.0629 | -0.1484 | 1303 | -0.0585 | -0.0619 | 1803 | 0.056 | 0.0172 |
| 304 | -0.1374 | -0.0813 | 804 | 0.1375 | 0.013 | 1304 | -0.0654 | 0.0298 | 1804 | -0.0872 | -0.0204 |
| 305 | -0.0813 | 0 | 805 | 0.0522 | 0.1305 | 1305 | 0.069 | 0.0781 | 1805 | 0.0014 | -0.0576 |
| 306 | -0.1374 | 0.0813 | 806 | -0.0414 | -0.045 | 1306 | 0.0687 | -0.0012 | 1806 | 0.1688 | 0.0318 |
| 307 | -0.0575 | -0.1626 | 807 | -0.06 | -0.0081 | 1307 | 0.007 | 0.0521 | 1807 | -0.0784 | 0.0679 |
| 308 | 0.1191 | -0.0813 | 808 | -0.0566 | 0.0224 | 1308 | -0.0648 | -0.0704 | 1808 | -0.0776 | -0.0205 |
| 309 | 0 | 0 | 809 | 0.0598 | 0.0598 | 1309 | -0.0744 | -0.1195 | 1809 | 0.035 | 0.1105 |
| 310 | -0.1191 | 0.0813 | 810 | 0.0155 | -0.0049 | 1310 | 0.0074 | 0.0233 | 1810 | 0.0339 | 0.0799 |
| 311 | 0.0575 | 0.1626 | 811 | -0.0226 | -0.0673 | 1311 | 0.0403 | -0.0202 | 1811 | 0.1051 | 0.0423 |
| 312 | 0.1374 | -0.0813 | 812 | 0.015 | 0.0565 | 1312 | 0.1407 | 0.0104 | 1812 | -0.0707 | 0.1072 |
| 313 | 0.0813 | 0 | 813 | -0.0467 | -0.0839 | 1313 | 0.1067 | 0.1547 | 1813 | -0.1841 | 0.0006 |
| 314 | 0.1374 | 0.0813 | 814 | 0.1573 | -0.059 | 1314 | -0.0732 | 0.0226 | 1814 | 0.003 | 0.0275 |
| 315 | 0.0575 | -0.1626 | 815 | 0.1444 | 0.0127 | 1315 | -0.0236 | -0.0788 | 1815 | -0.0147 | 0.0007 |
| 316 | -0.1191 | -0.0813 | 816 | -0.1217 | -0.14 | 1316 | 0.0675 | -0.0157 | 1816 | -0.1272 | -0.0454 |
| 317 | 0 | 0 | 817 | 0.0313 | -0.0625 | 1317 | -0.0475 | 0.0136 | 1817 | -0.0183 | 0.0221 |
| 318 | 0.1191 | 0.0813 | 818 | -0.0006 | 0.025 | 1318 | -0.1519 | 0.0065 | 1818 | -0.012 | 0.038 |
| 319 | -0.0575 | 0.1626 | 819 | -0.1219 | 0.082 | 1319 | -0.0584 | -0.0901 | 1819 | -0.0464 | 0.0979 |
| 320 | -0.1374 | -0.0813 | 820 | 0.0495 | 0.0813 | 1320 | -0.0183 | -0.06 | 1820 | 0.1697 | 0.092 |
| 321 | 0.1563 | 0 | 821 | 0.0031 | 0.098 | 1321 | -0.1261 | -0.0286 | 1821 | 0.1512 | 0.0272 |
| 322 | -0.0402 | 0.0646 | 822 | -0.0026 | 0.1524 | 1322 | -0.1166 | -0.0765 | 1822 | -0.0432 | -0.0517 |
| 323 | -0.0705 | 0.0541 | 823 | 0.0457 | 0.029 | 1323 | 0.0425 | -0.0067 | 1823 | 0.0368 | 0.0509 |
| 324 | 0.1074 | -0.0685 | 824 | -0.0298 | 0.0372 | 1324 | 0.0217 | -0.0318 | 1824 | 0.0191 | 0.1887 |
| 325 | 0.0014 | 0.0557 | 825 | 0.0507 | 0.0065 | 1325 | -0.1892 | 0.0535 | 1825 | -0.0792 | 0.0221 |
| 326 | -0.1219 | 0.057 | 826 | 0.0509 | 0.0136 | 1326 | -0.0713 | 0.1676 | 1826 | -0.0747 | -0.0249 |
| 327 | -0.1488 | -0.0466 | 827 | -0.0303 | 0.0947 | 1327 | 0.0478 | 0.1264 | 1827 | -0.097 | 0.005 |
| 328 | -0.097 | 0.0093 | 828 | -0.0486 | -0.0557 | 1328 | -0.0464 | 0.0834 | 1828 | -0.0252 | 0.0435 |
| 329 | 0.0129 | 0.0792 | 829 | 0.0469 | -0.0075 | 1329 | 0.0221 | -0.1326 | 1829 | 0.0543 | 0.0673 |
| 330 | 0.0359 | 0.1521 | 830 | 0.2658 | 0.0401 | 1330 | -0.1423 | -0.0403 | 1830 | 0.1257 | -0.053 |
| 331 | 0.0527 | 0.0848 | 831 | 0.1314 | 0.048 | 1331 | -0.1304 | 0.165 | 1831 | 0.1154 | 0.0111 |
| 332 | 0.1105 | 0.0535 | 832 | -0.0975 | 0.1723 | 1332 | 0.1453 | -0.0236 | 1832 | -0.0188 | -0.0811 |
| 333 | 0.0659 | -0.0284 | 833 | 0 | 0.0313 | 1333 | -0.0391 | 0.0017 | 1833 | 0.035 | -0.1821 |
| 334 | -0.0789 | -0.0645 | 834 | -0.0119 | -0.0407 | 1334 | -0.0371 | 0.0354 | 1834 | 0.032 | -0.0642 |
| 335 | -0.0423 | 0.1205 | 835 | -0.0975 | 0.0535 | 1335 | 0.1421 | -0.1161 | 1835 | -0.0357 | -0.0469 |
| 336 | 0.0432 | -0.0167 | 836 | 0.0099 | 0.0725 | 1336 | 0.0065 | -0.1181 | 1836 | 0.051 | 0.1044 |
| 337 | 0.0313 | -0.1563 | 837 | 0.117 | -0.0497 | 1337 | -0.0873 | -0.0781 | 1837 | -0.0197 | -0.0308 |
| 338 | 0.1375 | -0.0603 | 838 | 0.0517 | -0.1471 | 1338 | -0.0132 | -0.0437 | 1838 | -0.0962 | -0.2398 |
| 339 | 0.0578 | 0.0368 | 839 | -0.0718 | 0.0177 | 1339 | 0.0575 | -0.0381 | 1839 | -0.0639 | 0.0173 |
| 340 | 0.0002 | 0.145 | 840 | -0.0687 | 0.0308 | 1340 | -0.072 | -0.0088 | 1840 | -0.073 | 0.0773 |
| 341 | 0.1475 | 0.0524 | 841 | -0.0286 | -0.0286 | 1341 | 0.0043 | 0.0129 | 1841 | 0.0129 | -0.0975 |
| 342 | -0.0582 | -0.0479 | 842 | 0.0133 | 0.0133 | 1342 | 0.1845 | 0.0829 | 1842 | -0.0112 | -0.1012 |
| 343 | -0.0778 | -0.0422 | 843 | 0.0524 | -0.1118 | 1343 | 0.0462 | 0.1305 | 1843 | 0.0127 | 0.0563 |
| 344 | 0.1014 | 0.0803 | 844 | 0.0148 | -0.1294 | 1344 | 0.0291 | 0.0381 | 1844 | 0.0886 | 0.0334 |
| 345 | -0.0754 | 0.1417 | 845 | -0.0158 | -0.067 | 1345 | 0.0625 | 0.0221 | 1845 | -0.1026 | 0.0246 |
| 346 | -0.0674 | 0.0259 | 846 | -0.0624 | -0.1531 | 1346 | 0.0933 | -0.1316 | 1846 | -0.1542 | 0.0135 |
| 347 | -0.0396 | 0.1282 | 847 | -0.0895 | -0.0609 | 1347 | 0.1292 | -0.1184 | 1847 | 0.0728 | -0.035 |
| 348 | -0.0666 | 0.044 | 848 | -0.0599 | 0.1006 | 1348 | 0.0373 | 0.1174 | 1848 | 0.0957 | -0.0159 |
| 349 | 0.0352 | -0.1135 | 849 | 0 | -0.0312 | 1349 | 0.0033 | 0.0306 | 1849 | 0.0038 | 0.0092 |
| 350 | -0.0693 | 0.0526 | 850 | 0.1073 | 0.0367 | 1350 | -0.0811 | 0.0904 | 1850 | 0.0721 | 0.0639 |
| 351 | 0.0184 | 0.0222 | 851 | 0.0056 | 0.1638 | 1351 | 0.0266 | 0.1431 | 1851 | 0.0563 | 0.0834 |
| 352 | 0.0634 | -0.0674 | 852 | -0.1113 | -0.058 | 1352 | 0.0502 | -0.0151 | 1852 | 0.0085 | 0.084 |
| 353 | -0.0938 | 0 | 853 | 0.0777 | 0.0712 | 1353 | -0.1574 | -0.0598 | 1853 | 0.1643 | 0.0425 |
| 354 | 0.0634 | 0.0674 | 854 | 0.124 | 0.1214 | 1354 | 0.0205 | -0.1286 | 1854 | 0.005 | 0.0138 |
| 355 | 0.0184 | -0.0222 | 855 | -0.0281 | -0.1011 | 1355 | 0.0698 | -0.0074 | 1855 | -0.2221 | 0.0627 |
| 356 | -0.0693 | -0.0526 | 856 | -0.065 | 0.012 | 1356 | 0.0204 | 0.0433 | 1856 | -0.0639 | 0.0822 |
| 357 | 0.0352 | 0.1135 | 857 | -0.0819 | -0.0377 | 1357 | 0.0826 | -0.0352 | 1857 | -0.0442 | 0.0663 |
| 358 | -0.0666 | -0.044 | 858 | -0.0769 | 0.0118 | 1358 | -0.0935 | -0.0238 | 1858 | -0.0452 | -0.0379 |
| 359 | -0.0396 | -0.1282 | 859 | 0.0263 | 0.0585 | 1359 | -0.0093 | -0.1117 | 1859 | -0.0419 | -0.051 |
| 360 | -0.0674 | -0.0259 | 860 | 0.0368 | -0.157 | 1360 | 0.0804 | 0.0315 | 1860 | -0.0492 | -0.0248 |
| 361 | -0.0754 | -0.1417 | 861 | -0.1094 | 0.0334 | 1361 | 0.208 | -0.1547 | 1861 | 0.0998 | -0.1128 |
| 362 | 0.1014 | -0.0803 | 862 | -0.0656 | 0.0903 | 1362 | 0.0522 | -0.0784 | 1862 | 0.005 | -0.0851 |
| 363 | -0.0778 | 0.0422 | 863 | 0.053 | -0.0623 | 1363 | 0.0339 | 0.0827 | 1863 | -0.091 | 0.0164 |
| 364 | -0.0582 | 0.0479 | 864 | -0.1044 | -0.0586 | 1364 | 0.0801 | -0.0392 | 1864 | -0.1229 | 0.1298 |
| 365 | 0.1475 | -0.0524 | 865 | -0.1563 | -0.0625 | 1365 | -0.1404 | -0.0533 | 1865 | -0.0938 | 0 |
| 366 | 0.0002 | -0.145 | 866 | -0.0243 | -0.0548 | 1366 | -0.0552 | 0.1085 | 1866 | 0.0929 | -0.1555 |
| 367 | 0.0578 | -0.0368 | 867 | 0.0629 | -0.1484 | 1367 | 0.0296 | 0.1149 | 1867 | 0.0033 | 0.0215 |
| 368 | 0.1375 | 0.0603 | 868 | 0.1375 | 0.013 | 1368 | 0.0178 | -0.0208 | 1868 | 0.0371 | -0.0155 |
| 369 | 0.0313 | 0.1563 | 869 | 0.0522 | 0.1305 | 1369 | 0.0625 | -0.0754 | 1869 | 0.0438 | -0.0904 |
| 370 | 0.0432 | 0.0167 | 870 | -0.0414 | -0.045 | 1370 | 0.0731 | -0.0059 | 1870 | -0.1889 | -0.1286 |
| 371 | -0.0423 | -0.1205 | 871 | -0.06 | -0.0081 | 1371 | 0.1624 | 0.0976 | 1871 | -0.0481 | -0.1242 |
| 372 | -0.0789 | 0.0645 | 872 | -0.0566 | 0.0224 | 1372 | 0.1678 | 0.125 | 1872 | -0.0216 | 0.1548 |
| 373 | 0.0659 | 0.0284 | 873 | 0.0598 | 0.0598 | 1373 | 0.0153 | 0.1493 | 1873 | -0.1638 | 0.0092 |
| 374 | 0.1105 | -0.0535 | 874 | 0.0155 | -0.0049 | 1374 | -0.0583 | 0.0339 | 1874 | -0.0671 | -0.0443 |
| 375 | 0.0527 | -0.0848 | 875 | -0.0226 | -0.0673 | 1375 | 0.0935 | -0.1232 | 1875 | -0.0042 | 0.1364 |
| 376 | 0.0359 | -0.1521 | 876 | 0.015 | 0.0565 | 1376 | 0.0913 | 0.1282 | 1876 | -0.0341 | -0.1034 |
| 377 | 0.0129 | -0.0792 | 877 | -0.0467 | -0.0839 | 1377 | -0.0313 | 0.1105 | 1877 | -0.0041 | -0.0063 |
| 378 | -0.097 | -0.0093 | 878 | 0.1573 | -0.059 | 1378 | 0.062 | -0.2049 | 1878 | 0.1315 | 0.1624 |
| 379 | -0.1488 | 0.0466 | 879 | 0.1444 | 0.0127 | 1379 | -0.0095 | -0.1226 | 1879 | 0.0657 | 0.0559 |
| 380 | -0.1219 | -0.057 | 880 | -0.1217 | -0.14 | 1380 | -0.0861 | -0.0978 | 1880 | -0.0107 | 0.0374 |
| 381 | 0.0014 | -0.0557 | 881 | -0.1563 | 0.0313 | 1381 | -0.0502 | -0.0533 | 1881 | 0.0533 | -0.0221 |
| 382 | 0.1074 | 0.0685 | 882 | -0.0992 | 0.0638 | 1382 | -0.0712 | -0.0047 | 1882 | -0.0325 | 0.0492 |
| 383 | -0.0705 | -0.0541 | 883 | 0.0293 | -0.0653 | 1383 | 0.04 | -0.0456 | 1883 | 0.01 | 0.0177 |
| 384 | -0.0402 | -0.0646 | 884 | -0.1525 | -0.0939 | 1384 | -0.0037 | 0.0606 | 1884 | 0.0476 | -0.0003 |
| 385 | 0.1563 | 0 | 885 | -0.0475 | 0.0449 | 1385 | -0.0442 | -0.1509 | 1885 | 0.0179 | 0.1213 |
| 386 | -0.0402 | 0.0646 | 886 | 0.0522 | -0.0123 | 1386 | 0.0466 | -0.1163 | 1886 | -0.002 | -0.0485 |
| 387 | -0.0705 | 0.0541 | 887 | 0.048 | -0.0827 | 1387 | -0.0297 | 0.1602 | 1887 | -0.077 | -0.0946 |
| 388 | 0.1074 | -0.0685 | 888 | 0.0962 | 0.0415 | 1388 | -0.0166 | 0.0282 | 1888 | 0.0931 | -0.0059 |
| 389 | 0.0014 | 0.0557 | 889 | 0.0103 | 0.0598 | 1389 | 0.012 | 0.0968 | 1889 | 0 | -0.0221 |
| 390 | -0.1219 | 0.057 | 890 | 0.0876 | -0.0116 | 1390 | -0.0333 | 0.0787 | 1890 | -0.1465 | 0.003 |
| 391 | -0.1488 | -0.0466 | 891 | 0.0405 | -0.0867 | 1391 | -0.0274 | -0.0568 | 1891 | 0.1659 | -0.0351 |
| 392 | -0.097 | 0.0093 | 892 | 0.0358 | -0.1026 | 1392 | -0.0214 | 0.0744 | 1892 | 0.1638 | -0.039 |
| 393 | 0.0129 | 0.0792 | 893 | 0.2111 | 0.0231 | 1393 | 0.0313 | -0.0221 | 1893 | 0.0252 | -0.0381 |
| 394 | 0.0359 | 0.1521 | 894 | 0.0588 | 0.0003 | 1394 | -0.0462 | -0.121 | 1894 | 0.0152 | -0.0195 |
| 395 | 0.0527 | 0.0848 | 895 | -0.075 | -0.1682 | 1395 | -0.1645 | 0.0248 | 1895 | -0.0551 | 0.106 |
| 396 | 0.1105 | 0.0535 | 896 | -0.0357 | -0.1576 | 1396 | -0.0229 | 0.097 | 1896 | 0.0455 | 0.0917 |
| 397 | 0.0659 | -0.0284 | 897 | 0.0313 | 0.0938 | 1397 | 0.1145 | -0.0533 | 1897 | 0.0183 | -0.0313 |
| 398 | -0.0789 | -0.0645 | 898 | 0.0241 | 0.1875 | 1398 | 0.0255 | -0.0589 | 1898 | -0.0758 | -0.0424 |
| 399 | -0.0423 | 0.1205 | 899 | 0.0069 | -0.0611 | 1399 | -0.077 | 0.089 | 1899 | 0.063 | -0.016 |
| 400 | 0.0432 | -0.0167 | 900 | 0.1349 | -0.0622 | 1400 | -0.1231 | -0.05 | 1900 | 0.076 | 0.029 |
| 401 | 0.0313 | -0.1563 | 901 | 0.0833 | 0.0296 | 1401 | -0.0625 | -0.0129 | 1901 | -0.0309 | 0.1476 |
| 402 | 0.1375 | -0.0603 | 902 | 0.0224 | -0.1022 | 1402 | 0.0851 | 0.067 | 1902 | 0.0481 | 0.0036 |
| 403 | 0.0578 | 0.0368 | 903 | 0.0118 | 0.0141 | 1403 | 0.0291 | -0.0689 | 1903 | 0.0896 | -0.2006 |
| 404 | 0.0002 | 0.145 | 904 | -0.0988 | 0.0541 | 1404 | 0.017 | 0.043 | 1904 | -0.0001 | -0.0838 |
| 405 | 0.1475 | 0.0524 | 905 | -0.0065 | -0.0377 | 1405 | 0.0731 | 0.0198 | 1905 | 0.0129 | -0.0975 |
| 406 | -0.0582 | -0.0479 | 906 | 0.0906 | 0.0281 | 1406 | -0.0315 | -0.0618 | 1906 | -0.0112 | -0.1012 |
| 407 | -0.0778 | -0.0422 | 907 | -0.011 | -0.0556 | 1407 | 0.0084 | 0.0754 | 1907 | 0.0127 | 0.0563 |
| 408 | 0.1014 | 0.0803 | 908 | -0.0718 | 0.0827 | 1408 | -0.0462 | 0.0597 | 1908 | 0.0886 | 0.0334 |
| 409 | -0.0754 | 0.1417 | 909 | -0.0047 | 0.1984 | 1409 | -0.1196 | -0.0221 | 1909 | -0.1026 | 0.0246 |
| 410 | -0.0674 | 0.0259 | 910 | -0.0329 | -0.0728 | 1410 | -0.0055 | 0.0129 | 1910 | -0.1542 | 0.0135 |
| 411 | -0.0396 | 0.1282 | 911 | -0.1305 | 0.0152 | 1411 | -0.0733 | 0.0669 | 1911 | 0.0728 | -0.035 |
| 412 | -0.0666 | 0.044 | 912 | -0.092 | 0.1101 | 1412 | -0.0719 | -0.0496 | 1912 | 0.0957 | -0.0159 |
| 413 | 0.0352 | -0.1135 | 913 | -0.0625 | -0.0625 | 1413 | -0.0123 | -0.0533 | 1913 | 0.0038 | 0.0092 |
| 414 | -0.0693 | 0.0526 | 914 | -0.0469 | -0.0431 | 1414 | 0.0384 | 0.102 | 1914 | 0.0721 | 0.0639 |
| 415 | 0.0184 | 0.0222 | 915 | -0.0197 | -0.1031 | 1415 | 0.0957 | 0.0185 | 1915 | 0.0563 | 0.0834 |
| 416 | 0.0634 | -0.0674 | 916 | 0.034 | -0.1078 | 1416 | -0.021 | -0.022 | 1916 | 0.0085 | 0.084 |
| 417 | -0.0938 | 0 | 917 | 0.0033 | 0.0618 | 1417 | -0.0442 | -0.0259 | 1917 | 0.1643 | 0.0425 |
| 418 | 0.0634 | 0.0674 | 918 | -0.043 | 0.1069 | 1418 | -0.1424 | -0.1053 | 1918 | 0.005 | 0.0138 |
| 419 | 0.0184 | -0.0222 | 919 | 0.1088 | 0.0688 | 1419 | -0.1252 | -0.0639 | 1919 | -0.2221 | 0.0627 |
| 420 | -0.0693 | -0.0526 | 920 | -0.0591 | -0.0412 | 1420 | 0.0576 | -0.0549 | 1920 | -0.0639 | 0.0822 |
| 421 | 0.0352 | 0.1135 | 921 | -0.1665 | -0.0286 | 1421 | -0.012 | -0.0526 | 1921 | 0.0625 | 0 |
| 422 | -0.0666 | -0.044 | 922 | 0.0144 | 0.0783 | 1422 | 0.0606 | 0.0005 | 1922 | -0.0042 | 0.0198 |
| 423 | -0.0396 | -0.1282 | 923 | -0.0548 | 0.0577 | 1423 | 0.0139 | 0.1047 | 1923 | -0.0603 | -0.1183 |
| 424 | -0.0674 | -0.0259 | 924 | 0.1151 | 0.085 | 1424 | -0.0185 | 0.0716 | 1924 | -0.0763 | -0.0257 |
| 425 | -0.0754 | -0.1417 | 925 | 0.0647 | 0.1019 | 1425 | 0.208 | -0.1547 | 1925 | -0.1164 | 0.0002 |
| 426 | 0.1014 | -0.0803 | 926 | -0.1762 | 0.0044 | 1426 | 0.0522 | -0.0784 | 1926 | -0.0869 | -0.0093 |
| 427 | -0.0778 | 0.0422 | 927 | 0.0722 | -0.0874 | 1427 | 0.0339 | 0.0827 | 1927 | -0.0893 | -0.0689 |
| 428 | -0.0582 | 0.0479 | 928 | 0.0379 | -0.0296 | 1428 | 0.0801 | -0.0392 | 1928 | 0.0516 | -0.0319 |
| 429 | 0.1475 | -0.0524 | 929 | -0.0625 | 0.0625 | 1429 | -0.1404 | -0.0533 | 1929 | 0.0911 | 0.0911 |
| 430 | 0.0002 | -0.145 | 930 | -0.0272 | -0.0382 | 1430 | -0.0552 | 0.1085 | 1930 | 0.0023 | -0.0734 |
| 431 | 0.0578 | -0.0368 | 931 | -0.079 | 0.042 | 1431 | 0.0296 | 0.1149 | 1931 | 0.1004 | -0.103 |
| 432 | 0.1375 | 0.0603 | 932 | -0.0097 | 0.1823 | 1432 | 0.0178 | -0.0208 | 1932 | 0.0277 | 0.0604 |
| 433 | 0.0313 | 0.1563 | 933 | -0.0391 | -0.0113 | 1433 | 0.0625 | -0.0754 | 1933 | 0.1101 | 0.0971 |
| 434 | 0.0432 | 0.0167 | 934 | 0.0979 | -0.0469 | 1434 | 0.0731 | -0.0059 | 1934 | 0.1305 | -0.0019 |
| 435 | -0.0423 | -0.1205 | 935 | 0.1591 | 0.0256 | 1435 | 0.1624 | 0.0976 | 1935 | -0.1113 | 0.0213 |
| 436 | -0.0789 | 0.0645 | 936 | 0.0072 | -0.0205 | 1436 | 0.1678 | 0.125 | 1936 | 0.0571 | 0.1835 |
| 437 | 0.0659 | 0.0284 | 937 | 0.0377 | 0.0065 | 1437 | 0.0153 | 0.1493 | 1937 | -0.0221 | -0.0221 |
| 438 | 0.1105 | -0.0535 | 938 | -0.0434 | -0.088 | 1438 | -0.0583 | 0.0339 | 1938 | -0.1445 | -0.1275 |
| 439 | 0.0527 | -0.0848 | 939 | -0.0371 | -0.1029 | 1439 | 0.0935 | -0.1232 | 1939 | 0.1129 | -0.0286 |
| 440 | 0.0359 | -0.1521 | 940 | 0.091 | 0.0166 | 1440 | 0.0913 | 0.1282 | 1940 | 0.0598 | -0.1039 |
| 441 | 0.0129 | -0.0792 | 941 | 0.1038 | -0.0734 | 1441 | 0.0442 | -0.0092 | 1941 | -0.0144 | 0.0625 |
| 442 | -0.097 | -0.0093 | 942 | 0.0208 | -0.0542 | 1442 | -0.0081 | 0.03 | 1942 | -0.0939 | 0.0953 |
| 443 | -0.1488 | 0.0466 | 943 | -0.0694 | 0.0896 | 1443 | -0.0309 | -0.0338 | 1943 | -0.0621 | -0.0626 |
| 444 | -0.1219 | -0.057 | 944 | -0.0325 | 0.0433 | 1444 | -0.036 | 0.0494 | 1944 | 0.1351 | -0.0694 |
| 445 | 0.0014 | -0.0557 | 945 | -0.1563 | 0.0313 | 1445 | 0.0184 | 0.0439 | 1945 | -0.0027 | -0.1094 |
| 446 | 0.1074 | 0.0685 | 946 | -0.0992 | 0.0638 | 1446 | 0.1178 | -0.0108 | 1946 | -0.0039 | 0.0135 |
| 447 | -0.0705 | -0.0541 | 947 | 0.0293 | -0.0653 | 1447 | 0.012 | 0.1872 | 1947 | 0.0354 | 0.0939 |
| 448 | -0.0402 | -0.0646 | 948 | -0.1525 | -0.0939 | 1448 | -0.073 | 0.0811 | 1948 | -0.0674 | -0.0196 |
| 449 | 0.1563 | 0 | 949 | -0.0475 | 0.0449 | 1449 | -0.0377 | -0.0598 | 1949 | 0.0274 | -0.0038 |
| 450 | -0.0402 | 0.0646 | 950 | 0.0522 | -0.0123 | 1450 | 0.0236 | 0.0885 | 1950 | -0.0249 | 0.1956 |
| 451 | -0.0705 | 0.0541 | 951 | 0.048 | -0.0827 | 1451 | 0.0264 | 0.0399 | 1951 | -0.1516 | 0.2089 |
| 452 | 0.1074 | -0.0685 | 952 | 0.0962 | 0.0415 | 1452 | -0.027 | -0.0259 | 1952 | -0.0437 | -0.0292 |
| 453 | 0.0014 | 0.0557 | 953 | 0.0103 | 0.0598 | 1453 | 0.0482 | -0.0124 | 1953 | 0.1067 | 0.0442 |
| 454 | -0.1219 | 0.057 | 954 | 0.0876 | -0.0116 | 1454 | 0.0006 | -0.0509 | 1954 | 0.045 | 0.0015 |
| 455 | -0.1488 | -0.0466 | 955 | 0.0405 | -0.0867 | 1455 | -0.0328 | -0.0642 | 1955 | -0.0408 | -0.1441 |
| 456 | -0.097 | 0.0093 | 956 | 0.0358 | -0.1026 | 1456 | 0.0838 | -0.0336 | 1956 | 0.0191 | 0.1359 |
| 457 | 0.0129 | 0.0792 | 957 | 0.2111 | 0.0231 | 1457 | -0.0092 | 0.0442 | 1957 | -0.0657 | 0.0311 |
| 458 | 0.0359 | 0.1521 | 958 | 0.0588 | 0.0003 | 1458 | 0.0006 | 0.1033 | 1958 | -0.1381 | -0.1324 |
| 459 | 0.0527 | 0.0848 | 959 | -0.075 | -0.1682 | 1459 | 0.1381 | 0.0788 | 1959 | -0.105 | 0.0748 |
| 460 | 0.1105 | 0.0535 | 960 | -0.0357 | -0.1576 | 1460 | 0.036 | -0.0404 | 1960 | -0.1169 | 0.0074 |
| 461 | 0.0659 | -0.0284 | 961 | 0.035 | 0.0092 | 1461 | -0.0228 | -0.0216 | 1961 | -0.0652 | -0.0027 |
| 462 | -0.0789 | -0.0645 | 962 | -0.0001 | -0.0251 | 1462 | 0.066 | 0.0499 | 1962 | -0.0155 | 0.0563 |
| 463 | -0.0423 | 0.1205 | 963 | -0.077 | -0.1073 | 1463 | 0.0869 | -0.0833 | 1963 | -0.0164 | -0.0053 |
| 464 | 0.0432 | -0.0167 | 964 | -0.2123 | -0.0922 | 1464 | 0.1188 | -0.0719 | 1964 | -0.0562 | 0.061 |
| 465 | 0.0313 | -0.1563 | 965 | -0.0705 | 0.0299 | 1465 | 0.021 | 0.0819 | 1965 | 0.0354 | 0.0225 |
| 466 | 0.1375 | -0.0603 | 966 | 0.0202 | 0.2039 | 1466 | -0.2108 | 0.0416 | 1966 | 0.1615 | -0.0778 |
| 467 | 0.0578 | 0.0368 | 967 | -0.0444 | -0.0132 | 1467 | -0.0612 | -0.0541 | 1967 | -0.0173 | -0.0444 |
| 468 | 0.0002 | 0.145 | 968 | -0.0458 | -0.0719 | 1468 | 0.1236 | -0.0749 | 1968 | -0.1477 | 0.0044 |
| 469 | 0.1475 | 0.0524 | 969 | 0.0469 | 0.0339 | 1469 | -0.0018 | -0.1356 | 1969 | -0.0221 | 0.0663 |
| 470 | -0.0582 | -0.0479 | 970 | 0.1251 | 0.0444 | 1470 | -0.0741 | -0.1099 | 1970 | 0.0699 | 0.0723 |
| 471 | -0.0778 | -0.0422 | 971 | 0.0742 | 0.0846 | 1471 | -0.1319 | 0.0313 | 1971 | 0.0508 | -0.0367 |
| 472 | 0.1014 | 0.0803 | 972 | 0.0284 | -0.0775 | 1472 | -0.0219 | 0.062 | 1972 | 0.0791 | -0.088 |
| 473 | -0.0754 | 0.1417 | 973 | 0.0161 | 0.0678 | 1473 | 0.0884 | 0.0975 | 1973 | 0.1081 | -0.0938 |
| 474 | -0.0674 | 0.0259 | 974 | 0.0182 | 0.1366 | 1474 | -0.0275 | -0.0237 | 1974 | 0.0062 | -0.0352 |
| 475 | -0.0396 | 0.1282 | 975 | 0.0388 | -0.1515 | 1475 | -0.043 | -0.1254 | 1975 | 0.0171 | 0.0825 |
| 476 | -0.0666 | 0.044 | 976 | 0.0253 | -0.1164 | 1476 | -0.0608 | 0.0333 | 1976 | 0.0597 | 0.0601 |
| 477 | 0.0352 | -0.1135 | 977 | 0.0442 | -0.0442 | 1477 | -0.0055 | -0.1194 | 1977 | 0.0286 | 0.1094 |
| 478 | -0.0693 | 0.0526 | 978 | -0.0053 | -0.0138 | 1478 | -0.0414 | 0.0038 | 1978 | 0.051 | 0.0375 |
| 479 | 0.0184 | 0.0222 | 979 | -0.0105 | -0.1557 | 1479 | -0.1423 | 0.2773 | 1979 | -0.0051 | -0.1882 |
| 480 | 0.0634 | -0.0674 | 980 | 0.1147 | -0.1803 | 1480 | 0.056 | -0.055 | 1980 | 0.0142 | -0.0201 |
| 481 | 0.0313 | 0.0625 | 981 | 0.0693 | 0.0533 | 1481 | 0.0065 | 0.0027 | 1981 | 0.0922 | 0.0609 |
| 482 | 0.0353 | -0.0245 | 982 | 0.0452 | -0.1503 | 1482 | -0.0771 | 0.1202 | 1982 | 0.0456 | -0.0342 |
| 483 | 0.1127 | 0 | 983 | 0.099 | -0.0594 | 1483 | 0.0777 | -0.069 | 1983 | -0.0109 | -0.0349 |
| 484 | -0.0812 | -0.0568 | 984 | 0.009 | 0.1465 | 1484 | 0.0769 | 0.0752 | 1984 | 0.0049 | -0.1249 |
| 485 | 0.0586 | -0.0945 | 985 | -0.0065 | -0.0873 | 1485 | 0.0143 | 0.0932 | 1985 | 0.0625 | 0 |
| 486 | 0.1806 | 0.1341 | 986 | 0.0076 | 0.0061 | 1486 | -0.1206 | 0.0359 | 1986 | -0.0042 | 0.0198 |
| 487 | -0.1053 | 0.0103 | 987 | -0.0151 | 0.0142 | 1487 | -0.0881 | -0.0527 | 1987 | -0.0603 | -0.1183 |
| 488 | -0.1111 | -0.0162 | 988 | 0.0819 | -0.0674 | 1488 | -0.035 | -0.089 | 1988 | -0.0763 | -0.0257 |
| 489 | 0.0404 | 0.0884 | 989 | 0.1119 | 0.1347 | 1489 | -0.1417 | 0.0884 | 1989 | -0.1164 | 0.0002 |
| 490 | 0.0132 | 0.0311 | 990 | -0.0518 | 0.1242 | 1490 | -0.1004 | -0.044 | 1990 | -0.0869 | -0.0093 |
| 491 | 0.0236 | 0.1247 | 991 | -0.1453 | 0.0456 | 1491 | -0.0825 | -0.0887 | 1991 | -0.0893 | -0.0689 |
| 492 | 0.104 | 0.0807 | 992 | -0.0485 | 0.0371 | 1492 | 0.0858 | -0.0048 | 1992 | 0.0516 | -0.0319 |
| 493 | 0.0565 | -0.0037 | 993 | -0.0533 | -0.0533 | 1493 | 0.1166 | -0.0356 | 1993 | 0.0911 | 0.0911 |
| 494 | -0.1119 | 0.0068 | 994 | -0.0545 | -0.0592 | 1494 | -0.0684 | 0.0882 | 1994 | 0.0023 | -0.0734 |
| 495 | -0.1248 | -0.0569 | 995 | 0.0825 | 0.0124 | 1495 | 0.0617 | 0.038 | 1995 | 0.1004 | -0.103 |
| 496 | -0.0305 | -0.107 | 996 | -0.0148 | -0.0154 | 1496 | -0.0656 | -0.0529 | 1996 | 0.0277 | 0.0604 |
| 497 | -0.0313 | -0.0313 | 997 | -0.0804 | -0.004 | 1497 | -0.1848 | 0.0194 | 1997 | 0.1101 | 0.0971 |
| 498 | -0.086 | 0.1224 | 998 | 0.0192 | 0.0267 | 1498 | 0.0613 | 0.0743 | 1998 | 0.1305 | -0.0019 |
| 499 | -0.0852 | 0.0661 | 999 | -0.0451 | 0.0179 | 1499 | 0.0638 | 0.039 | 1999 | -0.1113 | 0.0213 |
| 500 | -0.0113 | -0.0267 | 1000 | -0.0326 | 0.0748 | 1500 | 0.0148 | -0.1002 | 2000 | 0.0571 | 0.1835 |