IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | OFDM Frame Example |  |
| Date Submitted | 15 September, 2010 |  |
| Source | [Emmanuel Monnerie] [Landis+Gyr] | Voice: [ +1 678 641 1986 ] Fax: [ ] E-mail: [ Emmanuel . monnerie @ landisgyr . com ] |
| Re: | Comment Resolution for TG4g Draft D1 |  |
| Abstract | This document presents an example of OFDM Frame, built from the payload all the way to the time domain samples. It is similar to the 802.11-2007 Annex G. |  |
| Purpose | Proposed resolution for Comments ID 1780 through 1783 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Annex X

(informative)

An example of encoding a frame for OFDM PHY

X.1 Introduction

The purpose of this annex is to show an example of encoding a frame for the OFDM PHY, as described in

Clause 6.12b. This example covers all the encoding details defined by this standard.

The encoding illustration goes through the following stages:

- Generating the short training sequence section of the preamble;

- Generating the long preamble sequence section of the preamble;

- Generating the OFDM Header and the corresponding HCS;

- Setting the six tail bits to zeros;

- Encoding the Header with a convolutional encoder and puncturing;

- The PSDU from the MAC should already contain an FCS, which is assumed to be four bytes here.  Six tail bits and PAD bits are appended to form the data field.

- Scrambling the data field;

- Resetting the six tail bits back to zeros;

- Encoding the data with a convolutional encoder and puncturing;

- Interleaving the data field;

- Mapping into complex symbols;

- Frequency Spreading;

- Concatenating the preamble, the OFDM header and the data field;

- Pilot, guard and DC tone insertion;

- Transforming from frequency to time domain using the IFFT and adding a circular prefix;

- Concatenating the OFDM symbols into a single, time-domain signal.

In the description of time domain waveforms, a complex baseband signal at 666.666 ksample/s is used. This example uses the 400 kbps data rate (QPSK  ½ rate modulation), which corresponds to OFDM Option 2 and MCS level 3, and a message of 72 octets. The OFDM Header uses the 50 kbps data rate (BPSK 1/2 rate coded with 4x frequency repetition), which corresponds to MCS level 0.

X.2 The Message

The message being encoded is the same one than the one used in 802.11-2007 annex G. It consists of the first 72 characters of the well-known “Ode to Joy” by F. Schiller:

Joy, bright spark of divinity,

Daughter of Elysium,

Fire-insired we tread

Thy sanctuary.

Thy magic power re-unites

All that custom has divided,

All men become brothers

Under the sway of thy gentle wings…

The message is converted to ASCII and a CRC32 is added as defined in clause 7.2.1.9;

Note: the MAC header will not be included. It is assumed to be part of the message in this annex.

The resulting 76 octets PSDU is shown in Table X.1.

Table X.1 – The message

| Octet # | Value (Hex) | Octet # | Value (Hex) | Octet # | Value (Hex) | Octet # | Value (Hex) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4A | 21 | 20 | 41 | 6F | 61 | 69 |
| 2 | 6F | 22 | 64 | 42 | 66 | 62 | 72 |
| 3 | 79 | 23 | 69 | 43 | 20 | 63 | 65 |
| 4 | 2C | 24 | 76 | 44 | 45 | 64 | 64 |
| 5 | 20 | 25 | 69 | 45 | 6C | 65 | 20 |
| 6 | 62 | 26 | 6E | 46 | 79 | 66 | 77 |
| 7 | 72 | 27 | 69 | 47 | 73 | 67 | 65 |
| 8 | 69 | 28 | 74 | 48 | 69 | 68 | 20 |
| 9 | 67 | 29 | 79 | 49 | 75 | 69 | 74 |
| 10 | 68 | 30 | 2C | 50 | 6D | 70 | 72 |
| 11 | 74 | 31 | A | 51 | 2C | 71 | 65 |
| 12 | 20 | 32 | 44 | 52 | A | 72 | 61 |
| 13 | 73 | 33 | 61 | 53 | 46 | 73 | 42 |
| 14 | 70 | 34 | 75 | 54 | 69 | 74 | E2 |
| 15 | 61 | 35 | 67 | 55 | 72 | 75 | 94 |
| 16 | 72 | 36 | 68 | 56 | 65 | 76 | C0 |
| 17 | 6B | 37 | 74 | 57 | 2D |  |  |
| 18 | 20 | 38 | 65 | 58 | 69 |  |  |
| 19 | 6F | 39 | 72 | 59 | 6E |  |  |
| 20 | 66 | 40 | 20 | 60 | 73 |  |  |

X.3 Generation of the OFDM Header

X.3.1 HCS and PAD bits insertion

In this example the payload data has a size of 76 octets and it will be encoded using QPSK modulation, ½ rate coding (Rate field value = 3). The scrambler index will be the first one (Scrambler ID = 0).

The corresponding OFDM Header, including the HCS is represented in table X.2

Table X.2 – OFDM Header

| Field Name | Bit # | Bit Value |  | Field Name | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- |
| Rate | 1 | 0 |  | Scrambler | 20 | 0 |
|  | 2 | 0 |  |  | 21 | 0 |
|  | 3 | 0 |  | RFU | 22 | 0 |
|  | 4 | 1 |  | HCS | 23 | 1 |
|  | 5 | 1 |  |  | 24 | 1 |
| RFU | 6 | 0 |  |  | 25 | 1 |
| Length | 7 | 0 |  |  | 26 | 1 |
|  | 8 | 0 |  |  | 27 | 1 |
|  | 9 | 0 |  |  | 28 | 1 |
|  | 10 | 0 |  |  | 29 | 0 |
|  | 11 | 1 |  |  | 30 | 1 |
|  | 12 | 0 |  | Tail | 31 | 0 |
|  | 13 | 0 |  |  | 32 | 0 |
|  | 14 | 1 |  |  | 33 | 0 |
|  | 15 | 1 |  |  | 34 | 0 |
|  | 16 | 0 |  |  | 35 | 0 |
|  | 17 | 0 |  |  | 36 | 0 |
| RFU | 18 | 0 |  |  |  |  |
|  | 19 | 0 |  |  |  |  |

In this configuration, no extra PAD bit is necessary. The size of the header will fill up exactly 6 OFDM symbols.

X.3.2 Convolutional Encoding

After convolutional encoding of the OFDM header, the size is now doubled and the corresponding bits are represented in table X.3. No puncturing is applied in this configuration.

Table X.3 – OFDM Header after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 19 | 0 | 37 | 1 | 55 | 0 |
| 2 | 0 | 20 | 1 | 38 | 0 | 56 | 0 |
| 3 | 0 | 21 | 0 | 39 | 0 | 57 | 0 |
| 4 | 0 | 22 | 0 | 40 | 1 | 58 | 0 |
| 5 | 0 | 23 | 0 | 41 | 1 | 59 | 1 |
| 6 | 0 | 24 | 1 | 42 | 1 | 60 | 0 |
| 7 | 1 | 25 | 1 | 43 | 0 | 61 | 1 |
| 8 | 1 | 26 | 1 | 44 | 0 | 62 | 1 |
| 9 | 1 | 27 | 0 | 45 | 1 | 63 | 1 |
| 10 | 0 | 28 | 0 | 46 | 1 | 64 | 0 |
| 11 | 1 | 29 | 1 | 47 | 1 | 65 | 1 |
| 12 | 0 | 30 | 0 | 48 | 0 | 66 | 0 |
| 13 | 0 | 31 | 0 | 49 | 0 | 67 | 1 |
| 14 | 0 | 32 | 0 | 50 | 1 | 68 | 1 |
| 15 | 1 | 33 | 1 | 51 | 1 | 69 | 1 |
| 16 | 1 | 34 | 1 | 52 | 0 | 70 | 0 |
| 17 | 1 | 35 | 1 | 53 | 1 | 71 | 1 |
| 18 | 0 | 36 | 1 | 54 | 0 | 72 | 1 |

X.3.3 Interleaving

In this configuration, no bit interleaving is necessary for the OFDM header. The formula defined in 6.12b.2.4 will lead to an interleaving that does not change the position of the bits.

X.3.4 Bit mapping

The 72 bits are split into 6 OFDM symbols. The bit mapping for the OFDM header in this example is BPSK, therefore Q is always 0. The I value is mapped as defined in figure 65.l.

Table X.4 – Bit mapping for the OFDM Header

|  | Symbol 0 |  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 2 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 3 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 4 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 5 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 6 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 7 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 8 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 9 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 10 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 11 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 12 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |

X.3.5 Frequency Spreading

In this example, a frequency spreading of 4 is applied to the OFDM header. The original 12 bits in each symbol (Bin # 25 through 36 in table X.5) are duplicated within the same symbol. The resulting symbols have 48 data bits each. The duplicated bits have a phase rotation as defined in 6.12b.2.5.

Table X.5 –OFDM Header in the frequency domain

|  | Symbol 0 |  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 2 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 3 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 4 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 5 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 6 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 7 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 8 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 9 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 10 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 11 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 12 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 13 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 14 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 15 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 16 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 17 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 18 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 19 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 20 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 21 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 22 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 23 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 24 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 25 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 26 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 27 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 28 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 29 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 30 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 31 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 32 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 33 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 34 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 35 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 36 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 37 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 38 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 39 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 40 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 41 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 42 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 43 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 44 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 45 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 46 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 47 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 48 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |

X.4 Generation of the data Symbols

X.4.1 PAD Insertion and Data Scrambling

The original 76 octets of data, as defined in table X.0, are concatenated with 6 zero tail bits and 10 pad bits (as calculated by the formulas in 6.12b.2.9). The 6 tail bits are forced to 0 after scrambling. The resulting 624 bits are represented in table X.6 (First and last 48 bits only).

Table X.6 – First and last 48 bits after pad insertion and scrambling

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 577 | 1 | 601 | 0 |
| 2 | 0 | 26 | 0 | 578 | 0 | 602 | 1 |
| 3 | 1 | 27 | 0 | 579 | 1 | 603 | 1 |
| 4 | 0 | 28 | 0 | 580 | 0 | 604 | 1 |
| 5 | 1 | 29 | 0 | 581 | 1 | 605 | 1 |
| 6 | 1 | 30 | 0 | 582 | 0 | 606 | 1 |
| 7 | 0 | 31 | 1 | 583 | 0 | 607 | 1 |
| 8 | 0 | 32 | 1 | 584 | 0 | 608 | 0 |
| 9 | 0 | 33 | 0 | 585 | 1 | 609 | 0 |
| 10 | 1 | 34 | 0 | 586 | 0 | 610 | 0 |
| 11 | 1 | 35 | 0 | 587 | 1 | 611 | 0 |
| 12 | 1 | 36 | 0 | 588 | 1 | 612 | 0 |
| 13 | 1 | 37 | 0 | 589 | 0 | 613 | 0 |
| 14 | 1 | 38 | 1 | 590 | 0 | 614 | 0 |
| 15 | 0 | 39 | 0 | 591 | 1 | 615 | 0 |
| 16 | 1 | 40 | 1 | 592 | 0 | 616 | 0 |
| 17 | 0 | 41 | 1 | 593 | 1 | 617 | 0 |
| 18 | 1 | 42 | 0 | 594 | 0 | 618 | 0 |
| 19 | 0 | 43 | 1 | 595 | 1 | 619 | 0 |
| 20 | 1 | 44 | 0 | 596 | 1 | 620 | 1 |
| 21 | 0 | 45 | 1 | 597 | 1 | 621 | 1 |
| 22 | 0 | 46 | 1 | 598 | 1 | 622 | 0 |
| 23 | 1 | 47 | 0 | 599 | 1 | 623 | 1 |
| 24 | 0 | 48 | 0 | 600 | 0 | 624 | 1 |

X.4.2 Convolutional Encoding and puncturing

After convolutional encoding of the payload, the size is now doubled and the corresponding bits are represented in table X.7. No puncturing is applied in this configuration.

Table X.7 – First and last 48 bits after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 1201 | 0 | 1225 | 1 |
| 2 | 1 | 26 | 0 | 1202 | 1 | 1226 | 1 |
| 3 | 0 | 27 | 1 | 1203 | 0 | 1227 | 0 |
| 4 | 1 | 28 | 0 | 1204 | 1 | 1228 | 0 |
| 5 | 0 | 29 | 1 | 1205 | 1 | 1229 | 0 |
| 6 | 0 | 30 | 1 | 1206 | 1 | 1230 | 0 |
| 7 | 1 | 31 | 1 | 1207 | 0 | 1231 | 0 |
| 8 | 0 | 32 | 0 | 1208 | 0 | 1232 | 0 |
| 9 | 0 | 33 | 1 | 1209 | 0 | 1233 | 0 |
| 10 | 0 | 34 | 1 | 1210 | 1 | 1234 | 0 |
| 11 | 1 | 35 | 0 | 1211 | 1 | 1235 | 0 |
| 12 | 1 | 36 | 1 | 1212 | 0 | 1236 | 0 |
| 13 | 0 | 37 | 1 | 1213 | 0 | 1237 | 0 |
| 14 | 1 | 38 | 1 | 1214 | 0 | 1238 | 0 |
| 15 | 1 | 39 | 1 | 1215 | 0 | 1239 | 1 |
| 16 | 0 | 40 | 1 | 1216 | 0 | 1240 | 1 |
| 17 | 0 | 41 | 0 | 1217 | 0 | 1241 | 1 |
| 18 | 0 | 42 | 0 | 1218 | 1 | 1242 | 0 |
| 19 | 0 | 43 | 0 | 1219 | 1 | 1243 | 1 |
| 20 | 1 | 44 | 0 | 1220 | 0 | 1244 | 0 |
| 21 | 1 | 45 | 1 | 1221 | 0 | 1245 | 1 |
| 22 | 1 | 46 | 0 | 1222 | 1 | 1246 | 1 |
| 23 | 1 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 0 | 48 | 0 | 1224 | 1 | 1248 | 1 |

X.4.3 Interleaving

A 2-step interleaver is applied to the data, as defined in 6.12b.2.4. The resulting data (first and last 48 bits only) is represented in table X.8.

Table X.8 – First and last 48 bits after interleaving

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 1201 | 1 | 1225 | 0 |
| 2 | 0 | 26 | 0 | 1202 | 1 | 1226 | 1 |
| 3 | 1 | 27 | 0 | 1203 | 1 | 1227 | 1 |
| 4 | 1 | 28 | 1 | 1204 | 0 | 1228 | 1 |
| 5 | 1 | 29 | 1 | 1205 | 0 | 1229 | 1 |
| 6 | 0 | 30 | 0 | 1206 | 1 | 1230 | 1 |
| 7 | 0 | 31 | 0 | 1207 | 0 | 1231 | 0 |
| 8 | 0 | 32 | 1 | 1208 | 1 | 1232 | 1 |
| 9 | 1 | 33 | 0 | 1209 | 0 | 1233 | 0 |
| 10 | 1 | 34 | 0 | 1210 | 1 | 1234 | 0 |
| 11 | 0 | 35 | 1 | 1211 | 1 | 1235 | 0 |
| 12 | 1 | 36 | 0 | 1212 | 0 | 1236 | 1 |
| 13 | 0 | 37 | 1 | 1213 | 0 | 1237 | 1 |
| 14 | 0 | 38 | 1 | 1214 | 0 | 1238 | 0 |
| 15 | 1 | 39 | 0 | 1215 | 0 | 1239 | 0 |
| 16 | 1 | 40 | 1 | 1216 | 0 | 1240 | 0 |
| 17 | 0 | 41 | 0 | 1217 | 0 | 1241 | 0 |
| 18 | 1 | 42 | 0 | 1218 | 1 | 1242 | 1 |
| 19 | 1 | 43 | 1 | 1219 | 0 | 1243 | 1 |
| 20 | 1 | 44 | 0 | 1220 | 0 | 1244 | 1 |
| 21 | 0 | 45 | 1 | 1221 | 0 | 1245 | 0 |
| 22 | 1 | 46 | 0 | 1222 | 0 | 1246 | 1 |
| 23 | 0 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 0 | 48 | 0 | 1224 | 1 | 1248 | 1 |

X.4.4 Bit mapping

The bit mapping for the OFDM header in this example is QPSK (2 data bits per vector). The I and Q vectors are mapped as defined in figure 65.l. The resulting data is represented in table X.9 (first two and last two symbols).

Table X.9 – Bit mapping for the OFDM payload

| Vector # | I | Q | Vector # | I | Q | Vector # | I | Q | Vector # | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.707 | -0.707 | 49 | -0.707 | -0.707 | 529 | -0.707 | -0.707 | 577 | 0.707 | -0.707 |
| 2 | 0.707 | 0.707 | 50 | -0.707 | -0.707 | 530 | -0.707 | 0.707 | 578 | 0.707 | -0.707 |
| 3 | 0.707 | -0.707 | 51 | -0.707 | 0.707 | 531 | 0.707 | -0.707 | 579 | -0.707 | -0.707 |
| 4 | -0.707 | -0.707 | 52 | -0.707 | -0.707 | 532 | -0.707 | 0.707 | 580 | 0.707 | -0.707 |
| 5 | 0.707 | 0.707 | 53 | -0.707 | -0.707 | 533 | -0.707 | -0.707 | 581 | 0.707 | -0.707 |
| 6 | -0.707 | 0.707 | 54 | -0.707 | 0.707 | 534 | 0.707 | -0.707 | 582 | -0.707 | -0.707 |
| 7 | -0.707 | -0.707 | 55 | 0.707 | -0.707 | 535 | 0.707 | -0.707 | 583 | 0.707 | -0.707 |
| 8 | 0.707 | 0.707 | 56 | -0.707 | 0.707 | 536 | -0.707 | 0.707 | 584 | 0.707 | -0.707 |
| 9 | -0.707 | 0.707 | 57 | -0.707 | -0.707 | 537 | 0.707 | 0.707 | 585 | 0.707 | -0.707 |
| 10 | 0.707 | 0.707 | 58 | 0.707 | 0.707 | 538 | 0.707 | 0.707 | 586 | 0.707 | 0.707 |
| 11 | -0.707 | 0.707 | 59 | 0.707 | 0.707 | 539 | 0.707 | -0.707 | 587 | -0.707 | -0.707 |
| 12 | -0.707 | -0.707 | 60 | 0.707 | -0.707 | 540 | 0.707 | -0.707 | 588 | -0.707 | 0.707 |
| 13 | 0.707 | -0.707 | 61 | 0.707 | 0.707 | 541 | 0.707 | 0.707 | 589 | 0.707 | 0.707 |
| 14 | -0.707 | 0.707 | 62 | -0.707 | 0.707 | 542 | -0.707 | 0.707 | 590 | -0.707 | 0.707 |
| 15 | 0.707 | -0.707 | 63 | 0.707 | 0.707 | 543 | 0.707 | -0.707 | 591 | 0.707 | -0.707 |
| 16 | -0.707 | 0.707 | 64 | 0.707 | 0.707 | 544 | -0.707 | 0.707 | 592 | -0.707 | 0.707 |
| 17 | -0.707 | -0.707 | 65 | 0.707 | 0.707 | 545 | -0.707 | 0.707 | 593 | 0.707 | -0.707 |
| 18 | 0.707 | -0.707 | 66 | -0.707 | 0.707 | 546 | -0.707 | 0.707 | 594 | 0.707 | 0.707 |
| 19 | 0.707 | 0.707 | 67 | -0.707 | -0.707 | 547 | 0.707 | 0.707 | 595 | 0.707 | -0.707 |
| 20 | -0.707 | 0.707 | 68 | 0.707 | -0.707 | 548 | 0.707 | 0.707 | 596 | -0.707 | 0.707 |
| 21 | -0.707 | -0.707 | 69 | 0.707 | 0.707 | 549 | -0.707 | -0.707 | 597 | 0.707 | -0.707 |
| 22 | 0.707 | -0.707 | 70 | 0.707 | 0.707 | 550 | -0.707 | -0.707 | 598 | 0.707 | -0.707 |
| 23 | 0.707 | -0.707 | 71 | 0.707 | -0.707 | 551 | 0.707 | 0.707 | 599 | 0.707 | 0.707 |
| 24 | 0.707 | -0.707 | 72 | -0.707 | -0.707 | 552 | -0.707 | -0.707 | 600 | -0.707 | -0.707 |
| 25 | 0.707 | -0.707 | 73 | 0.707 | -0.707 | 553 | -0.707 | 0.707 | 601 | 0.707 | 0.707 |
| 26 | 0.707 | -0.707 | 74 | 0.707 | 0.707 | 554 | -0.707 | 0.707 | 602 | 0.707 | -0.707 |
| 27 | -0.707 | -0.707 | 75 | -0.707 | 0.707 | 555 | -0.707 | 0.707 | 603 | -0.707 | 0.707 |
| 28 | -0.707 | -0.707 | 76 | 0.707 | -0.707 | 556 | -0.707 | -0.707 | 604 | -0.707 | 0.707 |
| 29 | -0.707 | 0.707 | 77 | -0.707 | -0.707 | 557 | -0.707 | 0.707 | 605 | -0.707 | 0.707 |
| 30 | -0.707 | -0.707 | 78 | 0.707 | -0.707 | 558 | -0.707 | -0.707 | 606 | 0.707 | -0.707 |
| 31 | 0.707 | -0.707 | 79 | 0.707 | 0.707 | 559 | -0.707 | -0.707 | 607 | -0.707 | -0.707 |
| 32 | -0.707 | -0.707 | 80 | -0.707 | 0.707 | 560 | -0.707 | -0.707 | 608 | -0.707 | -0.707 |
| 33 | -0.707 | 0.707 | 81 | -0.707 | -0.707 | 561 | 0.707 | 0.707 | 609 | -0.707 | 0.707 |
| 34 | 0.707 | 0.707 | 82 | 0.707 | 0.707 | 562 | -0.707 | 0.707 | 610 | -0.707 | -0.707 |
| 35 | 0.707 | 0.707 | 83 | -0.707 | 0.707 | 563 | 0.707 | 0.707 | 611 | -0.707 | -0.707 |
| 36 | -0.707 | -0.707 | 84 | 0.707 | -0.707 | 564 | 0.707 | -0.707 | 612 | -0.707 | 0.707 |
| 37 | -0.707 | 0.707 | 85 | 0.707 | 0.707 | 565 | 0.707 | -0.707 | 613 | -0.707 | 0.707 |
| 38 | 0.707 | -0.707 | 86 | 0.707 | 0.707 | 566 | 0.707 | -0.707 | 614 | 0.707 | 0.707 |
| 39 | 0.707 | 0.707 | 87 | -0.707 | -0.707 | 567 | -0.707 | 0.707 | 615 | 0.707 | 0.707 |
| 40 | 0.707 | 0.707 | 88 | -0.707 | -0.707 | 568 | -0.707 | -0.707 | 616 | -0.707 | 0.707 |
| 41 | 0.707 | 0.707 | 89 | 0.707 | -0.707 | 569 | 0.707 | 0.707 | 617 | -0.707 | -0.707 |
| 42 | -0.707 | 0.707 | 90 | 0.707 | -0.707 | 570 | 0.707 | -0.707 | 618 | -0.707 | 0.707 |
| 43 | 0.707 | 0.707 | 91 | -0.707 | 0.707 | 571 | -0.707 | 0.707 | 619 | 0.707 | -0.707 |
| 44 | 0.707 | 0.707 | 92 | -0.707 | 0.707 | 572 | 0.707 | 0.707 | 620 | -0.707 | -0.707 |
| 45 | 0.707 | -0.707 | 93 | 0.707 | -0.707 | 573 | 0.707 | 0.707 | 621 | -0.707 | 0.707 |
| 46 | 0.707 | -0.707 | 94 | 0.707 | 0.707 | 574 | -0.707 | -0.707 | 622 | 0.707 | 0.707 |
| 47 | -0.707 | -0.707 | 95 | -0.707 | -0.707 | 575 | -0.707 | 0.707 | 623 | -0.707 | 0.707 |
| 48 | -0.707 | -0.707 | 96 | -0.707 | 0.707 | 576 | 0.707 | 0.707 | 624 | -0.707 | 0.707 |

X.4.5 Frequency Spreading

In this example, no frequency spreading is applied to the OFDM payload. The vectors from table X.9 are mapped directly into the frequency domain. The next paragraphs are illustrating the mapping in the frequency domain, taking into account the pilot tones, the DC tone and the guard tones.

X.5 Conversion from frequency domain to time domain

X.5.1 Pilot, DC and Guard tone insertion

The following steps are applied to both the OFDM header and the OFDM payload. Before going to the next steps, table X.5 and X.9 should be appended, resulting in 19 symbols of 48 bins in the frequency domain. The 48 bins are mapped in the frequency domain by inserting pilot tones, guard tones and a DC tone, as defined in 6.12b.2.6.

Table X.10 Complete Packet in the frequency domain (first and last three symbols)

| Subcarrier | Symbol 0 | Symbol 1 | Symbol 2 | Symbol 16 | Symbol 17 | Symbol 18 |
| --- | --- | --- | --- | --- | --- | --- |
| -32 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -31 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -30 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -29 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -28 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -27 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -26 | 0 - 1i | 0 - 1i | 0 + 1i | -0.707 + 0.707i | 1 + 0i | 0.707 - 0.707i |
| -25 | 0 + 1i | 0 + 1i | 0 - 1i | 0.707 + 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -24 | 0 - 1i | 0 + 1i | 0 - 1i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| -23 | 0 + 1i | 0 - 1i | 0 + 1i | -0.707 - 0.707i | 0.707 - 0.707i | 0.707 - 0.707i |
| -22 | 0 - 1i | 1 + 0i | 0 + 1i | 0.707 - 0.707i | -0.707 + 0.707i | 1 + 0i |
| -21 | 0 + 1i | 0 + 1i | 0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -20 | 0 + 1i | 0 + 1i | 0 - 1i | -0.707 - 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| -19 | 0 - 1i | 0 - 1i | 0 + 1i | 0.707 - 0.707i | 0.707 - 0.707i | 0.707 - 0.707i |
| -18 | 0 + 1i | 0 - 1i | -1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 - 0.707i |
| -17 | 0 + 1i | 0 - 1i | 0 + 1i | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| -16 | 0 + 1i | 0 + 1i | 0 - 1i | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 + 0.707i |
| -15 | 0 + 1i | 0 - 1i | 0 + 1i | 0.707 + 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| -14 | -1 + 0i | 0 - 1i | 0 - 1i | -0.707 + 0.707i | 1 + 0i | -0.707 + 0.707i |
| -13 | 1 + 0i | 1 + 0i | -1 + 0i | 0.707 - 0.707i | 0.707 - 0.707i | 0.707 + 0.707i |
| -12 | 0 - 1i | 0 - 1i | 0 + 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| -11 | -1 + 0i | 1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -10 | 0 + 1i | 1 + 0i | 0 + 1i | -0.707 + 0.707i | 0.707 - 0.707i | -1 + 0i |
| -9 | 1 + 0i | 0 - 1i | -1 + 0i | -0.707 + 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -8 | 0 - 1i | -1 + 0i | 0 - 1i | -0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -7 | 1 + 0i | 0 - 1i | -1 + 0i | 0.707 - 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| -6 | 0 - 1i | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 + 0.707i | 0.707 - 0.707i |
| -5 | -1 + 0i | 0 - 1i | 0 + 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| -4 | 0 - 1i | 1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -3 | 1 + 0i | 0 - 1i | 0 + 1i | -0.707 + 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -2 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 + 0.707i | -1 + 0i | 0.707 + 0.707i |
| -1 | 0 + 1i | 0 - 1i | 0 - 1i | 0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 0 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 1 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | -0.707 - 0.707i | 0.707 + 0.707i |
| 2 | -1 + 0i | 1 + 0i | 1 + 0i | 0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| 3 | -1 + 0i | -1 + 0i | -1 + 0i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| 4 | -1 + 0i | 1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| 5 | -1 + 0i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 6 | -1 + 0i | 1 + 0i | 1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 - 0.707i |
| 7 | 1 + 0i | -1 + 0i | -1 + 0i | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 8 | 1 + 0i | -1 + 0i | -1 + 0i | -0.707 + 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 9 | 1 + 0i | 1 + 0i | -1 + 0i | 0.707 + 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 10 | -1 + 0i | -1 + 0i | 1 + 0i | 0.707 - 0.707i | 0.707 + 0.707i | -1 + 0i |
| 11 | -1 + 0i | -1 + 0i | 1 + 0i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 12 | 1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 13 | -1 + 0i | 1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| 14 | -1 + 0i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| 15 | 0 - 1i | -1 + 0i | 0 + 1i | -0.707 + 0.707i | 0.707 - 0.707i | 0.707 + 0.707i |
| 16 | 1 + 0i | 0 - 1i | 1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| 17 | 0 + 1i | -1 + 0i | 0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 18 | -1 + 0i | 0 - 1i | 1 + 0i | 1 + 0i | 0.707 + 0.707i | -0.707 - 0.707i |
| 19 | 0 - 1i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| 20 | -1 + 0i | 0 - 1i | 0 - 1i | -0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| 21 | 0 - 1i | 1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 22 | -1 + 0i | 0 - 1i | 0 + 1i | -0.707 - 0.707i | 0.707 + 0.707i | 1 + 0i |
| 23 | 1 + 0i | -1 + 0i | 1 + 0i | 0.707 + 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 24 | 0 - 1i | 0 - 1i | 0 + 1i | -0.707 - 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| 25 | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 - 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| 26 | 0 + 1i | 0 - 1i | 0 - 1i | 0.707 + 0.707i | -1 + 0i | -0.707 + 0.707i |
| 27 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 28 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 29 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 30 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 31 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |

Note: in table X.10, the pilot tones are represented in bold and the DC/Guard Tones are represented in italic.

X.5.2 Time domain OFDM Header and Payload

The data from table X10 is converted by an IFFT of size 64. Most IFFTs require a reordering of the data. Typically, the order of the frequencies within each symbol should be as follows:

0, 1, 2, … 31, -32, -31, …, -1

After the IFFT, each symbol is extended by a cyclic prefix of 16 samples. Each OFDM symbol then has a size of 80 samples (64 +16).

The resulting data in the time domain has 1520 samples (19*80) and is represented in paragraph X.7 (complete packet).

X.6 Generation of the preamble

X.6.1 Generation of the STF

As defined in clause 6.3.4a.1, the STF consists of 4 symbols. The resulting STF is equivalent to 20 repetitions of the following 16-sample time domain pattern:

Table X.11 – STF time domain pattern

| Sample # | I | Q |
| --- | --- | --- |
| 1 | 0.6505 | 0 |
| 2 | 1.0989 | 0.6505 |
| 3 | 0.46 | -1.301 |
| 4 | -0.9531 | -0.6505 |
| 5 | 0 | 0 |
| 6 | 0.9531 | 0.6505 |
| 7 | -0.46 | 1.301 |
| 8 | -1.0989 | -0.6505 |
| 9 | -0.6505 | 0 |
| 10 | -1.0989 | 0.6505 |
| 11 | -0.46 | -1.301 |
| 12 | 0.9531 | -0.6505 |
| 13 | 0 | 0 |
| 14 | -0.9531 | 0.6505 |
| 15 | 0.46 | 1.301 |
| 16 | 1.0989 | -0.6505 |

Note: the last 2 repetitions are negated.

X.6.2 Generation of the LTF

As defined in clause 6.3.4a.2, the LTF consists of the following time domain pattern:

Table X.12 – LTF in time domain

| Sample # | I | Q | Sample # | I | Q |
| --- | --- | --- | --- | --- | --- |
| 33 | -0.75 | 0 | 65 | 1.25 | 0 |
| 34 | 0.5069 | -0.5395 | 66 | -0.3216 | -0.5171 |
| 35 | 0.147 | 0.178 | 67 | -0.564 | -0.4328 |
| 36 | -0.5541 | 0.4205 | 68 | 0.8594 | 0.5478 |
| 37 | 0.2817 | -0.9077 | 69 | 0.0111 | -0.4458 |
| 38 | -0.5326 | 0.3518 | 70 | -0.9756 | -0.4563 |
| 39 | -0.3165 | 1.0258 | 71 | -1.1903 | 0.3727 |
| 40 | -0.5391 | 0.207 | 72 | -0.7761 | -0.0741 |
| 41 | -0.6036 | 1.1339 | 73 | 0.1036 | -0.6339 |
| 42 | 0.8113 | 0.6423 | 74 | 0.2871 | -1.2165 |
| 43 | -0.6222 | -0.3373 | 75 | 0.4219 | -0.6784 |
| 44 | -0.4653 | -0.3832 | 76 | 0.8836 | -0.4284 |
| 45 | 1.1802 | 0.4189 | 77 | 0.5269 | 0.2276 |
| 46 | 0.0015 | 1.1597 | 78 | -0.6312 | 0.5158 |
| 47 | 0.4623 | 0.2948 | 79 | -0.3382 | -0.9638 |
| 48 | 1.0998 | -0.4827 | 80 | 0.3459 | 0.1333 |
| 49 | 0.25 | -1.25 | 81 | 0.25 | 1.25 |
| 50 | 0.3459 | -0.1333 | 82 | 1.0998 | 0.4827 |
| 51 | -0.3382 | 0.9638 | 83 | 0.4623 | -0.2948 |
| 52 | -0.6312 | -0.5158 | 84 | 0.0015 | -1.1597 |
| 53 | 0.5269 | -0.2276 | 85 | 1.1802 | -0.4189 |
| 54 | 0.8836 | 0.4284 | 86 | -0.4653 | 0.3832 |
| 55 | 0.4219 | 0.6784 | 87 | -0.6222 | 0.3373 |
| 56 | 0.2871 | 1.2165 | 88 | 0.8113 | -0.6423 |
| 57 | 0.1036 | 0.6339 | 89 | -0.6036 | -1.1339 |
| 58 | -0.7761 | 0.0741 | 90 | -0.5391 | -0.207 |
| 59 | -1.1903 | -0.3727 | 91 | -0.3165 | -1.0258 |
| 60 | -0.9756 | 0.4563 | 92 | -0.5326 | -0.3518 |
| 61 | 0.0111 | 0.4458 | 93 | 0.2817 | 0.9077 |
| 62 | 0.8594 | -0.5478 | 94 | -0.5541 | -0.4205 |
| 63 | -0.564 | 0.4328 | 95 | 0.147 | -0.178 |
| 64 | -0.3216 | 0.5171 | 96 | 0.5069 | 0.5395 |

The samples 1 through 32 are part of the cyclic prefix for the LTF. They are simply a copy of samples 65 through 96. The samples 97 through 160 are a repetition of samples 33 through 96.

X.7 The entire packet

The complete packet in the time domain is represented in table X.13. The STF is from sample 1 to 320. The LFT is from sample 321 to 480. The OFDM header and payload is from sample 481 to 2000.

Table X.13 Complete Packet

| Sample | I | Q | Sample | I | Q | Sample | I | Q | Sample | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.6505 | 0 | 501 | -0.2547 | 0.1926 | 1001 | -0.2714 | 0.2286 | 1501 | -0.1464 | -0.7724 |
| 2 | 1.0989 | 0.6505 | 502 | -0.3761 | -0.1501 | 1002 | -0.072 | -0.331 | 1502 | 0.5391 | -0.2494 |
| 3 | 0.46 | -1.301 | 503 | -0.101 | -0.5548 | 1003 | -0.8165 | -0.2358 | 1503 | 1.1942 | -0.9895 |
| 4 | -0.9531 | -0.6505 | 504 | 0.6743 | 0.0203 | 1004 | -0.5143 | 0.4789 | 1504 | 0.7536 | -0.5666 |
| 5 | 0 | 0 | 505 | 0.9786 | 0.2286 | 1005 | 1.141 | -0.1779 | 1505 | 0.4268 | 0 |
| 6 | 0.9531 | 0.6505 | 506 | 1.225 | 0.1847 | 1006 | 0.3893 | 0.2462 | 1506 | -0.3316 | 0.3052 |
| 7 | -0.46 | 1.301 | 507 | 0.053 | -0.4205 | 1007 | -0.4088 | 0.5578 | 1507 | 0.0807 | -0.2264 |
| 8 | -1.0989 | -0.6505 | 508 | -0.2954 | -0.2119 | 1008 | -0.7299 | -0.2261 | 1508 | -0.1238 | -0.0443 |
| 9 | -0.6505 | 0 | 509 | -0.8605 | 0.9157 | 1009 | -0.3536 | 0.3536 | 1509 | -0.25 | 0.5991 |
| 10 | -1.0989 | 0.6505 | 510 | -0.5877 | 0.0836 | 1010 | 1.0205 | 0.6015 | 1510 | 1.1138 | 0.2278 |
| 11 | -0.46 | -1.301 | 511 | 0.8106 | -0.2012 | 1011 | -0.3728 | 0.4701 | 1511 | 0.1059 | 1.2188 |
| 12 | 0.9531 | -0.6505 | 512 | 0.2814 | 0.0023 | 1012 | -0.4247 | 0.327 | 1512 | -0.4941 | 0.8154 |
| 13 | 0 | 0 | 513 | 1.25 | -0.75 | 1013 | 0.0867 | -0.4596 | 1513 | 0.1768 | -0.6768 |
| 14 | -0.9531 | 0.6505 | 514 | 0.5322 | 0.1674 | 1014 | -1.5333 | -0.4899 | 1514 | -0.1687 | 0.2863 |
| 15 | 0.46 | 1.301 | 515 | -1.0086 | 0.6874 | 1015 | -0.6249 | 0.0207 | 1515 | -0.146 | 0.4847 |
| 16 | 1.0989 | -0.6505 | 516 | 0.8607 | 0.5348 | 1016 | 0.6767 | 0.6416 | 1516 | -0.228 | -0.2491 |
| 17 | 0.6505 | 0 | 517 | 0.7917 | -0.0383 | 1017 | 0.4786 | -0.5821 | 1517 | 0.1464 | 0.0923 |
| 18 | 1.0989 | 0.6505 | 518 | 0.7509 | -1.4469 | 1018 | -0.6711 | -1.8065 | 1518 | 0.2005 | -0.0101 |
| 19 | 0.46 | -1.301 | 519 | 0.8453 | -0.191 | 1019 | -0.8513 | 0.3592 | 1519 | -0.0248 | -0.6194 |
| 20 | -0.9531 | -0.6505 | 520 | -0.7056 | -0.1001 | 1020 | 0.3586 | 0.714 | 1520 | 0.6483 | -0.3495 |
| 21 | 0 | 0 | 521 | -0.7589 | -1.0089 | 1021 | -0.2321 | 0.0007 | 1521 | -0.6339 | 0.7071 |
| 22 | 0.9531 | 0.6505 | 522 | -0.1941 | 0.7677 | 1022 | 0.4426 | 1.048 | 1522 | -0.1568 | -0.3811 |
| 23 | -0.46 | 1.301 | 523 | 0.6274 | -0.5028 | 1023 | 1.2279 | 0.2189 | 1523 | 0.222 | -0.0242 |
| 24 | -1.0989 | -0.6505 | 524 | 0.377 | -1.0892 | 1024 | 0.2012 | 0.1533 | 1524 | -0.4875 | 1.2456 |
| 25 | -0.6505 | 0 | 525 | 0.2674 | 1.2566 | 1025 | 0.1768 | 0.5303 | 1525 | 0.3018 | -0.936 |
| 26 | -1.0989 | 0.6505 | 526 | 0.4434 | -0.1274 | 1026 | -0.1936 | -1.0679 | 1526 | -0.0937 | -0.3909 |
| 27 | -0.46 | -1.301 | 527 | -0.1671 | -0.9099 | 1027 | -0.5774 | -1.2458 | 1527 | -0.8418 | 0.4067 |
| 28 | 0.9531 | -0.6505 | 528 | 0.543 | -0.1811 | 1028 | -0.9144 | -0.2802 | 1528 | 0.8863 | -0.8438 |
| 29 | 0 | 0 | 529 | -0.75 | -0.75 | 1029 | -0.5359 | 0.8797 | 1529 | 0.5 | 0 |
| 30 | -0.9531 | 0.6505 | 530 | -0.9658 | -0.0364 | 1030 | 0.2703 | 0.8711 | 1530 | -0.5427 | -0.3335 |
| 31 | 0.46 | 1.301 | 531 | 0.4097 | -0.0141 | 1031 | -0.33 | -0.2903 | 1531 | 0.1013 | -0.0141 |
| 32 | 1.0989 | -0.6505 | 532 | -1.0412 | -0.322 | 1032 | 0.3665 | 0.1225 | 1532 | 0.0897 | 0.8094 |
| 33 | 0.6505 | 0 | 533 | 0.1512 | 0.0574 | 1033 | 0.9786 | 0.4786 | 1533 | -0.1153 | -0.3657 |
| 34 | 1.0989 | 0.6505 | 534 | 0.7049 | -0.1131 | 1034 | 0.5476 | 0.8578 | 1534 | -0.9697 | -0.496 |
| 35 | 0.46 | -1.301 | 535 | -0.9058 | 0.8703 | 1035 | 0.3213 | 0.6323 | 1535 | -0.7685 | 0.1056 |
| 36 | -0.9531 | -0.6505 | 536 | 0.2346 | 0.835 | 1036 | -0.1501 | -0.5993 | 1536 | 0.0207 | 0.2378 |
| 37 | 0 | 0 | 537 | 0.2714 | -0.4786 | 1037 | 0.2732 | 0.5314 | 1537 | -0.7803 | -0.3536 |
| 38 | 0.9531 | 0.6505 | 538 | -0.5515 | 0.1159 | 1038 | 0.2482 | 0.5598 | 1538 | 0.0141 | -0.3306 |
| 39 | -0.46 | 1.301 | 539 | -0.2286 | 0.6118 | 1039 | 0.0056 | -1.1233 | 1539 | 0.4437 | -0.1241 |
| 40 | -1.0989 | -0.6505 | 540 | 0.7134 | -0.0055 | 1040 | 0.275 | -0.9356 | 1540 | -0.4243 | -0.3051 |
| 41 | -0.6505 | 0 | 541 | 0.964 | 0.1272 | 1041 | -1.9142 | 1.4142 | 1541 | 0.0518 | -0.1268 |
| 42 | -1.0989 | 0.6505 | 542 | -0.2043 | 0.5779 | 1042 | -1.4964 | -0.6893 | 1542 | 0.139 | -1.3244 |
| 43 | -0.46 | -1.301 | 543 | -0.1864 | -0.461 | 1043 | 0.6753 | -0.9189 | 1543 | 0.4635 | -0.3935 |
| 44 | 0.9531 | -0.6505 | 544 | 0.4537 | -1.4 | 1044 | 1.0615 | 0.2237 | 1544 | 0.2001 | 1.4385 |
| 45 | 0 | 0 | 545 | 1 | 0 | 1045 | 0.0238 | -1.0687 | 1545 | -0.5 | 0.4571 |
| 46 | -0.9531 | 0.6505 | 546 | 0.501 | 0.6969 | 1046 | 0.6491 | -1.29 | 1546 | 0.2312 | 0.7694 |
| 47 | 0.46 | 1.301 | 547 | -1.1224 | -0.2254 | 1047 | 0.4977 | -0.1082 | 1547 | 0.1391 | 0.5106 |
| 48 | 1.0989 | -0.6505 | 548 | -1.0814 | -0.2119 | 1048 | 0.0751 | -0.1479 | 1548 | 1.2015 | -1.2846 |
| 49 | 0.6505 | 0 | 549 | -0.1882 | 0.2883 | 1049 | -0.5518 | -0.1982 | 1549 | 1.5586 | -1.2129 |
| 50 | 1.0989 | 0.6505 | 550 | 0.0736 | 0.3085 | 1050 | -0.509 | -0.0244 | 1550 | -0.8141 | -0.5768 |
| 51 | 0.46 | -1.301 | 551 | -0.5456 | -0.4174 | 1051 | 0.3506 | 0.4551 | 1551 | -0.2901 | -1.373 |
| 52 | -0.9531 | -0.6505 | 552 | -0.1083 | -0.6019 | 1052 | 0.0884 | 1.0909 | 1552 | 0.1332 | -1.3046 |
| 53 | 0 | 0 | 553 | 1.0089 | 0.7589 | 1053 | 0.6961 | -0.3134 | 1553 | -0.6339 | 1.0607 |
| 54 | 0.9531 | 0.6505 | 554 | 0.2911 | 1.1748 | 1054 | -0.0983 | -1.0174 | 1554 | 0.5345 | 0.1127 |
| 55 | -0.46 | 1.301 | 555 | -0.0376 | 0.3115 | 1055 | -0.6893 | 0.3594 | 1555 | 0.084 | -1.9763 |
| 56 | -1.0989 | -0.6505 | 556 | -0.425 | 0.5361 | 1056 | 0.7119 | 0.119 | 1556 | -0.2994 | -0.4379 |
| 57 | -0.6505 | 0 | 557 | -0.871 | 1.2005 | 1057 | 0.6768 | 0.1768 | 1557 | 0.3018 | 0.8325 |
| 58 | -1.0989 | 0.6505 | 558 | 0.1953 | 0.4533 | 1058 | 1.2595 | 0.3413 | 1558 | -0.0308 | 1.2627 |
| 59 | -0.46 | -1.301 | 559 | 0.25 | -0.135 | 1059 | 0.2883 | -1.1577 | 1559 | 0.3825 | 0.5527 |
| 60 | 0.9531 | -0.6505 | 560 | -0.959 | 0.4254 | 1060 | -0.803 | -0.5515 | 1560 | 0.1451 | 0.1383 |
| 61 | 0 | 0 | 561 | -0.75 | -0.25 | 1061 | 0.6961 | -0.0877 | 1561 | -0.5 | 0.5 |
| 62 | -0.9531 | 0.6505 | 562 | 1.6213 | -0.3351 | 1062 | 0.2958 | -0.5621 | 1562 | -0.5745 | -0.2756 |
| 63 | 0.46 | 1.301 | 563 | 0.5915 | -0.9327 | 1063 | -0.4173 | 0.6564 | 1563 | -0.4866 | 0.2461 |
| 64 | 1.0989 | -0.6505 | 564 | 0.2006 | 0.5837 | 1064 | -0.6627 | 0.007 | 1564 | 0.4982 | -0.1417 |
| 65 | 0.6505 | 0 | 565 | 0.5555 | 0.7233 | 1065 | -0.0518 | -0.6982 | 1565 | 0.4259 | -0.445 |
| 66 | 1.0989 | 0.6505 | 566 | -0.1333 | -0.9424 | 1066 | 0.0329 | 0.1007 | 1566 | -0.2009 | 0.4534 |
| 67 | 0.46 | -1.301 | 567 | 0.2771 | -0.5983 | 1067 | -1.6032 | -0.4381 | 1567 | -0.2701 | 0.0503 |
| 68 | -0.9531 | -0.6505 | 568 | -0.2692 | 0.79 | 1068 | -0.4564 | 0.2594 | 1568 | -0.7687 | 1.0792 |
| 69 | 0 | 0 | 569 | -0.9053 | 0.1982 | 1069 | 0.0238 | 0.9001 | 1569 | -0.7803 | 0 |
| 70 | 0.9531 | 0.6505 | 570 | -0.5737 | 0.2053 | 1070 | -0.111 | -0.3727 | 1570 | -0.8283 | -1.6853 |
| 71 | -0.46 | 1.301 | 571 | -0.0029 | 0.534 | 1071 | 0.4902 | -0.1793 | 1571 | -0.2498 | 0.6246 |
| 72 | -1.0989 | -0.6505 | 572 | 0.5572 | -0.5 | 1072 | -0.7099 | 0.6597 | 1572 | 0.7238 | 0.8578 |
| 73 | -0.6505 | 0 | 573 | -0.2738 | -0.9125 | 1073 | 0.2071 | 0 | 1573 | 0.0518 | -0.4767 |
| 74 | -1.0989 | 0.6505 | 574 | 0.2412 | -0.6094 | 1074 | 0.718 | -0.4613 | 1574 | -0.0684 | -0.3666 |
| 75 | -0.46 | -1.301 | 575 | 0.9646 | 0.62 | 1075 | -0.002 | 0.892 | 1575 | 0.4958 | -0.0658 |
| 76 | 0.9531 | -0.6505 | 576 | -0.1945 | 0.0298 | 1076 | 0.4238 | 0.7902 | 1576 | 0.5116 | 0.2448 |
| 77 | 0 | 0 | 577 | 0.25 | -1.75 | 1077 | 0.0798 | -0.742 | 1577 | 0.5 | 0.4571 |
| 78 | -0.9531 | 0.6505 | 578 | -0.2651 | -0.6623 | 1078 | 0.3012 | 0.589 | 1578 | -0.0917 | 0.7097 |
| 79 | 0.46 | 1.301 | 579 | -1.3472 | 0.2189 | 1079 | -0.3624 | 0.8265 | 1579 | -0.2538 | 0.7574 |
| 80 | 1.0989 | -0.6505 | 580 | 0.1179 | 0.2276 | 1080 | -0.7822 | -0.3665 | 1580 | 0.1122 | 0.6707 |
| 81 | 0.6505 | 0 | 581 | 0.0808 | 0.4902 | 1081 | 0.1982 | 0.5518 | 1581 | 0.252 | -0.0977 |
| 82 | 1.0989 | 0.6505 | 582 | -0.4816 | 0.5522 | 1082 | 0.0233 | -0.0344 | 1582 | 0.6243 | 0.0244 |
| 83 | 0.46 | -1.301 | 583 | 0.0108 | 0.2277 | 1083 | -0.2914 | -0.0201 | 1583 | 0.8287 | 0.7171 |
| 84 | -0.9531 | -0.6505 | 584 | -0.7583 | -0.603 | 1084 | -0.8343 | 1.0544 | 1584 | 0.286 | 0.4241 |
| 85 | 0 | 0 | 585 | -1.0821 | -0.4786 | 1085 | -0.0925 | -0.2902 | 1585 | -0.6339 | 0.7071 |
| 86 | 0.9531 | 0.6505 | 586 | -0.1977 | -0.3853 | 1086 | 0.6329 | -0.2973 | 1586 | -0.1568 | -0.3811 |
| 87 | -0.46 | 1.301 | 587 | 0.1769 | 0.3607 | 1087 | -0.02 | 0.2034 | 1587 | 0.222 | -0.0242 |
| 88 | -1.0989 | -0.6505 | 588 | -0.5974 | 0.6036 | 1088 | 0.7577 | -0.0506 | 1588 | -0.4875 | 1.2456 |
| 89 | -0.6505 | 0 | 589 | -0.3425 | -0.3571 | 1089 | 0.3232 | 0.5303 | 1589 | 0.3018 | -0.936 |
| 90 | -1.0989 | 0.6505 | 590 | 1.332 | 0.2587 | 1090 | -0.2741 | 0.2195 | 1590 | -0.0937 | -0.3909 |
| 91 | -0.46 | -1.301 | 591 | 0.8698 | -0.3452 | 1091 | 1.0384 | 0.4775 | 1591 | -0.8418 | 0.4067 |
| 92 | 0.9531 | -0.6505 | 592 | -0.1892 | 0.3432 | 1092 | 0.9584 | 0.362 | 1592 | 0.8863 | -0.8438 |
| 93 | 0 | 0 | 593 | 0 | 1.5 | 1093 | -0.0925 | -0.223 | 1593 | 0.5 | 0 |
| 94 | -0.9531 | 0.6505 | 594 | -0.3966 | -0.5919 | 1094 | -0.039 | 0.9799 | 1594 | -0.5427 | -0.3335 |
| 95 | 0.46 | 1.301 | 595 | -0.4382 | -0.1826 | 1095 | 0.282 | 0.3325 | 1595 | 0.1013 | -0.0141 |
| 96 | 1.0989 | -0.6505 | 596 | -0.0298 | -0.1894 | 1096 | -0.6026 | -0.2094 | 1596 | 0.0897 | 0.8094 |
| 97 | 0.6505 | 0 | 597 | 0.3409 | -1.1805 | 1097 | -0.3018 | 1.0518 | 1597 | -0.1153 | -0.3657 |
| 98 | 1.0989 | 0.6505 | 598 | 0.3403 | -0.347 | 1098 | 0.6599 | 0.1336 | 1598 | -0.9697 | -0.496 |
| 99 | 0.46 | -1.301 | 599 | -0.8378 | -0.8704 | 1099 | -0.4559 | -0.704 | 1599 | -0.7685 | 0.1056 |
| 100 | -0.9531 | -0.6505 | 600 | -0.8078 | -0.2194 | 1100 | -0.8526 | -0.8149 | 1600 | 0.0207 | 0.2378 |
| 101 | 0 | 0 | 601 | 0.1553 | 0.5518 | 1101 | 0.0798 | -1.0037 | 1601 | 0.3536 | -0.3536 |
| 102 | 0.9531 | 0.6505 | 602 | -0.1642 | 0.197 | 1102 | 0.7836 | -0.4436 | 1602 | -0.442 | 0.4763 |
| 103 | -0.46 | 1.301 | 603 | -0.2265 | -0.6133 | 1103 | 0.2191 | -0.6764 | 1603 | 0.7355 | -0.2103 |
| 104 | -1.0989 | -0.6505 | 604 | -0.1336 | -1.4482 | 1104 | -1.2015 | 0.4029 | 1604 | 1.9837 | 0.7731 |
| 105 | -0.6505 | 0 | 605 | -0.3298 | -0.0446 | 1105 | -1.9142 | 1.4142 | 1605 | 0.3739 | 1.6218 |
| 106 | -1.0989 | 0.6505 | 606 | 0.6265 | -0.1509 | 1106 | -1.4964 | -0.6893 | 1606 | -0.2014 | 0.117 |
| 107 | -0.46 | -1.301 | 607 | 0.596 | -0.1872 | 1107 | 0.6753 | -0.9189 | 1607 | 0.1229 | -0.2384 |
| 108 | 0.9531 | -0.6505 | 608 | -0.9301 | 0.712 | 1108 | 1.0615 | 0.2237 | 1608 | 0.4433 | -0.5897 |
| 109 | 0 | 0 | 609 | 0 | 0 | 1109 | 0.0238 | -1.0687 | 1609 | 1.3321 | -0.0214 |
| 110 | -0.9531 | 0.6505 | 610 | 1.2349 | 0.936 | 1110 | 0.6491 | -1.29 | 1610 | 0.07 | 0.9654 |
| 111 | 0.46 | 1.301 | 611 | 0.1939 | 0.8963 | 1111 | 0.4977 | -0.1082 | 1611 | -0.4094 | 0.0328 |
| 112 | 1.0989 | -0.6505 | 612 | 0.7787 | 0.0314 | 1112 | 0.0751 | -0.1479 | 1612 | 0.1847 | 0.448 |
| 113 | 0.6505 | 0 | 613 | 1.5227 | 0.4669 | 1113 | -0.5518 | -0.1982 | 1613 | -0.1926 | -0.134 |
| 114 | 1.0989 | 0.6505 | 614 | 0.2386 | 0.4665 | 1114 | -0.509 | -0.0244 | 1614 | -1.0129 | -1.5114 |
| 115 | 0.46 | -1.301 | 615 | -0.5715 | 0.534 | 1115 | 0.3506 | 0.4551 | 1615 | -1.2185 | 0.3601 |
| 116 | -0.9531 | -0.6505 | 616 | -0.8495 | -0.2382 | 1116 | 0.0884 | 1.0909 | 1616 | 0.8747 | 0.6872 |
| 117 | 0 | 0 | 617 | 0.3321 | 0.2286 | 1117 | 0.6961 | -0.3134 | 1617 | 0.1768 | -0.5303 |
| 118 | 0.9531 | 0.6505 | 618 | 0.7412 | 0.6362 | 1118 | -0.0983 | -1.0174 | 1618 | -0.9933 | 0.1831 |
| 119 | -0.46 | 1.301 | 619 | -0.9475 | -0.2814 | 1119 | -0.6893 | 0.3594 | 1619 | 0.2062 | 0.7803 |
| 120 | -1.0989 | -0.6505 | 620 | -0.0653 | 0.6913 | 1120 | 0.7119 | 0.119 | 1620 | -0.6677 | 0.5914 |
| 121 | -0.6505 | 0 | 621 | 0.4461 | 0.8142 | 1121 | 1.1339 | -0.1768 | 1621 | 0.5081 | -0.245 |
| 122 | -1.0989 | 0.6505 | 622 | -0.1637 | 0.7723 | 1122 | 1.1313 | 1.2471 | 1622 | 0.2672 | -0.8433 |
| 123 | -0.46 | -1.301 | 623 | 0.6909 | 0.6194 | 1123 | 1.4896 | 1.3609 | 1623 | -1.6491 | -0.6035 |
| 124 | 0.9531 | -0.6505 | 624 | -0.8299 | -0.8143 | 1124 | -0.296 | -0.1571 | 1624 | 0.3299 | -0.3792 |
| 125 | 0 | 0 | 625 | -0.75 | -0.25 | 1125 | -0.7138 | 0.2534 | 1625 | 0.0214 | -0.3321 |
| 126 | -0.9531 | 0.6505 | 626 | 1.6213 | -0.3351 | 1126 | -0.2728 | 0.8307 | 1626 | -0.4931 | -0.1019 |
| 127 | 0.46 | 1.301 | 627 | 0.5915 | -0.9327 | 1127 | -0.8852 | -0.0213 | 1627 | 0.1538 | 1.0218 |
| 128 | 1.0989 | -0.6505 | 628 | 0.2006 | 0.5837 | 1128 | -0.583 | -0.457 | 1628 | -0.4558 | 0.6282 |
| 129 | 0.6505 | 0 | 629 | 0.5555 | 0.7233 | 1129 | -0.1036 | 0.2197 | 1629 | 0.2883 | -0.4236 |
| 130 | 1.0989 | 0.6505 | 630 | -0.1333 | -0.9424 | 1130 | 0.2851 | 0.6204 | 1630 | 0.2017 | 0.8911 |
| 131 | 0.46 | -1.301 | 631 | 0.2771 | -0.5983 | 1131 | 0.1569 | 0.492 | 1631 | 0.0564 | -0.024 |
| 132 | -0.9531 | -0.6505 | 632 | -0.2692 | 0.79 | 1132 | -0.0799 | 0.2557 | 1632 | -0.2333 | -1.657 |
| 133 | 0 | 0 | 633 | -0.9053 | 0.1982 | 1133 | -0.6466 | -0.9654 | 1633 | -0.7071 | 0 |
| 134 | 0.9531 | 0.6505 | 634 | -0.5737 | 0.2053 | 1134 | -0.5333 | 0.2915 | 1634 | 0.0711 | 0.0718 |
| 135 | -0.46 | 1.301 | 635 | -0.0029 | 0.534 | 1135 | 0.8123 | 1.8979 | 1635 | 0.0705 | -0.9076 |
| 136 | -1.0989 | -0.6505 | 636 | 0.5572 | -0.5 | 1136 | 0.0169 | -0.3785 | 1636 | 0.9751 | -0.7302 |
| 137 | -0.6505 | 0 | 637 | -0.2738 | -0.9125 | 1137 | -0.0732 | -0.5303 | 1637 | 1.0832 | -0.6647 |
| 138 | -1.0989 | 0.6505 | 638 | 0.2412 | -0.6094 | 1138 | 0.8643 | 0.3542 | 1638 | -0.0837 | 0.0695 |
| 139 | -0.46 | -1.301 | 639 | 0.9646 | 0.62 | 1139 | -0.2893 | -0.4356 | 1639 | -0.2915 | 0.2358 |
| 140 | 0.9531 | -0.6505 | 640 | -0.1945 | 0.0298 | 1140 | -0.3872 | 0.3077 | 1640 | -0.7155 | -0.0002 |
| 141 | 0 | 0 | 641 | -0.5 | -0.75 | 1141 | 0.4126 | 0.645 | 1641 | 0.0821 | -0.2714 |
| 142 | -0.9531 | 0.6505 | 642 | -1.3453 | -0.8079 | 1142 | 0.2822 | -0.214 | 1642 | -0.0977 | -1.1857 |
| 143 | 0.46 | 1.301 | 643 | 0.3543 | -0.0011 | 1143 | -0.6713 | -0.7942 | 1643 | -1.5119 | -0.5714 |
| 144 | 1.0989 | -0.6505 | 644 | -0.0655 | -0.6599 | 1144 | -1.1536 | 0.1404 | 1644 | -0.8887 | 0.218 |
| 145 | 0.6505 | 0 | 645 | 0.8478 | -0.0701 | 1145 | -0.3536 | 0.7803 | 1645 | -0.0574 | 0.384 |
| 146 | 1.0989 | 0.6505 | 646 | 1.3194 | 0.5807 | 1146 | -0.0486 | -0.5349 | 1646 | -0.2881 | 1.1774 |
| 147 | 0.46 | -1.301 | 647 | 0.4496 | -0.3948 | 1147 | 0.0341 | -0.5827 | 1647 | -0.2186 | 0.2165 |
| 148 | -0.9531 | -0.6505 | 648 | 1.1167 | 0.1884 | 1148 | -0.3459 | 0.0346 | 1648 | 1.2753 | -0.9266 |
| 149 | 0 | 0 | 649 | -0.5518 | -0.1982 | 1149 | -0.0045 | 0.2328 | 1649 | 0.8839 | 0.1768 |
| 150 | 0.9531 | 0.6505 | 650 | -0.9666 | -1.1823 | 1150 | 1.1031 | -0.0628 | 1650 | -0.4963 | 0.4221 |
| 151 | -0.46 | 1.301 | 651 | 0.5378 | -0.1182 | 1151 | 0.0103 | -0.3159 | 1651 | 0.4878 | -0.1624 |
| 152 | -1.0989 | -0.6505 | 652 | 0.4216 | -0.1038 | 1152 | -0.059 | 0.3286 | 1652 | 0.0343 | 0.5951 |
| 153 | -0.6505 | 0 | 653 | 0.5238 | 0.2054 | 1153 | 0.7803 | -0.1768 | 1653 | -0.551 | 0.7021 |
| 154 | -1.0989 | 0.6505 | 654 | 0.2801 | 0.9888 | 1154 | -0.3173 | 0.2729 | 1654 | 0.0814 | -0.1137 |
| 155 | -0.46 | -1.301 | 655 | -0.3866 | -0.7397 | 1155 | -0.1672 | 0.5698 | 1655 | -0.0966 | 0.1062 |
| 156 | 0.9531 | -0.6505 | 656 | -0.215 | -1.3768 | 1156 | 0.3998 | -0.4727 | 1656 | -0.2691 | 0.1223 |
| 157 | 0 | 0 | 657 | 0.5 | 0.75 | 1157 | -0.1398 | -0.3999 | 1657 | -0.7286 | -0.0821 |
| 158 | -0.9531 | 0.6505 | 658 | 0.1364 | 1.3545 | 1158 | 0.6563 | -1.3301 | 1658 | -0.0331 | 0.1689 |
| 159 | 0.46 | 1.301 | 659 | -0.3352 | -0.2131 | 1159 | 0.5765 | -0.5616 | 1659 | 0.2675 | 0.0168 |
| 160 | 1.0989 | -0.6505 | 660 | 0.7349 | -0.6323 | 1160 | -1.3623 | 0.2202 | 1660 | -0.7515 | 0.4764 |
| 161 | 0.6505 | 0 | 661 | 0.9398 | -0.2196 | 1161 | -0.6036 | -0.7803 | 1661 | -0.0383 | 0.1736 |
| 162 | 1.0989 | 0.6505 | 662 | 0.2738 | -0.2465 | 1162 | 1.102 | 0.9416 | 1662 | 0.6216 | -0.7865 |
| 163 | 0.46 | -1.301 | 663 | 0.1904 | 0.2184 | 1163 | 0.9709 | 0.5149 | 1663 | 0.4664 | -0.0526 |
| 164 | -0.9531 | -0.6505 | 664 | 0.3874 | 0.1291 | 1164 | 0.9316 | -0.2313 | 1664 | 0.709 | -0.2568 |
| 165 | 0 | 0 | 665 | -0.0821 | -0.2286 | 1165 | 0.0859 | 0.6118 | 1665 | 0.3536 | -0.3536 |
| 166 | 0.9531 | 0.6505 | 666 | -0.457 | -0.7465 | 1166 | -0.2443 | -1.4109 | 1666 | -0.442 | 0.4763 |
| 167 | -0.46 | 1.301 | 667 | 0.1796 | -1.4307 | 1167 | 0.1497 | -1.0982 | 1667 | 0.7355 | -0.2103 |
| 168 | -1.0989 | -0.6505 | 668 | 0.1883 | 0.4316 | 1168 | -0.3842 | -0.0036 | 1668 | 1.9837 | 0.7731 |
| 169 | -0.6505 | 0 | 669 | 0.5925 | 1.0642 | 1169 | -0.4268 | -0.5303 | 1669 | 0.3739 | 1.6218 |
| 170 | -1.0989 | 0.6505 | 670 | 0.4015 | -0.4616 | 1170 | -0.9713 | 0.0497 | 1670 | -0.2014 | 0.117 |
| 171 | -0.46 | -1.301 | 671 | -0.6443 | 0.7304 | 1171 | -0.6188 | -0.788 | 1671 | 0.1229 | -0.2384 |
| 172 | 0.9531 | -0.6505 | 672 | -0.5165 | 0.7559 | 1172 | -0.1891 | -0.6018 | 1672 | 0.4433 | -0.5897 |
| 173 | 0 | 0 | 673 | -1.25 | -0.5 | 1173 | -0.9732 | 0.9156 | 1673 | 1.3321 | -0.0214 |
| 174 | -0.9531 | 0.6505 | 674 | -1.0813 | 0.4665 | 1174 | 0.0415 | 0.096 | 1674 | 0.07 | 0.9654 |
| 175 | 0.46 | 1.301 | 675 | -0.7079 | 0.2754 | 1175 | -0.0201 | -0.33 | 1675 | -0.4094 | 0.0328 |
| 176 | 1.0989 | -0.6505 | 676 | -1.2372 | -0.188 | 1176 | -0.456 | 0.479 | 1676 | 0.1847 | 0.448 |
| 177 | 0.6505 | 0 | 677 | -0.2442 | 0.5272 | 1177 | -0.3536 | -0.2197 | 1677 | -0.1926 | -0.134 |
| 178 | 1.0989 | 0.6505 | 678 | -0.2931 | 0.7052 | 1178 | -0.6314 | -0.951 | 1678 | -1.0129 | -1.5114 |
| 179 | 0.46 | -1.301 | 679 | 0.0281 | 0.4329 | 1179 | 1.2523 | 0.283 | 1679 | -1.2185 | 0.3601 |
| 180 | -0.9531 | -0.6505 | 680 | 0.5297 | -0.5243 | 1180 | 0.5525 | 0.865 | 1680 | 0.8747 | 0.6872 |
| 181 | 0 | 0 | 681 | -0.1982 | -0.5518 | 1181 | -0.8491 | 0.1207 | 1681 | 0.6339 | -1.2374 |
| 182 | 0.9531 | 0.6505 | 682 | 0.1552 | 0.6055 | 1182 | 0.3816 | -0.2004 | 1682 | -0.8861 | 1.1308 |
| 183 | -0.46 | 1.301 | 683 | -0.1843 | 0.8798 | 1183 | 0.0278 | -0.1909 | 1683 | -0.85 | 1.101 |
| 184 | -1.0989 | -0.6505 | 684 | 0.3234 | 0.2142 | 1184 | 0.5669 | -0.3292 | 1684 | -0.3513 | 0.5142 |
| 185 | -0.6505 | 0 | 685 | 0.5798 | -0.6625 | 1185 | 1.1339 | -0.1768 | 1685 | 0.3433 | 0.668 |
| 186 | -1.0989 | 0.6505 | 686 | -0.9626 | -0.0236 | 1186 | 1.1313 | 1.2471 | 1686 | -0.0812 | 1.4014 |
| 187 | -0.46 | -1.301 | 687 | -0.1673 | 1.0483 | 1187 | 1.4896 | 1.3609 | 1687 | -0.6562 | 0.9057 |
| 188 | 0.9531 | -0.6505 | 688 | 1.0747 | 0.0517 | 1188 | -0.296 | -0.1571 | 1688 | -0.0389 | -0.0311 |
| 189 | 0 | 0 | 689 | 0.75 | -1 | 1189 | -0.7138 | 0.2534 | 1689 | 0.3536 | -0.3232 |
| 190 | -0.9531 | 0.6505 | 690 | 0.1055 | -0.1843 | 1190 | -0.2728 | 0.8307 | 1690 | 0.2536 | -0.9151 |
| 191 | 0.46 | 1.301 | 691 | -0.0183 | 0.6459 | 1191 | -0.8852 | -0.0213 | 1691 | -0.112 | -0.338 |
| 192 | 1.0989 | -0.6505 | 692 | -0.6753 | -0.597 | 1192 | -0.583 | -0.457 | 1692 | -0.5836 | 0.471 |
| 193 | 0.6505 | 0 | 693 | -1.0433 | -0.7375 | 1193 | -0.1036 | 0.2197 | 1693 | -0.4965 | -0.2061 |
| 194 | 1.0989 | 0.6505 | 694 | 0.2674 | 1.3621 | 1194 | 0.2851 | 0.6204 | 1694 | -0.3258 | -0.4564 |
| 195 | 0.46 | -1.301 | 695 | -0.2539 | 0.7435 | 1195 | 0.1569 | 0.492 | 1695 | -0.2768 | 0.5475 |
| 196 | -0.9531 | -0.6505 | 696 | 0.3678 | -0.4053 | 1196 | -0.0799 | 0.2557 | 1696 | 1.052 | -0.0094 |
| 197 | 0 | 0 | 697 | 1.3321 | 0.4786 | 1197 | -0.6466 | -0.9654 | 1697 | 1.4874 | -0.8839 |
| 198 | 0.9531 | 0.6505 | 698 | -0.3753 | 0.0802 | 1198 | -0.5333 | 0.2915 | 1698 | -0.7639 | 0.1041 |
| 199 | -0.46 | 1.301 | 699 | 0.1739 | -0.038 | 1199 | 0.8123 | 1.8979 | 1699 | -0.3589 | 0.2733 |
| 200 | -1.0989 | -0.6505 | 700 | -0.1045 | 0.5351 | 1200 | 0.0169 | -0.3785 | 1700 | 1.3958 | 0.7797 |
| 201 | -0.6505 | 0 | 701 | -0.1961 | -0.1071 | 1201 | -0.5303 | 0.5303 | 1701 | -0.4516 | 0.8276 |
| 202 | -1.0989 | 0.6505 | 702 | 0.5419 | -0.491 | 1202 | -0.3525 | -0.013 | 1702 | -0.7757 | -0.2358 |
| 203 | -0.46 | -1.301 | 703 | -1.2161 | -0.0391 | 1203 | 0.3556 | -1.0579 | 1703 | 0.5487 | 0.077 |
| 204 | 0.9531 | -0.6505 | 704 | -0.3305 | 0.1812 | 1204 | -0.2295 | 0.0651 | 1704 | -0.276 | -0.308 |
| 205 | 0 | 0 | 705 | -0.5 | -0.75 | 1205 | -0.3743 | 0.4012 | 1705 | -0.3536 | 0.2197 |
| 206 | -0.9531 | 0.6505 | 706 | -1.3453 | -0.8079 | 1206 | -0.7553 | -1.0523 | 1706 | -0.0551 | 0.3914 |
| 207 | 0.46 | 1.301 | 707 | 0.3543 | -0.0011 | 1207 | -1.6344 | 0.0051 | 1707 | -0.3938 | -0.4973 |
| 208 | 1.0989 | -0.6505 | 708 | -0.0655 | -0.6599 | 1208 | 0.3033 | -0.6299 | 1708 | -0.4229 | 0.8183 |
| 209 | 0.6505 | 0 | 709 | 0.8478 | -0.0701 | 1209 | 0.9053 | -0.9053 | 1709 | -0.9722 | 0.5327 |
| 210 | 1.0989 | 0.6505 | 710 | 1.3194 | 0.5807 | 1210 | -0.9391 | 0.7791 | 1710 | -1.0468 | 0.4292 |
| 211 | 0.46 | -1.301 | 711 | 0.4496 | -0.3948 | 1211 | -0.4072 | -0.034 | 1711 | -0.5396 | 0.3808 |
| 212 | -0.9531 | -0.6505 | 712 | 1.1167 | 0.1884 | 1212 | 0.5069 | 0.1243 | 1712 | -0.3833 | -0.8847 |
| 213 | 0 | 0 | 713 | -0.5518 | -0.1982 | 1213 | -0.0555 | 0.1115 | 1713 | -0.0732 | 0.5303 |
| 214 | 0.9531 | 0.6505 | 714 | -0.9666 | -1.1823 | 1214 | -0.3317 | 0.0226 | 1714 | 0.3085 | 0.1151 |
| 215 | -0.46 | 1.301 | 715 | 0.5378 | -0.1182 | 1215 | -0.1444 | 0.726 | 1715 | 0.1941 | -0.416 |
| 216 | -1.0989 | -0.6505 | 716 | 0.4216 | -0.1038 | 1216 | -0.5611 | -0.7942 | 1716 | 0.08 | 0.7971 |
| 217 | -0.6505 | 0 | 717 | 0.5238 | 0.2054 | 1217 | -1.0607 | -0.3536 | 1717 | 0.6139 | -0.0645 |
| 218 | -1.0989 | 0.6505 | 718 | 0.2801 | 0.9888 | 1218 | -0.5099 | 1.0547 | 1718 | 0.1161 | -0.4098 |
| 219 | -0.46 | -1.301 | 719 | -0.3866 | -0.7397 | 1219 | 0.0792 | 0.7436 | 1719 | -0.2808 | -0.6288 |
| 220 | 0.9531 | -0.6505 | 720 | -0.215 | -1.3768 | 1220 | 0.7428 | 0.8556 | 1720 | 0.8487 | -0.2671 |
| 221 | 0 | 0 | 721 | -0.75 | 0.25 | 1221 | 0.5507 | 0.6453 | 1721 | 0.3536 | 0.1768 |
| 222 | -0.9531 | 0.6505 | 722 | -0.4556 | 0.0638 | 1222 | -0.7502 | -0.7817 | 1722 | 0.1073 | -0.3552 |
| 223 | 0.46 | 1.301 | 723 | 0.1309 | -0.6857 | 1223 | -0.309 | -1.8683 | 1723 | 0.6886 | -0.2095 |
| 224 | 1.0989 | -0.6505 | 724 | 0.2138 | -0.8391 | 1224 | 0.4214 | 0.0119 | 1724 | -0.1544 | -0.9222 |
| 225 | 0.6505 | 0 | 725 | 0.8235 | -0.8919 | 1225 | 0.6553 | 0.7589 | 1725 | 0.5394 | -0.3974 |
| 226 | 1.0989 | 0.6505 | 726 | 0.8588 | 0.3181 | 1226 | 0.8966 | -0.8771 | 1726 | 0.1375 | -0.1519 |
| 227 | 0.46 | -1.301 | 727 | 0.3631 | 0.8435 | 1227 | 0.1801 | -0.4463 | 1727 | -1.045 | -0.5793 |
| 228 | -0.9531 | -0.6505 | 728 | 0.516 | 0.3508 | 1228 | 0.6387 | 0.0821 | 1728 | 1.0292 | 1.3828 |
| 229 | 0 | 0 | 729 | 0.1553 | 0.3018 | 1229 | 1.0227 | 0.473 | 1729 | 0.7803 | 0.8839 |
| 230 | 0.9531 | 0.6505 | 730 | 0.9712 | -0.5524 | 1230 | 0.6087 | 0.4292 | 1730 | -0.4016 | -1.0794 |
| 231 | -0.46 | 1.301 | 731 | 0.1407 | -0.201 | 1231 | 0.1327 | -0.4792 | 1731 | 0.5148 | -1.1655 |
| 232 | -1.0989 | -0.6505 | 732 | -1.4512 | 0.1551 | 1232 | -1.7307 | -0.0701 | 1732 | -0.223 | -0.4378 |
| 233 | -0.6505 | 0 | 733 | 0.7738 | -0.2883 | 1233 | -0.8839 | 0.1768 | 1733 | 0.2016 | -0.017 |
| 234 | -1.0989 | 0.6505 | 734 | 0.4634 | 0.6783 | 1234 | 0.4807 | 0.0031 | 1734 | 1.2282 | -0.1025 |
| 235 | -0.46 | -1.301 | 735 | -1.0074 | -0.3821 | 1235 | -0.8445 | -0.2084 | 1735 | 0.4741 | 0.8532 |
| 236 | 0.9531 | -0.6505 | 736 | -0.1049 | -1.1917 | 1236 | 0.2875 | 0.0128 | 1736 | 0.1374 | -0.1232 |
| 237 | 0 | 0 | 737 | -0.25 | 0.75 | 1237 | 0.6849 | -0.0047 | 1737 | -0.3536 | -0.7803 |
| 238 | -0.9531 | 0.6505 | 738 | -0.0915 | 1.2764 | 1238 | -0.3319 | -0.0293 | 1738 | 0.0231 | 0.6082 |
| 239 | 0.46 | 1.301 | 739 | 0.0518 | 1.1138 | 1239 | 0.1836 | 1.1281 | 1739 | 0.3172 | -0.1623 |
| 240 | 1.0989 | -0.6505 | 740 | 0.1918 | 0.4751 | 1240 | -0.6194 | 0.0897 | 1740 | -0.3265 | -0.0204 |
| 241 | 0.6505 | 0 | 741 | 0.4281 | -0.0891 | 1241 | 0.1553 | -0.1553 | 1741 | 0.2222 | 0.0708 |
| 242 | 1.0989 | 0.6505 | 742 | 0.5872 | 0.2301 | 1242 | 1.1077 | 0.855 | 1742 | -0.6665 | -0.4743 |
| 243 | 0.46 | -1.301 | 743 | 0.667 | -0.4001 | 1243 | -0.273 | -0.5759 | 1743 | -1.0528 | -0.142 |
| 244 | -0.9531 | -0.6505 | 744 | -0.6902 | -0.1423 | 1244 | 0.0694 | -0.4624 | 1744 | 1.0452 | -1.7593 |
| 245 | 0 | 0 | 745 | -0.2286 | -0.2286 | 1245 | 0.1591 | -0.2151 | 1745 | 0.6339 | -1.2374 |
| 246 | 0.9531 | 0.6505 | 746 | 0.328 | -0.5995 | 1246 | 0.2483 | -0.6074 | 1746 | -0.8861 | 1.1308 |
| 247 | -0.46 | 1.301 | 747 | -0.1253 | 0.5268 | 1247 | 0.8088 | -0.0907 | 1747 | -0.85 | 1.101 |
| 248 | -1.0989 | -0.6505 | 748 | 1.0311 | -0.1122 | 1248 | -0.2704 | -0.0488 | 1748 | -0.3513 | 0.5142 |
| 249 | -0.6505 | 0 | 749 | 0.8425 | -0.0574 | 1249 | 0.3536 | 0.3536 | 1749 | 0.3433 | 0.668 |
| 250 | -1.0989 | 0.6505 | 750 | 0.0754 | 0.8898 | 1250 | 0.8817 | -0.162 | 1750 | -0.0812 | 1.4014 |
| 251 | -0.46 | -1.301 | 751 | -0.8348 | 0.3158 | 1251 | -0.2975 | 0.1085 | 1751 | -0.6562 | 0.9057 |
| 252 | 0.9531 | -0.6505 | 752 | -1.1035 | 0.7008 | 1252 | 0.158 | 0.4903 | 1752 | -0.0389 | -0.0311 |
| 253 | 0 | 0 | 753 | 0 | -0.5 | 1253 | 1.26 | -0.3346 | 1753 | 0.3536 | -0.3232 |
| 254 | -0.9531 | 0.6505 | 754 | -0.8483 | -1.2034 | 1254 | 1.3373 | 0.4394 | 1754 | 0.2536 | -0.9151 |
| 255 | 0.46 | 1.301 | 755 | -0.6017 | 0.6946 | 1255 | 0.0526 | -0.2649 | 1755 | -0.112 | -0.338 |
| 256 | 1.0989 | -0.6505 | 756 | 0.4951 | -0.1135 | 1256 | -0.2988 | -0.3544 | 1756 | -0.5836 | 0.471 |
| 257 | 0.6505 | 0 | 757 | -0.427 | -0.5652 | 1257 | 0.4053 | 1.0089 | 1757 | -0.4965 | -0.2061 |
| 258 | 1.0989 | 0.6505 | 758 | -0.6898 | 0.1942 | 1258 | -0.5653 | -0.6397 | 1758 | -0.3258 | -0.4564 |
| 259 | 0.46 | -1.301 | 759 | 0.0734 | -0.9227 | 1259 | -0.207 | -1.3581 | 1759 | -0.2768 | 0.5475 |
| 260 | -0.9531 | -0.6505 | 760 | 0.1352 | -0.8017 | 1260 | 0.8262 | -0.1679 | 1760 | 1.052 | -0.0094 |
| 261 | 0 | 0 | 761 | -0.9053 | -0.0518 | 1261 | -0.4192 | 0.3377 | 1761 | -0.6036 | 0.3536 |
| 262 | 0.9531 | 0.6505 | 762 | -0.4855 | -0.2317 | 1262 | -1.0252 | 0.5795 | 1762 | -0.4762 | 0.2229 |
| 263 | -0.46 | 1.301 | 763 | 0.6368 | -0.3851 | 1263 | -0.5042 | 0.8439 | 1763 | -0.1965 | 0.3008 |
| 264 | -1.0989 | -0.6505 | 764 | 0.7883 | -0.2406 | 1264 | -0.2443 | 0.7958 | 1764 | -0.3252 | -0.1379 |
| 265 | -0.6505 | 0 | 765 | 0.8298 | 0.0383 | 1265 | -0.5303 | 0.5303 | 1765 | -0.1273 | -0.0238 |
| 266 | -1.0989 | 0.6505 | 766 | -0.2263 | -0.4159 | 1266 | -0.3525 | -0.013 | 1766 | 0.8762 | -0.7397 |
| 267 | -0.46 | -1.301 | 767 | -0.353 | -0.7332 | 1267 | 0.3556 | -1.0579 | 1767 | 1.3059 | -0.2971 |
| 268 | 0.9531 | -0.6505 | 768 | -0.1346 | -1.3 | 1268 | -0.2295 | 0.0651 | 1768 | -0.421 | -0.6147 |
| 269 | 0 | 0 | 769 | -0.5 | -1 | 1269 | -0.3743 | 0.4012 | 1769 | -0.375 | -1.1857 |
| 270 | -0.9531 | 0.6505 | 770 | -0.6817 | 1.2333 | 1270 | -0.7553 | -1.0523 | 1770 | 0.402 | 0.0283 |
| 271 | 0.46 | 1.301 | 771 | -1.2881 | 0.5844 | 1271 | -1.6344 | 0.0051 | 1771 | -0.5232 | 0.1326 |
| 272 | 1.0989 | -0.6505 | 772 | 0.577 | -1.0584 | 1272 | 0.3033 | -0.6299 | 1772 | -0.1168 | 0.5552 |
| 273 | 0.6505 | 0 | 773 | 0.6755 | 0.0462 | 1273 | 0.9053 | -0.9053 | 1773 | 0.205 | -0.223 |
| 274 | 1.0989 | 0.6505 | 774 | -1.1441 | 0.3527 | 1274 | -0.9391 | 0.7791 | 1774 | -0.4809 | -1.5669 |
| 275 | 0.46 | -1.301 | 775 | 0.3107 | 0.4793 | 1275 | -0.4072 | -0.034 | 1775 | -0.4722 | -0.0379 |
| 276 | -0.9531 | -0.6505 | 776 | 0.4852 | 1.2876 | 1276 | 0.5069 | 0.1243 | 1776 | -0.4646 | 0.5746 |
| 277 | 0 | 0 | 777 | 0.4786 | 0.4786 | 1277 | -0.0555 | 0.1115 | 1777 | -0.6339 | -0.1768 |
| 278 | 0.9531 | 0.6505 | 778 | 0.2634 | -0.4006 | 1278 | -0.3317 | 0.0226 | 1778 | 0.0581 | 0.2723 |
| 279 | -0.46 | 1.301 | 779 | -0.945 | 0.3522 | 1279 | -0.1444 | 0.726 | 1779 | 1.3716 | 1.048 |
| 280 | -1.0989 | -0.6505 | 780 | 0.5682 | 0.7336 | 1280 | -0.5611 | -0.7942 | 1780 | 0.2887 | 0.2564 |
| 281 | -0.6505 | 0 | 781 | 0.0539 | -0.1926 | 1281 | -1.0607 | -0.5303 | 1781 | -0.9461 | -0.3425 |
| 282 | -1.0989 | 0.6505 | 782 | -0.9247 | 0.167 | 1282 | 0.5913 | -2.2691 | 1782 | 0.51 | 0.0308 |
| 283 | -0.46 | -1.301 | 783 | 0.781 | 0.7995 | 1283 | 0.0811 | -0.4603 | 1783 | 0.3751 | -0.5171 |
| 284 | 0.9531 | -0.6505 | 784 | 0.4825 | 0.0964 | 1284 | 0.086 | 0.7795 | 1784 | 0.2548 | -0.0121 |
| 285 | 0 | 0 | 785 | -0.75 | 0.25 | 1285 | 0.9539 | 0.097 | 1785 | 0.875 | 0.4786 |
| 286 | -0.9531 | 0.6505 | 786 | -0.4556 | 0.0638 | 1286 | 0.1315 | 0.4529 | 1786 | -0.5399 | 0.3311 |
| 287 | 0.46 | 1.301 | 787 | 0.1309 | -0.6857 | 1287 | 0.8155 | -0.1343 | 1787 | -0.043 | -0.2125 |
| 288 | 1.0989 | -0.6505 | 788 | 0.2138 | -0.8391 | 1288 | -0.5407 | -0.2969 | 1788 | 0.507 | -1.7043 |
| 289 | -0.6505 | 0 | 789 | 0.8235 | -0.8919 | 1289 | -1.0821 | -0.3018 | 1789 | -0.7088 | -0.3885 |
| 290 | -1.0989 | -0.6505 | 790 | 0.8588 | 0.3181 | 1290 | 0.1818 | -0.9225 | 1790 | 0.012 | 0.5853 |
| 291 | -0.46 | 1.301 | 791 | 0.3631 | 0.8435 | 1291 | -0.4342 | -0.0009 | 1791 | 0.3547 | 0.1336 |
| 292 | 0.9531 | 0.6505 | 792 | 0.516 | 0.3508 | 1292 | 0.4619 | 0.057 | 1792 | -0.4622 | 0.4521 |
| 293 | 0 | 0 | 793 | 0.1553 | 0.3018 | 1293 | -0.1194 | -0.1609 | 1793 | -0.9571 | -0.7071 |
| 294 | -0.9531 | -0.6505 | 794 | 0.9712 | -0.5524 | 1294 | -1.2634 | -0.1656 | 1794 | -0.4393 | -0.7653 |
| 295 | 0.46 | -1.301 | 795 | 0.1407 | -0.201 | 1295 | 0.3906 | -1.0262 | 1795 | 0.7234 | -1.1148 |
| 296 | 1.0989 | 0.6505 | 796 | -1.4512 | 0.1551 | 1296 | 0.5442 | 0.1137 | 1796 | -0.136 | -1.3912 |
| 297 | 0.6505 | 0 | 797 | 0.7738 | -0.2883 | 1297 | -0.5303 | 0.3536 | 1797 | -0.1834 | -0.0798 |
| 298 | 1.0989 | -0.6505 | 798 | 0.4634 | 0.6783 | 1298 | -0.1924 | -0.6398 | 1798 | 0.6958 | -0.5829 |
| 299 | 0.46 | 1.301 | 799 | -1.0074 | -0.3821 | 1299 | 0.6109 | 0.3646 | 1799 | -0.5877 | 0.3151 |
| 300 | -0.9531 | 0.6505 | 800 | -0.1049 | -1.1917 | 1300 | -0.131 | 0.1594 | 1800 | -0.6023 | 0.515 |
| 301 | 0 | 0 | 801 | -0.5 | 0.25 | 1301 | -0.0428 | 0.2693 | 1801 | 0.375 | -0.4357 |
| 302 | 0.9531 | -0.6505 | 802 | -0.3202 | 0.1747 | 1302 | 0.6369 | 1.2685 | 1802 | 0.249 | 1.144 |
| 303 | -0.46 | -1.301 | 803 | -1.1034 | -0.0021 | 1303 | -0.9171 | -0.1166 | 1803 | 0.1294 | 0.0566 |
| 304 | -1.0989 | 0.6505 | 804 | 0.1364 | 0.2026 | 1304 | -0.7903 | -0.2954 | 1804 | 0.2694 | -1.4241 |
| 305 | -0.6505 | 0 | 805 | 0.0304 | 0.9371 | 1305 | 0.5214 | 0.8018 | 1805 | 1.1057 | -0.0877 |
| 306 | -1.0989 | -0.6505 | 806 | -0.1242 | 0.1429 | 1306 | 0.3645 | 0.5467 | 1806 | 0.9682 | 0.2995 |
| 307 | -0.46 | 1.301 | 807 | -0.2426 | -0.3291 | 1307 | 0.1683 | 0.8151 | 1807 | -0.2715 | 0.2113 |
| 308 | 0.9531 | 0.6505 | 808 | -0.8903 | 0.9136 | 1308 | 0.3878 | 0.5399 | 1808 | -0.784 | 0.5347 |
| 309 | 0 | 0 | 809 | -0.7286 | -0.2714 | 1309 | 0.5687 | -0.6419 | 1809 | -0.6339 | 0.5303 |
| 310 | -0.9531 | -0.6505 | 810 | -0.6656 | -0.7258 | 1310 | -0.2562 | -0.1712 | 1810 | -0.0252 | 0.7701 |
| 311 | 0.46 | -1.301 | 811 | 0.344 | 0.948 | 1311 | 0.1113 | 0.5015 | 1811 | -0.1914 | 1.1802 |
| 312 | 1.0989 | 0.6505 | 812 | 0.5403 | 0.2809 | 1312 | 0.9442 | 0.4316 | 1812 | -0.2514 | 0.314 |
| 313 | 0.6505 | 0 | 813 | 0.5193 | -0.0339 | 1313 | 0 | 1.2374 | 1813 | -0.1575 | 0.4461 |
| 314 | 1.0989 | -0.6505 | 814 | 1.0857 | 0.1589 | 1314 | -0.4704 | 0.7551 | 1814 | -0.6581 | 0.7919 |
| 315 | 0.46 | 1.301 | 815 | -0.05 | -0.4792 | 1315 | -0.0072 | -1.0196 | 1815 | 0.6138 | -0.5009 |
| 316 | -0.9531 | 0.6505 | 816 | -0.6815 | 0.6047 | 1316 | 0.9274 | -0.5244 | 1816 | 0.6511 | 0.3052 |
| 317 | 0 | 0 | 817 | -0.25 | 1 | 1317 | 0.3567 | 0.153 | 1817 | -0.875 | -0.2714 |
| 318 | 0.9531 | -0.6505 | 818 | 0.2381 | -0.4162 | 1318 | -0.3106 | -0.5253 | 1818 | -0.2285 | -1.0034 |
| 319 | -0.46 | -1.301 | 819 | 0.1272 | -0.9576 | 1319 | 0.8969 | -0.6543 | 1819 | 0.1439 | 1.4376 |
| 320 | -1.0989 | 0.6505 | 820 | 0.4931 | -0.0878 | 1320 | 0.3378 | -0.4611 | 1820 | 0.7643 | 0.532 |
| 321 | 1.25 | 0 | 821 | 0.7772 | 0.7563 | 1321 | -0.3321 | -0.0518 | 1821 | 0.8123 | -0.7151 |
| 322 | -0.3216 | -0.5171 | 822 | -0.321 | 0.1379 | 1322 | -0.037 | -0.1536 | 1822 | -0.9231 | 0.1821 |
| 323 | -0.564 | -0.4328 | 823 | 0.8729 | -0.4917 | 1323 | -0.106 | -0.0171 | 1823 | 0.096 | 0.693 |
| 324 | 0.8594 | 0.5478 | 824 | 0.2117 | -0.1057 | 1324 | 0.4523 | 0.6783 | 1824 | 0.8282 | 1.2452 |
| 325 | 0.0111 | -0.4458 | 825 | -1.7589 | 0.5518 | 1325 | 0.0159 | -0.2962 | 1825 | -0.6036 | 0.3536 |
| 326 | -0.9756 | -0.4563 | 826 | -0.3649 | 0.4936 | 1326 | -0.9238 | -0.1452 | 1826 | -0.4762 | 0.2229 |
| 327 | -1.1903 | 0.3727 | 827 | -0.6043 | -0.6403 | 1327 | -0.4466 | 0.6996 | 1827 | -0.1965 | 0.3008 |
| 328 | -0.7761 | -0.0741 | 828 | 0.0148 | -0.3961 | 1328 | 0.2816 | -0.4208 | 1828 | -0.3252 | -0.1379 |
| 329 | 0.1036 | -0.6339 | 829 | 0.153 | 0.6321 | 1329 | -0.5303 | 0.3536 | 1829 | -0.1273 | -0.0238 |
| 330 | 0.2871 | -1.2165 | 830 | -1.0814 | 0.4243 | 1330 | 0.4056 | 1.2711 | 1830 | 0.8762 | -0.7397 |
| 331 | 0.4219 | -0.6784 | 831 | 0.9408 | 0.4458 | 1331 | 2.7295 | 0.1154 | 1831 | 1.3059 | -0.2971 |
| 332 | 0.8836 | -0.4284 | 832 | 0.2881 | 0.4621 | 1332 | 0.8659 | -0.2972 | 1832 | -0.421 | -0.6147 |
| 333 | 0.5269 | 0.2276 | 833 | -0.25 | 0 | 1333 | -0.5608 | -0.5193 | 1833 | -0.375 | -1.1857 |
| 334 | -0.6312 | 0.5158 | 834 | 1.1202 | -0.5698 | 1334 | -0.3583 | 0.2278 | 1834 | 0.402 | 0.0283 |
| 335 | -0.3382 | -0.9638 | 835 | -0.0413 | -0.7636 | 1335 | -0.7953 | 0.9052 | 1835 | -0.5232 | 0.1326 |
| 336 | 0.3459 | 0.1333 | 836 | -0.246 | -0.7034 | 1336 | -0.1062 | -0.3705 | 1836 | -0.1168 | 0.5552 |
| 337 | 0.25 | 1.25 | 837 | -0.4875 | -1.5407 | 1337 | -1.2286 | -0.4482 | 1837 | 0.205 | -0.223 |
| 338 | 1.0998 | 0.4827 | 838 | -0.2523 | -0.5558 | 1338 | -1.2577 | 0.412 | 1838 | -0.4809 | -1.5669 |
| 339 | 0.4623 | -0.2948 | 839 | 0.4223 | 0.9238 | 1339 | -0.2138 | 0.2029 | 1839 | -0.4722 | -0.0379 |
| 340 | 0.0015 | -1.1597 | 840 | -0.3731 | -0.6707 | 1340 | -0.6361 | -0.3925 | 1840 | -0.4646 | 0.5746 |
| 341 | 1.1802 | -0.4189 | 841 | -0.0214 | -0.9786 | 1341 | 0.242 | -0.3152 | 1841 | -0.8839 | 0 |
| 342 | -0.4653 | 0.3832 | 842 | 0.5854 | -0.0726 | 1342 | -0.0703 | 0.0582 | 1842 | -0.727 | 0.2676 |
| 343 | -0.6222 | 0.3373 | 843 | 1.0661 | -0.029 | 1343 | -0.0552 | -0.1748 | 1843 | 0.7192 | -0.2993 |
| 344 | 0.8113 | -0.6423 | 844 | 0.1828 | 1.1128 | 1344 | -0.2564 | 0.2993 | 1844 | 0.7677 | 0.0808 |
| 345 | -0.6036 | -1.1339 | 845 | -0.2693 | 1.1374 | 1345 | -1.0607 | -0.5303 | 1845 | -0.5563 | 0.7379 |
| 346 | -0.5391 | -0.207 | 846 | 1.5432 | 0.1468 | 1346 | 0.5913 | -2.2691 | 1846 | -0.2449 | 0.6577 |
| 347 | -0.3165 | -1.0258 | 847 | 0.0637 | -0.5034 | 1347 | 0.0811 | -0.4603 | 1847 | 0.5486 | -0.2859 |
| 348 | -0.5326 | -0.3518 | 848 | -1.2509 | -1.041 | 1348 | 0.086 | 0.7795 | 1848 | 0.1254 | -0.6719 |
| 349 | 0.2817 | 0.9077 | 849 | 0.5 | 0.25 | 1349 | 0.9539 | 0.097 | 1849 | -0.125 | 0.0518 |
| 350 | -0.5541 | -0.4205 | 850 | 0.3857 | 0.8525 | 1350 | 0.1315 | 0.4529 | 1850 | 0.6947 | -0.3826 |
| 351 | 0.147 | -0.178 | 851 | -0.1896 | -0.1908 | 1351 | 0.8155 | -0.1343 | 1851 | 0.6876 | 0.1691 |
| 352 | 0.5069 | 0.5395 | 852 | 0.148 | -0.2179 | 1352 | -0.5407 | -0.2969 | 1852 | 0.4274 | 0.5142 |
| 353 | -0.75 | 0 | 853 | 0.1799 | -0.6527 | 1353 | -1.0821 | -0.3018 | 1853 | 1.1677 | -0.2341 |
| 354 | 0.5069 | -0.5395 | 854 | 0.8148 | -1.5316 | 1354 | 0.1818 | -0.9225 | 1854 | -0.343 | 0.2796 |
| 355 | 0.147 | 0.178 | 855 | 1.1545 | -0.6029 | 1355 | -0.4342 | -0.0009 | 1855 | -1.9574 | 0.4341 |
| 356 | -0.5541 | 0.4205 | 856 | 0.0614 | 0.904 | 1356 | 0.4619 | 0.057 | 1856 | -0.6716 | 0.6353 |
| 357 | 0.2817 | -0.9077 | 857 | 0.0089 | 0.1982 | 1357 | -0.1194 | -0.1609 | 1857 | -0.3536 | 0.5303 |
| 358 | -0.5326 | 0.3518 | 858 | 0.0212 | -0.7364 | 1358 | -1.2634 | -0.1656 | 1858 | -0.6266 | -0.2263 |
| 359 | -0.3165 | 1.0258 | 859 | -0.5987 | 0.6355 | 1359 | 0.3906 | -1.0262 | 1859 | -0.4987 | -0.2304 |
| 360 | -0.5391 | 0.207 | 860 | 0.559 | 0.809 | 1360 | 0.5442 | 0.1137 | 1860 | -0.5543 | -0.1978 |
| 361 | -0.6036 | 1.1339 | 861 | 0.097 | -0.2357 | 1361 | 1.4874 | -1.0607 | 1861 | 0.5399 | -0.4218 |
| 362 | 0.8113 | 0.6423 | 862 | -0.6649 | 0.0765 | 1362 | 0.2441 | 0.082 | 1862 | 0.5699 | -0.7411 |
| 363 | -0.6222 | -0.3373 | 863 | -0.1617 | 0.0368 | 1363 | 0.1347 | 0.8622 | 1863 | -0.6214 | 0.0337 |
| 364 | -0.4653 | -0.3832 | 864 | -1.1939 | -0.067 | 1364 | 0.0907 | 0.1303 | 1864 | -1.258 | 0.7403 |
| 365 | 1.1802 | 0.4189 | 865 | -0.5 | 0.25 | 1365 | -1.2322 | -0.1218 | 1865 | -0.8321 | 0.0518 |
| 366 | 0.0015 | 1.1597 | 866 | -0.3202 | 0.1747 | 1366 | 0.19 | 0.1809 | 1866 | 0.8801 | -0.4239 |
| 367 | 0.4623 | 0.2948 | 867 | -1.1034 | -0.0021 | 1367 | 0.9158 | 0.561 | 1867 | 0.8642 | -0.0898 |
| 368 | 1.0998 | -0.4827 | 868 | 0.1364 | 0.2026 | 1368 | 0.7905 | -0.0141 | 1868 | 0.5226 | 0.9242 |
| 369 | 0.25 | -1.25 | 869 | 0.0304 | 0.9371 | 1369 | 0.7286 | -0.9053 | 1869 | 1.266 | 0.492 |
| 370 | 0.3459 | -0.1333 | 870 | -0.1242 | 0.1429 | 1370 | -0.0238 | 0.0268 | 1870 | 0.4312 | -1.0139 |
| 371 | -0.3382 | 0.9638 | 871 | -0.2426 | -0.3291 | 1371 | 0.5193 | 1.2465 | 1871 | 0.5189 | -1.2688 |
| 372 | -0.6312 | -0.5158 | 872 | -0.8903 | 0.9136 | 1372 | 0.8767 | 0.9139 | 1872 | 0.6881 | -0.2364 |
| 373 | 0.5269 | -0.2276 | 873 | -0.7286 | -0.2714 | 1373 | -0.0056 | 1.1477 | 1873 | -0.1768 | 0 |
| 374 | 0.8836 | 0.4284 | 874 | -0.6656 | -0.7258 | 1374 | -0.193 | 0.4268 | 1874 | 0.6177 | -1.339 |
| 375 | 0.4219 | 0.6784 | 875 | 0.344 | 0.948 | 1375 | 1.0811 | -1.0975 | 1875 | 0.3849 | 0.2724 |
| 376 | 0.2871 | 1.2165 | 876 | 0.5403 | 0.2809 | 1376 | 0.8354 | 0.9728 | 1876 | 0.0819 | 0.5726 |
| 377 | 0.1036 | 0.6339 | 877 | 0.5193 | -0.0339 | 1377 | -0.25 | 0.8839 | 1877 | 0.8063 | -1.5486 |
| 378 | -0.7761 | 0.0741 | 878 | 1.0857 | 0.1589 | 1378 | 0.5734 | -1.6055 | 1878 | -0.2192 | 0.7558 |
| 379 | -1.1903 | -0.3727 | 879 | -0.05 | -0.4792 | 1379 | 0.237 | -0.9335 | 1879 | -0.5375 | 0.6797 |
| 380 | -0.9756 | 0.4563 | 880 | -0.6815 | 0.6047 | 1380 | -0.3806 | -1.0783 | 1880 | -0.0338 | -0.674 |
| 381 | 0.0111 | 0.4458 | 881 | 0 | 0 | 1381 | -0.4152 | -0.4358 | 1881 | 0.125 | 0.3018 |
| 382 | 0.8594 | -0.5478 | 882 | -0.6766 | -0.9386 | 1382 | -0.8634 | 0.1658 | 1882 | 0.405 | -0.8886 |
| 383 | -0.564 | 0.4328 | 883 | 0.1426 | -0.3037 | 1383 | -0.3009 | -0.6844 | 1883 | 0.0307 | -0.2752 |
| 384 | -0.3216 | 0.5171 | 884 | -0.8266 | 0.9841 | 1384 | -0.581 | 0.833 | 1884 | 0.3646 | -0.1811 |
| 385 | 1.25 | 0 | 885 | -0.6253 | 1.1253 | 1385 | -0.2286 | -0.5089 | 1885 | -0.0035 | -0.3694 |
| 386 | -0.3216 | -0.5171 | 886 | 0.2358 | 0.1367 | 1386 | 0.7623 | -1.4569 | 1886 | -0.8022 | 0.8349 |
| 387 | -0.564 | -0.4328 | 887 | -0.2593 | -1.0642 | 1387 | 0.1091 | 1.1457 | 1887 | 0.5066 | -0.961 |
| 388 | 0.8594 | 0.5478 | 888 | 0.1653 | -0.2562 | 1388 | 0.2289 | 0.4916 | 1888 | 0.7084 | -0.0114 |
| 389 | 0.0111 | -0.4458 | 889 | 0.4053 | 0.3018 | 1389 | -0.0901 | -0.3195 | 1889 | 0 | 1.591 |
| 390 | -0.9756 | -0.4563 | 890 | -0.1525 | -0.2796 | 1390 | -0.6364 | 0.5335 | 1890 | -0.0707 | -0.2922 |
| 391 | -1.1903 | 0.3727 | 891 | 0.2305 | -0.3753 | 1391 | 0.0397 | 0.7231 | 1891 | -0.8983 | -0.4498 |
| 392 | -0.7761 | -0.0741 | 892 | 1.5309 | -0.3842 | 1392 | 0.1255 | 0.2904 | 1892 | -1.1019 | 0.3689 |
| 393 | 0.1036 | -0.6339 | 893 | 0.4167 | 0.2365 | 1393 | 0.4268 | 0 | 1893 | -0.7899 | 0.5253 |
| 394 | 0.2871 | -1.2165 | 894 | -0.1102 | -0.1295 | 1394 | -0.1817 | 0.0344 | 1894 | -0.0646 | 0.0443 |
| 395 | 0.4219 | -0.6784 | 895 | 1.2107 | -1.6825 | 1395 | -2.0243 | -0.9638 | 1895 | 0.9031 | -0.1347 |
| 396 | 0.8836 | -0.4284 | 896 | -0.0917 | -1.2367 | 1396 | -1.2467 | -0.0137 | 1896 | 0.1252 | -0.1112 |
| 397 | 0.5269 | 0.2276 | 897 | 0.75 | 0.25 | 1397 | 0.6716 | 0.4754 | 1897 | -0.5821 | 0.3018 |
| 398 | -0.6312 | 0.5158 | 898 | 1.4875 | 0.4778 | 1398 | 0.326 | -1.0396 | 1898 | -0.1732 | 0.8707 |
| 399 | -0.3382 | -0.9638 | 899 | -0.8735 | -0.2009 | 1399 | 0.091 | 0.5543 | 1899 | 0.1246 | -0.5112 |
| 400 | 0.3459 | 0.1333 | 900 | -0.1296 | -0.1562 | 1400 | 0.3882 | 0.9872 | 1900 | 0.492 | 0.3326 |
| 401 | 0.25 | 1.25 | 901 | 0.1237 | 0.3763 | 1401 | -0.0214 | -0.6553 | 1901 | 0.3982 | 0.8187 |
| 402 | 1.0998 | 0.4827 | 902 | -0.3174 | 0.2968 | 1402 | 0.356 | -0.2508 | 1902 | -0.3272 | -1.2316 |
| 403 | 0.4623 | -0.2948 | 903 | 0.8543 | 0.4276 | 1403 | -0.1023 | 0.3813 | 1903 | -0.7752 | 0.0886 |
| 404 | 0.0015 | -1.1597 | 904 | 0.1148 | 0.3519 | 1404 | -0.7895 | 0.0519 | 1904 | -0.6837 | 0.7435 |
| 405 | 1.1802 | -0.4189 | 905 | -0.2286 | 0.2286 | 1405 | -0.1409 | -0.2942 | 1905 | -0.8839 | 0 |
| 406 | -0.4653 | 0.3832 | 906 | 0.4142 | 0.0504 | 1406 | -0.1049 | 0.5445 | 1906 | -0.727 | 0.2676 |
| 407 | -0.6222 | 0.3373 | 907 | -0.263 | -0.42 | 1407 | -0.0743 | 0.2529 | 1907 | 0.7192 | -0.2993 |
| 408 | 0.8113 | -0.6423 | 908 | -1.1739 | 0.7534 | 1408 | -0.3821 | -0.6199 | 1908 | 0.7677 | 0.0808 |
| 409 | -0.6036 | -1.1339 | 909 | -0.6297 | 0.3591 | 1409 | -0.25 | 0.1768 | 1909 | -0.5563 | 0.7379 |
| 410 | -0.5391 | -0.207 | 910 | 0.4798 | -1.7205 | 1410 | 0.3642 | 0.1646 | 1910 | -0.2449 | 0.6577 |
| 411 | -0.3165 | -1.0258 | 911 | -0.1465 | -0.3491 | 1411 | -0.0545 | 0.4493 | 1911 | 0.5486 | -0.2859 |
| 412 | -0.5326 | -0.3518 | 912 | -1.2529 | 0.8542 | 1412 | 0.2301 | 0.745 | 1912 | 0.1254 | -0.6719 |
| 413 | 0.2817 | 0.9077 | 913 | -0.25 | 0.25 | 1413 | -0.4384 | 0.0822 | 1913 | -0.125 | 0.0518 |
| 414 | -0.5541 | -0.4205 | 914 | 0.6809 | 0.4392 | 1414 | -0.6526 | 0.0619 | 1914 | 0.6947 | -0.3826 |
| 415 | 0.147 | -0.178 | 915 | -0.9195 | -0.19 | 1415 | 0.0011 | -0.4308 | 1915 | 0.6876 | 0.1691 |
| 416 | 0.5069 | 0.5395 | 916 | -0.6366 | -0.6513 | 1416 | -1.1389 | -0.7162 | 1916 | 0.4274 | 0.5142 |
| 417 | -0.75 | 0 | 917 | 0.6253 | -0.1253 | 1417 | -0.4786 | -0.7589 | 1917 | 1.1677 | -0.2341 |
| 418 | 0.5069 | -0.5395 | 918 | -0.4719 | 0.6973 | 1418 | -0.0945 | -0.4089 | 1918 | -0.343 | 0.2796 |
| 419 | 0.147 | 0.178 | 919 | 0.3 | 1.0773 | 1419 | -0.2333 | 0.6406 | 1919 | -1.9574 | 0.4341 |
| 420 | -0.5541 | 0.4205 | 920 | 0.5184 | -0.0946 | 1420 | 0.9905 | 0.1736 | 1920 | -0.6716 | 0.6353 |
| 421 | 0.2817 | -0.9077 | 921 | -0.6553 | -0.0518 | 1421 | 0.2365 | -0.5341 | 1921 | -0.1768 | -0.1768 |
| 422 | -0.5326 | 0.3518 | 922 | -0.566 | 0.8343 | 1422 | -0.0657 | -0.288 | 1922 | -0.1258 | 0.2068 |
| 423 | -0.3165 | 1.0258 | 923 | -0.4916 | 0.2897 | 1423 | -0.3394 | 0.1215 | 1923 | -0.5438 | -0.7387 |
| 424 | -0.5391 | 0.207 | 924 | 0.6452 | 0.1053 | 1424 | -0.0376 | -0.3189 | 1924 | -1.2107 | -0.1258 |
| 425 | -0.6036 | 1.1339 | 925 | -0.5632 | -0.0901 | 1425 | 1.4874 | -1.0607 | 1925 | -0.5327 | 0.0933 |
| 426 | 0.8113 | 0.6423 | 926 | -1.3471 | -0.5585 | 1426 | 0.2441 | 0.082 | 1926 | 0.0915 | -0.0756 |
| 427 | -0.6222 | -0.3373 | 927 | 0.5985 | -0.5632 | 1427 | 0.1347 | 0.8622 | 1927 | -0.5814 | -0.832 |
| 428 | -0.4653 | -0.3832 | 928 | -0.1897 | -0.3782 | 1428 | 0.0907 | 0.1303 | 1928 | 0.807 | -0.3417 |
| 429 | 1.1802 | 0.4189 | 929 | 0.5 | 0.5 | 1429 | -1.2322 | -0.1218 | 1929 | 0.8536 | 0.7803 |
| 430 | 0.0015 | 1.1597 | 930 | 0.7924 | 1.0575 | 1430 | 0.19 | 0.1809 | 1930 | -0.7255 | -0.6879 |
| 431 | 0.4623 | 0.2948 | 931 | -0.5567 | 0.9017 | 1431 | 0.9158 | 0.561 | 1931 | 0.3656 | -0.6994 |
| 432 | 1.0998 | -0.4827 | 932 | 0.5568 | 0.8011 | 1432 | 0.7905 | -0.0141 | 1932 | 0.1176 | 0.6894 |
| 433 | 0.25 | -1.25 | 933 | -0.1237 | 0.6237 | 1433 | 0.7286 | -0.9053 | 1933 | 0.7129 | 0.7492 |
| 434 | 0.3459 | -0.1333 | 934 | -0.2658 | 0.0636 | 1434 | -0.0238 | 0.0268 | 1934 | 1.255 | 0.0034 |
| 435 | -0.3382 | 0.9638 | 935 | 0.6051 | -0.5265 | 1435 | 0.5193 | 1.2465 | 1935 | -0.6146 | 0.1877 |
| 436 | -0.6312 | -0.5158 | 936 | 0.3959 | -0.0549 | 1436 | 0.8767 | 0.9139 | 1936 | 0.4918 | 1.405 |
| 437 | 0.5269 | -0.2276 | 937 | 0.4786 | -0.4786 | 1437 | -0.0056 | 1.1477 | 1937 | -0.1768 | -0.1768 |
| 438 | 0.8836 | 0.4284 | 938 | -0.5658 | -1.641 | 1438 | -0.193 | 0.4268 | 1938 | -1.147 | -1.037 |
| 439 | 0.4219 | 0.6784 | 939 | -0.2687 | -0.7016 | 1439 | 1.0811 | -1.0975 | 1939 | 1.1722 | -0.2002 |
| 440 | 0.2871 | 1.2165 | 940 | 0.0337 | -0.038 | 1440 | 0.8354 | 0.9728 | 1940 | 0.751 | -0.9544 |
| 441 | 0.1036 | 0.6339 | 941 | -0.2238 | 0.4944 | 1441 | 0.4268 | 0 | 1941 | -0.1474 | 0.1519 |
| 442 | -0.7761 | 0.0741 | 942 | 0.3823 | 1.2141 | 1442 | -0.3316 | 0.3052 | 1942 | -0.7078 | 0.7978 |
| 443 | -1.1903 | -0.3727 | 943 | -0.1627 | -0.3195 | 1443 | 0.0807 | -0.2264 | 1943 | -0.8749 | -0.3524 |
| 444 | -0.9756 | 0.4563 | 944 | 0.3398 | -0.5997 | 1444 | -0.1238 | -0.0443 | 1944 | 0.2351 | -0.4928 |
| 445 | 0.0111 | 0.4458 | 945 | 0 | 0 | 1445 | -0.25 | 0.5991 | 1945 | -0.1464 | -0.1768 |
| 446 | 0.8594 | -0.5478 | 946 | -0.6766 | -0.9386 | 1446 | 1.1138 | 0.2278 | 1946 | 0.0778 | 0.4829 |
| 447 | -0.564 | 0.4328 | 947 | 0.1426 | -0.3037 | 1447 | 0.1059 | 1.2188 | 1947 | 0.2588 | 0.1616 |
| 448 | -0.3216 | 0.5171 | 948 | -0.8266 | 0.9841 | 1448 | -0.4941 | 0.8154 | 1948 | 0.3353 | -0.3618 |
| 449 | 1.25 | 0 | 949 | -0.6253 | 1.1253 | 1449 | 0.1768 | -0.6768 | 1949 | 0.945 | -0.4247 |
| 450 | -0.3216 | -0.5171 | 950 | 0.2358 | 0.1367 | 1450 | -0.1687 | 0.2863 | 1950 | -0.3833 | 0.6673 |
| 451 | -0.564 | -0.4328 | 951 | -0.2593 | -1.0642 | 1451 | -0.146 | 0.4847 | 1951 | -0.9891 | 2.1956 |
| 452 | 0.8594 | 0.5478 | 952 | 0.1653 | -0.2562 | 1452 | -0.228 | -0.2491 | 1952 | -0.4657 | 0.6128 |
| 453 | 0.0111 | -0.4458 | 953 | 0.4053 | 0.3018 | 1453 | 0.1464 | 0.0923 | 1953 | -0.1768 | 0.1768 |
| 454 | -0.9756 | -0.4563 | 954 | -0.1525 | -0.2796 | 1454 | 0.2005 | -0.0101 | 1954 | 0.0431 | 0.5691 |
| 455 | -1.1903 | 0.3727 | 955 | 0.2305 | -0.3753 | 1455 | -0.0248 | -0.6194 | 1955 | -0.4567 | -0.8986 |
| 456 | -0.7761 | -0.0741 | 956 | 1.5309 | -0.3842 | 1456 | 0.6483 | -0.3495 | 1956 | -0.4264 | -0.0361 |
| 457 | 0.1036 | -0.6339 | 957 | 0.4167 | 0.2365 | 1457 | -0.0732 | 0.3536 | 1957 | -0.0708 | 0.3639 |
| 458 | 0.2871 | -1.2165 | 958 | -0.1102 | -0.1295 | 1458 | 0.0278 | 0.939 | 1958 | -0.2156 | -0.479 |
| 459 | 0.4219 | -0.6784 | 959 | 1.2107 | -1.6825 | 1459 | 1.4508 | 0.5805 | 1959 | -0.511 | -0.0824 |
| 460 | 0.8836 | -0.4284 | 960 | -0.0917 | -1.2367 | 1460 | 0.3096 | -0.7283 | 1960 | -0.2589 | 0.2819 |
| 461 | 0.5269 | 0.2276 | 961 | 0.1768 | 0.5303 | 1461 | -0.4571 | -0.1895 | 1961 | -0.1464 | 0.2803 |
| 462 | -0.6312 | 0.5158 | 962 | -0.1936 | -1.0679 | 1462 | 0.4466 | 0.249 | 1962 | -0.8335 | -0.7488 |
| 463 | -0.3382 | -0.9638 | 963 | -0.5774 | -1.2458 | 1463 | 0.2013 | -0.7617 | 1963 | -0.863 | 0.024 |
| 464 | 0.3459 | 0.1333 | 964 | -0.9144 | -0.2802 | 1464 | 0.8698 | 0.0939 | 1964 | -1.0967 | 1.5492 |
| 465 | 0.25 | 1.25 | 965 | -0.5359 | 0.8797 | 1465 | 0.6768 | 0.9268 | 1965 | -0.4023 | 0.208 |
| 466 | 1.0998 | 0.4827 | 966 | 0.2703 | 0.8711 | 1466 | -1.5685 | 0.1626 | 1966 | 1.3362 | 0.1108 |
| 467 | 0.4623 | -0.2948 | 967 | -0.33 | -0.2903 | 1467 | -0.4047 | -0.7329 | 1967 | 0.4841 | 0.3186 |
| 468 | 0.0015 | -1.1597 | 968 | 0.3665 | 0.1225 | 1468 | 0.7748 | -0.8739 | 1968 | -0.4248 | -1.3239 |
| 469 | 1.1802 | -0.4189 | 969 | 0.9786 | 0.4786 | 1469 | -0.1464 | -0.5811 | 1969 | 0.5303 | -0.5303 |
| 470 | -0.4653 | 0.3832 | 970 | 0.5476 | 0.8578 | 1470 | -0.2962 | -0.72 | 1970 | 0.8417 | 0.4144 |
| 471 | -0.6222 | 0.3373 | 971 | 0.3213 | 0.6323 | 1471 | -1.1736 | 0.1311 | 1971 | 0.3283 | -0.7838 |
| 472 | 0.8113 | -0.6423 | 972 | -0.1501 | -0.5993 | 1472 | 0.4634 | 0.5279 | 1972 | 0.1155 | 0.0391 |
| 473 | -0.6036 | -1.1339 | 973 | 0.2732 | 0.5314 | 1473 | 1.1339 | -0.3536 | 1973 | 0.0439 | 0.8052 |
| 474 | -0.5391 | -0.207 | 974 | 0.2482 | 0.5598 | 1474 | -1.6651 | -0.5069 | 1974 | -0.2452 | -0.0138 |
| 475 | -0.3165 | -1.0258 | 975 | 0.0056 | -1.1233 | 1475 | -0.9459 | 0.0056 | 1975 | 0.0531 | 0.4739 |
| 476 | -0.5326 | -0.3518 | 976 | 0.275 | -0.9356 | 1476 | -0.3367 | 0.2824 | 1976 | 0.3701 | 0.1647 |
| 477 | 0.2817 | 0.9077 | 977 | 0.3536 | -0.3536 | 1477 | -0.25 | -0.2455 | 1977 | 0.8536 | -0.1768 |
| 478 | -0.5541 | -0.4205 | 978 | -0.0495 | -0.0312 | 1478 | 0.9207 | 1.0654 | 1978 | 0.8691 | -0.1995 |
| 479 | 0.147 | -0.178 | 979 | -0.5463 | -1.3031 | 1479 | -0.2385 | 1.2706 | 1979 | -0.2615 | -1.1075 |
| 480 | 0.5069 | 0.5395 | 980 | 0.691 | -1.4344 | 1480 | -0.1118 | -1.2791 | 1980 | 0.4143 | 0.2003 |
| 481 | 1 | 0 | 981 | 0.4133 | 0.5203 | 1481 | 0.1768 | -0.1768 | 1981 | 0.8657 | 0.8818 |
| 482 | 0.501 | 0.6969 | 982 | -0.2897 | -0.8886 | 1482 | -0.7123 | 0.4327 | 1982 | -0.1306 | -0.0109 |
| 483 | -1.1224 | -0.2254 | 983 | 0.3749 | -1.0862 | 1483 | -0.2953 | -0.3177 | 1983 | 0.2053 | -0.4948 |
| 484 | -1.0814 | -0.2119 | 984 | -0.6814 | 0.4839 | 1484 | 0.3306 | 1.0566 | 1984 | 0.2454 | -1.306 |
| 485 | -0.1882 | 0.2883 | 985 | 0.2286 | -0.8321 | 1485 | 0.1464 | 0.5542 | 1985 | -0.1768 | -0.1768 |
| 486 | 0.0736 | 0.3085 | 986 | 0.5834 | -0.3325 | 1486 | -1.2277 | 0.6094 | 1986 | -0.1258 | 0.2068 |
| 487 | -0.5456 | -0.4174 | 987 | -0.5678 | 0.0372 | 1487 | -0.4958 | 0.6849 | 1987 | -0.5438 | -0.7387 |
| 488 | -0.1083 | -0.6019 | 988 | 1.0765 | -0.823 | 1488 | 0.5363 | -0.4722 | 1988 | -1.2107 | -0.1258 |
| 489 | 1.0089 | 0.7589 | 989 | 0.9392 | 1.0599 | 1489 | -0.7803 | 0 | 1989 | -0.5327 | 0.0933 |
| 490 | 0.2911 | 1.1748 | 990 | -0.0029 | 1.2232 | 1490 | -0.3502 | -0.6423 | 1990 | 0.0915 | -0.0756 |
| 491 | -0.0376 | 0.3115 | 991 | -0.3246 | 0.1395 | 1491 | -0.0856 | -1.5667 | 1991 | -0.5814 | -0.832 |
| 492 | -0.425 | 0.5361 | 992 | -0.8996 | 0.8551 | 1492 | -0.678 | -0.9876 | 1992 | 0.807 | -0.3417 |
| 493 | -0.871 | 1.2005 | 993 | -0.1768 | 0.1768 | 1493 | -0.4571 | 0.543 | 1993 | 0.8536 | 0.7803 |
| 494 | 0.1953 | 0.4533 | 994 | -0.1653 | -0.8903 | 1494 | -0.1111 | 1.242 | 1994 | -0.7255 | -0.6879 |
| 495 | 0.25 | -0.135 | 995 | 0.5823 | -0.1284 | 1495 | 0.4314 | 0.4794 | 1995 | 0.3656 | -0.6994 |
| 496 | -0.959 | 0.4254 | 996 | 0.8774 | 0.617 | 1496 | -0.2514 | 0.816 | 1996 | 0.1176 | 0.6894 |
| 497 | -1 | 0 | 997 | -0.6712 | 0.4738 | 1497 | -0.3232 | -0.0732 | 1997 | 0.7129 | 0.7492 |
| 498 | 0.162 | -0.6567 | 998 | -0.5245 | 0.4303 | 1498 | 0.3544 | -0.5624 | 1998 | 1.255 | 0.0034 |
| 499 | -0.6929 | -0.4479 | 999 | 0.08 | 0.1487 | 1499 | 0.346 | 0.3588 | 1999 | -0.6146 | 0.1877 |
| 500 | -1.5225 | -0.2303 | 1000 | -0.2085 | -0.0948 | 1500 | 0.3657 | -0.8701 | 2000 | 0.4918 | 1.405 |