IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | OFDM Frame Example |  |
| Date Submitted | 25 March 2011 |  |
| Source | [Emmanuel Monnerie] [Landis+Gyr] | Voice: [ +1 678 641 1986 ] Fax: [ ] E-mail: [ Emmanuel . monnerie @ landisgyr . com ] |
| Re: | Comment Resolution for TG4g Draft D3 |  |
| Abstract | This document presents an example of OFDM Frame, built from the payload all the way to the time domain samples. It is similar to the 802.11-2007 Annex G. |  |
| Purpose | Proposed resolution for Comments ID 1780 through 1783 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Annex J

(informative)

An example of encoding a frame for OFDM PHY

J.1 Introduction

The purpose of this annex is to show an example of encoding a frame for the OFDM PHY, as described in

Clause 6.12b. This example covers all the encoding details defined by this standard.

The encoding illustration goes through the following stages:

- Generating the short training sequence section of the preamble;

- Generating the long preamble sequence section of the preamble;

- Generating the OFDM Header and the corresponding HCS;

- Setting the six tail bits to zeros;

- Encoding the header with a convolutional encoder and puncturing;

- Interleaving the header;

- The PSDU from the MAC should already contain an FCS, which is assumed to be four bytes here.  Six tail bits and PAD bits are appended to form the data field.

- Scrambling the data field;

- Resetting the six tail bits back to zeros;

- Encoding the data with a convolutional encoder and puncturing;

- Interleaving the data field;

- Mapping into complex symbols;

- Frequency Spreading;

- Concatenating the preamble, the OFDM header and the data field;

- Pilot, guard and DC tone insertion;

- Transforming from frequency to time domain using the IFFT and adding a circular prefix;

- Concatenating the OFDM symbols into a single, time-domain signal.

In the description of time domain waveforms, a complex baseband signal at 666.666 ksample/s is used. This example uses the 400 kbps data rate (QPSK  ½ rate modulation), which corresponds to OFDM Option 2 and MCS level 3, and a message of 72 octets. The OFDM Header uses the 50 kbps data rate (BPSK 1/2 rate coded with 4x frequency repetition), which corresponds to MCS level 0. This example uses 'PIB attribute phyOFDMInterleaving = 0'.

J.2 The Message

The message being encoded consists of the first 72 characters of the well-known “Ode to Joy” (“An die Freude”) by F. Schiller, in its original version (in German).

Freude, schöner Götterfunken,

Tochter aus Elysium,

Wir betreten feuertrunken,

Himmlische dein Heiligtum.

The message is converted to ASCII and a CRC32 is added as defined in clause 5.2.1.9;

Note: the MAC header will not be included. It is assumed to be part of the message in this annex J.

The resulting 76 octets PSDU is shown in Table J.1.

Table J.1 – The message

| Octet # | Value (Hex) | Octet # | Value (Hex) | Octet # | Value (Hex) | Octet # | Value (Hex) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 46 | 21 | 65 | 41 | 73 | 61 | 74 |
| 2 | 72 | 22 | 72 | 42 | 20 | 62 | 65 |
| 3 | 65 | 23 | 66 | 43 | 45 | 63 | 6E |
| 4 | 75 | 24 | 75 | 44 | 6C | 64 | 20 |
| 5 | 64 | 25 | 6E | 45 | 79 | 65 | 66 |
| 6 | 65 | 26 | 6B | 46 | 73 | 66 | 65 |
| 7 | 2C | 27 | 65 | 47 | 69 | 67 | 75 |
| 8 | 20 | 28 | 6E | 48 | 75 | 68 | 65 |
| 9 | 73 | 29 | 2C | 49 | 6D | 69 | 72 |
| 10 | 63 | 30 | A | 50 | 2C | 70 | 74 |
| 11 | 68 | 31 | 54 | 51 | A | 71 | 72 |
| 12 | F6 | 32 | 6F | 52 | 57 | 72 | 75 |
| 13 | 6E | 33 | 63 | 53 | 69 | 73 | 33 |
| 14 | 65 | 34 | 68 | 54 | 72 | 74 | 3C |
| 15 | 72 | 35 | 74 | 55 | 20 | 75 | 69 |
| 16 | 20 | 36 | 65 | 56 | 62 | 76 | 7C |
| 17 | 47 | 37 | 72 | 57 | 65 |  |  |
| 18 | F6 | 38 | 20 | 58 | 74 |  |  |
| 19 | 74 | 39 | 61 | 59 | 72 |  |  |
| 20 | 74 | 40 | 75 | 60 | 65 |  |  |

Note: to extract the bit stream from this table, the octet must be read LSB first.

J.3 Generation of the OFDM Header

J.3.1 HCS and PAD bits insertion

In this example the payload data has a size of 76 octets and it will be encoded using QPSK modulation, ½ rate coding (Rate field value = 3). The scrambler index will be the first one (Scrambler ID = 0).

The corresponding OFDM Header, including the HCS is represented in table J.2

Table J.2 – OFDM Header

| Field Name | Bit # | Bit Value |  | Field Name | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- |
| Rate | 1 | 0 |  | Scrambler | 20 | 0 |
|  | 2 | 0 |  |  | 21 | 0 |
|  | 3 | 0 |  | RFU | 22 | 0 |
|  | 4 | 1 |  | HCS | 23 | 1 |
|  | 5 | 1 |  |  | 24 | 1 |
| RFU | 6 | 0 |  |  | 25 | 1 |
| Length | 7 | 0 |  |  | 26 | 1 |
|  | 8 | 0 |  |  | 27 | 1 |
|  | 9 | 0 |  |  | 28 | 1 |
|  | 10 | 0 |  |  | 29 | 0 |
|  | 11 | 1 |  |  | 30 | 1 |
|  | 12 | 0 |  | Tail | 31 | 0 |
|  | 13 | 0 |  |  | 32 | 0 |
|  | 14 | 1 |  |  | 33 | 0 |
|  | 15 | 1 |  |  | 34 | 0 |
|  | 16 | 0 |  |  | 35 | 0 |
|  | 17 | 0 |  |  | 36 | 0 |
| RFU | 18 | 0 |  |  |  |  |
|  | 19 | 0 |  |  |  |  |

In this configuration, no extra PAD bit is necessary. The size of the header will fill up exactly 6 OFDM symbols.

J.3.2 Convolutional Encoding

After convolutional encoding of the OFDM header, the size is now doubled and the corresponding bits are represented in table J.3a. No puncturing is applied in this configuration.

Table J.3a – OFDM Header after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 19 | 0 | 37 | 1 | 55 | 0 |
| 2 | 0 | 20 | 1 | 38 | 0 | 56 | 0 |
| 3 | 0 | 21 | 0 | 39 | 0 | 57 | 0 |
| 4 | 0 | 22 | 0 | 40 | 1 | 58 | 0 |
| 5 | 0 | 23 | 0 | 41 | 1 | 59 | 1 |
| 6 | 0 | 24 | 1 | 42 | 1 | 60 | 0 |
| 7 | 1 | 25 | 1 | 43 | 0 | 61 | 1 |
| 8 | 1 | 26 | 1 | 44 | 0 | 62 | 1 |
| 9 | 1 | 27 | 0 | 45 | 1 | 63 | 1 |
| 10 | 0 | 28 | 0 | 46 | 1 | 64 | 0 |
| 11 | 1 | 29 | 1 | 47 | 1 | 65 | 1 |
| 12 | 0 | 30 | 0 | 48 | 0 | 66 | 0 |
| 13 | 0 | 31 | 0 | 49 | 0 | 67 | 1 |
| 14 | 0 | 32 | 0 | 50 | 1 | 68 | 1 |
| 15 | 1 | 33 | 1 | 51 | 1 | 69 | 1 |
| 16 | 1 | 34 | 1 | 52 | 0 | 70 | 0 |
| 17 | 1 | 35 | 1 | 53 | 1 | 71 | 1 |
| 18 | 0 | 36 | 1 | 54 | 0 | 72 | 1 |

J.3.3 Interleaving

A 2-step interleaver is applied to the data, as defined in 6.2.3.5. The resulting data is represented in table J.3b.

Table J.3b – OFDM Header after interleaving

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 19 | 1 | 37 | 1 | 55 | 0 |
| 2 | 0 | 20 | 0 | 38 | 1 | 56 | 1 |
| 3 | 1 | 21 | 1 | 39 | 0 | 57 | 1 |
| 4 | 0 | 22 | 0 | 40 | 1 | 58 | 0 |
| 5 | 0 | 23 | 0 | 41 | 0 | 59 | 0 |
| 6 | 0 | 24 | 1 | 42 | 1 | 60 | 0 |
| 7 | 1 | 25 | 1 | 43 | 0 | 61 | 1 |
| 8 | 1 | 26 | 0 | 44 | 1 | 62 | 0 |
| 9 | 0 | 27 | 0 | 45 | 0 | 63 | 1 |
| 10 | 0 | 28 | 1 | 46 | 1 | 64 | 0 |
| 11 | 1 | 29 | 1 | 47 | 1 | 65 | 1 |
| 12 | 0 | 30 | 1 | 48 | 0 | 66 | 1 |
| 13 | 0 | 31 | 0 | 49 | 0 | 67 | 1 |
| 14 | 1 | 32 | 1 | 50 | 0 | 68 | 1 |
| 15 | 0 | 33 | 0 | 51 | 0 | 69 | 1 |
| 16 | 0 | 34 | 0 | 52 | 0 | 70 | 0 |
| 17 | 0 | 35 | 1 | 53 | 1 | 71 | 1 |
| 18 | 1 | 36 | 1 | 54 | 1 | 72 | 1 |

J.3.4 Bit mapping

The 72 bits are split into 6 OFDM symbols. The bit mapping for the OFDM header in this example is BPSK, therefore Q is always 0. The I value is mapped as defined in figure 129.

Table J.4 – Bit mapping for the OFDM Header

|  | Symbol 0 |  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 2 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 3 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 4 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 5 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 6 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 7 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 8 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 9 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 10 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 11 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 12 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |

J.3.5 Frequency Spreading

In this example, a frequency spreading of 4 is applied to the OFDM header. The original 12 bits in each symbol (Bin # 25 through 36 in table J.5) are duplicated within the same symbol. The resulting symbols have 48 data bits each. The duplicated bits have a phase rotation as defined in 6.2.3.6.

Table J.5 –OFDM Header in the frequency domain

|  | Symbol 0 |  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 2 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 3 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 4 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 5 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 6 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 7 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 8 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 9 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 10 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 11 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 |
| 12 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 13 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 14 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 15 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 16 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 17 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 18 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 19 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 20 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 21 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 22 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 23 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 24 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 25 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 26 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 27 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 28 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 29 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 30 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 31 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 32 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 33 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 34 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 35 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 36 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 37 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 |
| 38 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 39 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 40 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 41 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 42 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 43 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 44 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 45 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 46 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 47 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 |
| 48 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |

J.4 Generation of the data Symbols

J.4.1 PAD Insertion and Data Scrambling

The original 76 octets of data, as defined in table J.0, are concatenated with 6 zero tail bits and 10 pad bits (as calculated by the formulas in 6.2.3.10). The 6 tail bits are forced to 0 after scrambling. The resulting 624 bits are represented in table J.6 (First and last 48 bits only).

Table J.6 – First and last 48 bits after pad insertion and scrambling

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 0 | 577 | 0 | 601 | 1 |
| 2 | 0 | 26 | 0 | 578 | 0 | 602 | 0 |
| 3 | 0 | 27 | 0 | 579 | 1 | 603 | 0 |
| 4 | 0 | 28 | 0 | 580 | 0 | 604 | 0 |
| 5 | 0 | 29 | 0 | 581 | 0 | 605 | 0 |
| 6 | 1 | 30 | 0 | 582 | 1 | 606 | 0 |
| 7 | 0 | 31 | 0 | 583 | 1 | 607 | 0 |
| 8 | 0 | 32 | 1 | 584 | 0 | 608 | 0 |
| 9 | 0 | 33 | 0 | 585 | 0 | 609 | 0 |
| 10 | 1 | 34 | 0 | 586 | 1 | 610 | 0 |
| 11 | 0 | 35 | 0 | 587 | 1 | 611 | 0 |
| 12 | 1 | 36 | 0 | 588 | 0 | 612 | 0 |
| 13 | 1 | 37 | 0 | 589 | 1 | 613 | 0 |
| 14 | 1 | 38 | 0 | 590 | 1 | 614 | 0 |
| 15 | 0 | 39 | 1 | 591 | 0 | 615 | 0 |
| 16 | 0 | 40 | 1 | 592 | 0 | 616 | 0 |
| 17 | 1 | 41 | 0 | 593 | 1 | 617 | 0 |
| 18 | 0 | 42 | 1 | 594 | 0 | 618 | 0 |
| 19 | 0 | 43 | 1 | 595 | 1 | 619 | 0 |
| 20 | 0 | 44 | 0 | 596 | 1 | 620 | 1 |
| 21 | 1 | 45 | 1 | 597 | 1 | 621 | 1 |
| 22 | 1 | 46 | 0 | 598 | 1 | 622 | 0 |
| 23 | 0 | 47 | 0 | 599 | 0 | 623 | 1 |
| 24 | 1 | 48 | 0 | 600 | 0 | 624 | 1 |

J.4.2 Convolutional Encoding and puncturing

After convolutional encoding of the payload, the size is now doubled and the corresponding bits are represented in table J.7. No puncturing is applied in this configuration.

Table J.7 – First and last 48 bits after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 0 | 1201 | 0 | 1225 | 0 |
| 2 | 1 | 26 | 1 | 1202 | 1 | 1226 | 0 |
| 3 | 0 | 27 | 0 | 1203 | 0 | 1227 | 0 |
| 4 | 1 | 28 | 1 | 1204 | 0 | 1228 | 0 |
| 5 | 1 | 29 | 1 | 1205 | 1 | 1229 | 0 |
| 6 | 1 | 30 | 1 | 1206 | 0 | 1230 | 0 |
| 7 | 1 | 31 | 1 | 1207 | 0 | 1231 | 0 |
| 8 | 1 | 32 | 1 | 1208 | 0 | 1232 | 0 |
| 9 | 0 | 33 | 1 | 1209 | 0 | 1233 | 0 |
| 10 | 0 | 34 | 0 | 1210 | 0 | 1234 | 0 |
| 11 | 0 | 35 | 0 | 1211 | 1 | 1235 | 0 |
| 12 | 1 | 36 | 0 | 1212 | 0 | 1236 | 0 |
| 13 | 1 | 37 | 1 | 1213 | 1 | 1237 | 0 |
| 14 | 0 | 38 | 0 | 1214 | 1 | 1238 | 0 |
| 15 | 1 | 39 | 0 | 1215 | 0 | 1239 | 1 |
| 16 | 1 | 40 | 0 | 1216 | 0 | 1240 | 1 |
| 17 | 1 | 41 | 1 | 1217 | 0 | 1241 | 1 |
| 18 | 1 | 42 | 1 | 1218 | 0 | 1242 | 0 |
| 19 | 1 | 43 | 0 | 1219 | 0 | 1243 | 1 |
| 20 | 1 | 44 | 0 | 1220 | 0 | 1244 | 0 |
| 21 | 1 | 45 | 0 | 1221 | 0 | 1245 | 1 |
| 22 | 1 | 46 | 1 | 1222 | 0 | 1246 | 1 |
| 23 | 1 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 1 | 48 | 1 | 1224 | 0 | 1248 | 1 |

J.4.3 Interleaving

A 2-step interleaver is applied to the data, as defined in 6.2.3.5. The resulting data (first and last 48 bits only) is represented in table J.8.

Table J.8 – First and last 48 bits after interleaving

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 1201 | 0 | 1225 | 0 |
| 2 | 1 | 26 | 1 | 1202 | 0 | 1226 | 0 |
| 3 | 0 | 27 | 1 | 1203 | 0 | 1227 | 1 |
| 4 | 1 | 28 | 0 | 1204 | 0 | 1228 | 0 |
| 5 | 1 | 29 | 1 | 1205 | 0 | 1229 | 0 |
| 6 | 0 | 30 | 1 | 1206 | 0 | 1230 | 0 |
| 7 | 1 | 31 | 1 | 1207 | 0 | 1231 | 0 |
| 8 | 0 | 32 | 0 | 1208 | 1 | 1232 | 1 |
| 9 | 1 | 33 | 1 | 1209 | 0 | 1233 | 1 |
| 10 | 0 | 34 | 1 | 1210 | 0 | 1234 | 1 |
| 11 | 1 | 35 | 1 | 1211 | 1 | 1235 | 1 |
| 12 | 0 | 36 | 1 | 1212 | 0 | 1236 | 1 |
| 13 | 0 | 37 | 1 | 1213 | 0 | 1237 | 1 |
| 14 | 0 | 38 | 0 | 1214 | 0 | 1238 | 0 |
| 15 | 0 | 39 | 1 | 1215 | 0 | 1239 | 0 |
| 16 | 1 | 40 | 1 | 1216 | 0 | 1240 | 0 |
| 17 | 0 | 41 | 1 | 1217 | 1 | 1241 | 1 |
| 18 | 1 | 42 | 1 | 1218 | 0 | 1242 | 1 |
| 19 | 0 | 43 | 1 | 1219 | 1 | 1243 | 1 |
| 20 | 0 | 44 | 1 | 1220 | 1 | 1244 | 0 |
| 21 | 0 | 45 | 0 | 1221 | 0 | 1245 | 0 |
| 22 | 1 | 46 | 1 | 1222 | 0 | 1246 | 0 |
| 23 | 1 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 0 | 48 | 0 | 1224 | 1 | 1248 | 1 |

J.4.4 Bit mapping

The bit mapping for the OFDM header in this example is QPSK (2 data bits per vector). The I and Q vectors are mapped as defined in figure 129. The resulting data is represented in table J.9 (first two and last two symbols).

Table J.9 – Bit mapping for the OFDM payload

| Vector # | I | Q | Vector # | I | Q | Vector # | I | Q | Vector # | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.707 | 0.707 | 49 | 0.707 | -0.707 | 529 | -0.707 | -0.707 | 577 | -0.707 | 0.707 |
| 2 | -0.707 | 0.707 | 50 | -0.707 | 0.707 | 530 | -0.707 | 0.707 | 578 | -0.707 | -0.707 |
| 3 | 0.707 | -0.707 | 51 | -0.707 | -0.707 | 531 | -0.707 | 0.707 | 579 | -0.707 | 0.707 |
| 4 | 0.707 | -0.707 | 52 | 0.707 | -0.707 | 532 | -0.707 | -0.707 | 580 | -0.707 | -0.707 |
| 5 | 0.707 | -0.707 | 53 | 0.707 | -0.707 | 533 | -0.707 | 0.707 | 581 | 0.707 | -0.707 |
| 6 | 0.707 | -0.707 | 54 | -0.707 | 0.707 | 534 | -0.707 | 0.707 | 582 | -0.707 | 0.707 |
| 7 | -0.707 | -0.707 | 55 | -0.707 | -0.707 | 535 | 0.707 | 0.707 | 583 | 0.707 | 0.707 |
| 8 | -0.707 | 0.707 | 56 | 0.707 | -0.707 | 536 | -0.707 | 0.707 | 584 | -0.707 | -0.707 |
| 9 | -0.707 | 0.707 | 57 | 0.707 | -0.707 | 537 | -0.707 | -0.707 | 585 | -0.707 | -0.707 |
| 10 | -0.707 | -0.707 | 58 | 0.707 | 0.707 | 538 | -0.707 | -0.707 | 586 | -0.707 | 0.707 |
| 11 | -0.707 | 0.707 | 59 | 0.707 | -0.707 | 539 | 0.707 | 0.707 | 587 | -0.707 | -0.707 |
| 12 | 0.707 | -0.707 | 60 | -0.707 | -0.707 | 540 | -0.707 | 0.707 | 588 | -0.707 | 0.707 |
| 13 | 0.707 | 0.707 | 61 | -0.707 | -0.707 | 541 | 0.707 | 0.707 | 589 | -0.707 | -0.707 |
| 14 | 0.707 | -0.707 | 62 | 0.707 | -0.707 | 542 | -0.707 | -0.707 | 590 | 0.707 | -0.707 |
| 15 | 0.707 | 0.707 | 63 | 0.707 | -0.707 | 543 | -0.707 | 0.707 | 591 | -0.707 | -0.707 |
| 16 | 0.707 | -0.707 | 64 | -0.707 | -0.707 | 544 | -0.707 | 0.707 | 592 | -0.707 | 0.707 |
| 17 | 0.707 | 0.707 | 65 | -0.707 | 0.707 | 545 | -0.707 | 0.707 | 593 | 0.707 | 0.707 |
| 18 | 0.707 | 0.707 | 66 | -0.707 | -0.707 | 546 | -0.707 | -0.707 | 594 | -0.707 | -0.707 |
| 19 | 0.707 | -0.707 | 67 | -0.707 | 0.707 | 547 | -0.707 | 0.707 | 595 | 0.707 | -0.707 |
| 20 | 0.707 | 0.707 | 68 | 0.707 | 0.707 | 548 | 0.707 | 0.707 | 596 | -0.707 | 0.707 |
| 21 | 0.707 | 0.707 | 69 | -0.707 | 0.707 | 549 | 0.707 | 0.707 | 597 | -0.707 | 0.707 |
| 22 | 0.707 | 0.707 | 70 | 0.707 | 0.707 | 550 | 0.707 | -0.707 | 598 | -0.707 | 0.707 |
| 23 | -0.707 | 0.707 | 71 | 0.707 | 0.707 | 551 | 0.707 | -0.707 | 599 | -0.707 | -0.707 |
| 24 | 0.707 | -0.707 | 72 | 0.707 | 0.707 | 552 | -0.707 | 0.707 | 600 | -0.707 | -0.707 |
| 25 | 0.707 | 0.707 | 73 | 0.707 | -0.707 | 553 | -0.707 | -0.707 | 601 | -0.707 | -0.707 |
| 26 | 0.707 | -0.707 | 74 | -0.707 | -0.707 | 554 | 0.707 | 0.707 | 602 | -0.707 | -0.707 |
| 27 | 0.707 | 0.707 | 75 | 0.707 | -0.707 | 555 | 0.707 | 0.707 | 603 | -0.707 | -0.707 |
| 28 | 0.707 | -0.707 | 76 | 0.707 | 0.707 | 556 | 0.707 | 0.707 | 604 | -0.707 | 0.707 |
| 29 | 0.707 | 0.707 | 77 | -0.707 | -0.707 | 557 | -0.707 | -0.707 | 605 | -0.707 | -0.707 |
| 30 | 0.707 | -0.707 | 78 | 0.707 | 0.707 | 558 | -0.707 | 0.707 | 606 | 0.707 | -0.707 |
| 31 | 0.707 | 0.707 | 79 | 0.707 | 0.707 | 559 | -0.707 | 0.707 | 607 | -0.707 | -0.707 |
| 32 | -0.707 | 0.707 | 80 | -0.707 | 0.707 | 560 | 0.707 | 0.707 | 608 | -0.707 | -0.707 |
| 33 | -0.707 | 0.707 | 81 | 0.707 | -0.707 | 561 | -0.707 | -0.707 | 609 | 0.707 | -0.707 |
| 34 | 0.707 | -0.707 | 82 | -0.707 | 0.707 | 562 | -0.707 | -0.707 | 610 | 0.707 | 0.707 |
| 35 | 0.707 | 0.707 | 83 | 0.707 | -0.707 | 563 | -0.707 | -0.707 | 611 | -0.707 | -0.707 |
| 36 | 0.707 | -0.707 | 84 | 0.707 | 0.707 | 564 | -0.707 | -0.707 | 612 | -0.707 | 0.707 |
| 37 | -0.707 | 0.707 | 85 | -0.707 | 0.707 | 565 | 0.707 | 0.707 | 613 | -0.707 | -0.707 |
| 38 | -0.707 | 0.707 | 86 | 0.707 | 0.707 | 566 | 0.707 | 0.707 | 614 | 0.707 | -0.707 |
| 39 | -0.707 | 0.707 | 87 | -0.707 | 0.707 | 567 | -0.707 | 0.707 | 615 | -0.707 | -0.707 |
| 40 | -0.707 | 0.707 | 88 | -0.707 | 0.707 | 568 | -0.707 | -0.707 | 616 | -0.707 | 0.707 |
| 41 | -0.707 | 0.707 | 89 | -0.707 | 0.707 | 569 | 0.707 | 0.707 | 617 | 0.707 | 0.707 |
| 42 | -0.707 | 0.707 | 90 | 0.707 | 0.707 | 570 | -0.707 | -0.707 | 618 | 0.707 | 0.707 |
| 43 | 0.707 | -0.707 | 91 | -0.707 | -0.707 | 571 | -0.707 | -0.707 | 619 | 0.707 | -0.707 |
| 44 | 0.707 | 0.707 | 92 | -0.707 | 0.707 | 572 | 0.707 | 0.707 | 620 | -0.707 | -0.707 |
| 45 | 0.707 | 0.707 | 93 | -0.707 | 0.707 | 573 | 0.707 | -0.707 | 621 | 0.707 | 0.707 |
| 46 | -0.707 | 0.707 | 94 | 0.707 | 0.707 | 574 | 0.707 | -0.707 | 622 | 0.707 | -0.707 |
| 47 | 0.707 | -0.707 | 95 | -0.707 | 0.707 | 575 | 0.707 | 0.707 | 623 | -0.707 | -0.707 |
| 48 | 0.707 | -0.707 | 96 | -0.707 | -0.707 | 576 | 0.707 | -0.707 | 624 | -0.707 | 0.707 |

J.4.5 Frequency Spreading

In this example, no frequency spreading is applied to the OFDM payload. The vectors from table J.9 are mapped directly into the frequency domain. The next paragraphs are illustrating the mapping in the frequency domain, taking into account the pilot tones, the DC tone and the guard tones.

J.5 Conversion from frequency domain to time domain

J.5.1 Pilot, DC and Guard tone insertion

The following steps are applied to both the OFDM header and the OFDM payload. Before going to the next steps, table J.5 and J.9 should be appended, resulting in 19 symbols of 48 bins in the frequency domain. The 48 bins are mapped in the frequency domain by inserting pilot tones, guard tones and a DC tone, as defined in 6.2.3.7.

Table J.10 Complete Packet in the frequency domain (first and last three symbols)

| Subcarrier | Symbol 0 | Symbol 1 | Symbol 2 | Symbol 16 | Symbol 17 | Symbol 18 |
| --- | --- | --- | --- | --- | --- | --- |
| -32 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -31 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -30 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -29 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -28 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -27 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| -26 | 0 - 1i | 0 - 1i | 0 + 1i | -0.707 + 0.707i | 1 + 0i | -0.707 + 0.707i |
| -25 | 0 + 1i | 0 - 1i | 0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| -24 | 0 + 1i | 0 - 1i | 0 - 1i | 0.707 + 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -23 | 0 + 1i | 0 + 1i | 0 - 1i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| -22 | 0 - 1i | 1 + 0i | 0 + 1i | 0.707 - 0.707i | -0.707 - 0.707i | 1 + 0i |
| -21 | 0 + 1i | 0 - 1i | 0 - 1i | 0.707 - 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -20 | 0 + 1i | 0 - 1i | 0 - 1i | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -19 | 0 - 1i | 0 + 1i | 0 - 1i | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 + 0.707i |
| -18 | 0 - 1i | 0 + 1i | -1 + 0i | 1 + 0i | -0.707 + 0.707i | -0.707 - 0.707i |
| -17 | 0 + 1i | 0 + 1i | 0 - 1i | 0.707 + 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| -16 | 0 + 1i | 0 + 1i | 0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| -15 | 0 + 1i | 0 - 1i | 0 + 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| -14 | -1 + 0i | 0 - 1i | 0 - 1i | 0.707 - 0.707i | 1 + 0i | -0.707 + 0.707i |
| -13 | 1 + 0i | 1 + 0i | -1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| -12 | 0 - 1i | 0 + 1i | 0 - 1i | -0.707 + 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| -11 | 1 + 0i | -1 + 0i | -1 + 0i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| -10 | 0 + 1i | 1 + 0i | 0 - 1i | 0.707 + 0.707i | -0.707 + 0.707i | -1 + 0i |
| -9 | 1 + 0i | 0 + 1i | -1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -8 | 0 - 1i | 1 + 0i | 0 + 1i | 0.707 + 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| -7 | 1 + 0i | 0 + 1i | -1 + 0i | 0.707 + 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| -6 | 0 - 1i | 1 + 0i | 1 + 0i | -1 + 0i | -0.707 + 0.707i | 0.707 - 0.707i |
| -5 | 1 + 0i | 0 + 1i | 0 - 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| -4 | 0 - 1i | -1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| -3 | 1 + 0i | 0 - 1i | 0 - 1i | 0.707 - 0.707i | 0.707 - 0.707i | -0.707 + 0.707i |
| -2 | -1 + 0i | -1 + 0i | 1 + 0i | 0.707 - 0.707i | -1 + 0i | -0.707 - 0.707i |
| -1 | 0 + 1i | 0 - 1i | 0 - 1i | 0.707 + 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| 0 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 1 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 + 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 2 | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 3 | 1 + 0i | 1 + 0i | -1 + 0i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 4 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| 5 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 6 | -1 + 0i | -1 + 0i | 1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 - 0.707i |
| 7 | 1 + 0i | 1 + 0i | 1 + 0i | 0.707 + 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 8 | 1 + 0i | 1 + 0i | -1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 9 | -1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| 10 | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 + 0.707i | -0.707 - 0.707i | -1 + 0i |
| 11 | -1 + 0i | -1 + 0i | -1 + 0i | 0.707 - 0.707i | -0.707 - 0.707i | 0.707 + 0.707i |
| 12 | 1 + 0i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 13 | -1 + 0i | 1 + 0i | 1 + 0i | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 14 | -1 + 0i | 1 + 0i | 1 + 0i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 15 | 0 - 1i | -1 + 0i | 0 - 1i | 0.707 + 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| 16 | -1 + 0i | 0 + 1i | 1 + 0i | 0.707 + 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 17 | 0 + 1i | 1 + 0i | 0 - 1i | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 18 | -1 + 0i | 0 + 1i | 1 + 0i | 1 + 0i | 0.707 + 0.707i | 0.707 + 0.707i |
| 19 | 0 - 1i | -1 + 0i | 1 + 0i | -0.707 + 0.707i | -0.707 - 0.707i | 0.707 + 0.707i |
| 20 | -1 + 0i | 0 + 1i | 0 + 1i | 0.707 - 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| 21 | 0 - 1i | -1 + 0i | 1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 22 | -1 + 0i | 0 + 1i | 0 - 1i | -0.707 - 0.707i | 0.707 - 0.707i | 1 + 0i |
| 23 | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 + 0.707i | 0.707 - 0.707i | 0.707 + 0.707i |
| 24 | 0 - 1i | 0 - 1i | 0 - 1i | 0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| 25 | -1 + 0i | 1 + 0i | -1 + 0i | 0.707 + 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| 26 | 0 + 1i | 0 - 1i | 0 - 1i | -0.707 + 0.707i | -1 + 0i | -0.707 + 0.707i |
| 27 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 28 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 29 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 30 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |
| 31 | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i | 0 + 0i |

Note: in table J.10, the pilot tones are represented in bold and the DC/Guard Tones are represented in italic.

J.5.2 Time domain OFDM Header and Payload

The data from table X10 is converted by an IFFT of size 64. Most IFFTs require a reordering of the data. Typically, the order of the frequencies within each symbol should be as follows:

0, 1, 2, … 31, -32, -31, …, -1

After the IFFT, each symbol is extended by a cyclic prefix of 16 samples. Each OFDM symbol then has a size of 80 samples (64 +16).

The resulting data in the time domain has 1520 samples (19*80) and is represented in paragraph J.7 (complete packet).

J.6 Generation of the preamble

J.6.1 Generation of the STF

As defined in clause 6.2.1.1, the STF consists of 4 symbols. The resulting STF is equivalent to 20 repetitions of the following 16-sample time domain pattern:

Table J.11 – STF time domain pattern

| Sample # | I | Q |
| --- | --- | --- |
| 1 | 0.6505 | 0 |
| 2 | 1.0989 | 0.6505 |
| 3 | 0.46 | -1.301 |
| 4 | -0.9531 | -0.6505 |
| 5 | 0 | 0 |
| 6 | 0.9531 | 0.6505 |
| 7 | -0.46 | 1.301 |
| 8 | -1.0989 | -0.6505 |
| 9 | -0.6505 | 0 |
| 10 | -1.0989 | 0.6505 |
| 11 | -0.46 | -1.301 |
| 12 | 0.9531 | -0.6505 |
| 13 | 0 | 0 |
| 14 | -0.9531 | 0.6505 |
| 15 | 0.46 | 1.301 |
| 16 | 1.0989 | -0.6505 |

Note: the last 2 repetitions are negated.

J.6.2 Generation of the LTF

As defined in clause 6.2.1.2, the LTF consists of the following time domain pattern:

Table J.12 – LTF in time domain

| Sample # | I | Q | Sample # | I | Q |
| --- | --- | --- | --- | --- | --- |
| 33 | -0.75 | 0 | 65 | 1.25 | 0 |
| 34 | 0.5069 | -0.5395 | 66 | -0.3216 | -0.5171 |
| 35 | 0.147 | 0.178 | 67 | -0.564 | -0.4328 |
| 36 | -0.5541 | 0.4205 | 68 | 0.8594 | 0.5478 |
| 37 | 0.2817 | -0.9077 | 69 | 0.0111 | -0.4458 |
| 38 | -0.5326 | 0.3518 | 70 | -0.9756 | -0.4563 |
| 39 | -0.3165 | 1.0258 | 71 | -1.1903 | 0.3727 |
| 40 | -0.5391 | 0.207 | 72 | -0.7761 | -0.0741 |
| 41 | -0.6036 | 1.1339 | 73 | 0.1036 | -0.6339 |
| 42 | 0.8113 | 0.6423 | 74 | 0.2871 | -1.2165 |
| 43 | -0.6222 | -0.3373 | 75 | 0.4219 | -0.6784 |
| 44 | -0.4653 | -0.3832 | 76 | 0.8836 | -0.4284 |
| 45 | 1.1802 | 0.4189 | 77 | 0.5269 | 0.2276 |
| 46 | 0.0015 | 1.1597 | 78 | -0.6312 | 0.5158 |
| 47 | 0.4623 | 0.2948 | 79 | -0.3382 | -0.9638 |
| 48 | 1.0998 | -0.4827 | 80 | 0.3459 | 0.1333 |
| 49 | 0.25 | -1.25 | 81 | 0.25 | 1.25 |
| 50 | 0.3459 | -0.1333 | 82 | 1.0998 | 0.4827 |
| 51 | -0.3382 | 0.9638 | 83 | 0.4623 | -0.2948 |
| 52 | -0.6312 | -0.5158 | 84 | 0.0015 | -1.1597 |
| 53 | 0.5269 | -0.2276 | 85 | 1.1802 | -0.4189 |
| 54 | 0.8836 | 0.4284 | 86 | -0.4653 | 0.3832 |
| 55 | 0.4219 | 0.6784 | 87 | -0.6222 | 0.3373 |
| 56 | 0.2871 | 1.2165 | 88 | 0.8113 | -0.6423 |
| 57 | 0.1036 | 0.6339 | 89 | -0.6036 | -1.1339 |
| 58 | -0.7761 | 0.0741 | 90 | -0.5391 | -0.207 |
| 59 | -1.1903 | -0.3727 | 91 | -0.3165 | -1.0258 |
| 60 | -0.9756 | 0.4563 | 92 | -0.5326 | -0.3518 |
| 61 | 0.0111 | 0.4458 | 93 | 0.2817 | 0.9077 |
| 62 | 0.8594 | -0.5478 | 94 | -0.5541 | -0.4205 |
| 63 | -0.564 | 0.4328 | 95 | 0.147 | -0.178 |
| 64 | -0.3216 | 0.5171 | 96 | 0.5069 | 0.5395 |

The samples 1 through 32 are part of the cyclic prefix for the LTF. They are simply a copy of samples 65 through 96. The samples 97 through 160 are a repetition of samples 33 through 96.

J.7 The entire packet

The complete packet in the time domain is represented in table J.13. The STF is from sample 1 to 320. The LTF is from sample 321 to 480. The OFDM header and payload is from sample 481 to 2000.

Table J.13 Complete Packet

| Sample | I | Q | Sample | I | Q | Sample | I | Q | Sample | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.6505 | 0 | 501 | -0.3152 | -0.0032 | 1001 | -0.2589 | 0.125 | 1501 | 1.0879 | -0.1802 |
| 2 | 1.0989 | 0.6505 | 502 | -0.7019 | 0.7465 | 1002 | 0.0957 | -0.0742 | 1502 | 0.5504 | -1.0303 |
| 3 | 0.46 | -1.301 | 503 | 0.3034 | -0.3115 | 1003 | 1.3264 | -0.645 | 1503 | -0.353 | -1.0804 |
| 4 | -0.9531 | -0.6505 | 504 | 0.2617 | -0.0489 | 1004 | 0.3176 | -0.1546 | 1504 | -0.1305 | 0.2871 |
| 5 | 0 | 0 | 505 | -0.4053 | 0.6553 | 1005 | 0.3753 | -0.1902 | 1505 | 0.0732 | 0.7071 |
| 6 | 0.9531 | 0.6505 | 506 | 0.9183 | -0.0648 | 1006 | 0.1755 | 0.693 | 1506 | -0.1505 | 0.1106 |
| 7 | -0.46 | 1.301 | 507 | 0.4545 | 0.023 | 1007 | -0.1535 | 1.4903 | 1507 | 0.7487 | -0.2539 |
| 8 | -1.0989 | -0.6505 | 508 | -0.1544 | 0.4795 | 1008 | 0.2928 | -0.4478 | 1508 | 0.5702 | -0.3195 |
| 9 | -0.6505 | 0 | 509 | -0.865 | 0.7468 | 1009 | 0.3536 | -0.8839 | 1509 | 0.2407 | -0.4889 |
| 10 | -1.0989 | 0.6505 | 510 | -0.4766 | 0.1583 | 1010 | 0.3706 | 0.447 | 1510 | 1.4049 | 0.235 |
| 11 | -0.46 | -1.301 | 511 | 0.7769 | -0.9179 | 1011 | -1.0583 | 0.5522 | 1511 | 0.267 | 0.8823 |
| 12 | 0.9531 | -0.6505 | 512 | -0.3158 | -0.6323 | 1012 | 0.014 | 0.5397 | 1512 | -0.1098 | 0.5457 |
| 13 | 0 | 0 | 513 | 1 | -0.5 | 1013 | 0.6459 | -0.24 | 1513 | 0 | 0.4571 |
| 14 | -0.9531 | 0.6505 | 514 | 1.1294 | -0.9673 | 1014 | -1.6167 | -1.0935 | 1514 | -1.0648 | 0.1166 |
| 15 | 0.46 | 1.301 | 515 | -0.475 | -0.0292 | 1015 | -0.6615 | -0.6868 | 1515 | -0.1968 | 0.4275 |
| 16 | 1.0989 | -0.6505 | 516 | 0.7496 | 1.1095 | 1016 | -0.4196 | -0.3639 | 1516 | -0.2262 | 0.5005 |
| 17 | 0.6505 | 0 | 517 | 0.2962 | -0.2072 | 1017 | -0.9053 | 0.3321 | 1517 | -0.1371 | -0.9889 |
| 18 | 1.0989 | 0.6505 | 518 | 0.61 | -1.2555 | 1018 | -0.5822 | 0.2493 | 1518 | 0.9962 | -0.8207 |
| 19 | 0.46 | -1.301 | 519 | 0.9438 | 0.2525 | 1019 | -0.9506 | -0.6197 | 1519 | 0.7019 | 0.0296 |
| 20 | -0.9531 | -0.6505 | 520 | -0.3988 | 0.1504 | 1020 | 0.4131 | 0.1353 | 1520 | -0.2743 | 0.2598 |
| 21 | 0 | 0 | 521 | 0.125 | -0.5821 | 1021 | -0.2272 | 0.0676 | 1521 | 0.6036 | 0 |
| 22 | 0.9531 | 0.6505 | 522 | 0.2185 | 0.1985 | 1022 | -0.5896 | -0.5819 | 1522 | -0.7102 | 0.0582 |
| 23 | -0.46 | 1.301 | 523 | 0.723 | -0.2595 | 1023 | 0.7024 | 0.5376 | 1523 | -0.8069 | 0.0666 |
| 24 | -1.0989 | -0.6505 | 524 | 0.7028 | 0.3073 | 1024 | -0.9078 | 0.6814 | 1524 | 0.7677 | 0.5468 |
| 25 | -0.6505 | 0 | 525 | -0.1721 | 1.0608 | 1025 | -0.8839 | 0.3536 | 1525 | 0.2524 | 0.3515 |
| 26 | -1.0989 | 0.6505 | 526 | 0.1245 | -0.6063 | 1026 | -0.0336 | 0.8687 | 1526 | 0.0091 | 1.0895 |
| 27 | -0.46 | -1.301 | 527 | -0.0681 | -0.9558 | 1027 | -0.7799 | 0.1728 | 1527 | -0.0781 | 1.2938 |
| 28 | 0.9531 | -0.6505 | 528 | 0.0339 | -0.7114 | 1028 | 0.1891 | -0.6159 | 1528 | 0.2749 | 0.1756 |
| 29 | 0 | 0 | 529 | -1.25 | -0.75 | 1029 | 0.6734 | -0.4006 | 1529 | 0.4268 | -0.7071 |
| 30 | -0.9531 | 0.6505 | 530 | -1.6118 | 0.2379 | 1030 | 0.2461 | 0.05 | 1530 | -0.6077 | -1.4293 |
| 31 | 0.46 | 1.301 | 531 | 0.1999 | 0.4937 | 1031 | 1.184 | 0.4273 | 1531 | 0.563 | -0.6991 |
| 32 | 1.0989 | -0.6505 | 532 | -0.5909 | 0.2326 | 1032 | 0.4654 | 0.1181 | 1532 | 0.8327 | -0.8248 |
| 33 | 0.6505 | 0 | 533 | -0.6419 | -0.6004 | 1033 | -1.5089 | -0.125 | 1533 | -0.4415 | -1.0406 |
| 34 | 1.0989 | 0.6505 | 534 | 0.3993 | -0.7404 | 1034 | -0.2443 | -0.146 | 1534 | -0.1301 | -0.7419 |
| 35 | 0.46 | -1.301 | 535 | 0.1517 | 0.8183 | 1035 | 0.7714 | -0.6208 | 1535 | -0.196 | -1.2772 |
| 36 | -0.9531 | -0.6505 | 536 | 0.3911 | 0.5603 | 1036 | -0.6118 | -0.675 | 1536 | 0.6182 | 0.3049 |
| 37 | 0 | 0 | 537 | 0.6553 | -0.4053 | 1037 | -0.8753 | -0.5169 | 1537 | 0.8107 | 0.3536 |
| 38 | 0.9531 | 0.6505 | 538 | 0.2064 | 0.0022 | 1038 | -0.5449 | -0.4082 | 1538 | -0.257 | -1.3845 |
| 39 | -0.46 | 1.301 | 539 | -0.1681 | -0.023 | 1039 | 0.4193 | 0.2374 | 1539 | 0.5271 | -0.5753 |
| 40 | -1.0989 | -0.6505 | 540 | 0.6483 | -0.2591 | 1040 | 1.6786 | 0.8281 | 1540 | -0.0303 | 0.2553 |
| 41 | -0.6505 | 0 | 541 | 0.115 | 0.1496 | 1041 | -1.3839 | -0.1768 | 1541 | -0.6329 | -0.3224 |
| 42 | -1.0989 | 0.6505 | 542 | -0.2533 | 0.6345 | 1042 | -0.6704 | 0.5355 | 1542 | 0.3853 | -0.5783 |
| 43 | -0.46 | -1.301 | 543 | 1.0385 | -0.2063 | 1043 | 0.5092 | -0.4634 | 1543 | 0.2062 | 1.2063 |
| 44 | 0.9531 | -0.6505 | 544 | 0.207 | -0.7164 | 1044 | -0.9994 | 0.5225 | 1544 | 0.5346 | 1.3008 |
| 45 | 0 | 0 | 545 | 0.75 | 0.75 | 1045 | 0.1117 | 0.5856 | 1545 | 0.7803 | -0.3964 |
| 46 | -0.9531 | 0.6505 | 546 | 0.7478 | 0.8804 | 1046 | 0.6351 | 0.1888 | 1546 | 0.1151 | 0.3834 |
| 47 | 0.46 | 1.301 | 547 | -1.8473 | 0.0292 | 1047 | -0.6357 | -0.2362 | 1547 | -0.634 | 0.429 |
| 48 | 1.0989 | -0.6505 | 548 | -1.0324 | 0.3448 | 1048 | -0.1339 | 0.1905 | 1548 | -0.6766 | 0.0394 |
| 49 | 0.6505 | 0 | 549 | 0.1609 | 0.3108 | 1049 | -0.7286 | 0.9786 | 1549 | 0.7144 | 0.1713 |
| 50 | 1.0989 | 0.6505 | 550 | 0.1388 | -0.4451 | 1050 | -0.7351 | -0.3456 | 1550 | -0.0891 | -0.0131 |
| 51 | 0.46 | -1.301 | 551 | -0.106 | -1.0522 | 1051 | 0.1451 | -0.4809 | 1551 | -0.4727 | 0.2019 |
| 52 | -0.9531 | -0.6505 | 552 | -0.8661 | -0.2156 | 1052 | -0.1864 | 0.2875 | 1552 | 1.1123 | -0.8002 |
| 53 | 0 | 0 | 553 | 0.125 | 0.8321 | 1053 | 0.0341 | 0.222 | 1553 | -0.1036 | -0.3536 |
| 54 | 0.9531 | 0.6505 | 554 | 0.1346 | 0.4 | 1054 | -0.7802 | -0.3408 | 1554 | 0.1428 | 0.8731 |
| 55 | -0.46 | 1.301 | 555 | -0.5951 | 0.2595 | 1055 | -0.2768 | -0.5178 | 1555 | 0.6424 | 0.5755 |
| 56 | -1.0989 | -0.6505 | 556 | -0.1195 | 0.4088 | 1056 | 1.9581 | 0.5282 | 1556 | -1.5436 | 0.1857 |
| 57 | -0.6505 | 0 | 557 | -0.5779 | 0.5428 | 1057 | 0.5 | 0.3536 | 1557 | -0.2095 | -0.9551 |
| 58 | -1.0989 | 0.6505 | 558 | -0.255 | 0.5079 | 1058 | -0.7115 | 0.0462 | 1558 | 0.6815 | -0.8154 |
| 59 | -0.46 | -1.301 | 559 | 0.9598 | 0.3728 | 1059 | 0.7485 | 0.6712 | 1559 | -0.7935 | 0.195 |
| 60 | 0.9531 | -0.6505 | 560 | -0.313 | 1.1997 | 1060 | 1.0929 | 0.5474 | 1560 | 0.0674 | 0.3238 |
| 61 | 0 | 0 | 561 | -0.5 | 0.5 | 1061 | 1.014 | -0.4339 | 1561 | -0.0732 | -0.2071 |
| 62 | -0.9531 | 0.6505 | 562 | -0.0269 | 0.5835 | 1062 | 0.7941 | -1.4585 | 1562 | -0.3058 | -0.3181 |
| 63 | 0.46 | 1.301 | 563 | 0.3762 | 0.8987 | 1063 | 0.2306 | -1.0711 | 1563 | 1.041 | 1.3381 |
| 64 | 1.0989 | -0.6505 | 564 | -0.3916 | -0.3543 | 1064 | 0.234 | -0.4601 | 1564 | 0.2113 | 0.5691 |
| 65 | 0.6505 | 0 | 565 | -1.6311 | 0.0394 | 1065 | 0.0214 | -0.125 | 1565 | -1.0156 | -0.77 |
| 66 | 1.0989 | 0.6505 | 566 | -0.1295 | 0.5707 | 1066 | -0.3034 | 0.1486 | 1566 | -0.5914 | -0.0458 |
| 67 | 0.46 | -1.301 | 567 | 0.8723 | -0.5243 | 1067 | 0.1528 | 0.2707 | 1567 | -0.4842 | -0.3492 |
| 68 | -0.9531 | -0.6505 | 568 | 0.4785 | -0.803 | 1068 | 0.3789 | 0.3591 | 1568 | -0.6408 | 0.3395 |
| 69 | 0 | 0 | 569 | 0.7286 | -0.625 | 1069 | -0.2942 | -0.6443 | 1569 | 0.1036 | 0.7071 |
| 70 | 0.9531 | 0.6505 | 570 | 0.4153 | 0.5426 | 1070 | -0.142 | -0.377 | 1570 | 0.8656 | -0.1366 |
| 71 | -0.46 | 1.301 | 571 | 0.4098 | -0.5751 | 1071 | 0.2126 | 0.3483 | 1571 | 0.3444 | -0.3598 |
| 72 | -1.0989 | -0.6505 | 572 | 0.3124 | -1.6609 | 1072 | -0.2751 | 0.0058 | 1572 | -1.0004 | -0.398 |
| 73 | -0.6505 | 0 | 573 | -0.2547 | 0.4696 | 1073 | 0.3839 | 0.1768 | 1573 | -0.8242 | 0.2188 |
| 74 | -1.0989 | 0.6505 | 574 | -0.4437 | -0.331 | 1074 | 0.7601 | -0.1322 | 1574 | 0.7306 | 0.0209 |
| 75 | -0.46 | -1.301 | 575 | -0.8561 | -0.5979 | 1075 | -0.3385 | 0.5893 | 1575 | -0.0417 | 0.0119 |
| 76 | 0.9531 | -0.6505 | 576 | -0.1274 | 0.6406 | 1076 | -0.1832 | 0.437 | 1576 | -0.9181 | 0.3307 |
| 77 | 0 | 0 | 577 | 0.25 | 0.25 | 1077 | 0.7418 | -0.5856 | 1577 | 0.2803 | 0.6036 |
| 78 | -0.9531 | 0.6505 | 578 | -0.3854 | 0.0544 | 1078 | 0.3944 | 1.0648 | 1578 | -0.2429 | 1.5395 |
| 79 | 0.46 | 1.301 | 579 | 0.2915 | -1.0776 | 1079 | -0.0763 | 1.0342 | 1579 | -0.263 | 0.6391 |
| 80 | 1.0989 | -0.6505 | 580 | -0.1043 | -0.2086 | 1080 | 0.6077 | -0.1183 | 1580 | 0.4391 | 0.0407 |
| 81 | 0.6505 | 0 | 581 | -0.7177 | 0.1911 | 1081 | 0.0214 | -0.2714 | 1581 | -0.6715 | -0.482 |
| 82 | 1.0989 | 0.6505 | 582 | 0.0456 | -0.867 | 1082 | -1.594 | -0.9907 | 1582 | 0.004 | -1.3302 |
| 83 | 0.46 | -1.301 | 583 | -0.6118 | 0.5262 | 1083 | -0.5865 | -0.3521 | 1583 | 0.4458 | 0.1316 |
| 84 | -0.9531 | -0.6505 | 584 | -0.5278 | 0.3099 | 1084 | 0.2947 | 0.8838 | 1584 | -0.0485 | 0.4391 |
| 85 | 0 | 0 | 585 | 1.0518 | 0.5518 | 1085 | -0.1805 | 0.278 | 1585 | 0.6036 | 0 |
| 86 | 0.9531 | 0.6505 | 586 | 0.4187 | 0.5543 | 1086 | 0.4864 | -0.6731 | 1586 | -0.7102 | 0.0582 |
| 87 | -0.46 | 1.301 | 587 | -0.9184 | -0.3628 | 1087 | 0.3354 | 0.9681 | 1587 | -0.8069 | 0.0666 |
| 88 | -1.0989 | -0.6505 | 588 | -0.2459 | 1.5139 | 1088 | 0.206 | 0.9803 | 1588 | 0.7677 | 0.5468 |
| 89 | -0.6505 | 0 | 589 | -0.1882 | 0.3201 | 1089 | 0.5 | -1.0607 | 1589 | 0.2524 | 0.3515 |
| 90 | -1.0989 | 0.6505 | 590 | -1.2673 | -1.1614 | 1090 | -0.3908 | -0.0033 | 1590 | 0.0091 | 1.0895 |
| 91 | -0.46 | -1.301 | 591 | -0.1584 | 0.0777 | 1091 | -0.4192 | -0.0042 | 1591 | -0.0781 | 1.2938 |
| 92 | 0.9531 | -0.6505 | 592 | 0.4695 | -0.3782 | 1092 | -0.7392 | -0.0292 | 1592 | 0.2749 | 0.1756 |
| 93 | 0 | 0 | 593 | -0.75 | -0.25 | 1093 | -1.1604 | 0.4339 | 1593 | 0.4268 | -0.7071 |
| 94 | -0.9531 | 0.6505 | 594 | -0.1427 | -0.3528 | 1094 | 0.0053 | -1.2729 | 1594 | -0.6077 | -1.4293 |
| 95 | 0.46 | 1.301 | 595 | 0.36 | -1.2903 | 1095 | -0.0186 | -0.5198 | 1595 | 0.563 | -0.6991 |
| 96 | 1.0989 | -0.6505 | 596 | 0.3983 | -0.2296 | 1096 | -0.6952 | -0.0583 | 1596 | 0.8327 | -0.8248 |
| 97 | 0.6505 | 0 | 597 | 0.7346 | 1.2106 | 1097 | -0.7286 | 0.125 | 1597 | -0.4415 | -1.0406 |
| 98 | 1.0989 | 0.6505 | 598 | -0.5309 | -0.1371 | 1098 | -0.769 | 0.3274 | 1598 | -0.1301 | -0.7419 |
| 99 | 0.46 | -1.301 | 599 | -0.4358 | -1.3615 | 1099 | -0.2115 | -1.6448 | 1599 | -0.196 | -1.2772 |
| 100 | -0.9531 | -0.6505 | 600 | 0.3291 | -0.1385 | 1100 | 0.756 | -0.5939 | 1600 | 0.6182 | 0.3049 |
| 101 | 0 | 0 | 601 | 0.0214 | -0.625 | 1101 | 1.1477 | 0.1443 | 1601 | 0.3536 | 0.7071 |
| 102 | 0.9531 | 0.6505 | 602 | 0.7796 | -0.5223 | 1102 | 0.1926 | 0.4544 | 1602 | -0.3547 | 0.6356 |
| 103 | -0.46 | 1.301 | 603 | 0.1606 | 0.2373 | 1103 | 0.2288 | 1.4085 | 1603 | 0.0026 | -0.5089 |
| 104 | -1.0989 | -0.6505 | 604 | -0.6509 | -0.4224 | 1104 | 0.5126 | -0.6539 | 1604 | 0.4591 | -0.9793 |
| 105 | -0.6505 | 0 | 605 | 0.1512 | 0.9875 | 1105 | -1.3839 | -0.1768 | 1605 | 0.0269 | -0.2041 |
| 106 | -1.0989 | 0.6505 | 606 | -0.5091 | 0.6844 | 1106 | -0.6704 | 0.5355 | 1606 | -1.055 | 0.1967 |
| 107 | -0.46 | -1.301 | 607 | -0.5042 | -0.4761 | 1107 | 0.5092 | -0.4634 | 1607 | 0.5081 | 0.1146 |
| 108 | 0.9531 | -0.6505 | 608 | 0.795 | 0.3131 | 1108 | -0.9994 | 0.5225 | 1608 | 1.3278 | -0.1783 |
| 109 | 0 | 0 | 609 | 0.5 | 0 | 1109 | 0.1117 | 0.5856 | 1609 | -0.7803 | -0.6036 |
| 110 | -0.9531 | 0.6505 | 610 | -0.315 | 0.1515 | 1110 | 0.6351 | 0.1888 | 1610 | 0.0162 | -0.7913 |
| 111 | 0.46 | 1.301 | 611 | -0.3206 | -0.2379 | 1111 | -0.6357 | -0.2362 | 1611 | -0.0462 | -0.069 |
| 112 | 1.0989 | -0.6505 | 612 | -0.3389 | -0.1851 | 1112 | -0.1339 | 0.1905 | 1612 | 0.1889 | 0.9928 |
| 113 | 0.6505 | 0 | 613 | -0.8858 | 1.0589 | 1113 | -0.7286 | 0.9786 | 1613 | 0.9413 | 0.1226 |
| 114 | 1.0989 | 0.6505 | 614 | 0.0198 | 0.3796 | 1114 | -0.7351 | -0.3456 | 1614 | -1.4154 | -0.4991 |
| 115 | 0.46 | -1.301 | 615 | 1.5895 | 0.3596 | 1115 | 0.1451 | -0.4809 | 1615 | -0.5254 | 0.8781 |
| 116 | -0.9531 | -0.6505 | 616 | 1.0805 | 0.6855 | 1116 | -0.1864 | 0.2875 | 1616 | -0.0592 | 0.0519 |
| 117 | 0 | 0 | 617 | 0.6982 | 0.1982 | 1117 | 0.0341 | 0.222 | 1617 | -0.3536 | -1.4142 |
| 118 | 0.9531 | 0.6505 | 618 | 0.6706 | 0.4031 | 1118 | -0.7802 | -0.3408 | 1618 | 1.2199 | -0.1272 |
| 119 | -0.46 | 1.301 | 619 | -0.359 | 0.4077 | 1119 | -0.2768 | -0.5178 | 1619 | -0.1617 | 0.5988 |
| 120 | -1.0989 | -0.6505 | 620 | -0.3933 | 0.1329 | 1120 | 1.9581 | 0.5282 | 1620 | 0.1019 | 0.3871 |
| 121 | -0.6505 | 0 | 621 | 0.7917 | -0.2772 | 1121 | 1.3107 | 0.7071 | 1621 | 0.4889 | 0.3932 |
| 122 | -1.0989 | 0.6505 | 622 | 1.4009 | -0.5524 | 1122 | 0.4326 | 0.6513 | 1622 | -0.3196 | 0.2501 |
| 123 | -0.46 | -1.301 | 623 | 0.1046 | -0.0037 | 1123 | -1.0584 | 0.2543 | 1623 | 0.4854 | 0.711 |
| 124 | 0.9531 | -0.6505 | 624 | -1.0834 | 0.785 | 1124 | 0.0874 | 0.4525 | 1624 | -0.6103 | 0.0647 |
| 125 | 0 | 0 | 625 | -0.5 | 0.5 | 1125 | 1.3684 | 0.2468 | 1625 | -0.6768 | -0.6036 |
| 126 | -0.9531 | 0.6505 | 626 | -0.0269 | 0.5835 | 1126 | 0.3107 | 0.0857 | 1626 | -0.1627 | 0.5331 |
| 127 | 0.46 | 1.301 | 627 | 0.3762 | 0.8987 | 1127 | -0.3262 | 0.6729 | 1627 | -0.4069 | 0.0439 |
| 128 | 1.0989 | -0.6505 | 628 | -0.3916 | -0.3543 | 1128 | -0.4839 | 0.6139 | 1628 | 0.1342 | -0.7233 |
| 129 | 0.6505 | 0 | 629 | -1.6311 | 0.0394 | 1129 | -1.125 | 1.2286 | 1629 | -0.2881 | 0.4492 |
| 130 | 1.0989 | 0.6505 | 630 | -0.1295 | 0.5707 | 1130 | -0.324 | -0.0743 | 1630 | 0.4181 | 0.5584 |
| 131 | 0.46 | -1.301 | 631 | 0.8723 | -0.5243 | 1131 | 0.4623 | -0.8069 | 1631 | 0.8552 | 0.0761 |
| 132 | -0.9531 | -0.6505 | 632 | 0.4785 | -0.803 | 1132 | -0.1397 | 1.1748 | 1632 | 0.3919 | -0.0307 |
| 133 | 0 | 0 | 633 | 0.7286 | -0.625 | 1133 | -0.7499 | -0.6194 | 1633 | 0.7071 | -0.7071 |
| 134 | 0.9531 | 0.6505 | 634 | 0.4153 | 0.5426 | 1134 | -0.1972 | -0.7251 | 1634 | 0.2109 | -1.4097 |
| 135 | -0.46 | 1.301 | 635 | 0.4098 | -0.5751 | 1135 | 0.9637 | 0.5971 | 1635 | 0.268 | -1.4598 |
| 136 | -1.0989 | -0.6505 | 636 | 0.3124 | -1.6609 | 1136 | 0.4623 | -0.5046 | 1636 | 0.9103 | 0.0464 |
| 137 | -0.6505 | 0 | 637 | -0.2547 | 0.4696 | 1137 | 0.2803 | 1.2374 | 1637 | 0.6802 | 0.9112 |
| 138 | -1.0989 | 0.6505 | 638 | -0.4437 | -0.331 | 1138 | 1.0275 | 0.5272 | 1638 | -0.5451 | -0.4129 |
| 139 | -0.46 | -1.301 | 639 | -0.8561 | -0.5979 | 1139 | 0.0113 | -0.9355 | 1639 | -0.2787 | -0.9936 |
| 140 | 0.9531 | -0.6505 | 640 | -0.1274 | 0.6406 | 1140 | -0.6204 | 0.7613 | 1640 | 1.0654 | -1.368 |
| 141 | 0 | 0 | 641 | 0.25 | -0.5 | 1141 | -0.89 | 0.3108 | 1641 | -0.2803 | -1.1036 |
| 142 | -0.9531 | 0.6505 | 642 | 0.403 | -0.2371 | 1142 | -1.2452 | -0.4992 | 1642 | -0.6351 | 1.079 |
| 143 | 0.46 | 1.301 | 643 | 1.0226 | -0.1746 | 1143 | -0.3086 | -1.0016 | 1643 | 0.3168 | 0.6773 |
| 144 | 1.0989 | -0.6505 | 644 | -0.1153 | 0.1551 | 1144 | -0.1656 | -0.5035 | 1644 | 0.2175 | -0.7115 |
| 145 | 0.6505 | 0 | 645 | 0.4778 | -0.134 | 1145 | 0.2714 | 0.625 | 1645 | 0.5587 | 0.5845 |
| 146 | 1.0989 | 0.6505 | 646 | 0.4828 | -0.8943 | 1146 | 0.4795 | 0.083 | 1646 | -0.004 | 0.698 |
| 147 | 0.46 | -1.301 | 647 | 0.2801 | 0.123 | 1147 | -0.6645 | -0.1462 | 1647 | -0.2452 | -0.1038 |
| 148 | -0.9531 | -0.6505 | 648 | -0.0148 | 1.1609 | 1148 | -0.0025 | -0.0595 | 1648 | -0.2382 | 0.1114 |
| 149 | 0 | 0 | 649 | -0.375 | 0.7286 | 1149 | -0.1067 | 0.7151 | 1649 | -0.7071 | 0 |
| 150 | 0.9531 | 0.6505 | 650 | 0.7524 | 0.5096 | 1150 | -0.852 | 0.3955 | 1650 | 0.0234 | 0.1846 |
| 151 | -0.46 | 1.301 | 651 | -0.8567 | 0.3234 | 1151 | 0.5795 | -0.6363 | 1651 | -0.1089 | -0.3372 |
| 152 | -1.0989 | -0.6505 | 652 | -0.7367 | -0.2166 | 1152 | 0.8012 | 0.0835 | 1652 | -0.4989 | -1.0441 |
| 153 | -0.6505 | 0 | 653 | 1.2458 | -0.0193 | 1153 | 0.25 | 0.3536 | 1653 | 0.2183 | 0.3139 |
| 154 | -1.0989 | 0.6505 | 654 | 0.2994 | 0.2395 | 1154 | 0.3127 | 0.5918 | 1654 | 0.1714 | 0.7904 |
| 155 | -0.46 | -1.301 | 655 | 0.4325 | -0.1663 | 1155 | 0.4818 | -0.2727 | 1655 | 0.2852 | -0.1249 |
| 156 | 0.9531 | -0.6505 | 656 | 0.5498 | -1.1755 | 1156 | 0.6819 | -0.8599 | 1656 | 1.272 | 0.3506 |
| 157 | 0 | 0 | 657 | 0.5 | -1.25 | 1157 | -0.2648 | -0.3504 | 1657 | 0.3232 | 0.8964 |
| 158 | -0.9531 | 0.6505 | 658 | 0.3124 | 0.9187 | 1158 | -0.2811 | -1.3707 | 1658 | -0.7321 | 0.3101 |
| 159 | 0.46 | 1.301 | 659 | -0.3425 | 0.7625 | 1159 | 0.148 | -0.1343 | 1659 | 0.1363 | -0.3594 |
| 160 | 1.0989 | -0.6505 | 660 | 0.5529 | -1.2669 | 1160 | 0.0583 | 0.9129 | 1660 | -1.0988 | -0.3824 |
| 161 | 0.6505 | 0 | 661 | -0.0529 | -0.1736 | 1161 | 1.125 | -0.0214 | 1661 | -1.2119 | 0.2579 |
| 162 | 1.0989 | 0.6505 | 662 | -0.4119 | -0.0433 | 1162 | 0.6191 | -0.3148 | 1662 | 0.3354 | 0.8325 |
| 163 | 0.46 | -1.301 | 663 | -0.5436 | -0.9554 | 1163 | -0.077 | -1.3215 | 1663 | -1.0846 | 0.8567 |
| 164 | -0.9531 | -0.6505 | 664 | -1.8697 | 0.0768 | 1164 | 0.1305 | -0.7331 | 1664 | -0.7351 | 0.5842 |
| 165 | 0 | 0 | 665 | -0.5518 | -0.3018 | 1165 | -1.2679 | -0.4841 | 1665 | 0.3536 | 0.7071 |
| 166 | 0.9531 | 0.6505 | 666 | 1.1419 | -0.446 | 1166 | -0.4491 | -0.5243 | 1666 | -0.3547 | 0.6356 |
| 167 | -0.46 | 1.301 | 667 | 0.8086 | 0.1402 | 1167 | 0.8201 | 0.9034 | 1667 | 0.0026 | -0.5089 |
| 168 | -1.0989 | -0.6505 | 668 | -0.1808 | -0.1731 | 1168 | -0.92 | -0.1987 | 1668 | 0.4591 | -0.9793 |
| 169 | -0.6505 | 0 | 669 | 0.3092 | 0.347 | 1169 | -1.1339 | -0.1768 | 1669 | 0.0269 | -0.2041 |
| 170 | -1.0989 | 0.6505 | 670 | 0.7407 | -0.2839 | 1170 | -0.1195 | 1.1895 | 1670 | -1.055 | 0.1967 |
| 171 | -0.46 | -1.301 | 671 | -0.4095 | -0.6668 | 1171 | 0.0654 | 0.0396 | 1671 | 0.5081 | 0.1146 |
| 172 | 0.9531 | -0.6505 | 672 | 0.324 | 0.6626 | 1172 | 0.1217 | -0.6245 | 1672 | 1.3278 | -0.1783 |
| 173 | 0 | 0 | 673 | 0 | 0.75 | 1173 | -0.2135 | -0.2072 | 1673 | -0.7803 | -0.6036 |
| 174 | -0.9531 | 0.6505 | 674 | -0.8544 | 0.7334 | 1174 | -0.0551 | -0.0275 | 1674 | 0.0162 | -0.7913 |
| 175 | 0.46 | 1.301 | 675 | 0.5255 | 0.2195 | 1175 | 0.4011 | -0.037 | 1675 | -0.0462 | -0.069 |
| 176 | 1.0989 | -0.6505 | 676 | -0.2896 | -0.4108 | 1176 | -0.0622 | -0.37 | 1676 | 0.1889 | 0.9928 |
| 177 | 0.6505 | 0 | 677 | -0.5814 | 0.384 | 1177 | -0.9786 | -1.125 | 1677 | 0.9413 | 0.1226 |
| 178 | 1.0989 | 0.6505 | 678 | 0.7117 | 0.4746 | 1178 | -0.4279 | -0.6538 | 1678 | -1.4154 | -0.4991 |
| 179 | 0.46 | -1.301 | 679 | -0.2389 | 0.6096 | 1179 | 0.7792 | 0.3604 | 1679 | -0.5254 | 0.8781 |
| 180 | -0.9531 | -0.6505 | 680 | -0.8804 | 0.3611 | 1180 | -0.259 | -0.1115 | 1680 | -0.0592 | 0.0519 |
| 181 | 0 | 0 | 681 | -0.375 | 0.0214 | 1181 | -0.7039 | 0.3885 | 1681 | -0.6036 | -1.0607 |
| 182 | 0.9531 | 0.6505 | 682 | -0.8699 | 0.7716 | 1182 | 0.7688 | 0.6657 | 1682 | -0.1873 | 1.0237 |
| 183 | -0.46 | 1.301 | 683 | -0.5328 | -0.715 | 1183 | 0.5509 | -0.3642 | 1683 | 0.8265 | -0.1228 |
| 184 | -1.0989 | -0.6505 | 684 | 0.5481 | -1.3786 | 1184 | 0.3098 | -0.0335 | 1684 | 0.222 | -0.2516 |
| 185 | -0.6505 | 0 | 685 | -0.8493 | 0.7693 | 1185 | 1.3107 | 0.7071 | 1685 | -0.5957 | 1.0412 |
| 186 | -1.0989 | 0.6505 | 686 | -1.1144 | 1.3178 | 1186 | 0.4326 | 0.6513 | 1686 | -0.63 | -0.2629 |
| 187 | -0.46 | -1.301 | 687 | 0.374 | 1.0109 | 1187 | -1.0584 | 0.2543 | 1687 | -0.7386 | -0.1001 |
| 188 | 0.9531 | -0.6505 | 688 | 0.1823 | 0.6881 | 1188 | 0.0874 | 0.4525 | 1688 | -0.468 | -0.2202 |
| 189 | 0 | 0 | 689 | 0.75 | 0.5 | 1189 | 1.3684 | 0.2468 | 1689 | -0.1036 | -0.2803 |
| 190 | -0.9531 | 0.6505 | 690 | 0.3684 | 0.0626 | 1190 | 0.3107 | 0.0857 | 1690 | 0.2743 | 0.6838 |
| 191 | 0.46 | 1.301 | 691 | -0.4985 | -1.1002 | 1191 | -0.3262 | 0.6729 | 1691 | -0.2272 | 0.1516 |
| 192 | 1.0989 | -0.6505 | 692 | 0.5538 | -0.1209 | 1192 | -0.4839 | 0.6139 | 1692 | -0.5012 | 0.4163 |
| 193 | 0.6505 | 0 | 693 | 0.6564 | 0.4236 | 1193 | -1.125 | 1.2286 | 1693 | 0.3932 | -0.3853 |
| 194 | 1.0989 | 0.6505 | 694 | 0.3707 | -0.3974 | 1194 | -0.324 | -0.0743 | 1694 | 0.1315 | -1.2505 |
| 195 | 0.46 | -1.301 | 695 | -0.4976 | 0.2228 | 1195 | 0.4623 | -0.8069 | 1695 | -0.5593 | 0.524 |
| 196 | -0.9531 | -0.6505 | 696 | -0.9433 | -0.0313 | 1196 | -0.1397 | 1.1748 | 1696 | -0.7824 | 0.5917 |
| 197 | 0 | 0 | 697 | -0.1982 | 0.0518 | 1197 | -0.7499 | -0.6194 | 1697 | -0.1036 | -0.7071 |
| 198 | 0.9531 | 0.6505 | 698 | -0.2538 | 0.1013 | 1198 | -0.1972 | -0.7251 | 1698 | 0.139 | -0.4854 |
| 199 | -0.46 | 1.301 | 699 | -0.1261 | -1.4557 | 1199 | 0.9637 | 0.5971 | 1699 | -0.4307 | 0.0293 |
| 200 | -1.0989 | -0.6505 | 700 | 0.0817 | -0.4166 | 1200 | 0.4623 | -0.5046 | 1700 | 0.9426 | 0.3115 |
| 201 | -0.6505 | 0 | 701 | 0.7944 | 0.403 | 1201 | 0.5303 | 0.7071 | 1701 | 0.3774 | 1.1601 |
| 202 | -1.0989 | 0.6505 | 702 | -0.0791 | -0.8272 | 1202 | 0.5446 | -0.1371 | 1702 | -0.9614 | 0.8328 |
| 203 | -0.46 | -1.301 | 703 | -1.397 | -0.1778 | 1203 | -0.3343 | -0.8645 | 1703 | 0.323 | -0.0744 |
| 204 | 0.9531 | -0.6505 | 704 | 0.2378 | 0.0857 | 1204 | -0.2946 | -1.1975 | 1704 | -0.0895 | 0.6163 |
| 205 | 0 | 0 | 705 | 0.25 | -0.5 | 1205 | -0.6705 | -1.132 | 1705 | -0.6036 | 0.8232 |
| 206 | -0.9531 | 0.6505 | 706 | 0.403 | -0.2371 | 1206 | -0.1481 | -1.0283 | 1706 | -0.2894 | 1.1625 |
| 207 | 0.46 | 1.301 | 707 | 1.0226 | -0.1746 | 1207 | 0.131 | 0.3293 | 1707 | -0.1444 | 1.1149 |
| 208 | 1.0989 | -0.6505 | 708 | -0.1153 | 0.1551 | 1208 | -0.7444 | 1.1245 | 1708 | 0.3446 | 0.0746 |
| 209 | 0.6505 | 0 | 709 | 0.4778 | -0.134 | 1209 | -0.7803 | -0.5303 | 1709 | 0.1495 | 0.9302 |
| 210 | 1.0989 | 0.6505 | 710 | 0.4828 | -0.8943 | 1210 | -0.5906 | 0.1619 | 1710 | 0.2154 | 0.3807 |
| 211 | 0.46 | -1.301 | 711 | 0.2801 | 0.123 | 1211 | -0.055 | 1.2353 | 1711 | -0.7559 | -1.4073 |
| 212 | -0.9531 | -0.6505 | 712 | -0.0148 | 1.1609 | 1212 | 0.1562 | -0.4363 | 1712 | -0.5459 | -1.1724 |
| 213 | 0 | 0 | 713 | -0.375 | 0.7286 | 1213 | 0.0415 | -0.8234 | 1713 | 0.8107 | -0.7071 |
| 214 | 0.9531 | 0.6505 | 714 | 0.7524 | 0.5096 | 1214 | 0.1156 | 0.2344 | 1714 | -0.2192 | -0.0629 |
| 215 | -0.46 | 1.301 | 715 | -0.8567 | 0.3234 | 1215 | 0.7563 | 0.3465 | 1715 | 0.3062 | -0.1527 |
| 216 | -1.0989 | -0.6505 | 716 | -0.7367 | -0.2166 | 1216 | 0.7471 | -0.388 | 1716 | 0.5388 | -0.5619 |
| 217 | -0.6505 | 0 | 717 | 1.2458 | -0.0193 | 1217 | -0.8839 | -0.3536 | 1717 | -0.4043 | -0.0412 |
| 218 | -1.0989 | 0.6505 | 718 | 0.2994 | 0.2395 | 1218 | 0.6253 | 0.7563 | 1718 | 0.4096 | -0.0019 |
| 219 | -0.46 | -1.301 | 719 | 0.4325 | -0.1663 | 1219 | 1.4158 | 0.9756 | 1719 | 0.3353 | 0.2709 |
| 220 | 0.9531 | -0.6505 | 720 | 0.5498 | -1.1755 | 1220 | -1.0077 | -0.2102 | 1720 | 0.0644 | -0.1024 |
| 221 | 0 | 0 | 721 | 1.25 | 0.25 | 1221 | -0.0912 | -0.6343 | 1721 | -0.6036 | -1.2803 |
| 222 | -0.9531 | 0.6505 | 722 | 0.5896 | -0.4771 | 1222 | 0.776 | -0.2064 | 1722 | -0.8985 | -0.4891 |
| 223 | 0.46 | 1.301 | 723 | 0.2416 | -0.9515 | 1223 | 0.7189 | -0.2537 | 1723 | -0.1401 | -0.0391 |
| 224 | 1.0989 | -0.6505 | 724 | -0.3164 | -0.4676 | 1224 | 0.8175 | 0.1801 | 1724 | -0.4531 | -1.2599 |
| 225 | 0.6505 | 0 | 725 | -0.1213 | -0.4968 | 1225 | -0.6339 | -0.0732 | 1725 | 0.3139 | -0.1147 |
| 226 | 1.0989 | 0.6505 | 726 | -0.3094 | -0.3462 | 1226 | -0.0827 | -0.1695 | 1726 | 0.6945 | 1.0481 |
| 227 | 0.46 | -1.301 | 727 | -0.6124 | 0.3067 | 1227 | 0.7858 | 1.0051 | 1727 | 0.5799 | 0.3413 |
| 228 | -0.9531 | -0.6505 | 728 | 0.6017 | 0.2498 | 1228 | 0.751 | 0.3911 | 1728 | 1.0788 | 0.9706 |
| 229 | 0 | 0 | 729 | 0.0518 | 0.0518 | 1229 | 0.5034 | 0.1723 | 1729 | -0.1036 | -0.3536 |
| 230 | 0.9531 | 0.6505 | 730 | 0.0178 | -0.737 | 1230 | 0.1235 | 0.1924 | 1730 | -0.1203 | -0.9216 |
| 231 | -0.46 | 1.301 | 731 | -0.0622 | -0.5425 | 1231 | 0.8943 | -0.8946 | 1731 | 0.798 | 1.1604 |
| 232 | -1.0989 | -0.6505 | 732 | -0.5637 | 0.8328 | 1232 | 0.212 | 0.0454 | 1732 | 0.3737 | -0.9757 |
| 233 | -0.6505 | 0 | 733 | 1.3807 | 0.4445 | 1233 | -0.1768 | 0.3536 | 1733 | -0.0845 | -1.453 |
| 234 | -1.0989 | 0.6505 | 734 | -0.006 | -0.3219 | 1234 | -0.1183 | -0.3198 | 1734 | 0.1046 | 0.9097 |
| 235 | -0.46 | -1.301 | 735 | -0.6906 | 0.0269 | 1235 | -0.6102 | 0.1182 | 1735 | 0.5803 | 0.4036 |
| 236 | 0.9531 | -0.6505 | 736 | 1.3586 | 0.0224 | 1236 | 0.9278 | -0.0763 | 1736 | -0.119 | 0.1525 |
| 237 | 0 | 0 | 737 | -0.25 | -0.25 | 1237 | 0.524 | -0.7822 | 1737 | -0.1036 | -0.6768 |
| 238 | -0.9531 | 0.6505 | 738 | -0.2619 | 0.8775 | 1238 | -0.3646 | -0.0952 | 1738 | 0.3015 | -0.4968 |
| 239 | 0.46 | 1.301 | 739 | 1.06 | 1.1668 | 1239 | 0.3261 | 1.6188 | 1739 | -0.9883 | 0.6868 |
| 240 | 1.0989 | -0.6505 | 740 | 0.1949 | -0.9882 | 1240 | -0.7508 | 0.5659 | 1740 | -0.4675 | -0.1675 |
| 241 | 0.6505 | 0 | 741 | -0.2424 | -0.2928 | 1241 | -0.2803 | -0.5303 | 1741 | 1.2647 | 0.2769 |
| 242 | 1.0989 | 0.6505 | 742 | -0.0352 | 1.424 | 1242 | 0.289 | 0.222 | 1742 | 1.0357 | 0.7582 |
| 243 | 0.46 | -1.301 | 743 | 0.426 | 0.1328 | 1243 | -1.2309 | -0.5937 | 1743 | 0.2353 | 0.042 |
| 244 | -0.9531 | -0.6505 | 744 | 0.2041 | 0.185 | 1244 | -0.7569 | -0.1461 | 1744 | -0.1384 | -1.2503 |
| 245 | 0 | 0 | 745 | -0.125 | 0.2286 | 1245 | 0.3121 | -0.0908 | 1745 | -0.6036 | -1.0607 |
| 246 | 0.9531 | 0.6505 | 746 | -0.1434 | -0.1718 | 1246 | 0.4104 | -0.9669 | 1746 | -0.1873 | 1.0237 |
| 247 | -0.46 | 1.301 | 747 | -0.1389 | 0.8722 | 1247 | 0.2008 | -0.5516 | 1747 | 0.8265 | -0.1228 |
| 248 | -1.0989 | -0.6505 | 748 | -0.2277 | -0.2568 | 1248 | -0.9587 | -1.3755 | 1748 | 0.222 | -0.2516 |
| 249 | -0.6505 | 0 | 749 | -0.0731 | -0.5227 | 1249 | -0.8839 | 0 | 1749 | -0.5957 | 1.0412 |
| 250 | -1.0989 | 0.6505 | 750 | 0.4728 | 1.0489 | 1250 | 0.1017 | 0.8539 | 1750 | -0.63 | -0.2629 |
| 251 | -0.46 | -1.301 | 751 | -1.0327 | 0.1196 | 1251 | 0.0287 | -1.0222 | 1751 | -0.7386 | -0.1001 |
| 252 | 0.9531 | -0.6505 | 752 | -0.7737 | -1.0561 | 1252 | 1.1451 | 0.1721 | 1752 | -0.468 | -0.2202 |
| 253 | 0 | 0 | 753 | 1 | -0.5 | 1253 | 0.9448 | 1.1343 | 1753 | -0.1036 | -0.2803 |
| 254 | -0.9531 | 0.6505 | 754 | -0.6231 | 0.6482 | 1254 | -1.0338 | 0.5594 | 1754 | 0.2743 | 0.6838 |
| 255 | 0.46 | 1.301 | 755 | -0.7036 | -0.0845 | 1255 | -0.2618 | 0.5126 | 1755 | -0.2272 | 0.1516 |
| 256 | 1.0989 | -0.6505 | 756 | 0.2741 | -1.1065 | 1256 | -0.4757 | 0.5893 | 1756 | -0.5012 | 0.4163 |
| 257 | 0.6505 | 0 | 757 | -0.3358 | 0.1004 | 1257 | -1.1339 | 0.4268 | 1757 | 0.3932 | -0.3853 |
| 258 | 1.0989 | 0.6505 | 758 | -0.3413 | -0.6247 | 1258 | 0.231 | -0.3677 | 1758 | 0.1315 | -1.2505 |
| 259 | 0.46 | -1.301 | 759 | 0.0041 | -1.0841 | 1259 | 0.0001 | 0.5604 | 1759 | -0.5593 | 0.524 |
| 260 | -0.9531 | -0.6505 | 760 | 0.4098 | 0.3478 | 1260 | 0.079 | 0.5031 | 1760 | -0.7824 | 0.5917 |
| 261 | 0 | 0 | 761 | -0.3018 | -0.3018 | 1261 | -0.1499 | -0.6723 | 1761 | -1.4874 | 0.3536 |
| 262 | 0.9531 | 0.6505 | 762 | -0.3041 | -0.3038 | 1262 | -0.879 | 0.3107 | 1762 | -0.8989 | 0.4187 |
| 263 | -0.46 | 1.301 | 763 | 0.2536 | -0.6519 | 1263 | 0.0628 | 0.3068 | 1763 | 0.2643 | -0.3137 |
| 264 | -1.0989 | -0.6505 | 764 | 0.1863 | -1.3532 | 1264 | 0.1529 | 0.2583 | 1764 | -0.5084 | 0.1167 |
| 265 | -0.6505 | 0 | 765 | 0.7835 | 0.6591 | 1265 | 0.5303 | 0.7071 | 1765 | -1.0687 | 0.403 |
| 266 | -1.0989 | 0.6505 | 766 | -0.1615 | 1.0382 | 1266 | 0.5446 | -0.1371 | 1766 | 0.4622 | -0.2318 |
| 267 | -0.46 | -1.301 | 767 | -0.3543 | 0.444 | 1267 | -0.3343 | -0.8645 | 1767 | -0.2149 | 0.7697 |
| 268 | 0.9531 | -0.6505 | 768 | -0.2568 | 0.0816 | 1268 | -0.2946 | -1.1975 | 1768 | 0.2876 | 0.3284 |
| 269 | 0 | 0 | 769 | -1.5 | -1 | 1269 | -0.6705 | -1.132 | 1769 | 0.9786 | 0.4482 |
| 270 | -0.9531 | 0.6505 | 770 | -0.8579 | -0.0613 | 1270 | -0.1481 | -1.0283 | 1770 | -0.2033 | 0.4965 |
| 271 | 0.46 | 1.301 | 771 | -0.5981 | -0.1308 | 1271 | 0.131 | 0.3293 | 1771 | 0.1658 | -1.2037 |
| 272 | 1.0989 | -0.6505 | 772 | 0.2936 | -1.3118 | 1272 | -0.7444 | 1.1245 | 1772 | 0.3518 | -0.0415 |
| 273 | 0.6505 | 0 | 773 | 1.1995 | -0.8108 | 1273 | -0.7803 | -0.5303 | 1773 | 0.5608 | 0.7043 |
| 274 | 1.0989 | 0.6505 | 774 | -0.0847 | -0.282 | 1274 | -0.5906 | 0.1619 | 1774 | 0.1553 | 0.1143 |
| 275 | 0.46 | -1.301 | 775 | -0.1105 | 0.9376 | 1275 | -0.055 | 1.2353 | 1775 | -0.9012 | 0.4369 |
| 276 | -0.9531 | -0.6505 | 776 | -0.279 | 0.9434 | 1276 | 0.1562 | -0.4363 | 1776 | 0.328 | -0.1142 |
| 277 | 0 | 0 | 777 | -0.125 | -0.4786 | 1277 | 0.0415 | -0.8234 | 1777 | 0.9571 | -0.1768 |
| 278 | 0.9531 | 0.6505 | 778 | 0.583 | -0.1889 | 1278 | 0.1156 | 0.2344 | 1778 | 0.0343 | 0.6611 |
| 279 | -0.46 | 1.301 | 779 | -0.0524 | 0.3223 | 1279 | 0.7563 | 0.3465 | 1779 | 0.132 | 0.6966 |
| 280 | -1.0989 | -0.6505 | 780 | -0.2553 | 0.8228 | 1280 | 0.7471 | -0.388 | 1780 | 0.6536 | -0.5701 |
| 281 | -0.6505 | 0 | 781 | -0.5911 | 0.9192 | 1281 | 0.1768 | -0.1768 | 1781 | 0.2659 | -0.4157 |
| 282 | -1.0989 | 0.6505 | 782 | -0.5347 | 0.4778 | 1282 | 1.1994 | -0.1394 | 1782 | -0.2429 | 0.4121 |
| 283 | -0.46 | -1.301 | 783 | 0.3705 | 1.1166 | 1283 | 0.7883 | -0.8558 | 1783 | -0.6262 | -0.9135 |
| 284 | 0.9531 | -0.6505 | 784 | 1.1496 | 1.0545 | 1284 | 0.1254 | 0.2224 | 1784 | -0.4221 | -0.1994 |
| 285 | 0 | 0 | 785 | 1.25 | 0.25 | 1285 | 0.4199 | 0.2061 | 1785 | 0.8321 | 0.8018 |
| 286 | -0.9531 | 0.6505 | 786 | 0.5896 | -0.4771 | 1286 | -0.3473 | -0.5966 | 1786 | -0.1098 | -0.7883 |
| 287 | 0.46 | 1.301 | 787 | 0.2416 | -0.9515 | 1287 | -0.5288 | 0.0012 | 1787 | -1.0394 | -0.4287 |
| 288 | 1.0989 | -0.6505 | 788 | -0.3164 | -0.4676 | 1288 | 0.125 | -0.0038 | 1788 | 0.2617 | 0.396 |
| 289 | -0.6505 | 0 | 789 | -0.1213 | -0.4968 | 1289 | 0.6036 | 0.0732 | 1789 | -0.0032 | 0.0409 |
| 290 | -1.0989 | -0.6505 | 790 | -0.3094 | -0.3462 | 1290 | 0.5678 | 0.2475 | 1790 | -0.1526 | 0.7973 |
| 291 | -0.46 | 1.301 | 791 | -0.6124 | 0.3067 | 1291 | -0.2018 | -0.1097 | 1791 | -0.4093 | 0.7031 |
| 292 | 0.9531 | 0.6505 | 792 | 0.6017 | 0.2498 | 1292 | -1.002 | 0.4166 | 1792 | -1.1569 | -0.862 |
| 293 | 0 | 0 | 793 | 0.0518 | 0.0518 | 1293 | -0.3908 | -0.0597 | 1793 | -0.0732 | -0.3536 |
| 294 | -0.9531 | -0.6505 | 794 | 0.0178 | -0.737 | 1294 | 0.7756 | -1.0674 | 1794 | 0.1982 | 0.7375 |
| 295 | 0.46 | -1.301 | 795 | -0.0622 | -0.5425 | 1295 | -0.0966 | -0.7552 | 1795 | 0.8536 | -0.3871 |
| 296 | 1.0989 | 0.6505 | 796 | -0.5637 | 0.8328 | 1296 | 0.0332 | -1.0148 | 1796 | 1.4144 | -0.5498 |
| 297 | 0.6505 | 0 | 797 | 1.3807 | 0.4445 | 1297 | 0.1768 | -0.5303 | 1797 | -0.742 | 0.347 |
| 298 | 1.0989 | -0.6505 | 798 | -0.006 | -0.3219 | 1298 | -1.6795 | 0.7331 | 1798 | -0.9928 | 0.1588 |
| 299 | 0.46 | 1.301 | 799 | -0.6906 | 0.0269 | 1299 | -0.4548 | 0.078 | 1799 | 0.3296 | -0.3776 |
| 300 | -0.9531 | 0.6505 | 800 | 1.3586 | 0.0224 | 1300 | 0.7439 | -0.3402 | 1800 | -0.2746 | -0.6705 |
| 301 | 0 | 0 | 801 | -0.75 | -0.5 | 1301 | -0.6905 | 0.2827 | 1801 | -0.2714 | -0.3018 |
| 302 | 0.9531 | -0.6505 | 802 | 0.6925 | 0.3787 | 1302 | -0.4462 | 0.4112 | 1802 | 0.4763 | 0.146 |
| 303 | -0.46 | -1.301 | 803 | 0.49 | -0.5588 | 1303 | 0.7675 | 0.3631 | 1803 | 0.3728 | 0.4649 |
| 304 | -1.0989 | 0.6505 | 804 | 0.0296 | -0.6947 | 1304 | 0.4473 | -0.4132 | 1804 | 0.689 | -0.3914 |
| 305 | -0.6505 | 0 | 805 | 0.1239 | 0.7468 | 1305 | -0.3964 | -0.8839 | 1805 | 0.0428 | -1.6614 |
| 306 | -1.0989 | -0.6505 | 806 | 0.4302 | 0.821 | 1306 | -0.1079 | 0.2809 | 1806 | -0.7666 | -0.9068 |
| 307 | -0.46 | 1.301 | 807 | -0.7965 | 0.3 | 1307 | -0.1765 | 0.7073 | 1807 | 0.0539 | -0.3132 |
| 308 | 0.9531 | 0.6505 | 808 | -0.7702 | -0.1656 | 1308 | 0.3647 | -0.481 | 1808 | -0.3577 | -0.0858 |
| 309 | 0 | 0 | 809 | 1.4053 | 0.4053 | 1309 | 0.3369 | -0.6744 | 1809 | -0.1036 | 0.1768 |
| 310 | -0.9531 | -0.6505 | 810 | 0.3555 | 1.2629 | 1310 | -1.3011 | 0.5666 | 1810 | 0.1887 | -0.0331 |
| 311 | 0.46 | -1.301 | 811 | -0.3756 | 1.0032 | 1311 | -0.1621 | 0.5236 | 1811 | -0.7499 | -0.2029 |
| 312 | 1.0989 | 0.6505 | 812 | -0.1669 | 0.606 | 1312 | -0.0199 | -0.1658 | 1812 | 0.7596 | -0.8572 |
| 313 | 0.6505 | 0 | 813 | -0.3919 | -0.4857 | 1313 | 0.1768 | 0.8839 | 1813 | 0.1306 | 0.3728 |
| 314 | 1.0989 | -0.6505 | 814 | 0.4783 | -0.7561 | 1314 | 1.8818 | 1.1359 | 1814 | -1.0869 | 0.9801 |
| 315 | 0.46 | 1.301 | 815 | -0.8041 | -0.0645 | 1315 | 0.1176 | -0.1622 | 1815 | 1.0115 | -0.2715 |
| 316 | -0.9531 | 0.6505 | 816 | -1.2097 | -0.5419 | 1316 | 0.3212 | -0.0358 | 1816 | 1.1933 | 0.0638 |
| 317 | 0 | 0 | 817 | -0.25 | 0 | 1317 | 0.0372 | 0.3974 | 1817 | 0.5821 | -0.9482 |
| 318 | 0.9531 | -0.6505 | 818 | -1.1991 | 0.684 | 1318 | -1.2523 | -0.096 | 1818 | -0.0997 | -1.2243 |
| 319 | -0.46 | -1.301 | 819 | -0.5867 | 0.0665 | 1319 | 1.1261 | -0.0446 | 1819 | 0.0008 | -0.0396 |
| 320 | -1.0989 | 0.6505 | 820 | 0.5047 | 0.4712 | 1320 | 0.5525 | 0.0939 | 1820 | 0.7925 | -0.5169 |
| 321 | 1.25 | 0 | 821 | -0.7581 | 0.5428 | 1321 | 0.1036 | -0.4268 | 1821 | -0.6004 | 0.2091 |
| 322 | -0.3216 | -0.5171 | 822 | -1.1718 | -0.626 | 1322 | 0.5888 | -0.4527 | 1822 | 0.2101 | 1.0901 |
| 323 | -0.564 | -0.4328 | 823 | 0.1328 | -1.1049 | 1323 | -0.2422 | 0.2831 | 1823 | 0.7565 | 1.3804 |
| 324 | 0.8594 | 0.5478 | 824 | -0.2339 | -0.215 | 1324 | 0.5516 | 0.0729 | 1824 | -1.1835 | 1.1254 |
| 325 | 0.0111 | -0.4458 | 825 | -0.125 | 1.0821 | 1325 | -0.7734 | -0.251 | 1825 | -1.4874 | 0.3536 |
| 326 | -0.9756 | -0.4563 | 826 | 1.4165 | 0.6882 | 1326 | -0.8906 | 0.9211 | 1826 | -0.8989 | 0.4187 |
| 327 | -1.1903 | 0.3727 | 827 | 0.4663 | -0.5682 | 1327 | -0.0387 | 0.827 | 1827 | 0.2643 | -0.3137 |
| 328 | -0.7761 | -0.0741 | 828 | -0.1519 | -0.5003 | 1328 | -1.2934 | -0.1539 | 1828 | -0.5084 | 0.1167 |
| 329 | 0.1036 | -0.6339 | 829 | 0.5462 | -0.2839 | 1329 | -0.5303 | 0.5303 | 1829 | -1.0687 | 0.403 |
| 330 | 0.2871 | -1.2165 | 830 | 0.65 | -0.3209 | 1330 | 0.0983 | 0.4014 | 1830 | 0.4622 | -0.2318 |
| 331 | 0.4219 | -0.6784 | 831 | 0.7639 | 0.6421 | 1331 | 0.5489 | -0.06 | 1831 | -0.2149 | 0.7697 |
| 332 | 0.8836 | -0.4284 | 832 | 0.444 | 0.8517 | 1332 | 0.6161 | 0.3291 | 1832 | 0.2876 | 0.3284 |
| 333 | 0.5269 | 0.2276 | 833 | 0.5 | -0.25 | 1333 | 0.2334 | -0.1792 | 1833 | 0.9786 | 0.4482 |
| 334 | -0.6312 | 0.5158 | 834 | -0.4142 | -0.2127 | 1334 | 0.5457 | 0.8712 | 1834 | -0.2033 | 0.4965 |
| 335 | -0.3382 | -0.9638 | 835 | -1.0935 | 0.0763 | 1335 | -1.3648 | 2.0945 | 1835 | 0.1658 | -1.2037 |
| 336 | 0.3459 | 0.1333 | 836 | 0.3203 | 0.1346 | 1336 | -1.0836 | 0.0399 | 1836 | 0.3518 | -0.0415 |
| 337 | 0.25 | 1.25 | 837 | 0.8332 | 0.1496 | 1337 | 1.1036 | -0.8839 | 1837 | 0.5608 | 0.7043 |
| 338 | 1.0998 | 0.4827 | 838 | 0.6291 | -1.301 | 1338 | 0.4514 | 0.2075 | 1838 | 0.1553 | 0.1143 |
| 339 | 0.4623 | -0.2948 | 839 | 0.3588 | -0.9125 | 1339 | -0.3796 | 0.1194 | 1839 | -0.9012 | 0.4369 |
| 340 | 0.0015 | -1.1597 | 840 | 0.3187 | 0.544 | 1340 | -0.7208 | -0.5982 | 1840 | 0.328 | -0.1142 |
| 341 | 1.1802 | -0.4189 | 841 | 0.3447 | -0.6553 | 1341 | -0.5869 | -1.1363 | 1841 | 0 | 0.1768 |
| 342 | -0.4653 | 0.3832 | 842 | 0.5711 | -0.6582 | 1342 | -0.0839 | -0.5959 | 1842 | -0.0727 | 0.2687 |
| 343 | -0.6222 | 0.3373 | 843 | 0.4791 | 0.0914 | 1343 | 0.2974 | -0.1812 | 1843 | 0.7228 | 0.96 |
| 344 | 0.8113 | -0.6423 | 844 | -1.2386 | -0.0474 | 1344 | 0.2389 | -0.7964 | 1844 | 1.0263 | 1.2571 |
| 345 | -0.6036 | -1.1339 | 845 | -0.0652 | 0.3821 | 1345 | 0.1768 | -0.1768 | 1845 | -0.2117 | -0.4154 |
| 346 | -0.5391 | -0.207 | 846 | 0.6937 | -0.5667 | 1346 | 1.1994 | -0.1394 | 1846 | -0.7483 | -0.1364 |
| 347 | -0.3165 | -1.0258 | 847 | -1.1061 | -0.7109 | 1347 | 0.7883 | -0.8558 | 1847 | 0.1785 | 0.1946 |
| 348 | -0.5326 | -0.3518 | 848 | 0.1804 | -0.3041 | 1348 | 0.1254 | 0.2224 | 1848 | 0.7065 | -0.5663 |
| 349 | 0.2817 | 0.9077 | 849 | 0 | -0.75 | 1349 | 0.4199 | 0.2061 | 1849 | -0.4053 | 0.4786 |
| 350 | -0.5541 | -0.4205 | 850 | -0.8096 | 1.0641 | 1350 | -0.3473 | -0.5966 | 1850 | -1.661 | -0.0392 |
| 351 | 0.147 | -0.178 | 851 | -0.0168 | 0.916 | 1351 | -0.5288 | 0.0012 | 1851 | -0.4411 | -0.4487 |
| 352 | 0.5069 | 0.5395 | 852 | -0.4307 | -0.4111 | 1352 | 0.125 | -0.0038 | 1852 | 1.1159 | 0.2276 |
| 353 | -0.75 | 0 | 853 | 0.301 | 1.0608 | 1353 | 0.6036 | 0.0732 | 1853 | 0.5193 | -0.6912 |
| 354 | 0.5069 | -0.5395 | 854 | 0.4539 | 0.1917 | 1354 | 0.5678 | 0.2475 | 1854 | 0.4769 | -0.3546 |
| 355 | 0.147 | 0.178 | 855 | 0.5119 | -1.1968 | 1355 | -0.2018 | -0.1097 | 1855 | 0.6412 | 0.4537 |
| 356 | -0.5541 | 0.4205 | 856 | 0.5681 | -0.6634 | 1356 | -1.002 | 0.4166 | 1856 | -0.8383 | -0.039 |
| 357 | 0.2817 | -0.9077 | 857 | -0.125 | -0.3321 | 1357 | -0.3908 | -0.0597 | 1857 | -0.8839 | 0.7071 |
| 358 | -0.5326 | 0.3518 | 858 | 0.3873 | 0.6213 | 1358 | 0.7756 | -1.0674 | 1858 | 0.4013 | 0.8995 |
| 359 | -0.3165 | 1.0258 | 859 | -0.3628 | -0.0264 | 1359 | -0.0966 | -0.7552 | 1859 | 0.3356 | -0.6632 |
| 360 | -0.5391 | 0.207 | 860 | 0.1335 | -0.5583 | 1360 | 0.0332 | -1.0148 | 1860 | 0.2511 | 0.149 |
| 361 | -0.6036 | 1.1339 | 861 | 0.4109 | 0.8874 | 1361 | -0.1036 | -0.1768 | 1861 | 0.3074 | 0.3488 |
| 362 | 0.8113 | 0.6423 | 862 | -1.1636 | 0.7296 | 1362 | -0.5071 | 1.4348 | 1862 | -0.4222 | -1.3687 |
| 363 | -0.6222 | -0.3373 | 863 | -0.0609 | 0.0475 | 1363 | -0.2218 | 0.3474 | 1863 | -0.667 | -0.3525 |
| 364 | -0.4653 | -0.3832 | 864 | -0.2974 | -0.5057 | 1364 | -0.4394 | -0.5773 | 1864 | -0.4294 | -0.4603 |
| 365 | 1.1802 | 0.4189 | 865 | -0.75 | -0.5 | 1365 | 0.1678 | -0.3457 | 1865 | -0.5518 | -0.375 |
| 366 | 0.0015 | 1.1597 | 866 | 0.6925 | 0.3787 | 1366 | -0.0687 | -0.7158 | 1866 | -0.8671 | 1.2446 |
| 367 | 0.4623 | 0.2948 | 867 | 0.49 | -0.5588 | 1367 | 0.1659 | -0.3205 | 1867 | -0.8479 | -0.3663 |
| 368 | 1.0998 | -0.4827 | 868 | 0.0296 | -0.6947 | 1368 | 0.4969 | -0.1672 | 1868 | -0.3676 | -1.0043 |
| 369 | 0.25 | -1.25 | 869 | 0.1239 | 0.7468 | 1369 | -0.2803 | 0.0732 | 1869 | -0.153 | -0.4651 |
| 370 | 0.3459 | -0.1333 | 870 | 0.4302 | 0.821 | 1370 | -0.4692 | 0.361 | 1870 | 0.4652 | 0.0878 |
| 371 | -0.3382 | 0.9638 | 871 | -0.7965 | 0.3 | 1371 | -0.5611 | -0.1364 | 1871 | 1.4612 | 0.7954 |
| 372 | -0.6312 | -0.5158 | 872 | -0.7702 | -0.1656 | 1372 | -0.4332 | -0.3624 | 1872 | 0.6684 | 0.2049 |
| 373 | 0.5269 | -0.2276 | 873 | 1.4053 | 0.4053 | 1373 | 0.4743 | -0.5845 | 1873 | 0 | 0.8839 |
| 374 | 0.8836 | 0.4284 | 874 | 0.3555 | 1.2629 | 1374 | 0.4699 | -0.1637 | 1874 | 1.0586 | -0.3127 |
| 375 | 0.4219 | 0.6784 | 875 | -0.3756 | 1.0032 | 1375 | -0.4154 | -0.188 | 1875 | 0.2661 | -0.7956 |
| 376 | 0.2871 | 1.2165 | 876 | -0.1669 | 0.606 | 1376 | -0.1374 | -0.3935 | 1876 | -1.3694 | 0.2729 |
| 377 | 0.1036 | 0.6339 | 877 | -0.3919 | -0.4857 | 1377 | -0.25 | 0.5303 | 1877 | -0.5383 | -1.3953 |
| 378 | -0.7761 | 0.0741 | 878 | 0.4783 | -0.7561 | 1378 | -0.8872 | 0.0904 | 1878 | 0.399 | 0.2903 |
| 379 | -1.1903 | -0.3727 | 879 | -0.8041 | -0.0645 | 1379 | 0.1095 | -0.6144 | 1879 | 0.2723 | 0.6389 |
| 380 | -0.9756 | 0.4563 | 880 | -1.2097 | -0.5419 | 1380 | 0.1946 | -0.3511 | 1880 | 0.0775 | -1.3992 |
| 381 | 0.0111 | 0.4458 | 881 | 0 | 0 | 1381 | -0.0558 | 0.1274 | 1881 | -0.6553 | 0.2286 |
| 382 | 0.8594 | -0.5478 | 882 | -0.7224 | -1.061 | 1382 | 0.7378 | 1.0306 | 1882 | -0.7675 | 0.0927 |
| 383 | -0.564 | 0.4328 | 883 | 0.0986 | -0.2501 | 1383 | 0.1524 | 0.7855 | 1883 | 0.2387 | -0.1194 |
| 384 | -0.3216 | 0.5171 | 884 | -0.6047 | 1.2378 | 1384 | -0.7686 | -0.1897 | 1884 | 0.5741 | -0.1968 |
| 385 | 1.25 | 0 | 885 | -0.3547 | -0.028 | 1385 | -1.0303 | -0.0732 | 1885 | -0.2693 | -0.8265 |
| 386 | -0.3216 | -0.5171 | 886 | 0.4239 | -0.645 | 1386 | -0.9171 | 0.8333 | 1886 | -1.3383 | 0.3229 |
| 387 | -0.564 | -0.4328 | 887 | 0.3278 | -0.6477 | 1387 | -0.3681 | 1.1834 | 1887 | -1.2295 | -0.1181 |
| 388 | 0.8594 | 0.5478 | 888 | -0.3022 | -0.2338 | 1388 | -0.1581 | 0.2614 | 1888 | -0.4267 | 0.1254 |
| 389 | 0.0111 | -0.4458 | 889 | 0.4053 | 1.0089 | 1389 | -0.6118 | -0.6114 | 1889 | -0.5303 | 0.3536 |
| 390 | -0.9756 | -0.4563 | 890 | 1.3666 | 0.6059 | 1390 | -0.1488 | 0.3559 | 1890 | -0.3639 | -0.3556 |
| 391 | -1.1903 | 0.3727 | 891 | 0.1337 | -0.0563 | 1391 | 1.1808 | 0.7177 | 1891 | 0.3827 | -0.2084 |
| 392 | -0.7761 | -0.0741 | 892 | 0.882 | -0.3037 | 1392 | 0.3623 | -1.1271 | 1892 | 0.3752 | -0.6378 |
| 393 | 0.1036 | -0.6339 | 893 | 0.4167 | -0.2635 | 1393 | 0.25 | -0.8839 | 1893 | 0.4426 | 0.7547 |
| 394 | 0.2871 | -1.2165 | 894 | -0.7497 | -0.0026 | 1394 | 1.396 | -0.5758 | 1894 | 0.4059 | 0.7148 |
| 395 | 0.4219 | -0.6784 | 895 | 0.8389 | -1.3511 | 1395 | 0.1183 | -1.1766 | 1895 | -0.0767 | -0.7739 |
| 396 | 0.8836 | -0.4284 | 896 | -0.0407 | -1.4127 | 1396 | -1.0588 | 0.6119 | 1896 | 0.2352 | 0.6192 |
| 397 | 0.5269 | 0.2276 | 897 | 0.75 | 0.25 | 1397 | -1.5214 | -0.1543 | 1897 | 0.1982 | 0.375 |
| 398 | -0.6312 | 0.5158 | 898 | 1.3445 | 0.4739 | 1398 | -0.3341 | -0.3884 | 1898 | -0.142 | -0.7982 |
| 399 | -0.3382 | -0.9638 | 899 | -0.8199 | 0.3431 | 1399 | 0.9788 | 1.7396 | 1899 | 0.7575 | 0.2273 |
| 400 | 0.3459 | 0.1333 | 900 | -0.0429 | 0.0252 | 1400 | -0.6621 | -0.1632 | 1900 | 0.8086 | 0.9322 |
| 401 | 0.25 | 1.25 | 901 | -0.5296 | -0.3943 | 1401 | -0.7803 | -0.9268 | 1901 | -0.097 | -0.1385 |
| 402 | 1.0998 | 0.4827 | 902 | -0.1959 | -0.2655 | 1402 | -0.8951 | 0.2553 | 1902 | 0.3477 | -0.5561 |
| 403 | 0.4623 | -0.2948 | 903 | 0.7708 | -0.1596 | 1403 | -0.9663 | 0.1538 | 1903 | 0.8342 | 0.5761 |
| 404 | 0.0015 | -1.1597 | 904 | -1.1196 | 0.2988 | 1404 | 0.9929 | 0.0152 | 1904 | 0.421 | 0.5152 |
| 405 | 1.1802 | -0.4189 | 905 | 0.4786 | 0.2286 | 1405 | 0.5864 | -0.1226 | 1905 | 0 | 0.1768 |
| 406 | -0.4653 | 0.3832 | 906 | 1.8203 | -0.2119 | 1406 | 0.0592 | 0.814 | 1906 | -0.0727 | 0.2687 |
| 407 | -0.6222 | 0.3373 | 907 | 0.0559 | 0.1768 | 1407 | -0.4049 | -0.4701 | 1907 | 0.7228 | 0.96 |
| 408 | 0.8113 | -0.6423 | 908 | -0.7192 | 0.4991 | 1408 | -0.8584 | -1.1959 | 1908 | 1.0263 | 1.2571 |
| 409 | -0.6036 | -1.1339 | 909 | -0.6297 | -0.1409 | 1409 | 0.8107 | 0.5303 | 1909 | -0.2117 | -0.4154 |
| 410 | -0.5391 | -0.207 | 910 | 0.2035 | -0.9139 | 1410 | 0.5755 | -0.5129 | 1910 | -0.7483 | -0.1364 |
| 411 | -0.3165 | -1.0258 | 911 | -0.3151 | 0.0227 | 1411 | -0.213 | -0.0564 | 1911 | 0.1785 | 0.1946 |
| 412 | -0.5326 | -0.3518 | 912 | -1.3792 | 0.8238 | 1412 | 0.6503 | 0.7214 | 1912 | 0.7065 | -0.5663 |
| 413 | 0.2817 | 0.9077 | 913 | -0.25 | 0.25 | 1413 | 1.4093 | -0.3345 | 1913 | -0.4053 | 0.4786 |
| 414 | -0.5541 | -0.4205 | 914 | 0.6563 | 0.5325 | 1414 | 0.7772 | 0.1274 | 1914 | -1.661 | -0.0392 |
| 415 | 0.147 | -0.178 | 915 | -0.3756 | 0.2564 | 1415 | -0.0901 | -0.2903 | 1915 | -0.4411 | -0.4487 |
| 416 | 0.5069 | 0.5395 | 916 | -0.6222 | -0.3347 | 1416 | 0.6632 | 0.3901 | 1916 | 1.1159 | 0.2276 |
| 417 | -0.75 | 0 | 917 | 0.3547 | 0.028 | 1417 | -0.0303 | 0.9268 | 1917 | 0.5193 | -0.6912 |
| 418 | 0.5069 | -0.5395 | 918 | -0.1308 | 0.2017 | 1418 | -0.2958 | -0.4719 | 1918 | 0.4769 | -0.3546 |
| 419 | 0.147 | 0.178 | 919 | -0.7871 | 1.1608 | 1419 | 0.6884 | 0.2991 | 1919 | 0.6412 | 0.4537 |
| 420 | -0.5541 | 0.4205 | 920 | -0.7915 | 0.6192 | 1420 | 0.2517 | 0.2667 | 1920 | -0.8383 | -0.039 |
| 421 | 0.2817 | -0.9077 | 921 | -0.6553 | -0.7589 | 1421 | 0.9654 | -0.8028 | 1921 | -0.1768 | 0.7071 |
| 422 | -0.5326 | 0.3518 | 922 | -0.3077 | 0.685 | 1422 | 0.5076 | 0.3542 | 1922 | 0.0188 | 0.0162 |
| 423 | -0.3165 | 1.0258 | 923 | 0.1051 | 0.4707 | 1423 | -0.1534 | 0.8546 | 1923 | -0.0149 | 0.0996 |
| 424 | -0.5391 | 0.207 | 924 | 0.765 | -1.2526 | 1424 | 0.9041 | -0.5678 | 1924 | 0.0536 | 0.6254 |
| 425 | -0.6036 | 1.1339 | 925 | -0.5632 | -0.5901 | 1425 | -0.1036 | -0.1768 | 1925 | 1.2606 | 0.0549 |
| 426 | 0.8113 | 0.6423 | 926 | -0.9437 | -0.1152 | 1426 | -0.5071 | 1.4348 | 1926 | 0.5406 | -0.1999 |
| 427 | -0.6222 | -0.3373 | 927 | 0.4703 | -0.3946 | 1427 | -0.2218 | 0.3474 | 1927 | -0.6483 | 0.1982 |
| 428 | -0.4653 | -0.3832 | 928 | -0.1704 | -0.2313 | 1428 | -0.4394 | -0.5773 | 1928 | 0.4289 | -1.1884 |
| 429 | 1.1802 | 0.4189 | 929 | 0.5 | 0.5 | 1429 | 0.1678 | -0.3457 | 1929 | -0.0214 | -0.8018 |
| 430 | 0.0015 | 1.1597 | 930 | 0.8652 | 1.0323 | 1430 | -0.0687 | -0.7158 | 1930 | 0.2803 | 0.4548 |
| 431 | 0.4623 | 0.2948 | 931 | -0.1102 | 0.8577 | 1431 | 0.1659 | -0.3205 | 1931 | 0.7002 | -0.3675 |
| 432 | 1.0998 | -0.4827 | 932 | 0.7064 | 1.19 | 1432 | 0.4969 | -0.1672 | 1932 | -1.0376 | -0.2012 |
| 433 | 0.25 | -1.25 | 933 | 0.5296 | 0.3943 | 1433 | -0.2803 | 0.0732 | 1933 | -0.8039 | -0.074 |
| 434 | 0.3459 | -0.1333 | 934 | 0.1419 | -0.6515 | 1434 | -0.4692 | 0.361 | 1934 | -0.3884 | 0.1453 |
| 435 | -0.3382 | 0.9638 | 935 | 0.1885 | 0.5606 | 1435 | -0.5611 | -0.1364 | 1935 | -0.6539 | 0.3117 |
| 436 | -0.6312 | -0.5158 | 936 | -0.1471 | 0.7344 | 1436 | -0.4332 | -0.3624 | 1936 | -1.1651 | -1.1555 |
| 437 | 0.5269 | -0.2276 | 937 | -0.2286 | -0.4786 | 1437 | 0.4743 | -0.5845 | 1937 | -1.7678 | -0.8839 |
| 438 | 0.8836 | 0.4284 | 938 | -0.1944 | -0.6425 | 1438 | 0.4699 | -0.1637 | 1938 | -0.7176 | 0.0897 |
| 439 | 0.4219 | 0.6784 | 939 | -0.0877 | -0.7983 | 1439 | -0.4154 | -0.188 | 1939 | 0.1292 | -0.3738 |
| 440 | 0.2871 | 1.2165 | 940 | -0.95 | -1.0612 | 1440 | -0.1374 | -0.3935 | 1940 | 0.8451 | -0.1536 |
| 441 | 0.1036 | 0.6339 | 941 | -0.2238 | -0.0056 | 1441 | 0.0732 | 0.7071 | 1941 | -0.0497 | 0.0946 |
| 442 | -0.7761 | 0.0741 | 942 | 0.4224 | 0.9778 | 1442 | -0.1505 | 0.1106 | 1942 | -0.8163 | -0.3697 |
| 443 | -1.1903 | -0.3727 | 943 | -0.494 | -0.1912 | 1443 | 0.7487 | -0.2539 | 1943 | 0.712 | -0.1527 |
| 444 | -0.9756 | 0.4563 | 944 | 0.5364 | -0.5984 | 1444 | 0.5702 | -0.3195 | 1944 | -0.2692 | 0.0432 |
| 445 | 0.0111 | 0.4458 | 945 | 0 | 0 | 1445 | 0.2407 | -0.4889 | 1945 | -0.4786 | -0.1982 |
| 446 | 0.8594 | -0.5478 | 946 | -0.7224 | -1.061 | 1446 | 1.4049 | 0.235 | 1946 | 0.9743 | -0.3294 |
| 447 | -0.564 | 0.4328 | 947 | 0.0986 | -0.2501 | 1447 | 0.267 | 0.8823 | 1947 | 0.9417 | -0.7351 |
| 448 | -0.3216 | 0.5171 | 948 | -0.6047 | 1.2378 | 1448 | -0.1098 | 0.5457 | 1948 | 0.5535 | -0.3754 |
| 449 | 1.25 | 0 | 949 | -0.3547 | -0.028 | 1449 | 0 | 0.4571 | 1949 | -1.1141 | -0.4582 |
| 450 | -0.3216 | -0.5171 | 950 | 0.4239 | -0.645 | 1450 | -1.0648 | 0.1166 | 1950 | -0.3284 | -0.1895 |
| 451 | -0.564 | -0.4328 | 951 | 0.3278 | -0.6477 | 1451 | -0.1968 | 0.4275 | 1951 | 0.7145 | 1.911 |
| 452 | 0.8594 | 0.5478 | 952 | -0.3022 | -0.2338 | 1452 | -0.2262 | 0.5005 | 1952 | -1.0893 | 0.7769 |
| 453 | 0.0111 | -0.4458 | 953 | 0.4053 | 1.0089 | 1453 | -0.1371 | -0.9889 | 1953 | 0.1768 | -0.3536 |
| 454 | -0.9756 | -0.4563 | 954 | 1.3666 | 0.6059 | 1454 | 0.9962 | -0.8207 | 1954 | 1.2181 | 0.7631 |
| 455 | -1.1903 | 0.3727 | 955 | 0.1337 | -0.0563 | 1455 | 0.7019 | 0.0296 | 1955 | -0.4021 | -0.3447 |
| 456 | -0.7761 | -0.0741 | 956 | 0.882 | -0.3037 | 1456 | -0.2743 | 0.2598 | 1956 | -0.923 | 0.2865 |
| 457 | 0.1036 | -0.6339 | 957 | 0.4167 | -0.2635 | 1457 | -0.7803 | 0.3536 | 1957 | -0.2606 | 0.6522 |
| 458 | 0.2871 | -1.2165 | 958 | -0.7497 | -0.0026 | 1458 | 0.2517 | -0.5182 | 1958 | 0.692 | -0.9556 |
| 459 | 0.4219 | -0.6784 | 959 | 0.8389 | -1.3511 | 1459 | -0.5378 | -0.1761 | 1959 | 0.3486 | -0.0292 |
| 460 | 0.8836 | -0.4284 | 960 | -0.0407 | -1.4127 | 1460 | -1.694 | -0.1293 | 1960 | 0.3173 | 0.7422 |
| 461 | 0.5269 | 0.2276 | 961 | -0.8839 | 0.3536 | 1461 | -0.4844 | -0.6802 | 1961 | 0.7286 | 0.4482 |
| 462 | -0.6312 | 0.5158 | 962 | -0.0336 | 0.8687 | 1462 | -0.9842 | 0.2458 | 1962 | -0.3733 | -0.2533 |
| 463 | -0.3382 | -0.9638 | 963 | -0.7799 | 0.1728 | 1463 | -1.1416 | -0.1229 | 1963 | -0.4999 | -0.1482 |
| 464 | 0.3459 | 0.1333 | 964 | 0.1891 | -0.6159 | 1464 | -0.0986 | 0.3233 | 1964 | 0.0756 | 0.04 |
| 465 | 0.25 | 1.25 | 965 | 0.6734 | -0.4006 | 1465 | -0.25 | 0.6036 | 1965 | 0.0968 | -1.1331 |
| 466 | 1.0998 | 0.4827 | 966 | 0.2461 | 0.05 | 1466 | -0.5928 | -1.5937 | 1966 | 0.9873 | 0.4546 |
| 467 | 0.4623 | -0.2948 | 967 | 1.184 | 0.4273 | 1467 | -0.1564 | -0.3116 | 1967 | 0.7369 | 1.2041 |
| 468 | 0.0015 | -1.1597 | 968 | 0.4654 | 0.1181 | 1468 | 0.4498 | 1.6195 | 1968 | -0.7251 | 0.0654 |
| 469 | 1.1802 | -0.4189 | 969 | -1.5089 | -0.125 | 1469 | -0.0273 | 0.4731 | 1969 | 0.3536 | 0.5303 |
| 470 | -0.4653 | 0.3832 | 970 | -0.2443 | -0.146 | 1470 | 0.5568 | -0.6178 | 1970 | 1.1877 | -0.4863 |
| 471 | -0.6222 | 0.3373 | 971 | 0.7714 | -0.6208 | 1471 | 0.3324 | -0.8678 | 1971 | -0.005 | -0.7952 |
| 472 | 0.8113 | -0.6423 | 972 | -0.6118 | -0.675 | 1472 | -0.7073 | 0.476 | 1972 | -0.224 | 0.1656 |
| 473 | -0.6036 | -1.1339 | 973 | -0.8753 | -0.5169 | 1473 | 0.4268 | 0 | 1973 | -0.9503 | 0.6125 |
| 474 | -0.5391 | -0.207 | 974 | -0.5449 | -0.4082 | 1474 | -0.4977 | -0.9329 | 1974 | -0.7091 | 0.6012 |
| 475 | -0.3165 | -1.0258 | 975 | 0.4193 | 0.2374 | 1475 | -0.6901 | 1.2402 | 1975 | 0.2949 | -0.6021 |
| 476 | -0.5326 | -0.3518 | 976 | 1.6786 | 0.8281 | 1476 | 0.7908 | 0.7534 | 1976 | -0.8776 | 0.0204 |
| 477 | 0.2817 | 0.9077 | 977 | 1.4142 | 0.8839 | 1477 | 0.32 | -0.2183 | 1977 | -0.2286 | 0.5518 |
| 478 | -0.5541 | -0.4205 | 978 | 0.5174 | 0.1582 | 1478 | 1.0434 | 0.7155 | 1978 | 0.8257 | -0.2548 |
| 479 | 0.147 | -0.178 | 979 | 0.9204 | -0.7007 | 1479 | 0.4069 | -0.0239 | 1979 | 0.5651 | -0.1633 |
| 480 | 0.5069 | 0.5395 | 980 | 0.9337 | -0.233 | 1480 | -1.1428 | 0.3316 | 1980 | 1.2426 | -0.3872 |
| 481 | 0.75 | 0.75 | 981 | -1.1459 | 0.74 | 1481 | 0 | 0.4571 | 1981 | 0.407 | 0.2511 |
| 482 | 0.7478 | 0.8804 | 982 | 0.0935 | 0.3738 | 1482 | 0.7672 | -0.3292 | 1982 | -0.5633 | 0.5135 |
| 483 | -1.8473 | 0.0292 | 983 | 1.4272 | -0.6626 | 1483 | 0.1381 | -0.0534 | 1983 | -0.0903 | -0.0125 |
| 484 | -1.0324 | 0.3448 | 984 | -0.8333 | -0.0067 | 1484 | -0.4874 | -0.8099 | 1984 | -0.0342 | 0.6959 |
| 485 | 0.1609 | 0.3108 | 985 | -0.1553 | 1.0821 | 1485 | -0.2164 | -0.7183 | 1985 | -0.1768 | 0.7071 |
| 486 | 0.1388 | -0.4451 | 986 | -0.1296 | -0.1823 | 1486 | -0.243 | -0.2394 | 1986 | 0.0188 | 0.0162 |
| 487 | -0.106 | -1.0522 | 987 | -0.6472 | -1.0287 | 1487 | -0.1813 | -0.2886 | 1987 | -0.0149 | 0.0996 |
| 488 | -0.8661 | -0.2156 | 988 | 0.8176 | -0.0763 | 1488 | -0.3657 | 0.4548 | 1988 | 0.0536 | 0.6254 |
| 489 | 0.125 | 0.8321 | 989 | 0.0201 | -0.0676 | 1489 | -1.1339 | -0.3536 | 1989 | 1.2606 | 0.0549 |
| 490 | 0.1346 | 0.4 | 990 | 0.0225 | 0.0677 | 1490 | 0.3331 | -0.3613 | 1990 | 0.5406 | -0.1999 |
| 491 | -0.5951 | 0.2595 | 991 | -0.0539 | 0.6489 | 1491 | 0.9791 | 0.397 | 1991 | -0.6483 | 0.1982 |
| 492 | -0.1195 | 0.4088 | 992 | -0.2031 | 0.0917 | 1492 | 0.7792 | -0.7507 | 1992 | 0.4289 | -1.1884 |
| 493 | -0.5779 | 0.5428 | 993 | 0.5303 | -0.3536 | 1493 | 0.6308 | -0.0269 | 1993 | -0.0214 | -0.8018 |
| 494 | -0.255 | 0.5079 | 994 | -0.4082 | -0.3206 | 1494 | -0.9103 | 1.0976 | 1994 | 0.2803 | 0.4548 |
| 495 | 0.9598 | 0.3728 | 995 | 0.4178 | 0.0615 | 1495 | -0.0323 | 0.0574 | 1995 | 0.7002 | -0.3675 |
| 496 | -0.313 | 1.1997 | 996 | 0.3409 | 0.0797 | 1496 | 0.4147 | -0.264 | 1996 | -1.0376 | -0.2012 |
| 497 | -1 | 0 | 997 | -0.8805 | -0.8065 | 1497 | 0.25 | 0.6036 | 1997 | -0.8039 | -0.074 |
| 498 | 0.6711 | -1.687 | 998 | -0.2006 | -0.1009 | 1498 | 1.3681 | 1.0939 | 1998 | -0.3884 | 0.1453 |
| 499 | -0.2918 | -0.4937 | 999 | -0.0355 | 0.8364 | 1499 | -0.2849 | 0.1445 | 1999 | -0.6539 | 0.3117 |
| 500 | -1.2035 | -0.2092 | 1000 | 0.3413 | 0.0992 | 1500 | -0.5966 | -0.4497 | 2000 | -1.1651 | -1.1555 |