IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | CRC-8-CCITT HEC for PPDU option fields |  |
| Date Submitted | September, 2010 |  |
| Source | [Roberts, Richard D] [<company>] [address] | Voice: [ ] Fax: [ ] E-mail: Richard.d.roberts@intel.com |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [Description of document contents.] |  |
| Purpose | [Description of what the author wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

5.4.2.x PPDU option fields header error control sequence generation

The PPDU option field HEC value is calculated across the compensation length, resync length and subframe length fields and inserted into the appropriate header field.

The HEC field shall be an 8-bit sequence. It shall be the remainder of the division (modulo 2) by the generator polynomial x8 + x2 + x + 1 of the product x8 multiplied by the content of the header excluding the HEC field.

The initial content of the register of the device computing the remainder of the division is preset to all 0s and is then modified by division of the header excluding the HEC field by the generator polynomial (as described above); the resulting remainder is the 8-bit HEC.

To significantly improve the cell delineation performance in the case of bit-slips, the following is

recommended:

- the check bits calculated by the use of the check polynomial are added (modulo 2) to an 8-bit pattern before being inserted in the last octet of the header;

- the recommended pattern is "0101 0101" (the left bit is the most significant bit);

- the receiver must subtract (equal to add modulo 2) the same pattern from the 8 HEC bits before calculating the syndrome of the header.

This operation in no way affects the error detection/correction capabilities of the HEC.

As an example, if the first 4 octets of the header were all zeros, the generated header before

scrambling would be "0000 0000 0000 0000 0000 0000 0000 0000 0101 0101". The starting value for the polynomial check is 0 (binary).

Reference ITU-T I.432.1, http://www.itu.int/rec/T-REC-I.432.1-199902-I/en