IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | CRC-8-CCITT HEC for PPDU option fields |  |
| Date Submitted | September, 2010 |  |
| Source | [Roberts, Richard D] [<company>] [address] | Voice: [ ] Fax: [ ] E-mail: Richard.d.roberts@intel.com |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [Description of document contents.] |  |
| Purpose | [Description of what the author wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

This document resolves T-CID 62

5.4.2.10 PPDU option fields check sequence generation

The PPDU option fields check sequence (OFCS) value is calculated across the compensation length, resync length and subframe length fields (as shown in Figure 23a) and inserted into the OFCS field.

The OFCS field shall be an 8-bit sequence. It shall be the remainder of the division (modulo 2) by the generator polynomial x8 + x2 + x + 1 of the product x8 multiplied by the content of the header excluding the OFCS field.

The initial content of the register of the device computing the remainder of the division is preset to all 1s and is then modified by division of the header, excluding the OFECS field, by the generator polynomial (as described above). The resulting remainder is the 8-bit OFECS.

Reorder the subclauses as shown below.

5.4.2	PHY header

5.4.2.1	Burst mode

5.4.2.2	Channel number

5.4.2.3	MCS ID

5.4.2.4	Length of PSDU field

5.4.2.5	Dimmed OOK extension

5.4.2.6	HCS

5.4.2.7	Compensation length

5.4.2.8	Resync length

5.4.2.9	Subframe length and generation

5.5.2.10 	OFCS field

5.4.2.11 	Channel estimation sequence

5.4.2.12 	PSDU field

Reference ITU-T I.432.1, http://www.itu.int/rec/T-REC-I.432.1-199902-I/en