IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Proposed Replacement Text for BAN Coexistence Subclause 7.12.4 |
| Date Submitted | September 13, 2010 |
| Source | Seunng-Hoon Park, Jin Meng Ho |
| Re: | Letter ballot #55 comments on 7.12.4 of IEEE P802.15.6/D01. |
| Abstract | This submission provides the replacement text and figures for the subclauses of IEEE P802.15.6/D01 identified in the above. |
| Purpose | To address letter ballot #55 comments on the subclauses of IEEE P802.15.6/D01 identified in the above. |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

— Command ID Field encoding

- — BAN Priority field encoding

|  | BAN services |
| --- | --- |

- Allocation reduction and restoration

A hub may reduce or remove the allocations to the services of its body area network (BAN) when other coexistence mechanisms are applicable or not applicable. A hub may restore the allocations by increasing the number of slots per allocation timing for the session. A hub may do this load control when coexistence information is exchanged, as specified in .

- — Allocation reduction for load control using BAN priority

| BAN services | BAN priority | Allocation reduction | Allocation restoration (# of slots increasing) (initial value is 1) |
| --- | --- | --- | --- |
| Non-medical services | Lowest | 100% | +1 |
| Mixed medical and non-medical services | Second lowest | 20% | +2 |
| General health services | Second highest | 10% | +3 |
| Highest priority medical services | Highest | 0% | 0 |

A hub may reduce or remove the allocations to the services of its body area network (BAN) when other coexistence mechanisms are applicable or not applicable. A hub may restore the allocations by increasing the number of slots per allocation timing for the session. A hub may do this load control when coexistence information is not exchanged and a hub recognizes the interference from the neighboring BAN, as specified in Table 22.

- 2 — Allocation reduction for load control when coexistence information is absence

| BAN services | traffic priority | Allocation reduction | Allocation restoration (# of slots increasing) (initial value is 1) |
| --- | --- | --- | --- |
| Background (BK) | 0 | 100 | + 1 |
| Best effort (BE) | 1 | 100 | + 2 |
| Excellent effort (EE) | 2 | 75 | + 1 |
| Controlled load (CL) | 3 | 75 | + 2 |
| Video (VI) | 4 | 50 | X 2 |
| Voice (VO) | 5 | 50 | X 2 |
| Medical data or network control | 6 | 10 | X 2 |
| Emergency or medical event report | 7 | 0 | 0 |