IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PIB Attribute Coordination |  |
| Date Submitted | 12 September 2010 |  |
| Source | Larry Taylor (DTC (UK)) | Voice: Fax: E-mail: larry.taylor@acm.org |
| Re: | PIB Attribute Coordination for 15.4g |  |
| Abstract | IEs and PIB Attribute encoding for PIB Attribute coordination |  |
| Purpose | Comment resolution |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

PIB Attribute Coordination Information Elements

The following Information Elements (IEs) are defined to enable coordination of PIB Attributes required for ordered operation of SUN networks. All of these IEs are declarative except for the Query IE. A device may include Declarative IEs in any MAC frame which may contain IEs (see xx.yy):

- At any time in order to declare PIB Attribute information in an unsolicited manner

- In response to receipt of a Query IE

Table x1 — PIB Attribute Coordination IEs

| IE ID | Content Length | Name | Description |
| --- | --- | --- | --- |
| Base + 0x00 | Variable | Query | Contains a list of IE IDs requested |
| Base + 0x01 | Variable | SUN PHY Capabilities | Declares the devices PHY Capabilities |
| Base + 0x02 | Variable | PIB Attribute | Declares the value of a PIB Attribute |

Editor’s notes:

- Base is the offset into the table of IEs to provide non-overlapping IE IDs to any other IEs defined in 15.4 enhancements

- Each IE diagram below will need to be made compatible with the actual IE structure agreed by 15.4e

xx.yy Query IE

Figure x1 Query IE Format

A Query IE contains a list of IE IDs requested by the originator and may be included in a MAC frame with unicast or broadcast destination address. Each IE is identified by its ID except for PIB Attribute IEs which require both the PIB Attribute IE ID and the ID of the PIB attribute requested (see PIB Attribute IE below).

A recipient device should include declarative IEs of the type requested in the Query in subsequent MAC frames. The MAC frames containing the requested IEs should be directed to the originator unless the frame was received via a broadcast address.

The Query IE Length field is set to the sum of the IE IDs contained in the Information Field (note that the PIB Attribute IE requires 2 octets, all others require 1 octet).

The Query IE Information Field is set to the list of IE IDs and/or PIB Attribute IE + PIB Attribute IDs requested.

xx.yy SUN PHY Capabilities IE

Figure x2 SUB PHY Capabilities IE Format

The SUN PHY Capabilities IE declares which SUN features a device supports.

The Length Filed is set to 2 + 2*Number of PHY Types supported.

Miscellaneous features are encoded as a bit map in the first octet of the Information Field. A bit set to 1 indicated the feature is supported otherwise the feature is not supported.

- Bit 4 indicates support for the Mode Switch feature (see 6.12a.3)

- Bit 3 indicates support for FEC Scheme 1 (6.12a.1.3)

- Bit 2 indicates support for FEC Scheme 0 (see 6.12a.1.3)

- Bit 1 indicates support for SFD Group 1 (see 6.3a.1.2)

- Bit 0 indicates support for Interleaving (see 6.12a.1.4)

| Bits 15..13 | Bits 12..0 |
| --- | --- |
| Rsvd | Frequency Band |

Figure x3 Frequency Bands Supported Format

The frequency bands defined in Table 4a are encoded into a 16-bit map as shown in Figure x3, with the Band ID in Table 4a corresponding to the bit-index into the map. The bits corresponding to undefined band identifiers are reserved. A bit set to 1 indicated the device supports operation in that frequency band otherwise it does not.

| Bits 15..12 | Bits 11..0 |
| --- | --- |
| PHY Type | PHY Modes |

Figure x4 PHY Type and PHY Modes Supported Format

PHY Types are encoded as 16-bit structures as shown in Figure x4. Bits 15-12 encode the Modulation Scheme as a 4-bit unsigned integer as defined by Table x2.

Table x2 — PHY Type Encoding

| PHY Type | Modulation Scheme |
| --- | --- |
| 0 | Narrowband GFSK |
| 1 | GFSK/FSK |
| 2 | O-QPSK (DSSS) |
| 3 | O-QPSK (MDSS) |
| 4 | OFDM Option 1 |
| 5 | OFDM Option 2 |
| 6 | OFDM Option 3 |
| 7 | OFDM Option 4 |
| 8 | OFDM Option 5 |
| 15..8 | Reserved |

Bits 11-0 provide a bitmap indicating which PHY options are supported.

Narrowband FSK PHY Modes are defined by Table x3.

Table x3 — Narrowband FSK PHY Mode Encoding

| Bit Position | Narrowband FSK PHY Mode |
| --- | --- |
| 0 | 5 kbps; GFSK; mod index = 0.5; channel spacing = 12.5 kHz |
| 1 | 10 kbps; GFSK; mod index = 0.5; channel spacing = 25 kHz |
| 2 | 20 kbps; GFSK; mod index = 0.5; channel spacing = 50 kHz |
| 15-3 | Reserved |

FSK PHY Modes are defined by Table x4.

Table x4 —FSK PHY Mode Encoding

| Bit Position | FSK PHY Mode |
| --- | --- |
| 0 | 40 kbps; GFSK; mod index = 0.5; channel spacing = 100 kHz |
| 1 | 50 kbps; GFSK; mod index = 1.0; channel spacing = 200 kHz |
| 2 | 50 kbps; FSK; mod index = 1.0; channel spacing = 200 kHz |
| 3 | 100 kbps; GFSK; mod index = 1.0; channel spacing = 400 kHz |
| 4 | 150 kbps; FSK; mod index = 0.5; channel spacing = 400 kHz |
| 5 | 200 kbps; GFSK; mod index = 0.5; channel spacing = 400 kHz |
| 6 | 200 kbps; 4-GFSK; mod index = 1/3; channel spacing = 400 kHz |
| 7 | 200 kbps; GFSK; mod index = 1.0; channel spacing = 600 kHz |
| 8 | 400 kbps; 4-GFSK; mod index = 0.33; channel spacing = 600 kHz |
| 9 | 50 kbps; GFSK; mod index = 1.0; channel spacing = 200/400 kHz |
| 15-10 | Reserved |

O-QPSK (DSSS) PHY Modes are defined in Table x5.

Table x5 — O-QPSK (DSSS) PHY Mode Encoding

| Bit Position | OQPSK DSSS PHY Mode |
| --- | --- |
| 0 | chip rate = 100 kchip/s; (4,1) spreading; rate 1/2 FEC; data rate = 12.5 kbps |
| 1 | chip rate = 100 kchip/s; no spreading; rate 1/2 FEC; data rate = 50 kbps |
| 2 | chip rate = 1000 kchip/s; (16,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 3 | chip rate = 1000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 4 | chip rate = 1000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 5 | chip rate = 1000 kchip/s; no spreading; rate 1/2 FEC; data rate = 500 kbps |
| 6 | chip rate = 2000 kchip/s; (32,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 7 | chip rate = 2000 kchip/s; (32,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 8 | chip rate = 2000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 9 | chip rate = 2000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 500 kbps |
| 15-10 | Reserved |

O-QPSK (MDSS) PHY Modes are defined in Table x6.

Table x6 — O-QPSK (MDSS) PHY Mode Encoding

| Bit Position | OQPSK MDSS PHY Mode |
| --- | --- |
| 0 | chip rate = 1000 kchip/s; (16,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 1 | chip rate = 1000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 2 | chip rate = 1000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 3 | chip rate = 1000 kchip/s; no spreading; rate 1/2 FEC; data rate = 500 kbps |
| 4 | chip rate = 2000 kchip/s; (32,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 5 | chip rate = 2000 kchip/s; (32,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 6 | chip rate = 2000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 7 | chip rate = 2000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 500 kbps |
| 15-8 | Reserved |

For each OFDM Option, bits 8..0 indicate which MCSs are supported as shown in Table x7.

Table x7 — OFDM PHY Mode Encoding

| Bit Position | OFDM PHY Modes (Option 1..5) |
| --- | --- |
| 0 | MCS 1 supported |
| 1 | MCS 2 supported |
| 2 | MCS 3 supported |
| 3 | MCS 4 supported |
| 4 | MCS 5 supported |
| 5 | MCS 6 supported |
| 6 | MCS 7 supported |
| 7 | MCS 8 supported |
| 8 | MCS 9 supported |
| 15-9 | Reserved |

In each encoding of PHY Modes in the PHY Type Supported structure, a bit set to 1 in bit position m for a given PHY Type in Table 3..Table 7 indicates the PHY Mode for that PHY Type corresponding to row m is supported. A value of 0 similarly indicated the PHY Mode is not supported.

xx.yy PIB Attribute IE

Figure x5 PIB Attribute IE Format

A PIB Attribute IE contains one or more {TLV} structures each describing one PIB Attribute. The {TLV} structure contains:

- PIB Attribute ID

- Length of Attribute data

- PIB Attribute data

The PIB Attribute IE Length is set to the sum of (2 + the length of the PIB Attribute data) for each PIB Attribute included in the IE Information Field

xx.yy.zz Generic PHY Descriptor PIB Attribute

The Generic PHY Descriptor is encoded as shown in Figure x6 :

| Octets:1 |  |  | 1 |  | 4 | 2 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: 7..4 | 3..2 | 1..0 | Bits: 7-6 | 5-0 | 1st Channel Centre Frequency | Number of Channels | Channel Spacing | Symbol Rate |
| Generic PHY Descriptor ID | Modulation Scheme | Modulation Order | BT | Modulation Index |  |  |  |  |

Figure x6 Generic PHY Descriptor Format

Generic PHY Descriptor ID is an unsigned 4 bit integer as defined in section 6.1.2.7.2.

Modulation Scheme is an unsigned 2 bit integer whose values are defined in Table 4b (undefined values are Reserved).

Modulation Order is a 2-bit unsigned integer as defined in Table 8:

Table x8 — Modulation Order Encoding

| Modulation Order Field | Modulation Order |
| --- | --- |
| 0 | 2-FSK |
| 1 | 4-FSK |
| 2-3 | Reserved |

BT is encoded as a 2-bit unsigned integer as defined in Table 9:

Table x9 — BT Encoding

| BT Field | BT |
| --- | --- |
| 0 | 0.5 |
| 1 | 1.0 |
| 2-3 | Reserved |

Modulation Index is encoded as a 6-bit unsigned integer with permissible value 0..45. The value of the modulation index (MI) is determined by the expression:

MI = 0.25 + Modulation Index * 0.05

1st Channel Centre Frequency is a 32-bit unsigned integer whose value is the centre frequency of the 1st channel in hertz.

Number of Channels is a 16-bit unsigned integer whose value is the number of channels for the Generic PHY

Channel Spacing is an unsigned 32-bit integer whose value is the difference between adjacent channel centre frequencies in hertz.

Symbol Rate is an unsigned 32-bit integer whose value is the number of symbols transmitted per second.

xx.yy.zz Mode Switch Parameter Entry PIB Attribute

The Mode Switch Parameter Entry is encoded as shown in Figure x7 :

| Octets:1 |  |  | 1 | 1 |
| --- | --- | --- | --- | --- |
| Bits 7..6 | Bits 5..1 | Bits 0 | Settling Delay | Secondary Preamble Length |
| Mode Switch Parameter Entry Index | Reserved | Secondary SFD |  |  |

Figure x7 Mode Swith Paramaters Format

Mode Switch Parameter Entry Index is a 2-bit unsigned integer whose value is the index into the ModeSwitchParameterEntries PIB Attribute array for the Mode Switch Parameter Entry.

Secondary SFD is a 1-bit integer set to 1 if a secondary SFD is present otherwise set to 0.

Settling Delay is an 8-bit unsigned integer whose value is the settling delay in microseconds.

Secondary Preamble Length is an 8-bit unsigned integer whose value is the number of secondary preamble cycles.