IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PIB Attribute Coordination |  |
| Date Submitted | 18 January 2011 |  |
| Source | Larry Taylor (DTC (UK)) | Voice: Fax: E-mail: larry.taylor@acm.org |
| Re: | PIB Attribute Coordination for 15.4g |  |
| Abstract | IEs and PIB Attribute encoding for PIB Attribute coordination |  |
| Purpose | Comment resolution |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Information Elements for coordinating SUN PHY Configuration Parameters

The following Nested Sub-Information Elements (IEs) are defined to enable coordination of PHY configuration parameters (defined as PIB Attributes) required for ordered operation of SUN networks.

Table x1 — SUN Configuration Parameter Coordination Nested IEs

| Group ID | Number | Content Length | Name | Description |
| --- | --- | --- | --- | --- |
| 0 | 1 | Variable | SUN PHY Capabilities | Declares the devices PHY Capabilities |
| 0 | 2 | Variable | Generic PHY Descriptor | Declares one Generic PHY Descriptor |
| 0 | 3 | Variable | Mode Switch Parameter Entry | Declares one Mode Switch Parameter Entry |

xx.yy SUN PHY Capabilities IE

| Octets:1 |  | 1 | 1 |  |  |  |  |  | 2 | 2 | 4 |  |  | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: 7-5 | Bits: 4-0 | Length (Varies with # PHY Types Supported) | Bits: 7..5 | 4 | 3 | 2 | 1 | 0 | Frequency Bands Supported | PHY Type Supported | Octets: 2 | Octets: 2 | … | PHY Type Supported |
| Group ID = 0 | No. = 1 |  | Rsvd | MS | FEC S1 | FEC S0 | SFD G1 | IL |  |  | PHY Type Supported | Frequency Bands Supported |  |  |

Figure x2 SUB PHY Capabilities IE Format

The SUN PHY Capabilities IE declares which SUN features a device supports.

The Length field is set to 3 + 2*Number of PHY Types Supported with All Frequency Bands = 1 + 4*Number of PHY Types Supported with All Frequency Bands = 0.

Miscellaneous features are encoded as a bit map in the first octet of the Information Field. A bit set to 1 indicates that the feature is supported otherwise the feature is not supported.

- Bit 4 indicates support for the Mode Switch feature (see 6.12a.3)

- Bit 3 indicates support for FEC Scheme 1 (6.12a.1.3)

- Bit 2 indicates support for FEC Scheme 0 (see 6.12a.1.3)

- Bit 1 indicates support for SFD Group 1 (see 6.3a.1.2)

- Bit 0 indicates support for Interleaving (see 6.12a.1.4)

| Bits 15..13 | Bits 12..0 |
| --- | --- |
| Rsvd | Frequency Bands Supported |

Figure x3 Frequency Bands Supported Format

The frequency bands defined in Table 4a are encoded into a 16-bit map as shown in Figure x3, with the Band ID in Table 4a corresponding to the bit-index into the Frequency Bands Supported bitmap. The bits corresponding to undefined band identifiers are reserved. A bit set to 1 indicated the device supports operation in that frequency band otherwise it does not.

PHY types supported are encoded as 16-bit structures as shown in Figure x4.

| Bits: 15..12 | 11 | 10..0 | 15..0 |
| --- | --- | --- | --- |
| PHY Type | All Frequency Bands | PHY Modes Supported | Frequency Bands Supported (Only present if All Frequency Bands = 0) |

Figure x4 PHY Type and PHY Modes Supported Format

For each PHY Types Supported field, bits 15-12 encode a Modulation Scheme supported as a 4-bit unsigned integer as defined by Table x2.

Table x2 — PHY Type Encoding

| PHY Type | Modulation Scheme |
| --- | --- |
| 0 | Filtered FSK-A |
| 1 | Filtered FSK-B |
| 2 | O-QPSK (DSSS) |
| 3 | O-QPSK (MDSS) |
| 4 | OFDM Option 1 |
| 5 | OFDM Option 2 |
| 6 | OFDM Option 3 |
| 7 | OFDM Option 4 |
| 8 | OFDM Option 5 |
| 15..9 | Reserved |

If Bit 11, All Frequency Bands, is set to 1, the PHY Modes declared in the PHY Modes Supported field are supported in each frequency band declared as supported in the Frequency Bands Supported field, otherwise the PHY Modes Supported field is followed by a 16-bit field identifying a subset of, and formatted in the same manner as, the Frequency Bands Supported field.

Bits 10-0 provide a bitmap indicating which PHY Modes or PHY options are supported for the modulation scheme encoded in the PHY Type field.

Filtered FSK PHY Modes are defined by Tables x3 and x4

Table x3 —Filtered FSK-A PHY Mode Encoding

| Bit Position | Narrowband FSK PHY Mode |
| --- | --- |
| 0 | 5 kbps; GFSK; mod index = 0.5; channel spacing = 12.5 kHz |
| 1 | 10 kbps; GFSK; mod index = 0.5; channel spacing = 25 kHz |
| 2 | 20 kbps; GFSK; mod index = 0.5; channel spacing = 50 kHz |
| 10-3 | Reserved |

FSK PHY Modes are defined by Table x4.

Table x4 —Filtered FSK-B PHY Mode Encoding

| Bit Position | FSK PHY Mode |
| --- | --- |
| 0 | 40 kbps; GFSK; mod index = 0.5; channel spacing = 100 kHz |
| 1 | 50 kbps; GFSK; mod index = 1.0; channel spacing = 200 kHz |
| 2 | 50 kbps; FSK; mod index = 1.0; channel spacing = 200 kHz |
| 3 | 100 kbps; GFSK; mod index = 1.0; channel spacing = 400 kHz |
| 4 | 150 kbps; FSK; mod index = 0.5; channel spacing = 400 kHz |
| 5 | 200 kbps; GFSK; mod index = 0.5; channel spacing = 400 kHz |
| 6 | 200 kbps; 4-GFSK; mod index = 1/3; channel spacing = 400 kHz |
| 7 | 200 kbps; GFSK; mod index = 1.0; channel spacing = 600 kHz |
| 8 | 400 kbps; 4-GFSK; mod index = 0.33; channel spacing = 600 kHz |
| 9 | 50 kbps; GFSK; mod index = 1.0; channel spacing = 200/400 kHz |
| 10 | Reserved |

O-QPSK (DSSS) PHY Modes are defined in Table x5.

Table x5 — O-QPSK (DSSS) PHY Mode Encoding

| Bit Position | OQPSK DSSS PHY Mode |
| --- | --- |
| 0 | chip rate = 100 kchip/s; (4,1) spreading; rate 1/2 FEC; data rate = 12.5 kbps |
| 1 | chip rate = 100 kchip/s; no spreading; rate 1/2 FEC; data rate = 50 kbps |
| 2 | chip rate = 1000 kchip/s; (16,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 3 | chip rate = 1000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 4 | chip rate = 1000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 5 | chip rate = 1000 kchip/s; no spreading; rate 1/2 FEC; data rate = 500 kbps |
| 6 | chip rate = 2000 kchip/s; (32,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 7 | chip rate = 2000 kchip/s; (32,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 8 | chip rate = 2000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 9 | chip rate = 2000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 500 kbps |
| 10 | Reserved |

O-QPSK (MDSS) PHY Modes are defined in Table x6.

Table x6 — O-QPSK (MDSS) PHY Mode Encoding

| Bit Position | OQPSK MDSS PHY Mode |
| --- | --- |
| 0 | chip rate = 1000 kchip/s; (16,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 1 | chip rate = 1000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 2 | chip rate = 1000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 3 | chip rate = 1000 kchip/s; no spreading; rate 1/2 FEC; data rate = 500 kbps |
| 4 | chip rate = 2000 kchip/s; (32,1) spreading; rate 1/2 FEC; data rate = 31.25 kbps |
| 5 | chip rate = 2000 kchip/s; (32,4) spreading; rate 1/2 FEC; data rate = 125 kbps |
| 6 | chip rate = 2000 kchip/s; (16,4) spreading; rate 1/2 FEC; data rate = 250 kbps |
| 7 | chip rate = 2000 kchip/s; (8,4) spreading; rate 1/2 FEC; data rate = 500 kbps |
| 10-8 | Reserved |

For each OFDM Option, bits 8..0 indicate which MCSs are supported as shown in Table x7.

Table x7 — OFDM PHY Mode Encoding

| Bit Position | OFDM PHY Modes (Option 1..5) |
| --- | --- |
| 0 | MCS 1 supported |
| 1 | MCS 2 supported |
| 2 | MCS 3 supported |
| 3 | MCS 4 supported |
| 4 | MCS 5 supported |
| 5 | MCS 6 supported |
| 6 | MCS 7 supported |
| 7 | MCS 8 supported |
| 8 | MCS 9 supported |
| 10-9 | Reserved |

In each encoding of PHY Modes in the PHY Type Supported structure, a bit set to 1 in bit position m for a given PHY Type in Table 3..Table 7 indicates the PHY Mode for that PHY Type corresponding to row m is supported. A value of 0 similarly indicates that the corresponding PHY Mode is not supported.

xx.yy.zz Generic PHY Descriptor Information Element

The Generic PHY Descriptor is encoded as shown in Figure x6 :

| Octets:1 |  | 1 | 1 |  |  | 1 |  | 4 | 2 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: 7-5 | Bits: 4-0 | Length | Bits: 7..4 | 3..2 | 1..0 | Bits: 7-6 | 5-0 | 1st Channel Centre Frequency | Number of Channels | Channel Spacing | Symbol Rate |
| Group ID = 0 | No. = 2 | = 16 | Generic PHY Descriptor ID | Modulation Scheme | Modulation Order | BT | Modulation Index |  |  |  |  |

Figure x6 Generic PHY Descriptor Format

Generic PHY Descriptor ID is an unsigned 4 bit integer as defined in section 6.1.2.7.2.

Modulation Scheme is an unsigned 2 bit integer whose values are defined in Table 4b (undefined values are Reserved).

Modulation Order is a 2-bit unsigned integer as defined in Table 8:

Table x8 — Modulation Order Encoding

| Modulation Order Field | Modulation Order |
| --- | --- |
| 0 | 2-FSK |
| 1 | 4-FSK |
| 2-3 | Reserved |

BT is encoded as a 2-bit unsigned integer as defined in Table 9:

Table x9 — BT Encoding

| BT Field | BT |
| --- | --- |
| 0 | 0.5 |
| 1 | 1.0 |
| 2-3 | Reserved |

Modulation Index is encoded as a 6-bit unsigned integer with permissible value 0..45. The value of the modulation index (MI) is determined by the expression:

MI = 0.25 + Modulation Index * 0.05

1st Channel Centre Frequency is a 32-bit unsigned integer whose value is the centre frequency of the 1st channel in hertz.

Number of Channels is a 16-bit unsigned integer whose value is the number of channels for the Generic PHY

Channel Spacing is an unsigned 32-bit integer whose value is the difference between adjacent channel centre frequencies in hertz.

Symbol Rate is an unsigned 32-bit integer whose value is the number of symbols transmitted per second.

xx.yy.zz Mode Switch Parameter Entry Information Element

The Mode Switch Parameter Entry is encoded as shown in Figure x7 :

s

| Octets: 1 |  | 1 | Octets:1 |  |  |  | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: 7..5 | Bits: 4..0 | Length = 3 | Bits 7..6 | Bit 5 | Bits 4..1 | Bits 0 | Settling Delay | Secondary Preamble Length |
| Group ID = 0 | No. = 3 |  | Mode Switch Parameter Entry Index | Source Mode | Target Mode | Secondary SFD |  |  |

Figure x7 Mode Swith Paramaters Format

Mode Switch Parameter Entry Index is a 2-bit unsigned integer whose value is the index into the ModeSwitchParameterEntries PIB Attribute array for the Mode Switch Parameter Entry.

Source Mode is set to 0 for 2-FSK or to 1 for 4-FSK.

Target Mode is a 4-bit unsigned integer indicating the PHY Mode of the following PPDU. The values are as defined in Table x2.

Secondary SFD is set to 1 if a secondary SFD is present otherwise set to 0.

Settling Delay is an 8-bit unsigned integer whose value is the settling delay in microseconds.

Secondary Preamble Length is an 8-bit unsigned integer whose value is the number of secondary preamble cycles.