- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [15th September, 2010] |  |
| Source | [Sang-Kyu Lim, Dae Ho Kim, Il Soon Jang, You Jin Kim, Tae-Gyu Kang] [ETRI] [Daejeon, Korea] | Voice: [ +82-42-860-1573] Fax: [+82-42-860-5218] E-mail: [sklim@etri.re.kr] |
| Re: | [Response to LB #57 comments] |  |
| Abstract | [This document describes the modified text of Annex G in D2 draft for the LB #57 comment resolution.] |  |
| Purpose | [To resolve the LB #57 comments] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Blue: new text

Red: deleted text

Black: not modified text

Related CIDs:  T-CIDs 402, 403, 404, 405, 406, 407, 408, 409, and 410

Annex G. Considerations for VLC LED signboard

(Informative)

Annex G describes some details that should be considered in the VLC system using a LED signboard.  In general an LED signboard consists of a host controller, a line scan controller, a display data buffer, and LED matrix, as shown in Figure G.1. The line scan controller selects a line to display some image or text as a signboard, and the display data buffer transmits some information, such as the on/off state or the color selection, to each LED pixel on the selected line. The line scan controller determines the active time of each line, where the active time says that the LED pixels on the selected line is operated by ON/OFF switching from the display data buffer only for the active time of the selected line. Therefore, the VLC using an LED signboard is tightly coupled with the active time.

Figure G.1 – General architecture of LED signboard

It is well known that most of PC monitors and TVs display some images through 60 displayed-frames per second. In case of PC monitors or TVs, the total displayed-frame number containing the different display information is 30 per second because each displayed-frame is transmitted in twice. The display mechanism of an LED signboard is similar to a PC monitor or TV. Assuming that an LED signboard consists of 16x16 lines and displays some images or text through 60 displayed-frames per second and 16 lines have the same active time per displayed-frame, the active time slot period assigned to each line for VLC is 1.042 ms per displayed-frame, as shown in Figure G.2. Therefore, an active time slot can transmit 130 byte at 1 Mbit/s, as shown in Figure G.3.

Figure G.2 – An example of LED signboard operation mechanism

Figure G.3 – An example of VLC data transmission on LED signboard

Figure G.4 – An example for the mitigation of the average brightness weakness on LED signboard

The total brightness during unit time period perceived by human eye of the VLC signboard can be reduced less than that of the non-VLC signboard. Therefore, it is important to minimize the perceivable brightness weakness on VLC applications using LED signboard because the LED signboard should keep the display function as its inherent role. Figure G.4 shows an example of operation mechanism to mitigate the total brightness weakness during unit time which can arise from when the 16x16 lines LED signboard operated as shown in Figures G.2 and G.3 is used for the VLC and the VLC data is carried during the active time of each line. The total brightness during unit time period in Figure G.4 is two times as large as that in Figure G.2 because the active time per displayed-frame per line in Figure G.4 is two times as long as that in Figure G.2.

The VLC LED signboard can be useful for the broadcast topology. The VLC broadcast topology in this standard consists of mainly the beacon and the downlink, as shown in Figure 106. Therefore, a broadcast topology can be constructed by assigning the active time slot #1 to the beacon and the active time slots from #2 to #8 to the downlink as shown in Figure G.5 and the use of GTS fields in the beacon frame.

The GTS fields of beacon frame can be used to indicate the GTS number, GTS length, and GTS direction for the broadcast topology. The multiple GTS slot can also be used, which depends on the service level, the subscriber’s grade, and the QoS policy.

Figure G.5 – An example of broadcast topology construction on VLC LED signboard