IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IE description for DSME |  |
| Date Submitted | [ 15 September 2010 ] |  |
| Source | [Wun-Cheol Jeong, Chang-Sub Shin, Se-Han Kim, In-Whan Lee, Cheol-Sig Pyo] [ETRI] [Gajeong dong Youseong gu, Daejeon, Korea] | Voice: [+82-42-860-5104] Fax: [ ] E-mail: [ wjeong@etri.re.kr ] |
| Re: |  |  |
| Abstract | Defines IEs for DSME |  |
| Purpose | Defines IEs for DSME in enhanced beacon and multi-purpose frame. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

- Consolidate Enhance Beacon Format (DSME related)

General EB format (from DCN#606r0):

| Octets: 1/2 | 0/1 |  |  | 1 | Variable | 0/2/4 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Ctrl | BSN | Addressing | Aux Sec | Beacon Type = 2 | Information Response (IE List) | FCS |
| MHR |  |  |  |  |  | MFR |

Information Response for DSME

| IE ID | Len | Name | Contents |
| --- | --- | --- | --- |
| xx | variable | DSME Beacon Information | See Table 1 |

IE Contents:

| octets:2 | variable | variable | variable | 9 | variable | variable |
| --- | --- | --- | --- | --- | --- | --- |
| Superframe Specification | Pending address fields | DSME Superframe Specification | ChannelHoppingSpecification | Time synchronization Specification | Beacon Bitmap | Beacon Payload |

Table 1

SuperframeSpecification:

See Figure 47 in IEEE802.15.4-2006.

Pending Address:

See Figure 46 in IEEE802.15.4-2006.

DSME Superframe Specification:

| bits:0-3 | 4 | 5 | 6 | 7-22 | 23-30 | 31 | 32-34 | 35-39 | variable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multi-superframe Order (MO) | CAP Reduction Flag | Embedded CAP/CFP Flag | ChannelDiversityMode | CAP Index | Number of Subslots | GACK Flag | ECFP Start Slot Length | Reserved | ECFP Start Slot |

ChannelHoppingSpecification:

| octets: 1 | 2 | 2 | variable |
| --- | --- | --- | --- |
| SequenceID | Channel Offset | ChannelOffset Bitmap Length | ChannelOffset Bitmap |

Time synchronization Specification:

| bits: 0 | 1-4 | 5-7 | 8-55 | 56-71 |
| --- | --- | --- | --- | --- |
| Deferred Beacon Flag | Deferred Beacon Time | Reserved | Beacon Timestamp | Beacon Timestamp offset |

Beacon Bitmap:

| octets: 2 | variable |
| --- | --- |
| SD Index | SD Bitmap |

- IE for Multi-purpose frame type format (DSME related)

Multi-purpose frame type (from DCN#603r3)

| Bit: 0-2 | 3 | 4-5 | 6-7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Type | long FC | Src mode | Dst mode | PAN ID | Security enabled | Aux sec header | Frame pending | Ack requested | IEs | Seq # | Version |
| 101 | 0 = no 1 = yes | 00 = none 01 = 1octet 10 = 2octet 11 = 8octet | 00 = none 01 = 1octet 10 = 2octet 11 = 8octet | 0 = none 1 = 2 in MHR | 0 = off 1 = on and no FCS | 0 = none, and frame counter must be reconstructed if security is on 1- in MHR | 0 = no 1 = yes | 0 = no 1 = yes | 0 = none 1 = present | 0 = absent 1 = present in MHR | 0 (version is 0 if 1 octet FC) |

IEs for DSME

| IE ID | Len (octet) | Name | Contents |
| --- | --- | --- | --- |
|  | variable | DSME Data2 | Table 2 |
|  | 0 | DSME ACK2 | Null |
|  | 3 | DSME Association Request 2 | Table 3 |
|  | variable | DSME Association Response 2 | Table 4 |
|  | variable | DSME-GTS Handshake 2 | Table 5 |
|  | 0 | DSME-GTS Information Request 2 | Null |
|  | variable | DSME-GTS Information Reply 2 | Table 6 |
|  | 2 | DSME-Beacon Allocation Notification 2 | Table 7 |
|  | 2 | DSME-Beacon Collision Notification 2 | Table 8 |
|  | variable | DSME-Link status report 2 | Table 9 |
|  | 4 | DSME-Asymmetic multi-channel beacon request 2 | Table 10 |
|  | 1 | DSME-Multi-channel hello 2 | Table 11 |
|  | 2 | DSME-Channel probe 2 | Table 12 |

Octets: variable

Data Payload

Table 2

| Octets: 1 | 2 |
| --- | --- |
| Capability Information | Channel Offset |

Table 3

| Octets: 2 | 1 | 2 | Variable |
| --- | --- | --- | --- |
| Short Address | Association Status | Channel Hopping Sequence Length | Channel Hopping Sequence |

Table 4

Octets: variable

DSME-GTS Characteristics

Table 5

| Octets: 3 | variable |
| --- | --- |
| Timestamp | DSME Characteristics |

Table 6

Octets: 2

Allocation Beacon SD Index

Table 7

Octets: 2

Collision SD Index

Table 8

Octets: variable

DSME-Link Status Specification

Table 9

Octets: 4

Scan Channels

Table 10

Octets: 1

Hello Specification

Table 11

Octets: 2

Channel Probe Specification

Table 12