IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Proposed resolution for CIDs 44 and 44 - Inner forward error correction encoder |  |
| Date Submitted | [09 November, 2010] |  |
| Source | [Anuj Batra] [Texas Instruments] [12500 TI Blvd, Dallas, TX 75243] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | [This document proposes a resolution for Letter Ballot 61 CIDs 44 and 47] |  |
| Abstract | [Proposed resolutions for CIDs 44 and 47] |  |
| Purpose | [For consideration by IEEE 802.15.7.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Editor should replace text in 5.6.4 with the following text:

5.6.4 Inner forward error correction encoder

The inner code is a based on a rate 1/3 mother convolutional code of constraint length seven (K=7) with generator polynomial g0 = 1338; g1 = 1718; g2 = 1658, as shown in Figure 1.

Figure 1 – Rate 1/3 mother convolutional code with constrain length 7

Six tail bits of zeros shall be added at the end of the encoding in order to terminate the convolutional encoder to an all zeros state. The tail bit of zeros shall be applied to both the header and the payload when the inner convolutional code is used.

5.6.4.1 Rate 1/4 code

The rate 1/4 code is obtained by puncturing the rate 1/3 mother code to a rate 1/2 code, as shown in Figure 2, and then using a simple repetition code as shown in Figure 3.

Figure 2 – puncturing pattern to obtain rate 1/2 code

Figure 2 – repetition pattern used to obtain the effective rate 1/4 code

5.6.4.2 Rate 1/3 code

The rate 1/3 code is obtained by using the outputs of the rate 1/3 mother code shown in Figure 1.

5.6.4.3 Rate 2/3 code

The rate 2/3 code is obtained by puncturing the rate 1/3 mother code, as shown in Figure 4.

Figure 4 – puncturing pattern to obtain rate 2/3 code