IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Proposed draft revision regarding CID797 for subclause reallocation |  |
| Date Submitted | [January 18th, 2011] |  |
| Source | [ Hiroshi Harada1, Fumihide Kojima1, Sum Chin Sean1, Kazuyuki Yasukawa2, Kazuo Kubo3, Tomonori Sato4, Kentaro Sakamoto5, Aiichiro Kashiwagi6, Takahiro Banno7, Hirohito Nishiyama8] [1NICT, 2Fuji Electric, 3Panasonic, 4Toshiba Toko Meter Systems, 5Tokyo Gas, 6Osaka Gas, 7Toho Gas, 8Mitsubishi Electric Corp.] [13-4, Hikari-no-oka,Yokosuka-shi,Kanagawa, 239-0847,Japan] | Voice: [1+81-46-847-5074] Fax: [1+81-46-847-5440] E-mail: [1{harada, f-kojima, sum}@nict.go.jp] |
| Re: | [] |  |
| Abstract | [This document contains proposed changes to the IEEE P802.15.4e Draft d2 to address comment resolution of CID797 in order to be discussed in LE subteam.] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Remove the following subclauses from the draft (from DSME).

7.5.10.17 DSME-GTS Superframe structure

7.5.10.18 DSME-GTS Contention access period (CAP)

7.5.10.19 DSME-GTS Incoming and outgoing superframe timing

Add the following texts at the end of 7.5.1.7.2

7.5.1.8 Low Energy Superframe

This subclause describes additional functionalities of devices supporting the two PIB attributes macLowEnergySuperframeSupported and macLowEnergySuperframeSyncInterval.

7.5.1.8.1 Low Energy Superframe structure

If macLowEnergySuperframeSupported is TRUE and macLowEnergySuperframeSyncInterval is not zero, the coordinator shall transmit beacon frames not in every beacon interval, but once in every beacon interval times  macLowEnergySuperframeSyncInterval, except when requested to do so. If macLowEnergySuperframeSupported is TRUE and macLowEnergySuperframeSyncInterval is zero, the coordinator shall transmit beacon frames only when it is requested to do so.

7.5.1.8.2 Low Energy Contention access period (CAP)

When macLowEnergySuperframeSupported is TRUE, transaction shall be ensured to be completed one IFS period before the end of the inactive period. If a device senses frame in CAP that does not end within CAP when macLowEnergySuperframeSupported is set to TRUE, the device may continue receiving the frame until it ends before the end of the inactive period. When macLowEnergySuperframeSupported is TRUE, the coordinator shall not locate GTSs in order to avoid the interference from the frames in CAP. When macLowEnergySuperframeSupported is TRUE, the coordinator shall notify the devices that already associated or intend to associate the condition of macLowEnergySuperframeSupported in the beacon frames.

7.5.1.8.3 Low Energy Incoming and outgoing superframe timing

If a device supports macLowEnergySuperframeSupported, the beacon order and superframe order may be equal for all superframes on a PAN. All devices supporting macLowEnergySuperframeSupported may interact with the PAN only during the active portion of a superframe.

Remove the following row in Table 86h.

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macLowEnergySuperframeSupported | 0x71 | Boolean | TRUE or FALSE | Indication of whether the low energy superframe is operational or not. If this attribute is TRUE, the coordinator shall not transmit beacon frames regardless of BO value. This attribute shall be set to FALSE if the device is aware of the existence of allocated GTS or DSME-GTS in its two-hop neighborhood. | Implementation specific |

Insert the following row at the bottom of Table 86i

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macLowEnergySuperframeSupported | 0x90 | Boolean | TRUE or FALSE | Indication of whether the low energy superframe is operational or not. If this attribute is TRUE, the coordinator shall not transmit beacon frames regardless of BO value. This attribute shall be set to FALSE if the device is aware of the existence of allocated GTS. | Implementation specific |
| macLowEnergySuperframeSyncInterval | 0x91 | Integer | 0x0000 -0xffff | Indication of beacon transmission interval when macLowEnergySuperframeSupported is TRUE. The beacon transmission interval shall be indicated by macLowEnergySuperframeSyncInterval times the beacon interval in the case this attribute is not zero. The beacon shall be transmitted only when requested in the case this attribute is zero. | Implementation specific |