IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Proposed Resolutions to Comments on Use of More Data Field |  |
| Date Submitted | January 11, 2011 |  |
| Source | Rick Powell Zarlink Semiconductor, Inc. 15822 Bernardo Center Dr, Ste B, San Diego, CA, 92127 858-675-3485 rick.powell@zarlink.com |  |
| Re: | LB# 66 comment resolution |  |
| Abstract | This submission describes proposed resolutions to MAC related LB#66 comments of IEEE P802.15.6/D02, related to unscheduled polling. It provided proposed test to defined an unscheduled polling IE and a definition of the use of unscheduled polling. |  |
| Purpose | To facilitate resolution of LB#66 comments |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

This document proposes changes to the function of the More Data bit and related fields in the MAC Header to reduce complexity, add consistency, and eliminate unnecessary modes.  A summary of the proposed changes is as follows:

- Polls should consistent behavior, whether they are a poll only, or a poll+ack.  The only difference between a poll and a poll+ack is that the poll+ack performs the acknowledgement function of the previous frame or block of frames.  If a poll is used to schedule a future post/poll, then the corresponding I/B-Ack+Poll should have the same functionality, with the addition of the acknowledgement function.

- Improvised Allocations should not span superframes.  This is unnecessary.  There is very little usefulness in having allocation intervals span superframe boundaries, and doing so, adds unnecessary complexity to the requirements.  This case would only be applicable in beaconless superframe mode, where the first slots of the superframe are not assigned to scheduled access.  This is an extremely limited case, and it can be satisfied by sending another "future post/poll" at the end of the superframe to schedule another poll at the beginning of the next superframe in the rare case where the first slots are not already scheduled and only in beaconless superframe mode.  This later capability already exists and must be supported, so adding the ability for an allocation interval to span superframe boundaries is unnecessary and adds complexity to all nodes that support type-I polling.

- Setting the More Data bit to one by the hub should have a consistent meaning.  In all cases, a Post or Poll Frame will be sent immediately or in the future.

- Indicating that there will be a post/poll frame sent to a node after the polled allocation interval currently being granted is unnecessary and adds complexity.  The indication of what will follow the Uplink or Polled Allocation Interval can be communicated with the subsequent I/B-Ack or I/B-Ack+Poll, and at least one frame requesting a I-Ack or B-Ack is required from the node in each uplink or bilink allocation interval, making this additional function redundant.

- There is no need for a poll the provides no polled allocation (Null Poll).  At Future Post/Poll location, the one slot is already consumed, and the frame transmitted by the hub is to be received by the node.  The Hub may assign a polled allocation of 1 slot, send a post, or schedule a future post/poll.  Providing a “Null Poll” provide no needed functionality.

Table 1 summarizes the current functionality define in the D02 draft.  Table 2 shows the proposed functionality.  Note that Table 2 provides a much simpler solution, while satisfying all necessary requirements.

Table 1: Current: Post – Poll Rules and Behavior Relative to More Data Bit, Post-Poll Window, and Next Fields

| Case | Frame Type | More Data | Access Mode | Poll-Post Timing | Sequence Number Post-Poll Window | Fragment Number Next | Action | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I/B-Ack | 0 | X | X | X | X | No Immediate or Future Post/Poll |  |
| 2 | I/B-Ack | 1 | X | X | 0 | X | Immediate Post/Poll after Allocation Interval |  |
| 3 | I/B-Ack | 1 | 0 | X | Starting Slot #S (>0) | Superframe Offset N | Future Post/Poll | This should be done with a Ack+Poll |
| 4 | I/B-Ack | 1 | 1 | X | X | X | Undefined | Behavior Not defined |
| 5 | Ack+Poll | 0 | 0 | X | Ending Slot #E | Ending SuperFrame #N | Immediate Type-I Polled Allocation, No Future Post/Poll | Allocation intervals should not span superframes. |
| 6 | Ack+Poll | 1 | 0 | X | Ending Slot #E | Ending SuperFrame #N | Immediate Type-I Polled Allocation, Immediate Post/Poll after Allocation Interval | Allocation intervals should not span superframes. Redundant to More Data = 0 |
| 7 | Ack+Poll | 0 | 1 | X | Number of Frames #M | X | Immediate Type-II Polled Allocation, No Future Post/Poll |  |
| 8 | Ack+Poll | 1 | 1 | X | Number of Frames #M | X | Immediate Type-II Polled Allocation, Immediate Post/Poll after Allocation Interval | Redundant to More Data = 0 |
| 9 | Poll/T-Poll | 0 | 0 | 0 | Ending Slot #E | Ending SuperFrame #N | Immediate Type-I Polled Allocation, No Future Post/Poll | Allocation intervals should not span superframes. |
| 10 | Poll/T-Poll | 0 | 0 | 1 | X | X | No Immediate and no Future Post/Poll | This is not needed |
| 11 | Poll/T-Poll | 1 | 0 | 0 | Ending Slot #E | Ending SuperFrame #N | Immediate Type-I Polled Allocation, Immediate Post/Poll after Allocation Interval | Allocation intervals should not span superframes. Redundant to More Data = 0. |
| 12 | Poll/T-Poll | 1 | 0 | 1 | Starting Slot #S (>0) | Superframe Offset N | Future Post/Poll |  |
| 13 | Poll/T-Poll | 0 | 1 | 0 | Number of Frames #M | X | Immediate Type-II Polled Allocation, No Future Post/Poll |  |
| 14 | Poll/T-Poll | 0 | 1 | 1 | X | X | Undefined | Behavior Not defined |
| 15 | Poll/T-Poll | 1 | 1 | 0 | Number of Frames #M | X | Immediate Type-II Polled Allocation, Immediate Post/Poll after Allocation Interval | This case does not make sense. Redundant to More Data = 0 |
| 16 | Poll/T-Poll | 1 | 1 | 1 | X | X | Undefined | Behavior Not defined |

Ack = I-Ack or B-Ack

X = Not Defined, Don’t Care, or Reserved

Access Mode: 0 = SuperFrame; 1 = non-SuperFrame

Poll-Post Timing: 0 = Immediate Poll Allocation; 1 = No Immediate Poll Allocation

Table 2: Proposed: Post – Poll Rules and Behavior Relative to More Data Bit, Access Mode, Post-Poll Window, and Next Fields

| Case | Frame Type | More Data | Access Mode | Poll-Post Timing | Sequence Number Post-Poll Window | Fragment Number Next | Action | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I/B-Ack | 0 | X | X | X | X | Frame Acknowledgement No Immediate Post or Poll Frame to follow |  |
| 2 | I/B-Ack | 1 | X | X | X | X | Frame Acknowledgement Immediate Post or Poll Frame after Current Allocation Interval** |  |
| 3 | I/B-Ack +Poll | 0 | 0 | X | Ending Slot #E | X | Frame Acknowledgement Immediate Type-I Polled Allocation |  |
| 4 | I/B-Ack +Poll | 1 | 0 | X | Starting Slot #S (>0) | SuperFrame Offset N | Frame Acknowledgement Future Post/Poll |  |
| 5 | I/B-Ack +Poll | X | 1 | X | Number of Frames #M | X | Frame Acknowledgement Immediate Type-II Polled Allocation |  |
| 6 | Poll/T-Poll | 0 | 0 | X | Ending Slot #E | X | Immediate Type-I Polled Allocation | Same as #3 without Ack |
| 7 | Poll/T-Poll | 1 | 0 | X | Starting Slot #S (>0) | SuperFrame Offset N | Future Post/Poll | Same as #4 without Ack |
| 8 | Poll/T-Poll | X | 1 | X | Number of Frames #M | X | Immediate Type-II Polled Allocation | Same as #5 without Ack |

** Immediately after current frame for non-superframe mode, and immediately after current allocation interval if time is available.

I/B-Ack = I-Ack or B-Ack

X = Not Defined, Don’t Care, or Reserved

Access Mode: 0 = SuperFrame; 1 = non-SuperFrame

Poll-Post Timing: Not needed

Note:

When More Data bit = 1, a Post or Poll Frame will be sent immediately or in the future.

I/B-Acks may only schedule immediate post/poll frames

I/B-Ack+Polls behave like I/B-Acks, except that they also provide Frame Acknowledgement.

When Access Mode = 1, Polls always grant an immediate Polled Allocation

Type-I/II Allocation capability in the MAC Capability field indicates the ability to receive polled allocations.  It is not an indication of support for Immediate or Future Post.