IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | LB63 MSK Comment Resolution |  |
| Date Submitted | [January 13, 2011] |  |
| Source | [Dalibor Pokrajac, GuardRFID] | Voice: [604-282-7584] Fax: [604-998-4018] E-mail: [dalibor.pokrajac@guardrfid.com] |
| Re: | [] |  |
| Abstract | [This document contains proposed changes to the IEEE P802.15.4f Draft d1 to address comment resolution of various CIDs relevant to 433 MHz PHY] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Overview

The following text addresses some of the comments from Letter Ballot LB63 (first letter ballot for 802.15.4f Draft Standard). All comments addressed in this document are related to 3 main categories of comments on MSK PHYs (433 MHz and 2450 MHz):

- Forward Error Correction (including Interleaving and Whitening)

- Definition of Transmitter / Receiver

- Definition of Preamble and associated SFD

Forward Error Correction

CID: 9, 11, 63, 64, 105, 106, 121, 263, 264, 267, 270, 272, 273, 301, 303

Draft D1 of 802.15.4f Draft standard specifies Forward Error Correction to be used optionally for 433 MHz PHY and 2450 MHz PHY. For this purpose, ½ rate convolutional coding is specified, the same coding as it is currently proposed to be used for FSK PHY in 802.15.4g.

After careful review of proposed coding and comments, the proposal is to remove Forward Error Correction  from 802.15.4f Draft standard altogether. The reasoning for this proposal is as follows:

- 802.15.4f PHYs (including MSK PHYs) are intended to be used for active RFID and RTLS applications which are extremely sensitive to power consumption and battery size. These devices usually operate in environment with many nodes (hundreds, thousands and tens of thousands) occupying the same channel and same physical space (e.g. a warehouse).

- Frame size is critical to be as short as possible to minimize power consumption and channel occupancy time.

- RFID /RTLS Tags are usually very small devices with low efficiency antenna with uneven transmit pattern. A change in Tag orientation or change in a surface to which Tag is attached can easily produce 10-30dB change in link budget.

- Under these circumstances, gain of 3-6 dB that potentially could be achieved with FEC, Interleaving and Whitening is not good trade-off, given that is increases frame length substantially, therefore degrading power consumption and channel occupancy time.

Resolution:

Based on above the following is proposed:

- Remove section 6.8b.2.4. Forward Error Correction

- Remove section 6.8b.2.5. Code-symbol Interleaving

- On Page 14, remove lines 17 – 23 which specify usage of FEC

- In Table 20b remove reference to coded SFD.

Definition of Transmitter and Receiver for 433 MHz PHY and 2450 MHz PHY

CID: 11, 12, 65, 107, 150, 151, 222, 223, 271,

Resolution:

- Add the following text  after “6.8b.2.3 MSK Modulation”:

6.8b.2.4. Operating frequency range

The 433 MHz PHY operates in the 433.05–434.79 MHz frequency band.

6.8b.2.5 Transmit PSD mask

The PSD mask for the 433 MHz PHY is specified as:

The transmitted spectral products shall be less than the limits specified in Table 39ad. For both relative and absolute limits, average spectral power shall be measured using a 100 kHz resolution bandwidth. For the relative limit, the reference level shall be the highest average spectral power measured within +/- 500 kHz or +/- 100 kHz of the carrier frequency (respective to datarate).

Table 39ad  433 MHz and 2450 MHz band MSK transmit PSD limit

| PHY | Data Rate | Frequency | Relative Limit | Absolute Limit |
| --- | --- | --- | --- | --- |
| 433 MHz | 250 ksymbols/s | ¦f – fc¦ > 1.0 MHz | -20 dB | -20 dB |
|  | 31.25 ksymbols/s | ¦f – fc¦ > 200 kHz | -20 dB | -20 dB |
| 2450 MHz | 250 ksymbols/s | ¦f – fc¦ > 1.0 MHz | -20 dB | -20 dB |

6.8b.2.6 Symbol rate

The 433 MHz PHY symbol rate shall be either 250 ksymbol/s or 31.25 ksymbol/s with an accuracy of ±40 ppm.

2450 MHz PHY symbol rate shall be 250 ksymbol/s with an accuracy of ±40 ppm.

6.8b.2.7 Receiver sensitivity

Under the conditions specified in 8.1.7 of IEEE 802.15.4i RevD3, a compliant device shall be capable of achieving a sensitivity of -85 dBm or better for 250 ksymbol/s (for both MSK PHYs) and -90 dBm or better for 31.25 ksymbol/s (specific to 433 MHz PHY).

6.8b.2.8 Receiver interference rejection

The minimum receiver interference rejection levels are given in Table 39ae. The adjacent channel is one on either side of the desired channel that is closest in frequency to the desired channel, and the alternate channel is one more removed from the adjacent channel. Only channels that operate at the same symbol rate and same PHY are taken into consideration (either 250 ksymbol/s or 31.25 ksymbol/s).

For example, when channel 7 is the desired channel (31.25 ksymbol/s), channel 6 and channel 9 are the adjacent channels (not channel 8, which is 250 ksymbol/s channel), and channel 5 and channel 10 are the alternate channels.

Table 39ae  Minimum Receiver Interference Rejection requirements for 433 MHz PHY and 2450 MHz PHY

| PHY | Data Rate | Adjacent Channel Rejection | Alternate Channel Rejection |
| --- | --- | --- | --- |
| 433 MHz | 250 ksymbols/s | 0 dB | 24 dB |
|  | 31.25 ksymbols/s | 0 dB | 16 dB |
| 2450 MHz | 250 ksymbols/s | 0 dB | 24 dB |

The adjacent channel rejection shall be measured as follows: the desired signal shall be a compliant 433 MHz PHY signal or 2450 MHz PHY, as defined by 6.8b.2.5, of pseudo-random data. The desired signal is input to the receiver at a level 3 dB above the maximum allowed receiver sensitivity given in 6.8b.2.7.

In either the adjacent or the alternate channel, a compliant signal, as defined by 6.8b.2.5, is input at the level specified in Table 39ae relative to the desired signal. The test shall be performed for only one interfering signal at a time. Under these conditions the receiver shall meet the error rate criteria defined in 8.1.7 of IEEE 802.15.4i RevD3.

6.8b.2.9. TX-to-RX turnaround time

The 433 MHz PHY and 2450 MHz PHY shall have a TX-to-RX turnaround time as defined in 8.2.1. of IEEE 802.15.4i RevD3 (12 symbol periods).

6.8b.2.10. Transmit center frequency tolerance

The 433 MHz PHY and 2450 MHz shall have a transmit center frequency tolerance as defined in 8.2.4. of IEEE 802.15.4i RevD3 (-40 ppm).

6.8b.2.11. Transmit power

The 433 MHz PHY and 2450 MHz PHY shall have a minimum transmit power as described in 8.2.5. of IEEE 802.15.4i RevD3 (minimum of -3 dBm and maximum as per local regulations).

6.8b.2.12 Receiver maximum input level of desired signal

The 433 MHz PHY and 2450 MHz PHY shall have a receiver maximum input level as described in 8.2.6. of IEEE 802.15.4i RevD3 (greater than or equal to -20 dBm).

6.8b.2.13 Receiver ED

The 433 MHz PHY and 2450 MHz PHY shall provide the receiver ED measurement as described in 8.2.7. of IEEE 802.15.4i RevD3.

6.8b.2.14 Link quality indicator (LQI)

The 433 MHz PHY and 2450 MHz PHY shall provide the LQI measurement as described in 8.2.8. of IEEE 802.15.4i RevD3.

6.8b.2.15 Clear channel assessment (CCA)

The 433 MHz PHY and 2450 MHz PHY shall use the one of the CCA methods as described in 8.2.9. of IEEE 802.15.4i RevD3.

6.8b.2.16 Modulation Frequency Deviation Tolerance

Modulation frequency tolerance is measured as a percentage of maximum frequency deviation ∆f.

Modulation frequency deviation shall be constrained within +/- 30% of maximum frequency deviation ∆f as defined in 6.8b.2.2.

6.8b.2.17 Zero Crossing Tolerance

All zero crossing shall be constrained within +/- 12.5% of symbol time (+/- 1/8th of the symbol).

Preamble and SFD

CID: 117

Preamble proposed in 802.15.4f D1 is “101010…..1010” (32-bit Preamble). This Preamble is in contrast with Preamble “010101….0101” as it is already defined in 802.15.4-2006 and proposed in 802.15.4g for constant envelope modulation. It would be desirable to have the same Preamble for all PHYs that use similar or the same modulation.

Change in Preamble must be accompanied with change of SFD in order to maximize energy peak obtained through correlation. The following SFD is proposed:

0100 1001 1000 1111

Correlation properties of proposed SFD are shown in diagram below:

Note: 18831 is decimal value. The same SFD expressed in BCD is 0x498F

Resolution:

- Change line 19 on Page 13 (Paragraph 6.3 PPDU Format) as follows:

18	The Preamble field for the 433 MHz PHY and 2450 MHz PHY is shown below:

19 		1   0   1   0   1   0   1   0 	[leftmost bit b0 transmitted first in time]

20 		b0.............................b7

- Change Table 20b to include new SFD as follows:

|  | SFD (b0 – b15) |
| --- | --- |
| SFD Value | 0100 1001 1000 1111 |