IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Description of Mode Switch Mechanism in Clause 5 |  |
| Date Submitted | January 2011 |  |
| Source | [Kuor-Hsin Chang1, Bob Mason1, Ed Calaway2, Phil Beecher3, Clint Powell4] Company [Elster Solutions1, Sensus2, PG&E3, SCE4] | Voice: [ ] Fax: [ ] E-mail: [kuor-hsin.chang@us.elster.com, robert.t.mason@us.elster.com, ed@sunrisemicro.com, pbeecher@ieee.org, cpowell@ieee.org] |
| Re: | Mode Switch Comment Resolution for LB59 |  |
| Abstract | This document proposes high-level description of mode switch mechanism in Clause 5. |  |
| Purpose | Propose resolution text for comments submitted against the TG4g letter ballot. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Insert the following new subclause (5.2b) after 5.2a:

5.2b Mode switch mechanism

The mode switch mechanism enables a device using the MR-FSK PHY to change its symbol rate and/or modulation scheme on a packet-by-packet basis. This is done as a PHY layer operation, requiring minimal involvement from the MAC layer. An additional packet header is used to inform the receiver of the mode switch and to specify the new PHY mode of the following PPDU.

Using the mode switch mechanism, the default operation mode (i.e., symbol rate or modulation scheme) can be overridden for a single packet. For example, the mode switch mechanism can be invoked by a device that is configured to operate at the mandatory MR-FSK mode (e.g., 50 kbps) to enable high data rate communications when needed. This might be used to facilitate a common communication PHY mode, while enabling higher data rate communications for those devices that are already established in the network. Also, this mechanism permits two devices using the FSK PHY to establish the communication in a different PHY mode.

To take advantage of this mechanism, the system implementer would typically configure devices with identical mode switch parameters. However, a source device wishing to use the mode switch mechanism would typically request SUN PHY capability information (including the mode switch parameter entries) from the destination device(s) to ensure that the mode switch mechanism, with the identical parameters, is supported by the destination device(s).