IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PIB Attribute Enumeration for Metrics |  |
| Date Submitted | 19 January 2011 |  |
| Source | Tim Godfrey (EPRI) | Voice: Fax: E-mail: tim.godfrey@ieee.org |
| Re: | PIB Attribute Enumeration for Metrics in 15.4e |  |
| Abstract |  |  |
| Purpose | Comment resolution |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Information Elements for communication of 802.15.4e MAC Metrics

The following Nested Information Elements (IEs) are defined.

Table x1 —Nested IEs

| Group ID | Number | Content Length | Name | Description |
| --- | --- | --- | --- | --- |
| 1 | <tbd> | Variable | MAC Metrics | Declares the value of a MAC Metrics counter |

xx.yy.zz MAC Metrics Information Element

The MAC Metrics Parameter Entry is encoded as shown in Figure x8:

| Octets: 1 |  | 1 | Octets:1 | Octets: 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: 7..5 | Bits: 4..0 | Length = 5 | PIB Identifier (index) | MSB |  |  | LSB |
| Group ID = 0 | No = 4 |  |  | Value |  |  |  |

Figure x8 MAC Metrics Parameters Format

PIB Identifier is the identifier of the specific Metrics PIB as described in Table 86j. The range of index values for the metrics is 0x90 through 0x99.

The four octets of the value field are encoded as Big Endian (Most Significant Byte first).

The value field can represent the value of the corresponding PIB counter for commands that read the PIB, or the value can represent the value to be written to the PIB for commands that set the PIB.