IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | TG6 Coexistence Assurance Document (NB PHY) |
| Date Submitted | March 2011 |
| Source | Mark Dawkins (Toumaz), Anuj Batra (TI), Kenichi Takizawa (NICT), Srinath Hosur (TI) |
| Re: |  |
| Abstract | Analysis on coexistence of 802.15.6 NB PHY with other 802 systems within the same frequency bands |
| Purpose | To address the coexistence capability of 802.15.6 NB PHY |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Scope	1

2	References	2

3	Acronyms and abbreviations	3

4	Overview of NB PHY	4

4.1	Operating Frequency Bands	4

4.2	Modulation Parameters	4

4.3	Coexistence Mechanisms	4

5	Other IEEE 802 Standards Occupying Same Frequency Band as IEEE 802.15.6	5

5.1	2400-2483.5 MHz Band (Worldwide)	5

5.2	863-870MHz (Europe), 902-928 MHz (United States) and 950-958MHz (Japan) ISM Bands	5

6	Coexistence Scenarios and Analysis	6

6.1	Methodology	6

6.1.1	Path Loss Model	6

6.1.2	PER for IEEE 802.15.6	7

6.2	Coexistence with IEEE 802.11	1

6.2.1	Standards characterized for coexistence	1

6.2.2	Assumptions in coexistence performance	1

6.2.3	Interference Modeling	2

6.2.4	Coexistence Simulation Results	3

6.3	Coexistence with IEEE 802.15.1	1

6.3.1	Standards characterized for coexistence	1

6.3.2	Assumptions in coexistence performance	1

6.3.3	Interference modeling	3

6.3.4	Coexistence simulation results	3

6.4	Coexistence with IEEE 802.15.4	4

6.4.1	Standards characterized for coexistence	4

6.4.2	Assumptions in coexistence performance	6

6.4.3	Coexistence performance	10

7	Discussions and Conclusion	16

# Scope

The IEEE 802.15 Task Group 6 defines a Medium Access Control (MAC) layer and several supporting PHY layers to enable Body Area Networks (BAN) used in, on, or around a body.

IEEE 802.15.6 specifies a total of three PHYs, namely Narrow Band (NB), Ultra Wide Band (UWB) and Human Body Communication (HBC). All three PHYs are specified to address different system demands and target applications. Each PHY is specified to operate in one of a unique set of frequency bands. Thus, mutual interference between BAN PHYs is avoided.

This document addresses the coexistence of IEEE 802.15.6 NB PHY systems with other IEEE 802 standards operating in the same frequency bands – specifically in terms of interference caused by IEEE 802.15.6 to these existing systems, and vice versa.

# References

[1] S. Shellhammer, “Writing a coexistence assurance document,” IEEE 802.19-09/0001r0, 2009.

[2] S. Shellhammer, “Estimating packet error rate caused by interference –A coexistence assurance methodology-,” IEEE 802.19-05/0029r0, 2005.

[3] IEEE Std 802.15.4TM-2006.

[4] IEEE Std 802.15.4dTM-2009.

[5] IEEE Std 802.15.2TM-2003.

[6] J .G. Proakis, Digital Communications 4th edition, 2000.

[7] C.-S. Sum, “TG4g coexistence assurance document,” IEEE 802.15-10-0668-03-004g, 2010

[8] IEEE Std 802.11TM-2007.

[9] IEEE Std 802.15.1TM-2005.

# Acronyms and abbreviations

AWN 			Affected Wireless Network

BER			Bit Error Rate

BPSK			Binary Phase Shift Keying

CCA			Clear Channel Assessment

D-MPSK		Differential M-ary Phase Shift Keying

DSSS-CCK		Direct Sequence Spread Spectrum – Complimentary Code Keying

FHSS 			Frequency Hopping Spread Spectrum

GMSK			Gaussian Minimum Shift Keying

IWN 			Interfering Wireless Network

LBT			Listen Before Talk

MBANS			Medical Body Area Network Service

MICS			Medical Implant Communication Service

MIMO			Multiple Input, Multiple Output

OFDM			Orthogonal Frequency Division Multiplexing

O-QPSK		Offset Quadrature Phase Shift Keying

PER			Packet Error Rate

PSSS-ASK		Parallel Sequence Spread Spectrum – Amplitude Shift Keying

QAM			Quadrature Amplitude Modulation

SIR			Signal to Interference Ratio

SRRC			Square Root Raised Cosine

WMTS			Wireless Medical Telemetry Service

# Overview of NB PHY

## Operating Frequency Bands

The allocated frequency bands for the 802.15.6 NB PHY are:

- 402-405MHz (MICS)

- 420-450MHz (Japan WMTS)

- 863-870MHz (EU ISM)

- 902-928MHz (US ISM)

- 950-958MHz (Japan ISM)

- 2360-2400MHz (MBANS)

- 2400-2483.5MHz (Worldwide ISM)

## Modulation Parameters

The 802.15.6 NB PHY has modulation parameters in the different frequency bands as shown in Table 1.

| Frequency Band | Symbol Rate (ksps) | Channel bandwidth (kHz) | Modulation |
| --- | --- | --- | --- |
| 402-405MHz | 187.5 | 300 | D-MPSK |
| 420-450MHz | 187.5 | 500 | GMSK |
| 863-870MHz 902-958MHz 950-958MHz | 250 | 400 | D-MPSK |
| 2360-2400MHz 2400-2483.5MHz | 600 | 1000 | D-MPSK |

Table 1

## Coexistence Mechanisms

The coexistence mechanisms for the NB PHY specified in the IEEE 802.15.6 standard are:

- Clear Channel Assessment (CCA)

  - Energy detection and/or carrier sense

  - LBT and other procedures as defined by local regulations

- Channel switching

  - >= 10 channels in each frequency band

- Low Duty cycle

- Limited transmission power

  - Narrow bandwidth via SRRC or Gaussian pulse shaping

- Optional slow channel hopping at MAC layer

# Other IEEE 802 Standards Occupying Same Frequency Band as IEEE 802.15.6

This clause lists the IEEE 802 standards that occupy the same frequency bands as the IEEE 802.15.6 NB PHY.  The common frequency bands are 2400-2483.5 MHz, 902-928 MHz, 863-870 MHz, and 950-958 MHz. There are no other IEEE 802 standards that operate in the other frequency bands supported by the IEEE 802.15.6 NB PHY.

## 2400-2483.5 MHz Band (Worldwide)

Table 2 shows the list of other IEEE 802 standards that share the 2400-2483.5 MHz band with the IEEE 802.15.6 NB PHY.

| Standard | PHY specification |
| --- | --- |
| 802.11b | DSSS – CCK |
| 802.11g | OFDM – QAM |
| 802.11n | MIMO OFDM – QAM |
| 802.15.1 | FHSS – GFSK |
| 802.15.4 | DSSS – O-QPSK |

Table 2

## 863-870MHz (Europe), 902-928 MHz (United States) and 950-958MHz (Japan) ISM Bands

Table 3 shows the list of other IEEE 802 standards that share the 863-870 MHz, 902-928 MHz or 950-958 MHz bands with the IEEE 802.15.6 NB PHY.

| Standard | PHY specification |
| --- | --- |
| 802.15.4 | DSSS – BPSK DSSS – O-QPSK PSSS – ASK |

Table 3

# Coexistence Scenarios and Analysis

This clause presents the analysis of the interference caused by IEEE 802.15.6 NB PHY systems to each of the IEEE 802 standards identified in section 5, and vice versa Note that the interferer parameters chosen in the following sub-clauses are chosen for the purpose of coexistence analysis in this CA document and are a sub-set of the available modes.

## Methodology

In order to quantify the coexistence performance, the techniques described in Shellhammer [1] and [2] have been adopted. The coexistence assurance methodology predicts the PER of an affected wireless network (AWN), in the presence of an interfering wireless network (IWN). In its simplest form, the methodology assumes an AWN and an IWN, each composed of a single transmitter and a receiver. The methodology takes as input a path loss model, a BER function for the AWN, and in some case predicted temporal models for packets generated by the AWN and for “pulses,” i.e., packets generated by the IWN. Based on these inputs, the methodology predicts the PER of the AWN as a function of the physical separation between the IWN transmitter and the AWN receiver.

The appeal of the coexistence assurance methodology is that multiple networking standards can be characterized and compared with just a few parameters, notably

- Bandwidth of AWN and IWN devices

- Path loss model for the networks

- PER as a function of SIR of AWN devices

### Path Loss Model

The coexistence methodology uses a variant of the path loss model described in IEEE 802.15.2-2003 [5], which stipulates a two-segment function with a path loss exponent of 2.0 for the first 8 meters and then a path loss exponent of 3.3 thereafter. The path loss model is based on the 2400 MHz. To adapt the model to the sub-GHz bands, the formula is modified as shown in Equation (1).

pld=pl(1)+10γ1log10d d≤8mpl(8)+10γ8log10d8 d>8m                 (1)

where pl(d) is the path loss at d meters, 1 and 8 is the path loss exponent at 1 and 8 meters, respectively. The pl(1) is computed as shown in Equation (2).

pl1=10γ1log104πfc                                 (2)

where f is the frequency and c is the speed of light. Plots of the path loss for the frequency bands are shown in Figure 1. The path loss model shown in Equation (1) is applied to compute the path loss between the AWN and IWN.

Figure 1 - Path loss curves for all frequency bands

### PER for IEEE 802.15.6

To calculate the PER for the IEEE 802.15.6 NB PHY, first of all the error probability of one BCH codeword is derived. According to Proakis [6], the codeword error probability of a perfect code for hard-decision decoding is given by a simple closed form, i.e.

Pw=1−m=0tNmPbm1−PbN−m                    (3)

where N is the number of symbols in a codeword, Pb is the bit error probability, and t is the number of error symbols possible to be corrected. Although the (63, 51)-BCH code itself is not a perfect code, as shown in Figure 2, the error probability shows good agreement with BER obtained by simulations. This closed form given in Equation (3) is used to compute the PER. The BER for the 15.6 PHY, /2-shift DBPSK, in the 2400-2483.5 MHz band is given by

Pb=0.5exp−2.42SIR                           (4)

For the 860-868 MHz, 902-928 MHz, and 950 -958 MHz bands, BER are computed by

Pb=0.5exp−1.21SIR                           (5)

These equations are based on the bit error probability of BPSK in differentially coherent detection [6]. The difference of the coefficients multiplied to SIR comes from the difference of the spreading factors. Then, PER for 15.6 NB-PHY is computed as shown in the Equation (6).

PER=1−1−PwNc                              (6)

where Nc is the number of BCH codewords in one 15.6 payload. For example, if the number of payload bits is 255*8 bits, the number of codewords Nc becomes ceil(255*8/51) = 40. The PER curve for the 15.6 NB-PHY in the 2400-2483.5 MHz band is displayed in Figure 2.

Figure 2 - PER as a function of SINR for 15.6 PHY (2400-2483.5 MHz)

## Coexistence with IEEE 802.11

This section considers issues regarding coexistence between IEEE 802.15.6 devices and IEEE 802.11 devices.

### Standards characterized for coexistence

The draft standard for IEEE 802.15.6 is specified for operation in the 402-405 MHz, 420-450 MHz, 863-868 MHz, 902-928 MHz, 950-958 MHz, 2360-2400 MHz, and 2400-2483.5 MHz bands.

IEEE 802.11 standards characterized in this document for coexistence with IEEE 802.15.6 are as follows:

- IEEE Std 802.11TM-2007 [8] (2400-2483.5 MHz, 5 GHz)

In the 2400-2483.5 MHz band, there are DSSS-CCK, OFDM-QAM and MIMO-OFDM-QAM PHYs. Their detailed specifications are summarized in Table 4. The details of the 15.6 PHY mode used here are given in Table 5. These PHYs will serve as both AWNs and IWNs.

| Modulation | Bit rate | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- |
| 802.11b: DSSS-CCK | 11 Mbps | 22 | 14 | -76 |
| 802.11g: OFDM-BPSK, R = 1/2 | 6 Mbps | 20 | 14 | -88 |
| 802.11n: OFDM-QPSK, R = 3/4 | 18 Mbps | 20 | 14 | -83 |

Table 4 - Specifications of 802.11 standard in 2400-2483.5MHz band

| Modulation | Bit rate | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- |
| D-BPSK, R = 51/63 | 121.4 kbps | 1 | -10 | -92 |

Table 5 - Specifications of 802.15.6 NB-PHY ((63,51)-BCH coded /2-shift DBPSK)

### Assumptions in coexistence performance

The assumptions made across all standards characterized for coexistence are described in the following sub-clauses.

#### Path loss model

The path loss model used in this section is given in section 6.1.1.

#### PER model

The PER for the 802.11 PHY modes can be calculated from the corresponding BER through the following relationship:

PER = 1 – (1-BER)L

where L is the average frame size. For 802.11b, L = 1024 bytes. For 802.11g, L = 1000 bytes. For 802.11n, L = 4096 bytes. The PER curves for 802.11 PHY modes are given in Figure 3.

Figure 3 - PER curves as a function of SINR for 802.11 PHYs

The PER curve for the 802.15.6 PHY mode is given in section 6.1.2.

### Interference Modeling

The effect of the interfering signal on the desired signal is assumed to be averaged to the bandwidth of the victim system. The victim receiver (with receive power Prv ) receives the desired signal from the victim transmitter (with transmit power Ptv), located at a distance dD, while an interferer transmitter (with  transmit power PTI) is located at a distance dU. The ratio between the desired and undesired power present at the victim receiver will be used as the SIR of the victim system.

At the victim receiver the received power from the desired signal is

And the received power from the interfering transmitter is

And the SIR is given as PRv-P’Rv.

### Coexistence Simulation Results

#### 802.15.6 121 kbps Mode as Victim Receiver

Figure 4 shows the relationship between the PER performance of the 802.15.6 victim receiver and the distance between the victim receiver to the interferer transmitter. The separation distance between the 802.15.6 victim transmitter and the 802.15.6 victim receiver is 3 meters. The list of interferers is given in the figure.

Figure 4 - PER vs. Distance between 802.11 Interferer to 802.15.6 Victim receiver

#### 802.11 PHY Modes as Victim Receiver

This sub-clause presents results obtained when setting the 802.11 system as the victim and the 802.15.6 system as the interferer. Figure 5, Figure 6 and Figure 7 show the relationship between the PER performances of the 802.11b/g/n victim receivers corresponding to the distance between the victim receivers and the 802.15.6 interferers. The separation distance between 802.11 victim transmitter and 802.11 victim receiver is 100 meters.

Figure 5 - PER vs. Distance between 802.15.6 Interferer to 802.11b Victim receiver

Figure 6 - PER vs. Distance between 802.15.6 Interferer to 802.11g Victim receiver

Figure 7 - PER vs. Distance between 802.15.6 Interferer to 802.11n Victim receiver

## Coexistence with IEEE 802.15.1

This section considers issues regarding coexistence between IEEE 802.15.6 devices and IEEE 802.15.1 devices.

### Standards characterized for coexistence

The draft standard for IEEE 802.15.6 is specified for operation in the 402-405 MHz, 420-450 MHz, 863-868 MHz, 902-928 MHz, 950-958 MHz, 2360-2400 MHz, and 2400-2483.5 MHz bands.

IEEE 802.15.1 standards characterized in this document for coexistence with IEEE 802.15.6 are as follows:

- IEEE Std 802.15.1TM-2005 [9]

This standard operates in the 2400-2483.5 MHz band using FHSS GFSK. The detailed specifications for this PHY are summarized in Table 6. The details of the 15.6 PHY mode used here are given in Table 7. These PHYs will serve as both AWNs and IWN.

| Modulation | Bit rate | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- |
| FHSS GFSK modulation factor 0.3, BT = 0.5 | 1 Mbps | 1 | 0 | -70 |

Table 6 - Specifications of 802.15.1 standard

| Modulation | Symbol rate | Bit rate | Spreading factor | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| D-BPSK, R = 51/63 | 600 ksps | 121.4 kbps | 4 | 1 | -10 | -92 |

Table 7 - Specifications of 802.15.6 NB-PHY ((63,51)-BCH coded /2-shift DBPSK)

### Assumptions in coexistence performance

The assumptions made in this analysis are described in the following sub-clauses.

#### Path loss model

The path loss model used in this section is given in section 6.1.1.

#### PER model

The PER for IEEE 802.15.1 is calculated from the BER model for the PHY. For the GFSK PHY, the BER is given as a function of SINR as follows:

Pb=Qa,b−12exp−a2+b22I0ab                   (7)

where

a=SIR21−1−ρ2                          (8)

b=SIR21+1−ρ2                          (9)

I0(x) is the modified Bessel function of order zero.  is the correlation coefficient given by

ρ=sin2πβ2πβ                                    (10)

where  is the modulation index. From the computed BER Pb, PER is obtained as shown in the Equation (11).

PER=1−1−PbL                               (11)

where L is the number of bits in a packet. The L is assumed to be 1024 octets for the 802.15.1 PHY. The PER curve as a function of SINR for the 15.1 PHY is plotted in Figure 8.

Figure 8 - BER/PER curves as a function of SINR for 15.1 PHY

The PER for the 802.15.6 PHY mode is given in section 6.1.2.

### Interference modeling

The effect of the interfering signal on the desired signal is assumed to be averaged to the bandwidth of the victim system. The victim receiver (with receive power Prv ) receives the desired signal from the victim transmitter (with transmit power Ptv), located at a distance dD, while an interferer transmitter (with  transmit power PTI) is located at a distance dU. The ratio between the desired and undesired power present at the victim receiver will be used as the SIR of the victim system.

At victim receiver the received power from the desired signal is

And the received power from the interfering transmitter is

And the SIR is given as PRv-P’Rv.

### Coexistence simulation results

Figure 9 shows the relationship between the PER performance of the 802.15.1 victim receiver corresponding to the distance between the victim receiver to the 802.15.6 interferer. The separation distance between 802.15.1 victim transmitter and 802.15.1 victim receiver is 10 meters.

Figure 9 - PER vs. Distance between 802.15.6 Interferer and 802.15.1 Victim receiver

Figure 10 shows the relationship between the PER performance of the 802.15.6 victim receiver corresponding to the distance between the victim receiver to the 802.15.1 interferer. The separation distance between 802.15.6 victim transmitter and 802.15.6 receiver is 3 meters.

Figure 10 - Victim PER vs. Distance between 802.15.1 Interferer and 802.15.6 Victim receiver

## Coexistence with IEEE 802.15.4

This section considers issues regarding coexistence between IEEE 802.15.6 devices and IEEE 802.15.4 devices.

### Standards characterized for coexistence

This standard is specified for operation in the 402-405 MHz, 420-450 MHz, 863-868 MHz, 902-928 MHz, 950-958 MHz, 2360-2400 MHz, and 2400-2483.5 MHz bands.

IEEE 802.15.4 standards characterized in this document for coexistence with IEEE 802.15.6 are as follows:

- IEEE Std 802.15.4TM-2006 [3] (863-868 MHz/902-928 MHz/2400-2483.5 MHz)

- IEEE Std 802.15.4dTM-2009 [4] (950-958 MHz)

In the 863-868 MHz and 902-928 MHz bands, there are BPSK, O-QPSK, and ASK PHYs. In the 950-958 MHz band, there are GFSK and BPSK PHYs. In the 2400-2483.5 MHz band, there is O-QSPK PHY. Their detailed specifications are summarized in Table 8. These PHYs will serve as both AWNs and IWNs. In the 402-405 MHz and 420-450 MHz bands, at the present, any IEEE 802 standards have not been addressed.

(a) 860-868 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4 BPSK | 20 kbps | 20 ksymbol/s | 300 kchip/s |
| 15.4 ASK | 250 kbps | 12.5 ksymbol/s | 400 kchip/s |
| 15.4 O-QPSK | 100 kbps | 25 ksymbol/s | 400 kchip/s |

(b) 902-928 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4 BPSK | 40 kbps | 40 ksymbol/s | 600 kchip/s |
| 15.4 ASK | 250 kbps | 50 ksymbol/s | 1600 kchip/s |
| 15.4 O-QPSK | 250 kbps | 62.5 ksymbol/s | 1000 kchip/s |

(c) 950-958 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4d BPSK | 40 kbps | 40 ksymbol/s | 600 kchip/s |
| 15.4d GFSK | 100 kbps | 100 ksymbol/s | (no spreading) |

(d) 2400-2483.5 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4 O-QPSK | 250 kbps | 62.5 ksymbol/s | 2000 kchip/s |

Table 8 - Specifications of 15.4 standards

| Frequency band | Bit rate | Symbol rate | Spreading factor |
| --- | --- | --- | --- |
| 868/915/950 MHz | 101.2 kbps | 250 ksymbol/s | 2 |
| 2450 MHz | 121.4 kbps | 600 ksymbol/s | 4 |

Table 9 - Specifications of 15.6 NB-PHY ((63,51)-BCH coded π/2-shift DBPSK)

### Assumptions in coexistence performance

The assumptions made across all standards characterized for coexistence are described in the following sub-clauses.

#### Bandwidth

The three IEEE 802.15.4 PHYs that operate in the 863-868 MHz band have one channel, approximately 600 kHz wide. The coexistence methodology assumes that any the 863-868 MHz device in an AWN will have the same bandwidth as a device in the IWN. Similarly, in the 902-928 MHz and 2400-2483.5 MHz band, each channel is 2MHz wide. In the 950-958 MHz band, there are 2 different bandwidths are accommodated; one is approximately 600 kHz wide for BPSK, and the other is approximately 200 kHz for GFSK.

IEEE 802.15.6 has several PHYs in each frequency band. The mandatory PHY, which is (63, 51)-BCH coded /2-shift DBPSK, will serve as both AWNs and IWNs. In the 863-868 MHz, 902-928 MHz, 950-958 MHz bands, each channel is 400 KHz wide. In the 2400-2483.5 MHz, each channel is 1 MHz wide.

#### Path loss model

The path loss model is given in section 6.1.1.

#### PER model

PER for each PHY in IEEE 802.15.4 is calculated from BER model for each PHY. The BER is given as a function of SIR as follows:

BPSK PHY [3]

Pb=0.5exp−11.25SIR                                 (12)

O-QPSK PHY [3]

Pb=815116j=2M−1j16jexp10SIR1j−1            (13)

ASK PHY [3]

Pb=0.4146exp−6.0871SIR                            (14)

GFSK PHY [5]

Pb=Qa,b−12exp−a2+b22I0ab                   (15)

where

a=SIR21−1−ρ2                          (!6)

b=SIR21+1−ρ2                          (17)

I0(x) is the modified Bessel function of order zero.  is the correlation coefficient given by

ρ=sin2πβ2πβ                                    (18)

where  is the modulation index. From the computed BER Pb, PER is obtained as shown in the Equation (19).

PER=1−1−PbL                               (19)

where L is the number of bits in a packet. The L is 22 octets in all the PHYs except GFSK PHY. GFSK PHY assumes a 256-octet payload as its typical number. The PER curves as a function of SIR for these 15.4 PHYs are plotted in Figure 11.

Figure 11 - PER as a function of SNR for 15.4 PHYs

The PER for 15.6 is given in section 6.1.2.

#### Computation of SIR

In this coexistence test, the receive signal power at a receiver in an AWN is assumed to be 10 dB higher than the receiver sensitivity SAWN. The physical separation d, between the AWN’s receiver and a transmitter in an IWN, determines the path loss pl(d) as shown in Equation (1). Assuming antenna gains for both devices are 0 dBi, SIR at the AWN’s receiver is simply given by

SIR=(SAWN+10)−(Ptx,IWN−pld)                       (20)

where Ptx,IWN is the transmit signal power from the IWN’s transmitter. The Ptx,IWN is assumed to be 0 dBm for the 15.4 transmitters, and -10 dBm for the 15.6 transmitters. According to [2], if its receiver bandwidth of an AWN’s device, BAWN, is narrower than the IWN’s signal bandwidth BIWN, the SIR is scaled by the ratio between these bandwidths:

SIR=(SAWN+10)−(Ptx,IWN−pld)+BAWNB           (21)

#### Temporal model

Both the 15.4 devices and 15.6 NB-PHY devices are not operated in full-duty since they have own MAC mechanism based on CSMA/CA. By taking into account the duty cycle, a temporal model given in [2] is introduced. In this temporal model, interference packets are uniformly generated in a given duty cycle. The time duration and idle duration is given by the bit rate, length of the payload, and duty ratio. Typical numbers for these parameters are listed in Table 3. The length of typical payload is set to 22 octets for all IEEE 802.15.4 [3] except GFSK in 15.4d having 255 octets [7]. The duty cycle is assumed to be 1 % for 15.4 devices operated in the 860-868 MHz band, and 10 % for devices in the other bands.

Assuming that 15.6 device has 255-octet payload, which is the maximum length of the payload. On the duty ratio, since 15.6 covers wide variety of applications, it is reasonable to assume the duty cycle at 10 % worst case except the 860-868 MHz band, in which the duty cycle is assumed to be 1 %.

(a) 860-868 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 101.2 kbps | 255 octets | 20.24 ms | 1 % |
| 15.4 BPSK | 20 kbps | 22 octets | 8.80 ms | 1 % |
| 15.4 ASK | 250 kbps | 22 octets | 0.70 ms | 1 % |
| 15.4 O-QPSK | 100 kbps | 22 octets | 1.76 ms | 1 % |

(b) 902-928 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 101.2 kbps | 255 octets | 20.24 ms | 10 % |
| 15.4 BPSK | 40 kbps | 22 octets | 4.40 ms | 10 % |
| 15.4 ASK | 250 kbps | 22 octets | 0.70 ms | 10 % |
| 15.4 O-QPSK | 250 kbps | 22 octets | 0.70 ms | 10 % |

(c) 950-958 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 101.2 kbps | 255 octets | 20.24 ms | 10 % |
| 15.4d BPSK | 20 kbps | 22 octets | 8.80 ms | 10 % |
| 15.4d GFSK | 100 kbps | 255 octets | 20.48 ms | 10 % |

(d) 2400-2483.5 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 121.4 kbps | 255 octets | 16.80 ms | 10 % |
| 15.4 O-QPSK | 250 kbps | 22 octets | 0.70 ms | 10 % |

Table 10 - Specifications related to set a temporal model

### Coexistence performance

This clause shows the results of the coexistence assurance methodology for each frequency band.

#### 2400 - 2483.5 MHz band

Figure 12-(a) shows the PER of the 15.4 O-QPSK receiver under interference caused by the 15.6 signal. The results show that the required physical separation between the 15.4 receiver and 15.6 transmitter is 12 meters to maintain PER of 10-2. On the other hand, the PER of the 15.6 receiver is displayed in Figure 12-(b) when interfered by 15.4 O-QOSK signal. The required separation is 40 meters to obtain PER of 10-2.

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 12 - PER for coexistence assurance results for 2400-2483.5 MHz

#### 950 - 958 MHz band

Figure 13-(a) shows the PER of the either 15.4 BPSK receiver or GFSK receiver under interference caused by the 15.6 signal. Simulation results show that the required physical separations for the 15.4 BPSK receiver and 15.4 receiver is 28 meters and 44 meters, respectively, to maintain PER of 10-2. On the other hand, the PER displayed in Figure 13-(b) show that the 15.6 receiver requires separation of 114 meters to obtain PER of 10-2 when interfered by the 15.4 BPSK or GFSK signal

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 13 - PER for coexistence assurance results for 950-958 MHz

#### 902 - 928 MHz band

Figure 14-(a) shows the PER of the 15.4 receivers under interference caused by the 15.6 signal. The results show that the required physical separation between the 15.4 receiver and 15.6 transmitter is 22 meters, 33 meters, and 42 meters to maintain PER of 10-2 in O-QPSK, BPSK, and ASK receivers, respectively. In Figure 14-(b), the PER of the 15.6 receiver is shown when interfered by 15.4 signal. The results demonstrate that the required separation is 90 meters, 104 meters, and 77 meters, respectively, to obtain PER of 10-2.

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 14 - PER for coexistence assurance results for 902-928 MHz

#### 860 - 868 MHz band

Figure 15-(a) illustrates the PER of the 15.4 receivers under interference caused by the 15.6 signal. The results show that the required physical separation between the 15.4 receiver and 15.6 transmitter is 20 meters, 26 meters, and 20 meters for the O-QPSK, BPSK, and ASK receiver, respectively, to maintain PER of 10-2. PER of the 15.6 receiver is displayed in Figure 15-(b) when interfered by 15.4 signal. The required separation to accommodate with one 15.4 transmitter in maintaining PER of 10-2 is 115 meters for the 15.4 signals.

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 15 - PER for coexistence assurance results for 860-868 MHz

# Discussions and Conclusion

This document has presented an analysis of the coexistence between the IEEE 802.15.6 NB PHY and other IEEE 802 systems.