IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Proposed Resolution Frame Reception Requirements |  |
| Date Submitted | February 1, 2011 |  |
| Source | Rick Powell Zarlink Semiconductor, Inc. 15822 Bernardo Center Dr, Ste B, San Diego, CA, 92127 858-675-3485 rick.powell@zarlink.com |  |
| Re: | LB# 66 comment resolution |  |
| Abstract | This submission describes proposed resolutions to MAC related LB#66 comments of IEEE P802.15.6/D02, related to Frame Reception Requirements before and during the Unconnected_NID assignment phase. |  |
| Purpose | To facilitate resolution of LB#66 comments S7-116, S7-117, S7-118, S7-119 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Frame Reception for Unconnected Nodes

Figure 1 shows the 3 Unconnected NID states of an Unconnected Node as it receives its assigned Connected_NID while it is in the connection/association process.  The Unconnected NID state determines the Frame Reception Requirements in regards to recipient NID and Frame Type.  Other requirements for Frame Reception are defined in 7.2.4.  The Unconnected NID state diagram shown in Figure 1 below operates in conjunction to those state diagrams shown in “MAC and security state diagrams” of Figure 4 in the D02 Draft.

An unconnected node may only receive Management Type Frames or Control Type Frames.

The unconnected node starts out in the Unconnected_NID state.  In this state, it can receive frames with the recipient NID set to the Unconnected_NID or to any Connected NID, which are either Management Frames, or I-Ack Frames.  I-Ack Frames are only received in response to the Management Frames sent by the node with the Unconnected_NID as the sender NID.  Other Control Type Frames, like Polls, may only be received with the Unconnected_NID while in the Unconnected NID state.

If the unconnected node in the Unconnected NID state receives and I-Ack with a Connected_NID in the recipient NID field, then it changes to the Temporary NID state and uses this NID as it Temporary Connected_NID.  In the Temporary NID state, the node receives frames as defines in 7.2.4, where the Temporary Connected_NID is used as the node’s NID.

In either the Unconnected NID state or the Temporary NID state, if the node receives a Management Frame addressed to the any Connected_NID and with the node’s EUI-48 address in the payload, the node goes to the Permanent Connected_NID state.

Figure 1: Unconnected NID State Diagram

Table 1 shows the Frame Reception Requirements for Unconnected Nodes for the different Unconnected NID states.

| Unconnected NID State | Action | Frame Reception Requirements |
| --- | --- | --- |
| Unconnected_NID | Node sends Management Frame with Unconnected_NID | Node receives I-Ack Frame within timeout, with 7.2.4 Requirements, with Unconnected_NID or any Connected NID for recipient NID. |
| Unconnected_NID | Node receives Management Type Frames | Node receives Management Frames with 7.2.4 Requirements, with Unconnected_NID or any Connected NID for recipient NID. |
| Unconnected_NID | Node receives non-I-Ack Control Frames | Node receives non-I-Ack Control Frames with 7.2.4 Requirements, with Unconnected_NID for recipient NID. |
| Temporary Connected_NID | Node sends Management Frame with Temporary Connected_NID | Node receives I-Ack within timeout, with 7.2.4 Requirements, where recipient NID is the Temporary Connected_NID |
| Temporary Connected_NID | Node receives Management or non-Ack Control Frames | Node receives only Management or non-Ack Control Frames with 7.2.4 Requirements, with Temporary Connected NID for recipient NID. |
| Permanent Connected_NID | Node sends Management Frame with Permanent Connected_NID | Node receives I-Ack within timeout, with 7.2.4 Requirements, where recipient NID is the Permanent Connected_NID |
| Permanent Connected_NID | Node receives Management or non-Ack Control Frames | Node receives only Management or non-Ack Control Frames with 7.2.4 Requirements, with Permanent Connected NID for recipient NID. |

Table 1: Frame Reception Requirements for Unconnected Node