IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | Proposed Clarification of Timeout Conditions |
| Date Submitted | February 23, 2011 |
| Source | Jin-Meng Ho, Okundu C. Omeni, Rick Powell |
| Re: | LB# 66 comment resolution |
| Abstract | This submission describes proposed clarification to the MAC portion of IEEE P802.15.6/D02, and provides a summary of various timeout conditions |
| Purpose | To facilitate resolution of several LB#66 comments related to timeout conditions |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table 1 gives a summary to different timeout conditions relative to the More Data and Last Frame bits for various access conditions.

| No. | Case | MD | LF | Superframe | Non-Superframe |
| --- | --- | --- | --- | --- | --- |
| 1a | Hub Retransmitting Unacknowledged Frame | 0 | 0 | pSIFS after end of expected Ack for retry | pSIFS after end of expected Ack for retry |
| 1b | Hub Retransmitting Unacknowledged Frame | 1 | 0 | pSIFS after end of expected Ack | pSIFS after end of expected Ack |
| 1c | Hub Retransmitting Unacknowledged Frame | 0 | 1 | In Next Allocation interval | In next allocation interval |
| 1d | Hub Retransmitting Unacknowledged Frame | 1 | 1 | In Next Allocation interval | In next allocation interval |
| 2a | Hub Reclaiming Allocation Interval after sending I/B-Ack | 0 | 0 | Beginning of MAC Header | Beginning of MAC Header1 |
| 2b | Hub Reclaiming Allocation Interval after sending I/B-Ack | 1 | 0 | End of Allocation Interval (accounting for appropriate guardtime) | Beginning of MAC Header1 |
| 2c | Hub Reclaiming Allocation Interval after sending I/B-Ack or receiving data/management frame | 0 | 1 | pSIFS after end of current frame transaction | pSIFS after end of current frame transaction |
| 2d | Hub Reclaiming Allocation Interval after sending I/B-Ack or receiving data/management frame | 1 | 1 | pSIFS after end of current frame transaction | pSIFS after end of current frame transaction |
| 3 | Hub reclaiming allocation interval after a poll with an immediate polled allocation but without a response | X | X | Beginning of MAC Header2 | Beginning of MAC Header2 |
| 4a | Node Retransmitting Unacknowledged Frame | 0 | 0 | pSIFS after end of expected Ack for retry | pSIFS after end of expected Ack for retry1 |
| 4b | Node Retransmitting Unacknowledged Frame | 1 | 0 | pSIFS after end of expected Ack | pSIFS after end of expected Ack1 |
| 4c | Node Retransmitting Unacknowledged Frame | 0 | 1 | In Next Allocation interval | In next allocation interval |
| 4d | Node Retransmitting Unacknowledged Frame | 1 | 1 | In Next Allocation interval | In next allocation interval |
| 5a | Node going to sleep after sending I/B-Ack with More=0 | 0 | 0 | Beginning of MAC Header for possible retry3 | Beginning of MAC Header for possible retry3 |
| 5b | Node going to sleep after sending I/B-Ack with More=0 | 1 | 0 | End of Allocation Interval (accounting for appropriate guardtime) | Beginning of MAC Header |
| 5c | Node going to sleep after sending I/B-Ack with More=0 | 0 | 1 | Immediately | Immediately |
| 5d | Node going to sleep after sending I/B-Ack with More=0 | 1 | 1 | Immediately | Immediately |
| 6a | Node going to sleep after sending I/B-Ack with More=1 | 0 | 0 | Beginning of MAC Header4 | Beginning of MAC Header4 |
| 6b | Node going to sleep after sending I/B-Ack with More=1 | 1 | 0 | Beginning of MAC Header4 | Beginning of MAC Header4 |
| 6c | Node going to sleep after sending I/B-Ack with More=1 | 0 | 1 | Immediately | Immediately |
| 6d | Node going to sleep after sending I/B-Ack with More=1 | 1 | 1 | Immediately | Immediately |
| 7a | Node going to sleep after receiving I/B-Ack with More=0 | 0 | X | Immediately | Immediately |
| 7b | Node going to sleep after receiving I/B-Ack with More=0 | X | 1 | Immediately | Immediately |
| 7c | Node going to sleep after receiving I/B-Ack with More=0 | 1 | 0 | No Timeout. Node sends another Frame pSIFS after end of expected Ack | Immediately |
| 8a | Node going to sleep after receiving I/B-Ack with More=1 | 0 | 0 | 2x Beginning of MAC Header5 | 2x Beginning of MAC Header5 |
| 8b | Node going to sleep after receiving I/B-Ack with More=1 | 1 | 0 | Beginning of MAC header after end of allocation interval (accounting for appropriate guardtime) | 2x Beginning of MAC Header5 |
| 8c | Node going to sleep after receiving I/B-Ack with More=1 | X | 1 | Beginning of MAC Header6 | Beginning of MAC Header6 |
| 9 | Node going to sleep after failing to receive a future poll/post outside a scheduled downlink/bilink allocation/ | NA | NA | Beginning of MAC header after announced beginning of the future poll/post (accounting for appropriate guardtime) | NA |

MD = More Data on last Data/Management Frame

LF = Last Frame on last Data/Management Frame

<text> = Proposed changes and/or clarifications

Note 1: Allows retransmission in non-superframe mode

Note 2: This only applies to the first uplink frame of a Polled Allocation Interval.  This is an exception to the rules for aborting a Polled Allocation Interval, as defined in 7.6.1.1.4: Aborting a polled allocation: Pg 100, Ln 33.

Note 3: This assumes possible retransmission.

Note 4: This assumes a possible Poll will be sent.

Note 5: Node expects Post/Poll, but hub must wait for start of MAC Header Timeout

Note 6: Node expects Post/Poll