IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | OFDM frame example when the PIB attribute phyOFDMInterleaving is one for LB67. |  |
| Date Submitted | 09 May, 2011 |  |
| Source | [Cheolho SHIN and Sangsung CHOI] [ETRI] | Voice: [ ] Fax: [ ] E-mail: [chshin@etri.re.kr sschoiQetri.re.kr] |
| Re: | Comment Resolution for CID 23 of LB 70 |  |
| Abstract | This document presents an example of OFDM Frame. |  |
| Purpose | Proposed Comment Resolution for LB 70 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Annex K

(informative)

This example sets the PIB attribute value of phyOFDMInterleaving to one. The other conditions are the same with Annex J.

K.1 Generation of the OFDM Header

K.1.1 HCS and PAD bits insertion

In this example the payload data has a size of 76 octets and it will be encoded using QPSK modulation, ½ rate coding (Rate field value = 3). The scrambler index will be the first one (Scrambler ID = 0).

The corresponding OFDM Header, including the HCS is represented in table K.1

Table K.1 – OFDM Header

| Field Name | Bit # | Bit Value |  | Field Name | Bit # | Bit Value |  | Field Name | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rate | 1 | 0 |  | Scrambler | 20 | 0 |  | Pad | 37 | 0 |
|  | 2 | 0 |  |  | 21 | 0 |  |  | 38 | 0 |
|  | 3 | 0 |  | RFU | 22 | 0 |  |  | 39 | 0 |
|  | 4 | 1 |  | HCS | 23 | 1 |  |  | 40 | 0 |
|  | 5 | 1 |  |  | 24 | 1 |  |  | 41 | 0 |
| RFU | 6 | 0 |  |  | 25 | 1 |  |  | 42 | 0 |
| Length | 7 | 0 |  |  | 26 | 1 |  |  | 43 | 0 |
|  | 8 | 0 |  |  | 27 | 1 |  |  | 44 | 0 |
|  | 9 | 0 |  |  | 28 | 1 |  |  | 45 | 0 |
|  | 10 | 0 |  |  | 29 | 0 |  |  | 46 | 0 |
|  | 11 | 1 |  |  | 30 | 1 |  |  | 47 | 0 |
|  | 12 | 0 |  | Tail | 31 | 0 |  |  | 48 | 0 |
|  | 13 | 0 |  |  | 32 | 0 |  |  |  |  |
|  | 14 | 1 |  |  | 33 | 0 |  |  |  |  |
|  | 15 | 1 |  |  | 34 | 0 |  |  |  |  |
|  | 16 | 0 |  |  | 35 | 0 |  |  |  |  |
|  | 17 | 0 |  |  | 36 | 0 |  |  |  |  |
| RFU | 18 | 0 |  |  |  |  |  |  |  |  |
|  | 19 | 0 |  |  |  |  |  |  |  |  |

In this configuration, PAD bits of 12 are necessary. The size of the header will fill up exactly eight OFDM symbols.

K.1.2 Convolutional Encoding

After convolutional encoding of the OFDM header, the size is now doubled and the corresponding bits are represented in table K.2. No puncturing is applied in this configuration.

Table K.2 – OFDM Header after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 25 | 1 | 49 | 0 | 73 | 0 |
| 2 | 0 | 26 | 1 | 50 | 1 | 74 | 0 |
| 3 | 0 | 27 | 0 | 51 | 1 | 75 | 0 |
| 4 | 0 | 28 | 0 | 52 | 0 | 76 | 0 |
| 5 | 0 | 29 | 1 | 53 | 1 | 77 | 0 |
| 6 | 0 | 30 | 0 | 54 | 0 | 78 | 0 |
| 7 | 1 | 31 | 0 | 55 | 0 | 79 | 0 |
| 8 | 1 | 32 | 0 | 56 | 0 | 80 | 0 |
| 9 | 1 | 33 | 1 | 57 | 0 | 81 | 0 |
| 10 | 0 | 34 | 1 | 58 | 0 | 82 | 0 |
| 11 | 1 | 35 | 1 | 59 | 1 | 83 | 0 |
| 12 | 0 | 36 | 1 | 60 | 0 | 84 | 0 |
| 13 | 0 | 37 | 1 | 61 | 1 | 85 | 0 |
| 14 | 0 | 38 | 0 | 62 | 1 | 86 | 0 |
| 15 | 1 | 39 | 0 | 63 | 1 | 87 | 0 |
| 16 | 1 | 40 | 1 | 64 | 0 | 88 | 0 |
| 17 | 1 | 41 | 1 | 65 | 1 | 89 | 0 |
| 18 | 0 | 42 | 1 | 66 | 0 | 90 | 0 |
| 19 | 0 | 43 | 0 | 67 | 1 | 91 | 0 |
| 20 | 1 | 44 | 0 | 68 | 1 | 92 | 0 |
| 21 | 0 | 45 | 1 | 69 | 1 | 93 | 0 |
| 22 | 0 | 46 | 1 | 70 | 0 | 94 | 0 |
| 23 | 0 | 47 | 1 | 71 | 1 | 95 | 0 |
| 24 | 1 | 48 | 0 | 72 | 1 | 96 | 0 |

K.1.3 Interleaving

The formula defined in 16.2.3.5 will lead to an interleaving. In this case, Ncbps is defined as 48 and Nrow is 3. The corresponding bits are represented in table K.3.

Table K.3 – OFDM Header after Interleaving

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 25 | 1 | 49 | 0 | 73 | 0 |
| 2 | 0 | 26 | 1 | 50 | 0 | 74 | 0 |
| 3 | 1 | 27 | 0 | 51 | 0 | 75 | 0 |
| 4 | 0 | 28 | 1 | 52 | 0 | 76 | 0 |
| 5 | 0 | 29 | 0 | 53 | 1 | 77 | 0 |
| 6 | 1 | 30 | 1 | 54 | 0 | 78 | 0 |
| 7 | 0 | 31 | 0 | 55 | 1 | 79 | 0 |
| 8 | 0 | 32 | 1 | 56 | 0 | 80 | 0 |
| 9 | 1 | 33 | 0 | 57 | 0 | 81 | 1 |
| 10 | 0 | 34 | 0 | 58 | 0 | 82 | 0 |
| 11 | 0 | 35 | 1 | 59 | 0 | 83 | 0 |
| 12 | 1 | 36 | 0 | 60 | 0 | 84 | 0 |
| 13 | 1 | 37 | 1 | 61 | 0 | 85 | 1 |
| 14 | 1 | 38 | 0 | 62 | 0 | 86 | 0 |
| 15 | 0 | 39 | 0 | 63 | 0 | 87 | 1 |
| 16 | 1 | 40 | 1 | 64 | 0 | 88 | 1 |
| 17 | 0 | 41 | 0 | 65 | 1 | 89 | 0 |
| 18 | 0 | 42 | 0 | 66 | 1 | 90 | 0 |
| 19 | 1 | 43 | 1 | 67 | 0 | 91 | 0 |
| 20 | 1 | 44 | 1 | 68 | 1 | 92 | 0 |
| 21 | 0 | 45 | 0 | 69 | 1 | 93 | 0 |
| 22 | 1 | 46 | 1 | 70 | 1 | 94 | 0 |
| 23 | 1 | 47 | 1 | 71 | 1 | 95 | 0 |
| 24 | 0 | 48 | 0 | 72 | 1 | 96 | 0 |

K.1.4 Bit mapping

The 96 bits are split into eight OFDM symbols. The bit mapping for the OFDM header in this example is BPSK, therefore Q is always 0. The I value is mapped as defined in Table K.4.

Table K.4 – Bit mapping for the OFDM Header

|  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  | Symbol 6 |  | Symbol 7 |  | Symbol 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 2 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 3 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 4 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 5 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 6 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 7 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 8 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 9 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 10 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 11 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 12 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |

K.1.5 Frequency Spreading

In this example, a frequency spreading of four is applied to the OFDM header. The original 12 bits in each symbol (Bin # 25 through 36 in table K.5) are duplicated within the same symbol. The resulting symbols have 48 data bits each. The duplicated bits have a phase rotation as defined in 16.2.3.6.

Table K.5 –OFDM Header in the frequency domain

|  | Symbol 1 |  | Symbol 2 |  | Symbol 3 |  | Symbol 4 |  | Symbol 5 |  | Symbol 6 |  | Symbol 7 |  | Symbol 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q | I | Q |
| 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 2 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 3 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 |
| 4 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 5 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 6 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 7 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 8 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 9 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 10 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 11 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 12 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 13 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 14 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 15 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 16 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 17 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 18 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 19 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 20 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 21 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 22 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 23 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 24 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 25 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 26 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 27 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 28 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 29 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 30 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 31 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 32 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 33 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 34 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 35 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 36 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 37 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 |
| 38 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 |
| 39 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 40 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 |
| 41 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 |
| 42 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 43 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 44 | 0 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |
| 45 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 |
| 46 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | -1 |
| 47 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| 48 | 0 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 0 | 1 | 0 | 1 |

K.2 Generation of the data Symbols

K.2.1 PAD Insertion and Data Scrambling

The original 76 octets of data are concatenated with six zero tail bits and 10 pad bits (as calculated by the formulas in 16.2.3.10). The six tail bits are forced to zero after scrambling. The resulting 624 bits are represented in table K.6 (First and last 48 bits only).

Table K.6 – First and last 48 bits after pad insertion and scrambling

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 0 | 577 | 0 | 601 | 1 |
| 2 | 0 | 26 | 0 | 578 | 0 | 602 | 0 |
| 3 | 0 | 27 | 0 | 579 | 1 | 603 | 0 |
| 4 | 0 | 28 | 0 | 580 | 0 | 604 | 0 |
| 5 | 0 | 29 | 0 | 581 | 0 | 605 | 0 |
| 6 | 1 | 30 | 0 | 582 | 1 | 606 | 0 |
| 7 | 0 | 31 | 0 | 583 | 1 | 607 | 0 |
| 8 | 0 | 32 | 1 | 584 | 0 | 608 | 0 |
| 9 | 0 | 33 | 0 | 585 | 0 | 609 | 0 |
| 10 | 1 | 34 | 0 | 586 | 1 | 610 | 0 |
| 11 | 0 | 35 | 0 | 587 | 1 | 611 | 0 |
| 12 | 1 | 36 | 0 | 588 | 0 | 612 | 0 |
| 13 | 1 | 37 | 0 | 589 | 1 | 613 | 0 |
| 14 | 1 | 38 | 0 | 590 | 1 | 614 | 0 |
| 15 | 0 | 39 | 1 | 591 | 0 | 615 | 0 |
| 16 | 0 | 40 | 1 | 592 | 0 | 616 | 0 |
| 17 | 1 | 41 | 0 | 593 | 1 | 617 | 0 |
| 18 | 0 | 42 | 1 | 594 | 0 | 618 | 0 |
| 19 | 0 | 43 | 1 | 595 | 1 | 619 | 0 |
| 20 | 0 | 44 | 0 | 596 | 1 | 620 | 1 |
| 21 | 1 | 45 | 1 | 597 | 1 | 621 | 1 |
| 22 | 1 | 46 | 0 | 598 | 1 | 622 | 0 |
| 23 | 0 | 47 | 0 | 599 | 0 | 623 | 1 |
| 24 | 1 | 48 | 0 | 600 | 0 | 624 | 1 |

K.2.2 Convolutional Encoding and puncturing

After convolutional encoding of the payload, the size is now doubled and the corresponding bits are represented in table K.7. No puncturing is applied in this configuration.

Table K.7 – First and last 48 bits after convolutional encoding

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 0 | 1201 | 0 | 1225 | 0 |
| 2 | 1 | 26 | 1 | 1202 | 1 | 1226 | 0 |
| 3 | 0 | 27 | 0 | 1203 | 0 | 1227 | 0 |
| 4 | 1 | 28 | 1 | 1204 | 0 | 1228 | 0 |
| 5 | 1 | 29 | 1 | 1205 | 1 | 1229 | 0 |
| 6 | 1 | 30 | 1 | 1206 | 0 | 1230 | 0 |
| 7 | 1 | 31 | 1 | 1207 | 0 | 1231 | 0 |
| 8 | 1 | 32 | 1 | 1208 | 0 | 1232 | 0 |
| 9 | 0 | 33 | 1 | 1209 | 0 | 1233 | 0 |
| 10 | 0 | 34 | 0 | 1210 | 0 | 1234 | 0 |
| 11 | 0 | 35 | 0 | 1211 | 1 | 1235 | 0 |
| 12 | 1 | 36 | 0 | 1212 | 0 | 1236 | 0 |
| 13 | 1 | 37 | 1 | 1213 | 1 | 1237 | 0 |
| 14 | 0 | 38 | 0 | 1214 | 1 | 1238 | 0 |
| 15 | 1 | 39 | 0 | 1215 | 0 | 1239 | 1 |
| 16 | 1 | 40 | 0 | 1216 | 0 | 1240 | 1 |
| 17 | 1 | 41 | 1 | 1217 | 0 | 1241 | 1 |
| 18 | 1 | 42 | 1 | 1218 | 0 | 1242 | 0 |
| 19 | 1 | 43 | 0 | 1219 | 0 | 1243 | 1 |
| 20 | 1 | 44 | 0 | 1220 | 0 | 1244 | 0 |
| 21 | 1 | 45 | 0 | 1221 | 0 | 1245 | 1 |
| 22 | 1 | 46 | 1 | 1222 | 0 | 1246 | 1 |
| 23 | 1 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 1 | 48 | 1 | 1224 | 0 | 1248 | 1 |

K.2.3 Interleaving

A two-step interleaver is applied to the data, as defined in 16.2.3.5. In this case, Ncbps is defined as 96 and Nrow  is 12. The resulting data (first and last 48 bits only) is represented in table K.8.

Table K.8 – First and last 48 bits after interleaving

| Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value | Bit # | Bit Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 25 | 1 | 1201 | 0 | 1225 | 0 |
| 2 | 1 | 26 | 1 | 1202 | 0 | 1226 | 0 |
| 3 | 0 | 27 | 1 | 1203 | 0 | 1227 | 1 |
| 4 | 1 | 28 | 0 | 1204 | 0 | 1228 | 0 |
| 5 | 1 | 29 | 1 | 1205 | 0 | 1229 | 0 |
| 6 | 0 | 30 | 1 | 1206 | 0 | 1230 | 0 |
| 7 | 1 | 31 | 1 | 1207 | 0 | 1231 | 0 |
| 8 | 0 | 32 | 0 | 1208 | 1 | 1232 | 1 |
| 9 | 1 | 33 | 1 | 1209 | 0 | 1233 | 1 |
| 10 | 0 | 34 | 1 | 1210 | 0 | 1234 | 1 |
| 11 | 1 | 35 | 1 | 1211 | 1 | 1235 | 1 |
| 12 | 0 | 36 | 1 | 1212 | 0 | 1236 | 1 |
| 13 | 0 | 37 | 1 | 1213 | 0 | 1237 | 1 |
| 14 | 0 | 38 | 0 | 1214 | 0 | 1238 | 0 |
| 15 | 0 | 39 | 1 | 1215 | 0 | 1239 | 0 |
| 16 | 1 | 40 | 1 | 1216 | 0 | 1240 | 0 |
| 17 | 0 | 41 | 1 | 1217 | 1 | 1241 | 1 |
| 18 | 1 | 42 | 1 | 1218 | 0 | 1242 | 1 |
| 19 | 0 | 43 | 1 | 1219 | 1 | 1243 | 1 |
| 20 | 0 | 44 | 1 | 1220 | 1 | 1244 | 0 |
| 21 | 0 | 45 | 0 | 1221 | 0 | 1245 | 0 |
| 22 | 1 | 46 | 1 | 1222 | 0 | 1246 | 0 |
| 23 | 1 | 47 | 1 | 1223 | 0 | 1247 | 0 |
| 24 | 0 | 48 | 0 | 1224 | 1 | 1248 | 1 |

K.2.4 Bit mapping

The bit mapping for the OFDM header in this example is QPSK (two data bits per vector). The I and Q vectors are mapped as defined in figure 131. The resulting data is represented in table K.9 (first two and last two symbols).

Table K.9 – Bit mapping for the OFDM payload

| Vector # | I | Q | Vector # | I | Q | Vector # | I | Q | Vector # | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.707 | 0.707 | 49 | 0.707 | -0.707 | 529 | -0.707 | -0.707 | 577 | -0.707 | 0.707 |
| 2 | -0.707 | 0.707 | 50 | -0.707 | 0.707 | 530 | -0.707 | 0.707 | 578 | -0.707 | -0.707 |
| 3 | 0.707 | -0.707 | 51 | -0.707 | -0.707 | 531 | -0.707 | 0.707 | 579 | -0.707 | 0.707 |
| 4 | 0.707 | -0.707 | 52 | 0.707 | -0.707 | 532 | -0.707 | -0.707 | 580 | -0.707 | -0.707 |
| 5 | 0.707 | -0.707 | 53 | 0.707 | -0.707 | 533 | -0.707 | 0.707 | 581 | 0.707 | -0.707 |
| 6 | 0.707 | -0.707 | 54 | -0.707 | 0.707 | 534 | -0.707 | 0.707 | 582 | -0.707 | 0.707 |
| 7 | -0.707 | -0.707 | 55 | -0.707 | -0.707 | 535 | 0.707 | 0.707 | 583 | 0.707 | 0.707 |
| 8 | -0.707 | 0.707 | 56 | 0.707 | -0.707 | 536 | -0.707 | 0.707 | 584 | -0.707 | -0.707 |
| 9 | -0.707 | 0.707 | 57 | 0.707 | -0.707 | 537 | -0.707 | -0.707 | 585 | -0.707 | -0.707 |
| 10 | -0.707 | -0.707 | 58 | 0.707 | 0.707 | 538 | -0.707 | -0.707 | 586 | -0.707 | 0.707 |
| 11 | -0.707 | 0.707 | 59 | 0.707 | -0.707 | 539 | 0.707 | 0.707 | 587 | -0.707 | -0.707 |
| 12 | 0.707 | -0.707 | 60 | -0.707 | -0.707 | 540 | -0.707 | 0.707 | 588 | -0.707 | 0.707 |
| 13 | 0.707 | 0.707 | 61 | -0.707 | -0.707 | 541 | 0.707 | 0.707 | 589 | -0.707 | -0.707 |
| 14 | 0.707 | -0.707 | 62 | 0.707 | -0.707 | 542 | -0.707 | -0.707 | 590 | 0.707 | -0.707 |
| 15 | 0.707 | 0.707 | 63 | 0.707 | -0.707 | 543 | -0.707 | 0.707 | 591 | -0.707 | -0.707 |
| 16 | 0.707 | -0.707 | 64 | -0.707 | -0.707 | 544 | -0.707 | 0.707 | 592 | -0.707 | 0.707 |
| 17 | 0.707 | 0.707 | 65 | -0.707 | 0.707 | 545 | -0.707 | 0.707 | 593 | 0.707 | 0.707 |
| 18 | 0.707 | 0.707 | 66 | -0.707 | -0.707 | 546 | -0.707 | -0.707 | 594 | -0.707 | -0.707 |
| 19 | 0.707 | -0.707 | 67 | -0.707 | 0.707 | 547 | -0.707 | 0.707 | 595 | 0.707 | -0.707 |
| 20 | 0.707 | 0.707 | 68 | 0.707 | 0.707 | 548 | 0.707 | 0.707 | 596 | -0.707 | 0.707 |
| 21 | 0.707 | 0.707 | 69 | -0.707 | 0.707 | 549 | 0.707 | 0.707 | 597 | -0.707 | 0.707 |
| 22 | 0.707 | 0.707 | 70 | 0.707 | 0.707 | 550 | 0.707 | -0.707 | 598 | -0.707 | 0.707 |
| 23 | -0.707 | 0.707 | 71 | 0.707 | 0.707 | 551 | 0.707 | -0.707 | 599 | -0.707 | -0.707 |
| 24 | 0.707 | -0.707 | 72 | 0.707 | 0.707 | 552 | -0.707 | 0.707 | 600 | -0.707 | -0.707 |
| 25 | 0.707 | 0.707 | 73 | 0.707 | -0.707 | 553 | -0.707 | -0.707 | 601 | -0.707 | -0.707 |
| 26 | 0.707 | -0.707 | 74 | -0.707 | -0.707 | 554 | 0.707 | 0.707 | 602 | -0.707 | -0.707 |
| 27 | 0.707 | 0.707 | 75 | 0.707 | -0.707 | 555 | 0.707 | 0.707 | 603 | -0.707 | -0.707 |
| 28 | 0.707 | -0.707 | 76 | 0.707 | 0.707 | 556 | 0.707 | 0.707 | 604 | -0.707 | 0.707 |
| 29 | 0.707 | 0.707 | 77 | -0.707 | -0.707 | 557 | -0.707 | -0.707 | 605 | -0.707 | -0.707 |
| 30 | 0.707 | -0.707 | 78 | 0.707 | 0.707 | 558 | -0.707 | 0.707 | 606 | 0.707 | -0.707 |
| 31 | 0.707 | 0.707 | 79 | 0.707 | 0.707 | 559 | -0.707 | 0.707 | 607 | -0.707 | -0.707 |
| 32 | -0.707 | 0.707 | 80 | -0.707 | 0.707 | 560 | 0.707 | 0.707 | 608 | -0.707 | -0.707 |
| 33 | -0.707 | 0.707 | 81 | 0.707 | -0.707 | 561 | -0.707 | -0.707 | 609 | 0.707 | -0.707 |
| 34 | 0.707 | -0.707 | 82 | -0.707 | 0.707 | 562 | -0.707 | -0.707 | 610 | 0.707 | 0.707 |
| 35 | 0.707 | 0.707 | 83 | 0.707 | -0.707 | 563 | -0.707 | -0.707 | 611 | -0.707 | -0.707 |
| 36 | 0.707 | -0.707 | 84 | 0.707 | 0.707 | 564 | -0.707 | -0.707 | 612 | -0.707 | 0.707 |
| 37 | -0.707 | 0.707 | 85 | -0.707 | 0.707 | 565 | 0.707 | 0.707 | 613 | -0.707 | -0.707 |
| 38 | -0.707 | 0.707 | 86 | 0.707 | 0.707 | 566 | 0.707 | 0.707 | 614 | 0.707 | -0.707 |
| 39 | -0.707 | 0.707 | 87 | -0.707 | 0.707 | 567 | -0.707 | 0.707 | 615 | -0.707 | -0.707 |
| 40 | -0.707 | 0.707 | 88 | -0.707 | 0.707 | 568 | -0.707 | -0.707 | 616 | -0.707 | 0.707 |
| 41 | -0.707 | 0.707 | 89 | -0.707 | 0.707 | 569 | 0.707 | 0.707 | 617 | 0.707 | 0.707 |
| 42 | -0.707 | 0.707 | 90 | 0.707 | 0.707 | 570 | -0.707 | -0.707 | 618 | 0.707 | 0.707 |
| 43 | 0.707 | -0.707 | 91 | -0.707 | -0.707 | 571 | -0.707 | -0.707 | 619 | 0.707 | -0.707 |
| 44 | 0.707 | 0.707 | 92 | -0.707 | 0.707 | 572 | 0.707 | 0.707 | 620 | -0.707 | -0.707 |
| 45 | 0.707 | 0.707 | 93 | -0.707 | 0.707 | 573 | 0.707 | -0.707 | 621 | 0.707 | 0.707 |
| 46 | -0.707 | 0.707 | 94 | 0.707 | 0.707 | 574 | 0.707 | -0.707 | 622 | 0.707 | -0.707 |
| 47 | 0.707 | -0.707 | 95 | -0.707 | 0.707 | 575 | 0.707 | 0.707 | 623 | -0.707 | -0.707 |
| 48 | 0.707 | -0.707 | 96 | -0.707 | -0.707 | 576 | 0.707 | -0.707 | 624 | -0.707 | 0.707 |

K.2.5 Frequency Spreading

In this example, no frequency spreading is applied to the OFDM payload. The vectors from table K.9 are mapped directly into the frequency domain. The next paragraphs are illustrating the mapping in the frequency domain, taking into account the pilot tones, the DC tone and the guard tones.

K.3 Conversion from frequency domain to time domain

K.3.1 Pilot, DC and Guard tone insertion

The following steps are applied to both the OFDM header and the OFDM payload. Before going to the next steps, table K.5 and K.9 should be appended, resulting in 21 symbols of 48 bins in the frequency domain. The 48 bins are mapped in the frequency domain by inserting pilot tones, guard tones and a DC tone, as defined in 16.2.3.7.

Table K.10 Complete Packet in the frequency domain (first and last three symbols)

| Subcarrier | Symbol 1 | Symbol 2 | Symbol 3 | Symbol 19 | Symbol 20 | Symbol 21 |
| --- | --- | --- | --- | --- | --- | --- |
| -32 | 0 | 0 | 0 | 0 | 0 | 0 |
| -31 | 0 | 0 | 0 | 0 | 0 | 0 |
| -30 | 0 | 0 | 0 | 0 | 0 | 0 |
| -29 | 0 | 0 | 0 | 0 | 0 | 0 |
| -28 | 0 | 0 | 0 | 0 | 0 | 0 |
| -27 | 0 | 0 | 0 | 0 | 0 | 0 |
| -26 | -0 - 1i | 0 + 1i | 0 + 1i | -0.707 + 0.707i | -0.707 - 0.707i | -1 |
| -25 | 0 + 1i | -0 - 1i | -0 - 1i | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -24 | 0 + 1i | -0 - 1i | -0 - 1i | 0.707 + 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| -23 | 0 + 1i | -0 - 1i | -0 - 1i | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| -22 | -0 - 1i | 1 | -0 - 1i | 1 | -0.707 + 0.707i | -0.707 - 0.707i |
| -21 | -0 - 1i | -0 - 1i | -0 - 1i | 0.707 - 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -20 | 0 - 1i | 0 + 1i | 0 - 1i | 0.707 - 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| -19 | 0 + 1i | -0 + 1i | -0 - 1i | -0.707 - 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| -18 | -0 + 1i | -0 - 1i | -1 | -0.707 - 0.707i | -1 | -0.707 - 0.707i |
| -17 | 0 + 1i | 0 - 1i | 0 - 1i | 0.707 + 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| -16 | 0 - 1i | -0 - 1i | 0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| -15 | -0 - 1i | -0 + 1i | -0 + 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| -14 | -1 | 0 + 1i | 0 + 1i | 0.707 - 0.707i | -0.707 + 0.707i | -0.707 + 0.707i |
| -13 | 1 - 0i | -1 + 0i | -1 + 0i | -0.707 - 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| -12 | -0 - 1i | 0 + 1i | 0 + 1i | -0.707 + 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| -11 | 1 - 0i | -1 + 0i | -1 + 0i | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| -10 | 0 + 1i | 1 | -0 - 1i | -1 | -0.707 + 0.707i | -0.707 + 0.707i |
| -9 | 1 - 0i | -0 - 1i | 1 - 0i | 0.707 + 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| -8 | -0 + 1i | 1 - 0i | -0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| -7 | -1 + 0i | 0 - 1i | -1 + 0i | 0.707 + 0.707i | -0.707 + 0.707i | 0.707 - 0.707i |
| -6 | 0 + 1i | 1 - 0i | 1 | 0.707 + 0.707i | 1 | -0.707 + 0.707i |
| -5 | -1 - 0i | -0 - 1i | -0 - 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| -4 | 0 - 1i | 1 + 0i | 1 + 0i | -0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| -3 | -1 + 0i | -0 + 1i | 0 - 1i | 0.707 - 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| -2 | -1 | 1 - 0i | 1 - 0i | 0.707 - 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| -1 | -0 - 1i | 0 + 1i | 0 + 1i | 0.707 + 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | -1 | 1 | 1 | -0.707 + 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 2 | -1 | 1 | 1 | 0.707 - 0.707i | 1 | -0.707 - 0.707i |
| 3 | 1 | 1 | -1 | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| 4 | -1 | -1 | 1 | -0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 5 | -1 | 1 | -1 | -0.707 - 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| 6 | 1 | -1 | 1 | 0.707 + 0.707i | -0.707 - 0.707i | -1 |
| 7 | -1 | -1 | 1 | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 8 | -1 | 1 | -1 | -0.707 - 0.707i | -0.707 + 0.707i | -0.707 - 0.707i |
| 9 | 1 | 1 | 1 | 0.707 + 0.707i | 0.707 + 0.707i | 0.707 - 0.707i |
| 10 | -1 | -1 | -1 | -1 | -0.707 - 0.707i | 0.707 + 0.707i |
| 11 | -1 | 1 | -1 | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 12 | -1 | 1 | 1 | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 + 0.707i |
| 13 | 1 | -1 | -1 | 0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 14 | -1 | 1 | 1 | 0.707 + 0.707i | 1 | 0.707 - 0.707i |
| 15 | -0 - 1i | 1 | 0 + 1i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 - 0.707i |
| 16 | -1 + 0i | 0 + 1i | 1 - 0i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| 17 | 0 + 1i | 1 - 0i | -0 - 1i | 0.707 - 0.707i | -0.707 + 0.707i | 0.707 + 0.707i |
| 18 | -1 + 0i | -0 - 1i | 1 | -0.707 + 0.707i | -0.707 - 0.707i | 1 |
| 19 | 0 + 1i | -1 + 0i | -1 + 0i | 0.707 - 0.707i | 0.707 + 0.707i | 0.707 + 0.707i |
| 20 | 1 - 0i | -0 - 1i | 0 + 1i | -0.707 - 0.707i | -0.707 - 0.707i | 0.707 - 0.707i |
| 21 | 0 + 1i | -1 + 0i | 1 - 0i | -0.707 - 0.707i | -0.707 - 0.707i | -0.707 - 0.707i |
| 22 | -1 | -0 - 1i | -0 - 1i | 1 | 0.707 + 0.707i | 0.707 + 0.707i |
| 23 | 1 - 0i | -1 + 0i | -1 + 0i | 0.707 + 0.707i | 0.707 - 0.707i | 0.707 - 0.707i |
| 24 | -0 - 1i | 0 + 1i | -0 - 1i | 0.707 - 0.707i | 0.707 - 0.707i | -0.707 - 0.707i |
| 25 | 1 - 0i | -1 + 0i | -1 + 0i | 0.707 + 0.707i | 0.707 + 0.707i | -0.707 + 0.707i |
| 26 | -0 - 1i | 0 + 1i | 0 + 1i | -0.707 + 0.707i | 0.707 - 0.707i | 1 |
| 27 | 0 | 0 | 0 | 0 | 0 | 0 |
| 28 | 0 | 0 | 0 | 0 | 0 | 0 |
| 29 | 0 | 0 | 0 | 0 | 0 | 0 |
| 30 | 0 | 0 | 0 | 0 | 0 | 0 |
| 31 | 0 | 0 | 0 | 0 | 0 | 0 |

K.3.2 Time domain OFDM Header and Payload

The data from table K.10 is converted by an IFFT of size 64. Most IFFTs require a reordering of the data. Typically, the order of the frequencies within each symbol should be as follows:

0, 1, 2, … 31, -32, -31, …, -1

After the IFFT, each symbol is extended by a cyclic prefix of 16 samples. Each OFDM symbol then has a size of 80 samples (64 +16).

The resulting data in the time domain has 1680 samples (21*80).