- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [14 March, 2011] |  |
| Source | [Sang-Kyu Lim, Il Soon Jang, Dae Ho Kim, You Jin Kim, Tae-Gyu Kang] [ETRI, 218 Gajeongno, Yuseong-Gu, Daejeon, Korea] | Voice: [ +82-42-860-1573] Fax: [+82-42-860-5218] E-mail: [sklim@etri.re.kr] |
| Re: | [Response to the SB 1st recirculation for the IEEE 802.15.7 standard] |  |
| Abstract | [This document describes the modified text in Annex F for the comment resolution of the SB 1st recirculation.] |  |
| Purpose | [Description of what the authors wants P802.15 to do with the contents in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Blue: text deleted from D6

Red: modified or inserted text

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

2nd paragraph and Table F.1 on page 281

Table F.1 describes the receiver input powers, calculated in Watts, from the assumption of 1 lumen on each of the 7 color bands (given in Table 75). The assumption that the lights have only their center wavelength monochromatic component (shown as example wavelength in Table F.1) on each color band is also used for simple calculations. V() is the human eye sensitivity function which indicates CIE sensitivity curves [B5].

- Table F.1—Calculated Color Channel Difference Table at Receiver Input

| Wavelength Band (nm) |  | Spectral Width (nm) | Center Wavelength (nm) | V() at C.W. | Receiver input power (Watts) @ lm |
| --- | --- | --- | --- | --- | --- |
| 380 | 450 | 70 | 415 | 0.0118 | 0.1241 |
| 450 | 510 | 60 | 480 | 0.1390 | 0.0105 |
| 510 | 560 | 50 | 535 | 0.9151 | 0.0016 |
| 560 | 600 | 40 | 580 | 0.8700 | 0.0017 |
| 600 | 650 | 50 | 625 | 0.3205 | 0.0046 |
| 650 | 710 | 60 | 680 | 0.0170 | 0.0861 |
| 710 | 780 | 70 | 745 | 0.0002 | 7.3206 |

- Table F.1—Calculated color channel power at receiver

| Wavelength band (nm) |  | Spectral width (nm) | Example wavelength (nm) | V() at example wavelength | Receiver input power (Watt) @ lm |
| --- | --- | --- | --- | --- | --- |
| 380 | 478 | 98 | 430 | 0.0273 | 0.0536 |
| 478 | 540 | 62 | 510 | 0.5030 | 0.0029 |
| 540 | 588 | 48 | 565 | 0.9788 | 0.0015 |
| 588 | 633 | 45 | 610 | 0.5030 | 0.0029 |
| 633 | 679 | 46 | 655 | 0.0817 | 0.0179 |
| 679 | 726 | 47 | 700 | 0.0041 | 0.3571 |
| 726 | 780 | 54 | 750 | 0.0001 | 14.6413 |

2nd paragraph, 3rd paragraph, Figure F.4 and Table F.2 on page 282

Table F.2 shows the photo-detector output current obtained from both the wavelength dependence of photo sensitivity shown in Figure F.4 and the conversion relations between the radiometric and photometric units described in Table F.1. The photo-detector output currents in Table F.2 were calculated only at 480 430nm, 535 510nm, and 625 655nm among the center example wavelengths in 7 color bands, as shown in Table F.1, according to VLC band plan for convenience.

Table F.2 indicates that a VLC receiver with Si photo-detector performs differently on multiple color channels even though the radiometric received powers are equal on each color channel. Therefore, two main factors, the unit conversion and the photo sensitivity of a photo-detector depending on wavelength, need to be sufficiently considered in order that the performance of a VLC receiver can be maintained uniformly on multiple color channels.

Figure F.4—Typical Si photo-detector wavelength sensitivity

Figure F.4—Typical Si photo-detector wavelength sensitivity

- Table F.2—Photo-detector current from Figure F.4 with conditions of Table F.1

| Center Wavelength (nm) | Receiver Input Power (Watt) @ lm | Photo Sensitivity (A/W) | Photo-detector Output Current (mA) @ 1 lm |
| --- | --- | --- | --- |
| 480 | 0.0105 | 0.27 | 2.84 |
| 535 | 0.0016 | 0.32 | 0.51 |
| 625 | 0.0046 | 0.43 | 1.98 |

- Table F.2—Photo-detector current from Figure F.4 with conditions of Table F.1

| Center wavelength (nm) | Receiver input power (Watts) @ lm | Photo sensitivity (A/W) | Photo-detector output current (mA) @ lm |
| --- | --- | --- | --- |
| 430 | 0.0536 | 0.21 | 11.26 |
| 510 | 0.0029 | 0.30 | 0.87 |
| 655 | 0.0179 | 0.46 | 8.23 |