IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.7 Task Group Visible-Light Communication (TG-VLC) |  |
| --- | --- | --- |
| Title | Text for resolution of CID 66 in the sponsor ballot 2 |  |
| Date Submitted | 20th January 2011 |  |
| Source | Joachim W. Walewski | Voice: +49-89-636-45850 Fax: E-mail: joachim.walewski (curly a) siemens.com |
| Re: |  |  |
| Abstract | I provide text for the draft 7 of IEEE 802.15.7. This text is intended for the resolution of CID 66 in sponsor ballot 2. |  |
| Purpose | Contribution of standard-text material for the preparation of IEEE 802.15.7 D7 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Document # | 15-11-0256-00-0007 |  |

Pink: Editorial instructions

Crossed out: deleted

Blue: new text

Orange: Labels (hot links) in framemaker

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Replace the text in D6, 5.1.1.1.3 (“Visibility support during channel access) with the below text.

The visibility Visibility slots can be used during in contention and uplink slots in star-topology mode, and in unused slots in all modes. Some of the purposes of so doing are to maintain visibility, reduce flicker, and to keep the transmitter always "ON" for the infrastructure. This is shown illustrated in Figure 15. Visibility support is a very important and distinguishing feature for VLC. Thus, although half-duplexing can be done for data communication, idle One may need to transmit idle patterns may need to be transmitted during receive and idle modes. This can be done by simultaneous reception of data and the transmission of visibility or idle patterns. using a “visibility-duplex” mode of operation that maintains visibility. This feature is made possible due to by the spatial separation of the light source and the receiving circuitry receiver. As shown in Figure 15, idle patterns are sent during contention, uplink slots and unused downlink slots by the infrastructure to maintain visibility. Idle patterns are can also be sent by a mobile device in during unused slots by the mobile device in order to help with pointing and alignment for optimal data transfer.

If the continuous visibility bit is set to one in the capabilities field shown in Table 15, then infrastructure devices shall provide continuous visibility.

In Section 3 on Page 2, delete the entire entry for “visibility-duplex”