IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | TG6 Coexistence Assurance Document (HBC PHY) |
| Date Submitted | March 2011 |
| Source | Seung-Hoon Park(Samsung Electronics) and Jung-Hwan Hwang (ETRI) |
| Re: |  |
| Abstract | Analysis on coexistence of 802.15.6 HBC PHY with other 802 systems within the same frequency bands |
| Purpose | To address the coexistence capability of 802.15.6 HBC PHY |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Scope	1

2	References	2

3	Acronyms and abbreviations	3

4	Overview of HBC PHY	4

4.1	Operating Frequency Bands	4

4.2	Modulation Parameters	4

4.3	Coexistence Mechanisms	4

5	Other IEEE 802 Standards Occupying Same Frequency Band as IEEE 802.15.6	5

6	Discussions and Conclusion	6

- Scope

The IEEE 802.15 Task Group 6 defines a Medium Access Control (MAC) layer and several supporting PHY layers to enable Body Area Networks (BAN) used in, on, or around a body.

IEEE 802.15.6 specifies a total of three PHYs, namely Narrow Band (NB), Ultra Wide Band (UWB) and Human Body Communication (HBC). All three PHYs are specified to address different system demands and target applications. Each PHY is specified to operate in one of a unique set of frequency bands. Thus, mutual interference between BAN PHYs is avoided.

This document addresses the coexistence of IEEE 802.15.6 HBC PHY systems with other IEEE 802 standards operating in the same frequency bands – specifically in terms of interference caused by IEEE 802.15.6 to these existing systems.

- References

TBD

- Acronyms and abbreviations

FSDT                 Frequency Selective Digital Transmission

- Overview of HBC PHY

- Operating Frequency Bands

The non-license frequency bands that can be used for the 802.15.6 HBC PHY are:

- 18.375-23.625MHz (USA, Japan, Korea)

- 28-36MHz (USA, Japan, Korea)

- Modulation Parameters

The 802.15.6 HBC PHY has modulation parameters as shown in Table 1.

| Frequency Band | Symbol Rate (ksps) | Channel bandwidth (MHz) | Modulation |
| --- | --- | --- | --- |
| 18.375-23.625MHz | 41/82/164/328 | 5.25 | FSDT |
| 28-36MHz | 62.5/125/250/500 | 8 | FSDT |

Table 1

- Coexistence Mechanisms

The coexistence mechanisms for the HBC PHY specified in the IEEE 802.15.6 standard are:

- Low duty cycle

- Limited transmission power

- Optional coexistence mechanisms at MAC layer

- Other IEEE 802 Standards Occupying Same Frequency Band as IEEE 802.15.6

There are no other IEEE 802 standards that occupy the same frequency bands as the IEEE 802.15.6 HBC PHY..

- Discussions and Conclusion

This document has presented an analysis of the coexistence between the IEEE 802.15.6 HBC PHY and other IEEE 802 systems.