IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.7 Task Group Visible-Light Communication (TG-VLC) |  |
| --- | --- | --- |
| Title | Text for resolution of CID 309,310 in the sponsor ballot 2 |  |
| Date Submitted | 17th March 2011 |  |
| Source | Jaeseung Son Joachim W. Walewski | Voice: +49-89-636-45850 Fax: E-mail: js1007.son@samsung.com joachim.walewski@siemens.com |
| Re: |  |  |
| Abstract | I provide text for the draft 7 of IEEE 802.15.7. This text is intended for the resolution of CID 309,310 in sponsor ballot 2. |  |
| Purpose | Contribution of standard-text material for the preparation of IEEE 802.15.7 D7 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |
| Document # | 15-11-0256-00-0007 |  |

Pink: Editorial instructions

Crossed out: deleted

Blue: new text

Orange: Labels (hot links) in framemaker

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Replace 5.3.9 (“Blinking notification command”) in D6 with the below contributioin.

- Blinking notification command

| octets: 7 | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier (see Error: Reference source not found) | Blinking frequency |
| - Blinking Notification Command |  |  |

The blinking notification command is sent by a coordinator when the device is not longer responding. Reasons for this might be a misalignment between the device TX and the coordinator RX (limited FOV of receiver, low device TX power, mobility of the device, etc.). In such cases, the device can change the visibility indication from continuous visibility for point-and-shoot to blinking indication. The device can then change from point-and-shoot mode to blinking mode in order to indicate to the user that the uplink to the coordinator is disconnected. This indication can be applied to both P2MP and P2P modes of operation.

The blinking notification bit shall be set when the MAC PIB attribute, macUseBlinkingNotification and macBlinkingNotificationfFrequency, as defined in Table 59 indicates the blinking notification usage.

To support the blinking notification, the period shall be chosen from the phyBlinkingnotificationFrequency PHY PIB attribute as shown in Table 99, using the MLME-SET.request and PLME-SET.request primitives

This feature can help to align the link and is only intended for mobile devices.

5.3.9.1 Blinking frequency

| bits: 0–3 | 4–7 |  |
| --- | --- | --- |
| Frequency | Reserved |  |
| - Blinking frequency field format |  |  |

The frequency subfield shall contain the frequency of blinking.

Add the text in table 59 in D6, 6.4.2 (“MAC PIB attributes”) on Page 178, Line 36 with the below text.

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macUseBlinkingNotification |  | Boolean | TRUE or FALSE | Shall be set to 1 when blinking notification is to be performed | 0 |
| macBlinkingNotificationFrequency |  | Integer | 0-10 | The frequency of blinking notification 0: 0.25Hz 1: 0.5Hz 2: 0.75Hz 3: 1Hz 4: 1.25Hz 5: 1.5Hz 6: 1.75Hz 7: 2Hz 8: 2.25Hz 9: 2.5Hz 10: 2.75Hz | 0 |

Add the text in table 99 in D6, 9.5.2 (“PHY PIB attributes”) on Page 240, Line 54 with the below text.

| Attribute | Identifier | Type | Range | Description |
| --- | --- | --- | --- | --- |
| phyBlinkingNotificationfFrequency |  | Integer | 0-10 | The frequency of blinking notification 0: 0.25Hz 1: 0.5Hz 2: 0.75Hz 3: 1Hz 4: 1.25Hz 5: 1.5Hz 6: 1.75Hz 7: 2Hz 8: 2.25Hz 9: 2.5Hz 10: 2.75Hz |