IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | TG6 Coexistence Assurance Document |
| Date Submitted | March 2011 |
| Source | Anuj Batra (TI), Marco Hernandez (NICT) Mark Dawkins (Toumaz), Srinath Hosur (TI) Jung-Hwan Hwang (ETRI) Daniel Lewis (NICTA) Seung-Hoon Park(Samsung Electronics) Kenichi Takizawa (NICT), |
| Re: | 802.15.6 d03 |
| Abstract | Analysis on coexistence of 802.15.6 with other 802 systems within the same frequency bands |
| Purpose | To address the coexistence capability of 802.15.6 NB PHY, UWB PHY & HBC PHY to satisfy requirements of the IEEE 802.19 Task Group and IEEE 802 Executive Committee to determine if a proposed IEEE 802 standard has made a reasonable effort to be able to coexist with devices compliant to other IEEE 802 standards in their operating band. |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Scope	1

2	References	2

3	Acronyms and abbreviations	3

4	802.15.6 Narrow Band PHY Overview	5

4.1	Operating Frequency Bands	5

4.2	Modulation Parameters	5

4.3	Coexistence Mechanisms	5

5	Other IEEE 802 Standards Occupying Same Frequency Bands as IEEE 802.15.6 Narrow Band PHY	6

5.1	2400-2483.5 MHz Band (Worldwide)	6

5.2	863-870MHz (Europe), 902-928 MHz (United States) and 950-958MHz (Japan) ISM Bands	6

6	Coexistence Scenarios and Analysis for Narrowband PHY	7

6.1	Methodology	7

6.1.1	Path Loss Model	7

6.1.2	PER for IEEE 802.15.6	8

6.2	Coexistence with IEEE 802.11	1

6.2.1	Standards characterized for coexistence	1

6.2.2	Assumptions in coexistence performance	1

6.2.3	Interference Modeling	2

6.2.4	Coexistence Simulation Results	3

6.3	Coexistence with IEEE 802.15.1	1

6.3.1	Standards characterized for coexistence	1

6.3.2	Assumptions in coexistence performance	1

6.3.3	Interference modeling	3

6.3.4	Coexistence simulation results	3

6.4	Coexistence with IEEE 802.15.4	4

6.4.1	Standards characterized for coexistence	4

6.4.2	Assumptions in coexistence performance	6

6.4.3	Coexistence performance	10

7	802.15.6 Ultra Wide Band PHY Overview	16

7.1	Operating Frequency Bands	16

7.2	Modulation Parameters	16

7.3	Coexistence Mechanisms	16

8	Other IEEE 802 Standards Occupying Same Frequency Bands as IEEE 802.15.6 Ultra Wide Band PHY	17

9	Coexistence Scenarios and Analysis for Ultra Wide Band PHY	18

9.1	Coexistence mechanisms for Ultra Wide Band	18

9.1.1	Low transmit power	18

9.1.2	Low duty cycle	18

9.1.3	Modulation	19

9.1.4	Time hopping	19

9.1.5	Clear channel assessment	20

9.1.6	Hybrid type II ARQ	20

9.2	IEEE Std 802.11-2007 (5 GHz) coexistence performance	20

9.3	IEEE Std 802.15.4a-2007 (UWB) coexistence performance	20

10	802.15.6 Human Body Communication PHY Overview	21

10.1	Operating Frequency Bands	21

10.2	Modulation Parameters	21

10.3	Coexistence Mechanisms	21

11	Other IEEE 802 Standards Occupying Same Frequency Band as IEEE 802.15.6	22

12	Discussions and Conclusion	23

# Scope

The IEEE 802.19 TAG has mandated that new wireless standards developed under IEEE 802 be accompanied by a Coexistence Assurance document. In Error: Reference source not found, guidelines are provided for how coexistence can be quantified based on predicted packet error rates among IEEE 802 wireless devices. A detailed discussion of coexistence and coexistence methods can be found in IEEE Std 802.15.2-2003.

Hence, this coexistence assurance document is provided by the IEEE 802.15.6 Task Group to satisfy the requirements of the IEEE 802.19 Task Group and IEEE 802 Executive Committee.

The IEEE 802.15 Task Group 6 defines a Medium Access Control (MAC) layer and several supporting PHY layers to enable Body Area Networks (BAN) used in, on, or around a body.

IEEE 802.15.6 specifies a total of three PHYs, namely Narrow Band (NB), Ultra Wide Band (UWB) and Human Body Communication (HBC). All three PHYs are specified to address different system demands and target applications.

Each PHY is specified to operate in one of a unique set of frequency bands. Thus, mutual interference between BAN PHYs is avoided.

This document addresses the coexistence of each of the three IEEE 802.15.6 PHY systems with other IEEE 802 standards operating in the same frequency bands.  Addressing interference caused by IEEE 802.15.6 to these existing systems, and interference from these existing standards with IEEE 802.15.6.

# References

[1] S. Shellhammer, “Writing a coexistence assurance document,” IEEE 802.19-09/0001r0, 2009.

[2] S. Shellhammer, “Estimating packet error rate caused by interference –A coexistence assurance methodology-,” IEEE 802.19-05/0029r0, 2005.

[3] IEEE Std 802.15.4TM-2006.

[4] IEEE Std 802.15.4dTM-2009.

[5] IEEE Std 802.15.2TM-2003.

[6] J .G. Proakis, Digital Communications 4th edition, 2000.

[7] C.-S. Sum, “TG4g coexistence assurance document,” IEEE 802.15-10-0668-03-004g, 2010

[8] IEEE Std 802.11TM-2007.

[9] IEEE Std 802.15.1TM-2005.

# Acronyms and abbreviations

AWN 			Affected Wireless Network

BAN			Body Area Network

BCH			Bose, Ray-Chaudhuri, Hocquenghem Code

BER			Bit Error Rate

BPSK			Binary Phase Shift Keying

CCA			Clear Channel Assessment

D-MPSK		Differential M-ary Phase Shift Keying

DAA			Detect And Avoid

DSSS-CCK		Direct Sequence Spread Spectrum – Complimentary Code Keying

EIRP			Equivalent Isotropically Radiated Power

FHSS 			Frequency Hopping Spread Spectrum

FM-UWB		Frequency Modulation Ultra-Wide Band.

FSDT			Frequency Selective Digital Transmission

GMSK			Gaussian Minimum Shift Keying

HARQ			Hybrid Automatic Repeat Request

IR-UWB		Impulse Radio Ultra-Wide Band

IWN 			Interfering Wireless Network

LBT			Listen Before Talk

LSB			Least Significant Bit

LFSR			Linear Feedback Shift Register

MBANS			Medical Body Area Network Service

MICS			Medical Implant Communication Service

MIMO			Multiple Input, Multiple Output

MPDU			MAC Protocol Data Unit

MSB			Most Significant Bit

OFDM			Orthogonal Frequency Division Multiplexing

O-QPSK		Offset Quadrature Phase Shift Keying

PER			Packet Error Rate

PHR			PHY Header

PLCP			Physical Layer Convergence Procedure

PPDU			PHY Protocol Data Unit

PSDU			PHY Service Data Unit

PSSS-ASK		Parallel Sequence Spread Spectrum – Amplitude Shift Keying

QAM			Quadrature Amplitude Modulation

SHR			Synchronization Header

SIR			Signal to Interference Ratio

SRRC			Square Root Raised Cosine

WMTS			Wireless Medical Telemetry Service

# 802.15.6 Narrow Band PHY Overview

## Operating Frequency Bands

The allocated frequency bands for the 802.15.6 NB PHY are:

- 402-405MHz (MICS)

- 420-450MHz (Japan WMTS)

- 863-870MHz (EU ISM)

- 902-928MHz (US ISM)

- 950-958MHz (Japan ISM)

- 2360-2400MHz (MBANS)

- 2400-2483.5MHz (Worldwide ISM)

## Modulation Parameters

The 802.15.6 NB PHY has modulation parameters in the different frequency bands as shown in Table 1.

| Frequency Band | Symbol Rate (ksps) | Channel bandwidth (kHz) | Modulation |
| --- | --- | --- | --- |
| 402-405MHz | 187.5 | 300 | D-MPSK |
| 420-450MHz | 187.5 | 500 | GMSK |
| 863-870MHz 902-958MHz 950-958MHz | 250 | 400 | D-MPSK |
| 2360-2400MHz 2400-2483.5MHz | 600 | 1000 | D-MPSK |

Table 1

## Coexistence Mechanisms

The coexistence mechanisms for the NB PHY specified in the IEEE 802.15.6 standard are:

- Clear Channel Assessment (CCA)

  - Energy detection and/or carrier sense

  - LBT and other procedures as defined by local regulations

- Channel switching

  - >= 10 channels in each frequency band

- Low Duty cycle

- Limited transmission power

  - Narrow bandwidth via SRRC or Gaussian pulse shaping

- Optional slow channel hopping at MAC layer

# Other IEEE 802 Standards Occupying Same Frequency Bands as IEEE 802.15.6 Narrow Band PHY

This clause lists the IEEE 802 standards that occupy the same frequency bands as the IEEE 802.15.6 NB PHY.  The common frequency bands are 2400-2483.5 MHz, 902-928 MHz, 863-870 MHz, and 950-958 MHz. There are no other IEEE 802 standards that operate in the other frequency bands supported by the IEEE 802.15.6 NB PHY.

## 2400-2483.5 MHz Band (Worldwide)

Table 2 shows the list of other IEEE 802 standards that share the 2400-2483.5 MHz band with the IEEE 802.15.6 NB PHY.

| Standard | PHY specification |
| --- | --- |
| 802.11b | DSSS – CCK |
| 802.11g | OFDM – QAM |
| 802.11n | MIMO OFDM – QAM |
| 802.15.1 | FHSS – GFSK |
| 802.15.4 | DSSS – O-QPSK |

Table 2

## 863-870MHz (Europe), 902-928 MHz (United States) and 950-958MHz (Japan) ISM Bands

Table 3 shows the list of other IEEE 802 standards that share the 863-870 MHz, 902-928 MHz or 950-958 MHz bands with the IEEE 802.15.6 NB PHY.

| Standard | PHY specification |
| --- | --- |
| 802.15.4 | DSSS – BPSK DSSS – O-QPSK PSSS – ASK |

Table 3

# Coexistence Scenarios and Analysis for Narrowband PHY

This clause presents the analysis of the interference caused by IEEE 802.15.6 NB PHY systems to each of the IEEE 802 standards identified in section 5, and vice versa Note that the interferer parameters chosen in the following sub-clauses are chosen for the purpose of coexistence analysis in this CA document and are a sub-set of the available modes.

## Methodology

In order to quantify the coexistence performance, the techniques described in Shellhammer [1] and [2] have been adopted. The coexistence assurance methodology predicts the PER of an affected wireless network (AWN), in the presence of an interfering wireless network (IWN). In its simplest form, the methodology assumes an AWN and an IWN, each composed of a single transmitter and a receiver. The methodology takes as input a path loss model, a BER function for the AWN, and in some case predicted temporal models for packets generated by the AWN and for “pulses,” i.e., packets generated by the IWN. Based on these inputs, the methodology predicts the PER of the AWN as a function of the physical separation between the IWN transmitter and the AWN receiver.

The appeal of the coexistence assurance methodology is that multiple networking standards can be characterized and compared with just a few parameters, notably

- Bandwidth of AWN and IWN devices

- Path loss model for the networks

- PER as a function of SIR of AWN devices

### Path Loss Model

The coexistence methodology uses a variant of the path loss model described in IEEE 802.15.2-2003 [5], which stipulates a two-segment function with a path loss exponent of 2.0 for the first 8 meters and then a path loss exponent of 3.3 thereafter. The path loss model is based on the 2400 MHz. To adapt the model to the sub-GHz bands, the formula is modified as shown in Equation (1).

pld=pl(1)+10γ1log10d d≤8mpl(8)+10γ8log10d8 d>8m                 (1)

where pl(d) is the path loss at d meters, 1 and 8 is the path loss exponent at 1 and 8 meters, respectively. The pl(1) is computed as shown in Equation (2).

pl1=10γ1log104πfc                                 (2)

where f is the frequency and c is the speed of light. Plots of the path loss for the frequency bands are shown in Figure 1. The path loss model shown in Equation (1) is applied to compute the path loss between the AWN and IWN.

Figure 1 - Path loss curves for all frequency bands

### PER for IEEE 802.15.6

To calculate the PER for the IEEE 802.15.6 NB PHY, first of all the error probability of one BCH codeword is derived. According to Proakis [6], the codeword error probability of a perfect code for hard-decision decoding is given by a simple closed form, i.e.

Pw=1−m=0tNmPbm1−PbN−m                    (3)

where N is the number of symbols in a codeword, Pb is the bit error probability, and t is the number of error symbols possible to be corrected. Although the (63, 51)-BCH code itself is not a perfect code, as shown in Figure 2, the error probability shows good agreement with BER obtained by simulations. This closed form given in Equation (3) is used to compute the PER. The BER for the 15.6 PHY, /2-shift DBPSK, in the 2400-2483.5 MHz band is given by

Pb=0.5exp−2.42SIR                           (4)

For the 860-868 MHz, 902-928 MHz, and 950 -958 MHz bands, BER are computed by

Pb=0.5exp−1.21SIR                           (5)

These equations are based on the bit error probability of BPSK in differentially coherent detection [6]. The difference of the coefficients multiplied to SIR comes from the difference of the spreading factors. Then, PER for 15.6 NB-PHY is computed as shown in the Equation (6).

PER=1−1−PwNc                              (6)

where Nc is the number of BCH codewords in one 15.6 payload. For example, if the number of payload bits is 255*8 bits, the number of codewords Nc becomes ceil(255*8/51) = 40. The PER curve for the 15.6 NB-PHY in the 2400-2483.5 MHz band is displayed in Figure 2.

Figure 2 - PER as a function of SINR for 15.6 PHY (2400-2483.5 MHz)

## Coexistence with IEEE 802.11

This section considers issues regarding coexistence between IEEE 802.15.6 devices and IEEE 802.11 devices.

### Standards characterized for coexistence

The draft standard for IEEE 802.15.6 is specified for operation in the 402-405 MHz, 420-450 MHz, 863-868 MHz, 902-928 MHz, 950-958 MHz, 2360-2400 MHz, and 2400-2483.5 MHz bands.

IEEE 802.11 standards characterized in this document for coexistence with IEEE 802.15.6 are as follows:

- IEEE Std 802.11TM-2007 [8] (2400-2483.5 MHz, 5 GHz)

In the 2400-2483.5 MHz band, there are DSSS-CCK, OFDM-QAM and MIMO-OFDM-QAM PHYs. Their detailed specifications are summarized in Table 4. The details of the 15.6 PHY mode used here are given in Table 5. These PHYs will serve as both AWNs and IWNs.

| Modulation | Bit rate | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- |
| 802.11b: DSSS-CCK | 11 Mbps | 22 | 14 | -76 |
| 802.11g: OFDM-BPSK, R = 1/2 | 6 Mbps | 20 | 14 | -88 |
| 802.11n: OFDM-QPSK, R = 3/4 | 18 Mbps | 20 | 14 | -83 |

Table 4 - Specifications of 802.11 standard in 2400-2483.5MHz band

| Modulation | Bit rate | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- |
| D-BPSK, R = 51/63 | 121.4 kbps | 1 | -10 | -92 |

Table 5 - Specifications of 802.15.6 NB-PHY ((63,51)-BCH coded /2-shift DBPSK)

### Assumptions in coexistence performance

The assumptions made across all standards characterized for coexistence are described in the following sub-clauses.

#### Path loss model

The path loss model used in this section is given in section 6.1.1.

#### PER model

The PER for the 802.11 PHY modes can be calculated from the corresponding BER through the following relationship:

PER = 1 – (1-BER)L

where L is the average frame size. For 802.11b, L = 1024 bytes. For 802.11g, L = 1000 bytes. For 802.11n, L = 4096 bytes. The PER curves for 802.11 PHY modes are given in Figure 3.

Figure 3 - PER curves as a function of SINR for 802.11 PHYs

The PER curve for the 802.15.6 PHY mode is given in section 6.1.2.

### Interference Modeling

The effect of the interfering signal on the desired signal is assumed to be averaged to the bandwidth of the victim system. The victim receiver (with receive power Prv ) receives the desired signal from the victim transmitter (with transmit power Ptv), located at a distance dD, while an interferer transmitter (with  transmit power PTI) is located at a distance dU. The ratio between the desired and undesired power present at the victim receiver will be used as the SIR of the victim system.

At the victim receiver the received power from the desired signal is

And the received power from the interfering transmitter is

And the SIR is given as PRv-P’Rv.

### Coexistence Simulation Results

#### 802.15.6 121 kbps Mode as Victim Receiver

Figure 4 shows the relationship between the PER performance of the 802.15.6 victim receiver and the distance between the victim receiver to the interferer transmitter. The separation distance between the 802.15.6 victim transmitter and the 802.15.6 victim receiver is 3 meters. The list of interferers is given in the figure.

Figure 4 - PER vs. Distance between 802.11 Interferer to 802.15.6 Victim receiver

#### 802.11 PHY Modes as Victim Receiver

This sub-clause presents results obtained when setting the 802.11 system as the victim and the 802.15.6 system as the interferer. Figure 5, Figure 6 and Figure 7 show the relationship between the PER performances of the 802.11b/g/n victim receivers corresponding to the distance between the victim receivers and the 802.15.6 interferers. The separation distance between 802.11 victim transmitter and 802.11 victim receiver is 100 meters.

Figure 5 - PER vs. Distance between 802.15.6 Interferer to 802.11b Victim receiver

Figure 6 - PER vs. Distance between 802.15.6 Interferer to 802.11g Victim receiver

Figure 7 - PER vs. Distance between 802.15.6 Interferer to 802.11n Victim receiver

## Coexistence with IEEE 802.15.1

This section considers issues regarding coexistence between IEEE 802.15.6 devices and IEEE 802.15.1 devices.

### Standards characterized for coexistence

The draft standard for IEEE 802.15.6 is specified for operation in the 402-405 MHz, 420-450 MHz, 863-868 MHz, 902-928 MHz, 950-958 MHz, 2360-2400 MHz, and 2400-2483.5 MHz bands.

IEEE 802.15.1 standards characterized in this document for coexistence with IEEE 802.15.6 are as follows:

- IEEE Std 802.15.1TM-2005 [9]

This standard operates in the 2400-2483.5 MHz band using FHSS GFSK. The detailed specifications for this PHY are summarized in Table 6. The details of the 15.6 PHY mode used here are given in Table 7. These PHYs will serve as both AWNs and IWN.

| Modulation | Bit rate | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- |
| FHSS GFSK modulation factor 0.3, BT = 0.5 | 1 Mbps | 1 | 0 | -70 |

Table 6 - Specifications of 802.15.1 standard

| Modulation | Symbol rate | Bit rate | Spreading factor | Receiver Bandwidth (MHz) | Transmit Power (dBm) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| D-BPSK, R = 51/63 | 600 ksps | 121.4 kbps | 4 | 1 | -10 | -92 |

Table 7 - Specifications of 802.15.6 NB-PHY ((63,51)-BCH coded /2-shift DBPSK)

### Assumptions in coexistence performance

The assumptions made in this analysis are described in the following sub-clauses.

#### Path loss model

The path loss model used in this section is given in section 6.1.1.

#### PER model

The PER for IEEE 802.15.1 is calculated from the BER model for the PHY. For the GFSK PHY, the BER is given as a function of SINR as follows:

Pb=Qa,b−12exp−a2+b22I0ab                   (7)

where

a=SIR21−1−ρ2                          (8)

b=SIR21+1−ρ2                          (9)

I0(x) is the modified Bessel function of order zero.  is the correlation coefficient given by

ρ=sin2πβ2πβ                                    (10)

where  is the modulation index. From the computed BER Pb, PER is obtained as shown in the Equation (11).

PER=1−1−PbL                               (11)

where L is the number of bits in a packet. The L is assumed to be 1024 octets for the 802.15.1 PHY. The PER curve as a function of SINR for the 15.1 PHY is plotted in Figure 8.

Figure 8 - BER/PER curves as a function of SINR for 15.1 PHY

The PER for the 802.15.6 PHY mode is given in section 6.1.2.

### Interference modeling

The effect of the interfering signal on the desired signal is assumed to be averaged to the bandwidth of the victim system. The victim receiver (with receive power Prv ) receives the desired signal from the victim transmitter (with transmit power Ptv), located at a distance dD, while an interferer transmitter (with  transmit power PTI) is located at a distance dU. The ratio between the desired and undesired power present at the victim receiver will be used as the SIR of the victim system.

At victim receiver the received power from the desired signal is

And the received power from the interfering transmitter is

And the SIR is given as PRv-P’Rv.

### Coexistence simulation results

Figure 9 shows the relationship between the PER performance of the 802.15.1 victim receiver corresponding to the distance between the victim receiver to the 802.15.6 interferer. The separation distance between 802.15.1 victim transmitter and 802.15.1 victim receiver is 10 meters.

Figure 9 - PER vs. Distance between 802.15.6 Interferer and 802.15.1 Victim receiver

Figure 10 shows the relationship between the PER performance of the 802.15.6 victim receiver corresponding to the distance between the victim receiver to the 802.15.1 interferer. The separation distance between 802.15.6 victim transmitter and 802.15.6 receiver is 3 meters.

Figure 10 - Victim PER vs. Distance between 802.15.1 Interferer and 802.15.6 Victim receiver

## Coexistence with IEEE 802.15.4

This section considers issues regarding coexistence between IEEE 802.15.6 devices and IEEE 802.15.4 devices.

### Standards characterized for coexistence

This standard is specified for operation in the 402-405 MHz, 420-450 MHz, 863-868 MHz, 902-928 MHz, 950-958 MHz, 2360-2400 MHz, and 2400-2483.5 MHz bands.

IEEE 802.15.4 standards characterized in this document for coexistence with IEEE 802.15.6 are as follows:

- IEEE Std 802.15.4TM-2006 [3] (863-868 MHz/902-928 MHz/2400-2483.5 MHz)

- IEEE Std 802.15.4dTM-2009 [4] (950-958 MHz)

In the 863-868 MHz and 902-928 MHz bands, there are BPSK, O-QPSK, and ASK PHYs. In the 950-958 MHz band, there are GFSK and BPSK PHYs. In the 2400-2483.5 MHz band, there is O-QSPK PHY. Their detailed specifications are summarized in Table 8. These PHYs will serve as both AWNs and IWNs. In the 402-405 MHz and 420-450 MHz bands, at the present, any IEEE 802 standards have not been addressed.

(a) 860-868 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4 BPSK | 20 kbps | 20 ksymbol/s | 300 kchip/s |
| 15.4 ASK | 250 kbps | 12.5 ksymbol/s | 400 kchip/s |
| 15.4 O-QPSK | 100 kbps | 25 ksymbol/s | 400 kchip/s |

(b) 902-928 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4 BPSK | 40 kbps | 40 ksymbol/s | 600 kchip/s |
| 15.4 ASK | 250 kbps | 50 ksymbol/s | 1600 kchip/s |
| 15.4 O-QPSK | 250 kbps | 62.5 ksymbol/s | 1000 kchip/s |

(c) 950-958 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4d BPSK | 40 kbps | 40 ksymbol/s | 600 kchip/s |
| 15.4d GFSK | 100 kbps | 100 ksymbol/s | (no spreading) |

(d) 2400-2483.5 MHz

| Modulation | Bit rate | Symbol rate | Chip rate |
| --- | --- | --- | --- |
| 15.4 O-QPSK | 250 kbps | 62.5 ksymbol/s | 2000 kchip/s |

Table 8 - Specifications of 15.4 standards

| Frequency band | Bit rate | Symbol rate | Spreading factor |
| --- | --- | --- | --- |
| 868/915/950 MHz | 101.2 kbps | 250 ksymbol/s | 2 |
| 2450 MHz | 121.4 kbps | 600 ksymbol/s | 4 |

Table 9 - Specifications of 15.6 NB-PHY ((63,51)-BCH coded π/2-shift DBPSK)

### Assumptions in coexistence performance

The assumptions made across all standards characterized for coexistence are described in the following sub-clauses.

#### Bandwidth

The three IEEE 802.15.4 PHYs that operate in the 863-868 MHz band have one channel, approximately 600 kHz wide. The coexistence methodology assumes that any the 863-868 MHz device in an AWN will have the same bandwidth as a device in the IWN. Similarly, in the 902-928 MHz and 2400-2483.5 MHz band, each channel is 2MHz wide. In the 950-958 MHz band, there are 2 different bandwidths are accommodated; one is approximately 600 kHz wide for BPSK, and the other is approximately 200 kHz for GFSK.

IEEE 802.15.6 has several PHYs in each frequency band. The mandatory PHY, which is (63, 51)-BCH coded /2-shift DBPSK, will serve as both AWNs and IWNs. In the 863-868 MHz, 902-928 MHz, 950-958 MHz bands, each channel is 400 KHz wide. In the 2400-2483.5 MHz, each channel is 1 MHz wide.

#### Path loss model

The path loss model is given in section 6.1.1.

#### PER model

PER for each PHY in IEEE 802.15.4 is calculated from BER model for each PHY. The BER is given as a function of SIR as follows:

BPSK PHY [3]

Pb=0.5exp−11.25SIR                                 (12)

O-QPSK PHY [3]

Pb=815116j=2M−1j16jexp10SIR1j−1            (13)

ASK PHY [3]

Pb=0.4146exp−6.0871SIR                            (14)

GFSK PHY [5]

Pb=Qa,b−12exp−a2+b22I0ab                   (15)

where

a=SIR21−1−ρ2                          (!6)

b=SIR21+1−ρ2                          (17)

I0(x) is the modified Bessel function of order zero.  is the correlation coefficient given by

ρ=sin2πβ2πβ                                    (18)

where  is the modulation index. From the computed BER Pb, PER is obtained as shown in the Equation (19).

PER=1−1−PbL                               (19)

where L is the number of bits in a packet. The L is 22 octets in all the PHYs except GFSK PHY. GFSK PHY assumes a 256-octet payload as its typical number. The PER curves as a function of SIR for these 15.4 PHYs are plotted in Figure 11.

Figure 11 - PER as a function of SNR for 15.4 PHYs

The PER for 15.6 is given in section 6.1.2.

#### Computation of SIR

In this coexistence test, the receive signal power at a receiver in an AWN is assumed to be 10 dB higher than the receiver sensitivity SAWN. The physical separation d, between the AWN’s receiver and a transmitter in an IWN, determines the path loss pl(d) as shown in Equation (1). Assuming antenna gains for both devices are 0 dBi, SIR at the AWN’s receiver is simply given by

SIR=(SAWN+10)−(Ptx,IWN−pld)                       (20)

where Ptx,IWN is the transmit signal power from the IWN’s transmitter. The Ptx,IWN is assumed to be 0 dBm for the 15.4 transmitters, and -10 dBm for the 15.6 transmitters. According to [2], if its receiver bandwidth of an AWN’s device, BAWN, is narrower than the IWN’s signal bandwidth BIWN, the SIR is scaled by the ratio between these bandwidths:

SIR=(SAWN+10)−(Ptx,IWN−pld)+BAWNB           (21)

#### Temporal model

Both the 15.4 devices and 15.6 NB-PHY devices are not operated in full-duty since they have own MAC mechanism based on CSMA/CA. By taking into account the duty cycle, a temporal model given in [2] is introduced. In this temporal model, interference packets are uniformly generated in a given duty cycle. The time duration and idle duration is given by the bit rate, length of the payload, and duty ratio. Typical numbers for these parameters are listed in Table 3. The length of typical payload is set to 22 octets for all IEEE 802.15.4 [3] except GFSK in 15.4d having 255 octets [7]. The duty cycle is assumed to be 1 % for 15.4 devices operated in the 860-868 MHz band, and 10 % for devices in the other bands.

Assuming that 15.6 device has 255-octet payload, which is the maximum length of the payload. On the duty ratio, since 15.6 covers wide variety of applications, it is reasonable to assume the duty cycle at 10 % worst case except the 860-868 MHz band, in which the duty cycle is assumed to be 1 %.

(a) 860-868 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 101.2 kbps | 255 octets | 20.24 ms | 1 % |
| 15.4 BPSK | 20 kbps | 22 octets | 8.80 ms | 1 % |
| 15.4 ASK | 250 kbps | 22 octets | 0.70 ms | 1 % |
| 15.4 O-QPSK | 100 kbps | 22 octets | 1.76 ms | 1 % |

(b) 902-928 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 101.2 kbps | 255 octets | 20.24 ms | 10 % |
| 15.4 BPSK | 40 kbps | 22 octets | 4.40 ms | 10 % |
| 15.4 ASK | 250 kbps | 22 octets | 0.70 ms | 10 % |
| 15.4 O-QPSK | 250 kbps | 22 octets | 0.70 ms | 10 % |

(c) 950-958 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 101.2 kbps | 255 octets | 20.24 ms | 10 % |
| 15.4d BPSK | 20 kbps | 22 octets | 8.80 ms | 10 % |
| 15.4d GFSK | 100 kbps | 255 octets | 20.48 ms | 10 % |

(d) 2400-2483.5 MHz

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| 15.6 /2-shift DBPSK | 121.4 kbps | 255 octets | 16.80 ms | 10 % |
| 15.4 O-QPSK | 250 kbps | 22 octets | 0.70 ms | 10 % |

Table 10 - Specifications related to set a temporal model

### Coexistence performance

This clause shows the results of the coexistence assurance methodology for each frequency band.

#### 2400 - 2483.5 MHz band

Figure 12-(a) shows the PER of the 15.4 O-QPSK receiver under interference caused by the 15.6 signal. The results show that the required physical separation between the 15.4 receiver and 15.6 transmitter is 12 meters to maintain PER of 10-2. On the other hand, the PER of the 15.6 receiver is displayed in Figure 12-(b) when interfered by 15.4 O-QOSK signal. The required separation is 40 meters to obtain PER of 10-2.

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 12 - PER for coexistence assurance results for 2400-2483.5 MHz

#### 950 - 958 MHz band

Figure 13-(a) shows the PER of the either 15.4 BPSK receiver or GFSK receiver under interference caused by the 15.6 signal. Simulation results show that the required physical separations for the 15.4 BPSK receiver and 15.4 receiver is 28 meters and 44 meters, respectively, to maintain PER of 10-2. On the other hand, the PER displayed in Figure 13-(b) show that the 15.6 receiver requires separation of 114 meters to obtain PER of 10-2 when interfered by the 15.4 BPSK or GFSK signal

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 13 - PER for coexistence assurance results for 950-958 MHz

#### 902 - 928 MHz band

Figure 14-(a) shows the PER of the 15.4 receivers under interference caused by the 15.6 signal. The results show that the required physical separation between the 15.4 receiver and 15.6 transmitter is 22 meters, 33 meters, and 42 meters to maintain PER of 10-2 in O-QPSK, BPSK, and ASK receivers, respectively. In Figure 14-(b), the PER of the 15.6 receiver is shown when interfered by 15.4 signal. The results demonstrate that the required separation is 90 meters, 104 meters, and 77 meters, respectively, to obtain PER of 10-2.

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 14 - PER for coexistence assurance results for 902-928 MHz

#### 860 - 868 MHz band

Figure 15-(a) illustrates the PER of the 15.4 receivers under interference caused by the 15.6 signal. The results show that the required physical separation between the 15.4 receiver and 15.6 transmitter is 20 meters, 26 meters, and 20 meters for the O-QPSK, BPSK, and ASK receiver, respectively, to maintain PER of 10-2. PER of the 15.6 receiver is displayed in Figure 15-(b) when interfered by 15.4 signal. The required separation to accommodate with one 15.4 transmitter in maintaining PER of 10-2 is 115 meters for the 15.4 signals.

(a) AWN: 15.4, IWN: 15.6

(b) AWN: 15.6, IWN: 15.4

Figure 15 - PER for coexistence assurance results for 860-868 MHz

# 802.15.6 Ultra Wide Band PHY Coexistence Overview

The IEEE 802.15.6 Task Group is developing a new UWB-PHY that operates in the designated UWB frequency bands between 3.1 and 10.6 GHz. In order to assure that such PHY will provide reasonable performance when operating near other wireless devices, the 802.15.6 Task Group has adopted the policies and conventions of the IEEE 802.19 Coexistence Technical Advisory Group (TAG).

## Operating Frequency Bands

The allocated frequency bands for the 802.15.6 UWB PHY are:

## Modulation Parameters

The 802.15.6 UWB PHY has modulation parameters in the different frequency bands as.

## Coexistence Mechanisms

The proposed standard IEEE 802.15.6 provides several mechanisms that enhance coexistence of its UWB PHYs with other wireless devices operating in the same spectrum. These mechanisms include:

- Very low transmit power

- Low duty cycle

- Modulation

- Time hopping

- Hybrid ARQ

- Clear channel assessment (CCA)

- Active and inactive frames periods

- Local regulations that may require detect-and-avoid (DAA) techniques

# Other IEEE 802 Standards Occupying Same Frequency Bands as IEEE 802.15.6 Ultra Wide Band

This clause enumerates IEEE-compliant devices that are characterized in the document and devices that are not characterized for operation in proximity to IEEE 802.15.6 devices.

IEEE Standards characterized for coexistence are as follows:

- IEEE Std 802.11-2007 (5 GHz)

- IEEE Std 802.11y-2008 (3 GHz)

- IEEE Std 802.11n-2009 (5 GHz)

- IEEE Std 802.16-2009 (below 11 GHz: primarily 3 GHz, 5-6 GHz)

- IEEE Std 802.15.4a-2007 (UWB band)

Proposed IEEE standard characterized for coexistence is as follows:

- IEEE 802.15.4f  (high band UWB)

# Coexistence Scenarios and Analysis for Ultra Wide Band PHY

## Coexistence mechanisms for Ultra Wide Band

### Low transmit power

The UWB PHYs operate under strict regulations for unlicensed UWB devices worldwide. The least restrictive regulations for UWB are available under the Federal Communications Commission (FCC) rules, US 47 CFR Part 15, subpart F. Under these rules, the highest allowable limit for UWB emissions are based on an effective isotropically radiated power (EIRP) of –41.3 dBm/MHz. Other future UWB regulations in other regions will be likely at this same level or even lower.

Under these limits, the allowable transmit power for a train of pulses with spectrumG(f), whose power spectral density (PSD) is centered at frequency fc and whose amplitude has been set to 1 for convenience is given by

PEIRP=10 Log100∞G2(f)df10−41.310

If all available spectrum in a 10 dB point bandwidth of 500 MHz were perfectly filled with the maximum allowed signal PSD, the total PEIRPmax= -14.3 dBm. This value represents the maximum possible EIRP limit for a UWB signal under this particular regulation and setting.

The maximum allowable EIRP for a compliant pulse shape is found by computing Equation  and satisfies PEIRP<PEIRPmax assuming any channel in the frequency band plan.

This transmit power level is at or below the limits for unintentional emissions from other electrical or electronic devices. In addition, this power level value is less than the out-of-band emission limits for other unlicensed devices operating in designated bands such as the 2.4 GHz ISM or 5 GHz UNII bands.

Additionally, since this transmits power is spread over at least 500 MHz of bandwidth, the highest power in the operating bandwidth of a typical narrowband 20 MHz victim system is less than –28.29 dBm. These very low power levels emitted into the operating band of any potential victim system with this characteristic will reduce the likelihood that UWB devices might interfere with other narrowband systems.

### Low duty cycle

The IR-UWB specifications of this standard are tailored for applications with low power and low data rates with a constant duty cycle of 3% for all data rates.  This will make IEEE 802.15.6 devices less likely to cause or be subject to interference by other standards.

On the other hand, at the MAC level, the maximum interference level to victim systems can be limited by controlling the duty cycle of packets or frames through active and inactive periods. The traffic can occur only in the active period. Victim systems are free of interference in the inactive period. The control of active and inactive periods is handled by the hub and a given application.

The interference level is restricted by the ratio of active period to the active plus inactive period. The possible packet collision in the active period can be mitigated by

- CSMA-CA mechanism. FM-UWB can implement carrier sense of a narrowband system.

- Slotted Aloha with channel indicator. This channel-dependent Aloha sets transmission probability related with channel’s quality, which can be obtained through listening to a beacon or preamble symbol from the hub by means of ED. The function to map channel quality to transmission probability is defined at application layer.

- Limit the number of node devices through association.

- Traffic shaping like a combination of short packet to large packet.

- —Concept of active and inactive periods

### Modulation

The IR-UWB PHY has a high QoS mode in which differentially encoded BPSK or QPSK combined with differential detection are employed. This strategy is the best compromise between performance and complexity. Performance is better and more robust to interference than on-off modulation, but slightly more complex.  Furthermore, the use of complaint chirp pulses opens the possibility of novel detection strategies that have been proof resilient against interference.

The FM-UWB PHY combines CP-2FSK modulation with wideband FM. The mandatory data rate is 250 kbps, the central frequency of CP-2FSK modulation is 1.5 MHz and bandwidth of 800 kHz, also known as subcarrier. Subsequently, the wideband FM signal has a transmission bandwidth given approximately by the Carlson’s rule:

BWFM≈(2β+1)fm

where βis the modulation index and fmis the largest frequency component of the CP-2FSK signal. Hence, if fm=1.9 MHz, then BWFM≈500 MHz for β=130.5.

The effect of spreading the data signal’s bandwidth of 800 kHz to 500 MHz transmission bandwidth is similar to spread spectrum. This high processing gain of FM-UWB allows resilience against interference. On the other hand, a BAN hub with a FM-UWB radio must implement an IR-UWB radio as well. Thus, the hub has control of both UWB technologies and can enforce low interference between them.

### Time hopping

A dynamic time hopping sequence (TM) is generated by a linear feedback shift register (LFSR). A hub initializes such TM generator according to the Kasami sequence number used to form the synchronization header (SHR). There are 8 possible sequences. Hence, a different TH sequence can be associated for a different BAN. Simulation results show the performance under multiple BANs improves significantly.

### Clear channel assessment

The receiver energy detection (ED) measurement for clear channel assessment (CCA) is an estimate of a (mostly) narrowband signal’s power around its central frequency. It is meaningless when the signal’s power is below the noise floor (ultra wideband signal). No attempt is made to identify or decode signals on the channel.

The FM-UWB PHY can perform carrier sense of a narrowband system after FM demodulation by ED over a certain threshold. Carrier sense cannot be applied for IR-UWB as the signal power level is below the noise floor. However, a hub with FM-UWB radio must implement an IR-UWB radio as well. Consequently, carrier sense by FM-UWB can be use for the IR-UWB radio as well.

The IR-UWB PHY can perform CCA by preamble detection. CCA shall report a busy medium upon detection of a synchronization symbolSi. Otherwise, slotted Aloha is employed.

### Hybrid type II ARQ

The high QoS mode employs a more powerful channel code based on the shortened BCH(126,63) in case of a packet is found in error by CRC-16 error detection mechanism. Thus, the BANs under high QoS mode is more robust to interference.

## IEEE Std 802.11-2007 (5 GHz) coexistence performance

The proposed standard IEEE 802.15.6 provides several mechanisms that enhance coexistence of its UWB PHYs with other wireless devices operating in the same spectrum. These mechanisms include:

- Very low transmit power

- Low duty cycle

- Modulation

- Time hopping

- Hybrid ARQ

- Clear channel assessment (CCA)

- Active and inactive frames periods

- Local regulations that may require detect-and-avoid (DAA) techniques

## IEEE Std 802.15.4a-2007 (UWB) coexistence performance

The proposed standard IEEE 802.15.6 provides several mechanisms that enhance coexistence of its UWB PHYs with other wireless devices operating in the same spectrum. These mechanisms include:

- Very low transmit power

- Low duty cycle

- Modulation

- Time hopping

- Hybrid ARQ

- Clear channel assessment (CCA)

- Active and inactive frames periods

- Local regulations that may require detect-and-avoid (DAA) techniques

# 802.15.6 Human Body Communication PHY Overview

## Operating Frequency Bands

The non-license frequency bands that can be used for the 802.15.6 HBC PHY are:

- 18.375-23.625MHz (USA, Japan, Korea)

- 28-36MHz (USA, Japan, Korea)

## Modulation Parameters

The 802.15.6 HBC PHY has modulation parameters as shown in Table 1.

| Frequency Band | Symbol Rate (ksps) | Channel bandwidth (MHz) | Modulation |
| --- | --- | --- | --- |
| 18.375-23.625MHz | 41/82/164/328 | 5.25 | FSDT |
| 28-36MHz | 62.5/125/250/500 | 8 | FSDT |

Table 10

## Coexistence Mechanisms

The coexistence mechanisms for the HBC PHY specified in the IEEE 802.15.6 standard are:

- Low duty cycle

- Limited transmission power

- Optional coexistence mechanisms at MAC layer

# Other IEEE 802 Standards Occupying Same Frequency Band as IEEE 802.15.6

There are no other IEEE 802 standards that occupy the same frequency bands as the IEEE 802.15.6 HBC PHY.

# Discussions and Conclusion

This document has presented an analysis of the coexistence between the IEEE 802.15.6 NB, UWB & HBC PHY and other IEEE 802 systems.