- IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802 |  |
| Date Submitted | [24 March, 2011] |  |
| Source | Sang-Kyu Lim, Il Soon Jang, Dae Ho Kim, You Jin Kim, Tae-Gyu Kang (ETRI, 218 Gajeongno, Yuseong-Gu, Daejeon, Korea) Richard D Roberts (Intel Corporation) | Voice: [ +82-42-860-1573] Fax: [+82-42-860-5218] E-mail: [sklim@etri.re.kr] |
| Re: | [Response to the SB 1st recirculation for the IEEE 802.15.7 standard] |  |
| Abstract | [This document describes the modified text of 8.5.2.3 and 10.6 for the comment resolution of the SB 1st recirculation.] |  |
| Purpose | [Description of what the authors wants P802.15 to do with the contents in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Black: original text in D6

Black: text deleted from D6

Red: new text

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

8.5.2.3 VPPM-mode dimming

The VPPM-mode dimming is described in 4.4.3.1.5. The VPPM PHY shall have basic dimming level support at 10% duty cycle resolution. To support higher resolution for dimming from 0 to 100% in steps of 0.1%, the VPPM PHY shall use the algorithm as provided in Figure 113. For example, for supporting 25% dimming, the PHY shall alternately send 20% and 30% duty cycle VPPM symbols. It is recommended that the receiver change the matched filtering in step with the change in the transmitter symbol shape to enable optimum detection.

By default, a 50% duty cycle shall be used for VPPM. If dimming is supported using VPPM modulation in PHY I or PHY II, a dimming notification command shall be sent by the MAC to the receiver. Both the TX and RX shall use the algorithm in Figure 113 for VPPM dimming. The transmitter shall honor all dimming requests from the upper layer. It is recommended that the transmitter use the receiver’s capabilities information as provided in 5.3.19.1.1 for VPPM dimming support. This information is obtained during the device discovery process provided in 5.1.2.4.

The VPPM-mode dimming is described in 4.4.3.1.5. VPPM modulation as implemented here (see 10.6) supports a dimming-level resolution of 10%. To support a dimming resolution of 0.1%, as prescribed in 5.3.10, the VPPM PHY shall use the algorithm provided below.

The algorithm relies on the following symbols. VS0, VS1, VS2, … VS10. VS0 corresponds to the light source being turned off (macDim = 0) and VS10 corresponds to the light source fully being turned on (macDim = 1,000). VS1 to VS9 are the VPPM symbols for d = 0.1 to 0.9 (see 10.6).

- Choose the diming level macDim (see Table 59).

- First, determine the type of the corresponding symbols, viz.  k1=macDim100 and k2=macDim100, where . stands for rounding to the next lower integer and  for rounding to the next higher integer.

- Next, calculate the number of how often each symbol is to be sent:   rep2=macDim−100⋅k1 and rep1=100−rep2

- Then, to achieve the desired dimming level macDim :

- Sequentially assign  rep_1 times, and then,

- Assign  rep_2 times, sequentially.

- If the number of VPPM data symbols to be sent is not the multiples of 100, then make add idle-pattern VPPM symbols so that the number of the sent symbols becomes multiples of 100. Also, in this case send repeat the above order number of times and are sent, as shown in Figure X.

Figure X— Sequential proportional cycling between two duty symbols to achieve fractional dimming with 0.1% accuracy in VPPM-mode.

Note that during data transmission time, only VPPM symbols between VS1 and VS9 can carry data information, as shown in Table 103.  This is because VS0 (light full off) and VS10 (light full on) can not carry data information because there are no transitions during these two symbols.  Therefore, when a macDim value less than 100 is required, data information is carried only by VS1 symbols. Similarly, data information is carried only by VS9 symbols when a macDim value greater than 900 is required.  All dimming requests must be honored even if data transmission is not possible. It is recommended that the receiver changes its matched filter in step with the change in the transmitter-symbol shape in order to enable optimum detection.

By default, a 50% duty cycle shall be used for VPPM. If dimming is supported using VPPM modulation in PHY I or PHY II, a dimming notification command shall be sent by the MAC. Both the TX and RX shall use the above for VPPM dimming. The transmitter shall honor all dimming requests from the upper layer. It is recommended that the transmitter uses the receiver’s capability information as provided in 5.3.19.1.1 for VPPM dimming support. This information is obtained during the device discovery process described in 5.1.2.4.

10.6 Data mapping for VPPM

The data mapping for VPPM shall be defined as in Table103. The physical value mapped from the logical data '0' has a transition from 'high' to 'low', and the physical value mapped from the logical data '1' has a transition from 'low' to 'high', as shown in Table103. 'Low' and 'high' values are defined in 8.3.2. The variable d in Table103 is the VPPM duty cycle, and it is assigned by the VPPM-mode dimming mechanism described in 8.5.2.3. It can be varied in steps of 0.1.