IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Resolution of CIDs 2 and 5 |  |
| Date Submitted | April 2011 |  |
| Source | [Clint Chaplin] [<company>] [address] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [Description of document contents.] |  |
| Purpose | [Description of what the author wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

CID 2: I can not find any reason about delelting sentence "The transmitter clock is restricted to the clocks supported by the optical source while the receiver clock is restricted by the photodetector."

Resolution: Agree. Make the following change to the first paragraph of 6.5.1:

Let us assume that Device 1 supports clock rates at the transmitter (C1, C2, … Ct1), where Ct1 is the maximum clock rate supported at the transmitter at of the coordinator. Also, C1 < C2 < ... < Ct1. Within a PHY type, the clock rates are integral multiples of each other to make the clock generation and selection simple at the transmitter (i.e. Ci+1/Ci = m which is an integer). The receiver may support more or less clock rates than the transmitter since the receiver optronics is physically independent of the transmitter clock. The transmitter clock is restricted to the clocks supported by the optical source while the receiver clock is restricted by the photodetector.  Let the clocks supported by the receiver of device 1 be C1, C2, … Cr1, where Cr1 is the maximum clock rate supported at the receiver of device 1. Similarly, let Ct2 and Cr2 be the maximum clock rates supported by the device 2. Support for the lowest clock rate C1 is mandatory at both the transmitter and receiver for all devices i.e. t1, t2, r1, r2  1. For every clock rate, there is an associated set of data rates at the physical layer. This data rate is dependent on the modulation, RLL coding, and FEC used at the physical layer for a given clock rate. Let the data rate be represented by rate{Ci,p}, where Ci is the chosen clock rate and 1  p  N(Ci), where N(Ci) is the number of physical-layer data rates associated with clock rate Ci.

CID 5: I can not find any reason about delelting sentence "communication for performance or throughput"

Resolution: Agree.   Make the following change to the first paragraph of 6.5.1.1:

In Figure 95, a device sends the association request at the lowest clock C1 at a physical layer data rate rate{C1,k}. The data rate index k is typically chosen to be the lowest data rate to guarantee maximum range and reliability for the given clock rate. In this association request, the device also informs the coordinator of the maximum clock rate supported by its transmitter and receiver (Ct2, Cr2). The maximum-clock-rate information is provided by the capabilities IE as shown in Table 15, which shall also be transmitted during this association request. The coordinator receives the association request and compares the received information about the supported clocks at the device and compares it with its supported clocks. In order for it to communicate, it shall select a clock rate C_SEL1 that is equal to or lower than MAX_C_SEL1, which is the minimum of its maximum transmitter clock and the maximum receiver clock supported by device 1. The decision to use clock rates lower than MAX_C_SEL1 and MAX_C_SEL2 at device and coordinator for transmission depends on the performance and throughput needs of the coordinator and devices. The receiver also sends an association grant back to the transmitter at the same lowest clock rate C1 supported by all devices. The devices then exchange the selected clock frequencies for future communication before they switch to the selected clock frequencies. The devices may also decide to change the clock rate anytime in future communication for performance or throughput, as long as it is below MAX_C_SEL1 and MAX_C_SEL2 for transmission at the coordinator and device, respectively.