IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | The process of interleaving for MR-OFDM |  |
| Date Submitted | [27 April 2011] |  |
| Source | [Cheolho SHIN and Sangsung CHOI] [ETRI] | Voice: [ ] Fax: [ ] E-mail: [chshin@etri.re.kr sschoi@etri.re.kr] |
| Re: |  |  |
| Abstract | Working doc of MR-OFDM Comment Resolutions for LB70 |  |
| Purpose | Comment Resolution |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Comment for LB 70

The structure of the interleaver is not clear, the process of interleaving should be defined.

Proposed Change

Insert the corresponding process of interleaving

16.2.3.5 Interleave

The index of the coded bit before the first permutation shall be denoted as k; i shall be the index after the first and before the second permution; and j shall be the index after second  permutation, just prior to modulation mapping. The index i is defined as follows:

i = (Ncbps / Nrow) ⅹk mod(Nrow) + floor(k / Nrow)

where

Ncbps is the number of coded bits before any frequency spreading.

k =0,1,2,...,(Ncbps - 1)

Nrow is 12 when no frequency spreading is used or 12/(spreading factor) when spreading is

used

The process of interleaving for the first permutation is illustrated in the following Figure.

Write

Read

The index j is defined as follows:

j = s ⅹ floor(i / s) + [i + Ncbps - floor((Nrow ⅹ i) / Ncbps)]mod(s)

where

Ncbps is the number of coded bits before any frequency spreading.

k =0,1,2,...,(Ncbps - 1)

Nrow is 12 when no frequency spreading is used or 12/(spreading factor) when spreading is

used