IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | DSME Comment Resolution for LB73 |  |
| Date Submitted | [11 May 2011] |  |
| Source | [Chang-Sub Shin, Wun-Cheol Jeong, Hyo-Yong Kang] [ETRI] [Gajeong dong Youseong gu, Daejeon, Korea] | Voice: [+82-42-860-1668] Fax: [ ] E-mail: [ shincs@etri.re.kr ] |
| Re: |  |  |
| Abstract | DSME comment resolution material for LB73 |  |
| Purpose | To provide materials to resolve DSME comments |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Resolution for CID 62

Resolution for CID 75

1. Delete the following parameters in MLME-DSME-GTS.request primitive. (page 135)

DSMESABSubblockLength

DSMESABSubblockIndex

2. Insert the following parameter in MLME-DSME-GTS.request primitive. (page 135)

DSMESABSpecification

3. Delete the following rows in Table 44q. (page 135)

Table 0a—MLME-DSME-GTS.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DSMESABSubblockLength | Integer | 0x00–0x0ff | The length of the DSME SAB sub-block. |
| DSMESABSubblockIndex | Integer | 0x00–0xfff | The index that indicates the beginning of the DSME SAB sub-block. |

4. Add the following row in Table 44q. (page 135)

Table 0b—MLME-DSME-GTS.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DSMESABSpecificaton | DSME SAB Specification Value | See Error: Reference source not found | If the ManagementType is 0x01, this parameter contains the information of the current DSME-GTS allocation status and slot availability in one hop neighborhood of the requesting device. If the ManagementType is not 0x01, this parameter indicates the DSME-GTSs to deallocate, notify duplicated allocation, reduce, or restart. |

Resolution for CID 82

Resolution for CID 85

Change the end of Error: Reference source not found as follows:

Table 0—MLME-BEACON-NOTIFY.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EBSN | Integer | 0x00-0xff | Beacon sequence number used for enhanced beacon frames |
| sdu | Set of octets | — | The set of octets comprising the beacon payload to be transferred from the MAC layer entity to the next higher layer including Payload IEs if present. |
| beaconType | Integer | 0x00-0x01 | Indicates a beacon (0x00) or enhanced beacon (0x01) was received |

Insert at the end of Table 55 the following rows:

Table 55—Elements of PANDescriptor

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DSMESuperframeSpecification | Bitmap | See 5.2.4.3.4.1. | The DSME superframe specification as specified in the received enhanced frame. This element is present when macDSMEenabled is TRUE. |
| TimeSynchronizationSpecification | Bitmap | See 5.2.4.3.4.2. | The time synchronization specification as specified in the received enhanced beacon frame. This element is present when macDSMEenabled is TRUE. |
| BeaconBitmap | Bitmap | See 5.2.4.3.4.3. | The beacon bitmap as specified in the received enhanced beacon frame. This element is present when macDSMEenabled is TRUE. |
| ChannelHoppingSpecification | Bitmap | See 5.2.4.3.4.4. | The channel hopping specification specified in the received enhanced beacon frame. This element is present when macDSMEenabled is TRUE and the value of Channel Diversity Mode field in the received enhanced beacon frame is 1. |
| GroupACKSpecification | Bitmap | See 5.2.4.3.4.5. | The group ACK specification specified in the received enhanced beacon frame. This element is present when macDSMEenabled is TRUE and the value of GACK flag field in the received enhanced beacon frame is 1. |