IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Template for TG4j MBAN draft amendment |  |
| Date Submitted | [4th November 2011] |  |
| Source | [David Evans] [Philips] [101 Cambridge Science Park, Cambridge, UK] | Voice: [+44 1293 886490] Fax: [ ] E-mail: [david.evans@philips.com] |
| Re: | [] |  |
| Abstract | [An outline template for the TG4j MBAN draft amendment based on the 802.15.4-2011.] |  |
| Purpose | [To provide the starting point for development of the TG4j MBAN draft amendment. This outline template identifies the clauses and subclauses where either edits or new work is to be performed.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

The outline template is in bold Roman text, italic text is used to indicate where the necessary text of the draft for the Amendment is to be developed and the normal text is an example of the text than can be used. Normal text that is underlined indicates changes or additions to the existing 802.15.4-2011 text.

### Front matter

The IEEE Editors will complete this

### Body of draft

The IEEE Editors will complete this

### 1. Overview

### 1.2 Scope

Insert the paragraph from the PAR scope:

This amendment defines a physical layer for IEEE 802.15.4 in the 2360 to 2400 MHz band which complies with Federal Communications Commission (FCC) MBAN rules. This amendment defines modifications to the MAC needed to support this new physical layer.

### 1.3 Purpose

Insert the following text:

─ 	2360-2400 MHz

### 2. Normative references

802.15.4-2011 has very little text here!

Any reference to the FCC MBAN rules, if required, can be in Annex A Bibliography

### 3. Definitions, acronyms and abbreviations

### 3.1 Definitions

### 3.2 Acronyms and abbreviations

Insert the following text:

MBANS	Medical body areas network services

### 4. General description

No text is required for the 15.4j Amendment.

### 5.  MAC protocol

This clause describes what the MAC does

### 5.1 MAC functional description

### 5.2 MAC frame format

### 5.3 MAC command frames

### 6. MAC services

This clause covers communications between the next higher layer and the MAC sub-layer. Any changes to primitives would be in this clause.  It also covers pass-through primitives to the PHY layer where the MAC generally is not impacted.

### 6.1 Overview

### 6.2 MAC management service

### 6.3 MAC data service

### 6.4 MAC constants and PIB attributes

There may be some changes in the list of PHY Information Base (PIB) attributes that the govern MAC operation.

### 7. Security

No text is required for the 15.4j Amendment.

### 8. General PHY requirements

### 8. 1 General requirements and definitionsChange the text in 8.1 as follows:

- O-QPSK PHY: direct sequency spread spectrum (DSSS) PHY employing offset quadrature phaseshift keying (O-QPSK) modulation, operating in the 780 MHz bands, 868 MHz, 915 MHz, 2380 MHz, and 2450 MHz, as defined in clause 10.

### 8.1.1 Operating frequency range

Insert in Table 66 the following text:

| PHY (MHz) | Frequency band (MHz) | Spreading parameters |  | Data parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chip rate (kchip/s) | Modulation | Bit rate (kb/s) | Symbol rate (ksymbol/s) | Symbols |
| 2380 DSSS | 2360-2400 | 2000 | O-QPSK | 250 | 62.5 | 16-ary Orthogonal |

### 8.1.2 Channel assignments

Insert the following text in 8.1.2:

### 8.1.2.6 Channel numbering for 2380 MHz band

To be agreed.

### 9.  PHY services

This clause addresses the primitives and their parameters for the PHY data service and PHY management service. It also contains enumeration descriptions. If there are changes in communication between the MAC sub-layer and PHY, as compared with 802.15.4-2011, this will require new content in this clause.

### 9.3 PHY PIB attributes

Change and add to the tables as required. It is expected that attributes may be added to interface to the PHY for whatever channel access mechanism is used.

### 10. O-QPSK PHY

10.2 Modulation and spreading

10.2.1 Data rate

Change the text in 10.2.1 as follows:

The data rate of the O-QPSK PHY shall be 250 kb/s when operating in the 2450 MHz, 915 MHz, or 780 MHz, or 2380 MHz bands and shall be 100 kb/s when operating in the 868 MHz band.

Support for the 2450 MHz O-QPSK PHY is mandatory when operating in the 2450 MHz band.

## 10.2.4 Symbol-to-chip mapping

Change the text in 10.2.4 as follows:

In the 2450 MHz and 2380 MHz bands, each data symbol shall be mapped into a 32-chip PN sequence as specified in Table 73. The PN sequences are related to each other through cyclic shifts and/or conjugation (i.e., inversion of odd-indexed chip values).

Table 73—Symbol-to-chip mapping for the 2450 MHz and 2380 MHz bands

## 10.2.5 O-QPSK modulation

Change the text in 10.2.5 as follows:

The chip sequences representing each data symbol are modulated onto the carrier using O-QPSK with halfsine pulse shaping. Even-indexed chips are modulated onto the in-phase (I) carrier, and odd-indexed chips are modulated onto the quadrature-phase (Q) carrier. In the 2450 MHz and 2380 MHz bands, each data symbol is represented by a 32-chip sequence, and so the chip rate is 32 times the symbol rate. In the 915 MHz, 868 MHz, and 780 MHz bands, each data symbol is represented by a 16-chip sequence, and so the chip rate is 16 times the symbol rate. To form the offset between I-phase and Q-phase chip modulation, the Q-phase chips shall be delayed by Tc with respect to the I-phase chips, as illustrated in Figure 70, where Tc is the inverse of the chip rate.

## 10.2.6 Pulse shape

Change the text in 10.2.6 as follows:

In the 2450 MHz, 915 MHz, and 868 MHz, and 2380 MHz bands, the half-sine pulse shape is used to represent each baseband chip and is given by:

## 10.2.7 Chip transmission order

Change the text in 10.2.7 as follows:

During each symbol period, the least significant chip, c0, is transmitted first and the most significant chip, either c31, for the 2450 MHz and 2380 MHz bands, or c15, for the 915 MHz, 868 MHz, and 780 MHz bands, is transmitted last.

10.3 O-QPSK PHY RF requirements

10.3.1 Operating frequency range

Change the text in 10.3.1 as follows:

The O-QPSK PHY operates in the following bands:

— 779–787 MHz

— 868.0–868.6 MHz

— 902–928 MHz

— 2360–2400 MHz

— 2400.0–2483.5 MHz

## 10.3.2 Transmit power spectral density (PSD) mask

Change the text in 10.3.2 as follows:

When operating in the 2380 MHz and 2450 MHz bands, the transmitted spectral products shall be less than the limits specified in Table 76. For both relative and absolute limits, average spectral power shall be measured using a 100 kHz resolution bandwidth. For the relative limit, the reference level shall be the highest average spectral power measured within ±1 MHz of the carrier frequency.

Table 76—2380 MHz and 2450 MHz bands O-QPSK transmit PSD limits

## 10.3.3 Symbol rate

Change the text in 10.3.3 as follows:

The O-QPSK PHY symbol rate shall be 25 ksymbol/s when operating in the 868 MHz band and 62.5 ksymbol/s when operating in the 780 MHz, 915 MHz, 2380 MHz, or 2450 MHz bands with an accuracy of ±40 ppm.

## 10.3.5 Receiver interference rejection

Change the text in 10.3.5 as follows:

This subclause applies only to the 780 MHz, 915 MHz, 2380 MHz, and 2450 MHz bands as there is only one channel available in the 868 MHz band.

Table 77—Minimum receiver interference rejection requirements for the 780 MHz, 915 MHz, 2380 MHz, and 2450 MHz bands

Annex A (informative) Bibliography

Add any references as required, for example the 15.4j coexistence assurance document.

Annex D (informative) Protocol implementation conformance statement (PICS) proforma

This seems to be just a few sentences even though there are several sections. This may be minimal if the 15.4 PHY is unchanged.

D.7 PICS proforma tables

D.7.2 Major capabilities of the PHY

D.7.2.1 PHY functions

Add content for the new PHY and assure that the mandatory/optional methodology is retained

D.7.2.2 Radio frequency (RF)

Add content in Table D3 for the new PHY (i.e. MBAN frequency band) and assure that the mandatory/optional methodology is retained

D.7.3 Major capabilities for the MAC sublayer

D.7.3.1 MAC sublayer functions

Probably no edits to table D.5

Annex I (informative) Consideration for MBANS band

This is a new annex, if needed, specifically addressing issues related to the FCC MBANS band.