IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comment Resolution for CID 261 |  |
| Date Submitted | September 2011 |  |
| Source | [Tim Schmidl] [Texas Instruments] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | Comment Resolution for TG4g Sponsor Ballot |  |
| Abstract | This document presents an example of the processing to generate an FSK packet. |  |
| Purpose | Proposed resolution for Comment ID 261 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Proposed resolution to CID 261:  Revised.  Create Annex M as shown below.

Annex M

(informative)

An example of encoding a packet for the MR-FSK PHY

M.1 Introduction

The purpose of this annex is to show an example of encoding a packet for the MR-FSK PHY, as described

in 16.1. In particular, generation of the PPDU bit sequence is described in detail.  The frequency band used in this example is the 915 MHz band, and this example uses Operating Mode #1.

For this example, selected PIB attributes are set as follows:

phyFSKPreambleLength=4

phyMRFSKSFD = 0

phyFSKFECEnabled = TRUE

phyFSKFECInterleaving = TRUE

phyFSKFECScheme=0

phyFSKScramblePSDU=TRUE

In the example mode switch is not used, and the 4-octet FCS is used.

The encoding illustration goes through the following stages:

a) Generating the bit sequence of the SHR

b) Generating the bit sequence of the PHR

c) Generating the bit sequence of the PSDU

d) Concatenating the PHR, PSDU, tail bits and pad bits

e) Encoding the bit sequence of the PSDU with a rate ½ convolutional encoder

f) Interleaving of the code-bit sequence

g) Concatenation to form the PPDU

In this example, all binary sequences of length n are treated as bit strings:

b0 b1 ......................... bn-1

The corresponding entries are processed b0 first to bn-1 last.

M.2 Generation of the SHR

The bit sequence of the SHR, consisting of four preamble octets and two SFD octets, is given as

0101 0101 0101 0101 0101 0101 0101 0101 0110 1111 0100 1110

M.3 Generation of the PHR

The Mode Switch (MS) field is set to (0) (no mode switch), the Reserved field entries are set to (0,0), the FCS Type (FCS) field is set to (0) corresponding to a 4-octet FCS, the Data Whitening (DW) field is set to (0) (data whitening is not used), and the Frame Length field entries are set to the binary representation of “7,” corresponding to the PSDU length of the packet. The complete PHR field is shown in Table M.1

Table M.1 – PHR for MR-FSK

| Bit string index | 0 | 1-2 | 3 | 4 | 5-15 |
| --- | --- | --- | --- | --- | --- |
| Bit mapping | MS | R1-R0 | FCS | DW | L10-L0 |
| Field name | Mode Switch | Reserved | FCS Type | Data Whitening | Frame Length |
| Value | 0 | 0 0 | 0 | 0 | 0 0 0 0 0 0 0 0 1 1 1 |

M.4 The message

The example payload of 7 octets is shown below. It constitutes an acknowledgment frame with a 3-octet MHR and a 4-octet FCS, as defined in 5.2.1.9.

The bit sequence of the example PSDU is

0100 0000 0000 0000 0101 0110 0101 1101 0010 1001 1111 1010 0010 1000

M.5 Concatenating the PHR, PSDU, tail bits, and pad bits

Concatenation of the PHR, PSDU, tail bits, and pad bits is performed as described in 16.1.2.4.  After concatenation the bit sequence is given as

0000 0000 0000 0111 0100 0000 0000 0000 0101 0110 0101 1101 0010 1001 1111 1010 0010 1000 0000 1011

M.6 Encoding of the bit sequence

Convolutional coding is performed as described in 16.1.2.4.  The bit sequence after convolutional coding is given as

1111 1111 1111 1111 1111 1111 1100 0110 1000 0100 0011 1111 1111 1111 1111 1111 1100 1011 0111 1001 1111 1011 1010 1000 0100 1110 1101 0011 0110 0101 0110 0001 0000 0010 1101 0000 1111 1111 0010 1110

M.7 Interleaving of the bit sequence

Interleaving is performed as described in 16.1.2.5.  The bit sequence after interleaving is given as

1011 1111 0111 1111 0011 1111 1111 1111 1111 1100 1111 1101 1111 1100 1111 0010 0011 0111 1010 1010 1011 1100 1011 0111 0101 1110 0001 0011 1010 0100 0101 1101 1011 0010 1111 0000 1011 0100 0011 1100

M.8 Concatenating the SHR with the PHR and PSDU

The bit sequence for the PPDU is given as

0101 0101 0101 0101 0101 0101 0101 0101 0110 1111 0100 1110 1011 1111 0111 1111 0011 1111 1111 1111 1111 1100 1111 1101 1111 1100 1111 0010 0011 0111 1010 1010 1011 1100 1011 0111 0101 1110 0001 0011 1010 0100 0101 1101 1011 0010 1111 0000 1011 0100 0011 1100