IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | THz IG Technical Expectations Document (TED) |  |
| Date Submitted | [Nov, 2011] |  |
| Source | Rick Roberts | E-mail: richard.d.roberts@intel.com |
| Re: |  |  |
| Abstract | [THz Technical Expectations] |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Document Overview

This document treats THz communications and related network applications operating in the THz frequency bands between 275 – 3000GHz. Such THz communications applications would include; component to component, board to board, machine to machine, human to machine and human to human, (indoor and outdoor) wireless communications. THz communication applications cover multiple categories with varying requirements. As envisioned, THz communications would overall employ wireless modulation methods of limited complexity, omni and/or directional antenna systems, and would typically offer very high data transfer rates in multiples of 10 Gbps, and up to possibly 1 Tbps, for parity with future fiber optic capacities.

| List of contributors |  |
| --- | --- |
| Ralf-Peter Braun | Deutsche Telekom |
| Michael Grigat | Deutsche Telekom |
| Thomas Schneider | Deutsche Telekom |
| Stefan Preußler | Deutsche Telekom |
| Cai Yunlong | Huawei |
| Thomas Kürner | TU Braunschweig |
| Sebastian Priebe | TU Braunschweig |
| Sebastian Rey | TU Braunschweig |
| Ed. Note: need to add additional names |  |

Table of Contents

1.	Definitions:	9

2.	General Guidelines	10

3.	Protocol Reference Model	11

4.	Use case summary	12

4.1	Outdoor Fixed Wireless Links	12

4.2	Indoor Fixed Wireless Links	12

4.3	Nano Cells	12

4.4	WLAN Types of Applications	12

4.5	WPAN Types of Applications	13

4.6	Connecting devices on short ranges	13

4.7	Kiosk Downloading	13

4.8	Board-to-Board Communications	13

4.9	Data Center/Server Farm Wireless Data Distribution	13

5.	TED Summary	14

6.	Topology	15

6.1	Outdoor Fixed Wireless Topology	15

6.2	Indoor Fixed Wireless Topology	15

6.3	Nano Cell Topology	15

6.4	WLAN Topology	15

6.5	WPAN Topology	15

6.6	Connecting devices on short range Topology	15

6.7	Kiosk Download Topology	15

6.8	Board-to-Board Communications Topology	15

6.9	Data Center/Server Farm Wireless Data Distribution	16

6.9.1	Data center infrastructure [29]	18

6.9.2	3-Tier Data center Infrastructure [29]	18

7.	Data Rates	20

7.1	Outdoor Fixed Wireless Rates	20

7.2	Indoor Fixed Wireless Rates	20

7.3	Nano Cell Rates	20

7.4	WLAN Rates	20

7.5	WPAN Rates	20

7.6	Connecting devices on short range Rates	20

7.7	Kiosk Download Rates	20

7.8	Board-to-Board Communications Rates	20

7.9	Data Center/Server Farm Wireless Data Distribution Rates	21

8.	Operational frequency bands	22

8.1	Outdoor Fixed Wireless Frequencies	22

8.1.1	Outdoor attenuation profile	22

8.1.2	Attenuation in THz transmission windows	25

8.2	Indoor Fixed Wireless Frequencies	26

8.3	Nano Cell Frequencies	26

8.4	WLAN Frequencies	26

8.5	WPAN Frequencies	26

8.6	Connecting devices on short range Frequencies	26

8.7	Kiosk Download Frequencies	26

8.8	Board-to-Board Communications Frequencies	26

8.9	Data Center/Server Farm Wireless Data Distribution Frequencies	26

9.	Coexistence/protection existing services and other use cases	27

9.1	Potentially Critical Interference Scenarios	27

9.1.1	Nomadic devices operated outdoors in rural environment	27

9.1.2	Nomadic devices operated outdoors in urban environment	28

9.1.3	Fixed links with scattering objects close to direct ray path	28

9.1.4	Relevant emission from antenna sidelobes	29

9.1.5	Airborne operated transmitters	29

9.1.6	Satellite in limb scanning mode	30

9.2	Spectral Masks between Active Services and Earth Exploration Services (EESS)	30

9.2.1	Maximum equivalent isotropically radiated power (EIRP)	30

9.2.2	Definition of transmit power spectrum masks	31

10.	Regulatory requirements	33

11.	Transmission range	34

11.1.	Outdoor Fixed Wireless Range	34

11.2.	Indoor Fixed Wireless Range	34

11.3.	Nano Cell Range	34

11.4.	WLAN Range	34

11.5.	WPAN Range	34

11.6.	Connecting devices on short range Range	34

11.7.	Kiosk Download Range	34

11.8	Board-to-Board Communications Range	34

11.9	Data Center/Server Farm Wireless Data Distribution Range	34

12.	Antenna gain and required alignment accuracy	35

12.1.	Outdoor Fixed Wireless Gain and Accuracy	35

12.2.	Indoor Fixed Wireless Gain and Accuracy	36

12.3.	Nano Cell Gain and Accuracy	36

12.4.	WLAN Gain and Accuracy	37

12.5.	WPAN Gain and Accuracy	38

12.6.	Connecting devices on short range Gain and Accuracy	38

12.7.	Kiosk Download Gain and Accuracy	38

12.8.	Board-to-Board Communications Gain and Accuracy	38

12.9.	Data Center/Server Farm Wireless Data Distribution Gain and Accuracy	38

13.	Channel models	39

13.1	Outdoor Fixed Wireless	39

13.1.1	Co-channel interference	39

13.1.2	Fading mechanisms	39

13.1.3	Spatial/temporal properties	39

13.2.	Indoor Fixed Wireless	39

13.2.1	Co-channel interference	39

13.2.2	Fading mechanisms	39

13.2.3	Spatial/temporal properties	39

13.3.	Nano Cells	40

13.3.1	Co-channel interference	40

13.3.2	Fading mechanisms	40

13.3.3	Spatial/temporal properties	40

13.4.	WLAN Types of Applications	40

13.5.	WPAN Types of Applications	41

13.5.1	Co-channel interference	41

13.5.2	Fading mechanisms	41

13.5.3	Spatial/temporal properties	41

13.6.	Connecting devices on short ranges	41

13.6.1	Co-channel interference	41

13.6.2	Fading mechanisms	41

13.6.3	Spatial/temporal properties	41

13.7.	Kiosk Downloading	41

13.7.1	Co-channel interference	41

13.7.2	Fading mechanisms	41

13.7.3	Spatial/temporal properties	42

13.8.	Board-to-Board Communications	42

13.8.1	Co-channel interference	42

13.8.2	Fading mechanisms	42

13.8.3	Spatial/temporal properties	42

13.9	Data Center/Server Farm Wireless Data Distribution Gain and Accuracy	42

13.9.1	Co-channel interference	42

13.9.2	Fading mechanisms	42

13.9.3	Spatial/temporal properties	43

14.	Link budget and SNR analysis	44

14.1	Example Link Budget	44

14.2.	Outdoor Fixed Wireless Link Budget	44

14.3.	Indoor Fixed Wireless Link Budget	44

14.4.	Nano Cell Link Budget	44

14.5.	WLAN Link Budget	44

14.6.	WPAN Link Budget	44

14.7.	Connecting devices on short range Link Budget	44

14.8.	Kiosk Download Link Budget	45

14.9.	Board-to-Board Communications Link Budget	45

14.10	Data Center/Server Farm Wireless Data Distribution Gain and Accuracy	45

15.	Size, Weight and Power	46

15.1.	Outdoor Fixed Wireless SWaP	46

15.2.	Indoor Fixed Wireless SWaP	46

15.3.	Nano Cell SWaP	46

15.4.	WLAN SWaP	46

15.5.	WPAN SWaP	46

15.6.	Connecting devices on short range SWaP	46

15.7.	Kiosk Download SWaP	46

15.8.	Board-to-Board Communications SWaP	46

15.9	Data Center/Server Farm Wireless Data Distribution SWaP	46

16.	Mobility and Link switching	47

16.1.	Outdoor Fixed Wireless Link Switching	47

16.2.	Indoor Fixed Wireless Link Switching	47

16.3.	Nano Cell Link Switching	47

16.4.	WLAN Link Switching	47

16.5.	WPAN Link Switching	47

16.6.	Connecting devices on short range Link Switching	47

16.7.	Kiosk Download Link Switching	47

16.8.	Board-to-Board Communications Link Switching	47

16.9.	Data Center/Server Farm Wireless Data Distribution Link Switching	47

17.	Beam steered or control channel assisted device discovery	48

17.1.	Outdoor Fixed Wireless Device Discovery	48

17.2.	Indoor Fixed Wireless Device Discovery	48

17.3.	Nano Cell Device Discovery	48

17.4.	WLAN Device Discovery	48

17.5.	WPAN Device Discovery	48

17.6.	Connecting devices on short range Device Discovery	48

17.7.	Kiosk Download Device Discovery	48

17.8.	Board-to-Board Communications Device Discovery	48

17.9.	Data Center/Server Farm Wireless Data Distribution Device Discovery	48

18.	Media access mechanism and superframe expectations	49

18.1	Multiband operation	50

19.	Security mechanisms	51

20.	I/O Interfaces and Memory Buffer Considerations	52

21.	References	53

# Definitions:

# General Guidelines

This technical expectations document (TED) describes the technical expectations for a THz standard, such as performance-related issues, reliability issues and availability issues. These types of issues are often called quality of service (QoS). Technical expectations are documented in the same manner as any specifications, including a description, an example, a source or references to related technical documents.

Difference between Expectations and Requirements (TED vs. TRD)

The THz IG group decided to use the term “expectations” instead of “requirements” in order to adopt a less rigid, informal process with the intention to be able to quickly develop actual requirements when the transition to a task group occurs. This document serves to provide guidance for development of IEEE802 THz technical contributions.

# Protocol Reference Model

The communication protocol reference model used for this document is shown in Figure 1.

PHY_SAP: PHY Service Access Point

PLCP: PHY Layer Convergence Protocol

MAC_SAP: MAC Service Access Point

PMD: Physical Medium Dependent (radio)

Figure 1: Reference partitioning

# Use case summary

The following six use case summaries were extracted from document 802.15-11-0749-00-0thz.

## Outdoor Fixed Wireless Links

- Operational environment: Links of backbone network; static use

- Typical range: A few hundred meters up to several kilometers

- Specific propagation conditions: LOS; Atmospheric attenuation becomes important

- Requirements for the antenna alignment: Highly directive antennas; alignment during the installation process by radio engineers

## Indoor Fixed Wireless Links

- Operational environment: Links of backbone network; static use

- Typical range: Up to a few hundred meters or more

- Specific propagation conditions: LOS/Obstructed LOS; Atmospheric attenuation becomes important

- Requirements for the antenna alignment: Highly directive antennas; alignment during the installation process by radio engineers

## Nano Cells

- Operational environment: Part of a hierarchical cellular network; potentially mobile users; indoor as well as outdoor

- Typical range: < 200m

- Specific propagation conditions: LOS/NLOS; dynamically changing conditions

- Requirements for the antenna alignment: automatic beamsteering required

## WLAN Types of Applications

- Operational environment: Connection to access points; nomadic users; mainly indoor

- Typical range: < 100m

- Specific propagation conditions: LOS/NLOS; heavily dynamically changing

- Requirements for the antenna alignment: automatic beamsteering required

## WPAN Types of Applications

- Operational environment: Adhoc networks; nomadic users; mainly indoor

- Typical range: < 10m

- Specific propagation conditions: LOS/NLOS; static to moderately dynamically changing

- Requirements for the antenna alignment: automatic beamsteering required

## Connecting devices on short ranges

- Operational environment: indoor (typically on a desktop), nomadic use

- Typical range: a few cm

- Specific propagation conditions: LOS, multi paths from nearby objects and multiple reflections from Tx and Rx

- Requirements for the antenna alignment: ideally by automatic beamsteering but with Gbps-Link, manual alignment may be possible

## Kiosk Downloading

- Operational environment: indoor, nomadic use

- Typical range: a few cm

- Specific propagation conditions: LOS, multiple reflections from Tx and Rx

- Requirements for the antenna alignment: automatic beamsteering (manual alignment may be very difficult)

## Board-to-Board Communications

- Operational environment: inside computers and inside devices, fixed use

- Typical range: a few cm

- Specific propagation conditions: LOS/NLOS, potentially strong multi paths

- Requirements for the antenna alignment: fixed alignment during design process possible (automatic beamsteering as an option)

## Data Center/Server Farm Wireless Data Distribution

- Enterprise type environment, less sensitivity to complexity issues

- Typical range: up to 100 meters

- Specific propagation conditions: LOS, Obstructed-LOS

- Requirements for the antenna alignment: switched beam, spatial separation for frequency reuse

- Network is self-organizing, fixed node

# TED Summary

This clause contains an overall summary of the rest of the document, mentioning the highlights such as PHY types (assuming multiple options), data rates, topology options (e.g. star, point-to-point, etc.), and MAC frame formats.

# Topology

This clause identifies operational topologies such as point-to-point, star, uni-cast, multi-cast, etc.  This clause also includes topologies that require packet forwarding between nodes.  These operational topologies are use case driven and reference should be included to the pivotal use case documents.

## Outdoor Fixed Wireless Topology

e.g. point-to-point access connections (could include hub-and-spoke (star) topologies), relay connections, repeater connections

## Indoor Fixed Wireless Topology

## Nano Cell Topology

## WLAN Topology

## WPAN Topology

## Connecting devices on short range Topology

## Kiosk Download Topology

## Board-to-Board Communications Topology

## Data Center/Server Farm Wireless Data Distribution

The following information is taken from [31].

In order to apply wireless links in data centers beamforming capabilities are  required, see Fig. 6-1 inclusding the following feature:

- Beamforming capabilities both in azimuth and elevation

- Ceiling reflectors (aluminium plates or other good reflecting materials)

- Electromagnetic absorbers on top of the racks to prevent local reflection/scattering around the antenna

Fig. 6-1 LOS and Indirect LOS Paths [33,34]

Traditional DCN architectures are based on layered 2-tier (3tier-) architectures with core, (aggregation) and access layers [32] A couple of specific arrangements of the servers racks exploring the possibilities to introduce wireless links are proposed as well.

In Fig. 6-2 to 6-4  some of these proposals are presented.

Fig. 6-2 Node Arrangements – Two Parallel Rows [32]

Fig. 6-3: Node Arrangements – Hexagonal Shape [32]

Fig. 6-4: Node Arrangements in a Cayley Data Center [35]

### Data center infrastructure [29]

### 3-Tier Data center Infrastructure [29]

#### Core Layer

The data center core is a Layer 3 domain built with high-bandwidth links (10 GE or a bunch of 10GE)

#### Aggregation Layer

Supports Layer 2 and Layer 3 functionality; using 10 Gbps links.

#### Access Layer/ToR

A Layer 2 domain

ToR using 1Gbps links

#### Topology is tradeoffs

Emerging 40G Ethernet , performance bottlenecks

# Data Rates

The data rates should be sufficient to support the proposed use cases in conjunction with the operational frequency plan and channel model.  This section discusses the data rates, which usually is the bases for different PHY type classes.  The data rates are use case driven.

## Outdoor Fixed Wireless Rates

## Indoor Fixed Wireless Rates

## Nano Cell Rates

## WLAN Rates

In reference [30] some first thoughts about modulation schemes and forward error corrections are presented based on link level simulations with realistic devices.  In particular phase noise will probably only allow for easy modulation schemes like BPSK or QPSK in the THz range.  Nevertheless, with a bandwidth of 50 GHz and QPSK, a gross data rate of 100 Gbit/s can be reached.  The forward error correction schemes will reduce the throughput significantly since the most advanced and efficient techniques may be too power (thermal problem) and chip area consuming. Some simple examples can also be found in reference [30], achieving a bit error rate of 10^-9 for a reasonable SNR in the order of 10dB.

## WPAN Rates

## Connecting devices on short range Rates

## Kiosk Download Rates

## Board-to-Board Communications Rates

## Data Center/Server Farm Wireless Data Distribution Rates

The intended target data rate is 10 Gbps, 40 Gbps, and 100 Gbps [30].  The expected BER for data center applications is 10^-12.

# Operational frequency bands

The THz frequency range is quite large, with some frequencies exhibiting better propagation qualities than others.  Also influencing this issue are practical limits on implementation physics.  In this section the operational frequency bands are discussed.  Each use case may operate in a different band.

## Outdoor Fixed Wireless Frequencies

The atmospheric attenuation related to the different frequencies is shown in Figure 2 with respect to reference [22]

### Outdoor attenuation profile

Figure 2: Atmospheric attenuation of electromagnetic waves in dB/km versus the Frequency in THz for different atmospheric parameters

The attenuation of electromagnetic waves in the atmosphere occurs due to interactions and resonances with the molecules of the atmosphere[22, 23]. Especially the water vapour has an important influence on THz-waves.

The specific attenuation can be calculated with the ITU-R [24] and am [25] models. The result up to a frequency of 1 THz at sea level, for an air pressure of 1013 hPa, a temperature of 15°C and a water vapor density of 7.5 g/m3 can be seen in Figure 3.

Figure 3: Atten. Specific attenuation of the earths atmosphere at sea level

However, fog and especially rain can lead to a scattering of the THz-waves at the water droplets, which reduces the power at the receiver [26, 27].

The additional attenuation due to fog, for water vapor densities of 0.05 g/m3 (vision range 300 m) and 0.5 g/m3 (vision range 50 m) is shown in Figure 4 .

Figure 4 – Additional attenuation due to fog

The additional attenuation due to rain depends on the frequency of the wave and the rain rate. As shown in Figure 5, the maximum attenuation is not in the THz band, but between 100 GHz and 200 GHz.

Figure 4: Specific attenuation of electromagnetic waves due to rain

### Attenuation in THz transmission windows

In most cases the atmospheric attenuation adds to the attenuation of either the fog or the rain, but not both together. The over all attenuation can be seen in Figure 6 [ 22,23]

Figure 5: Attenuation in a clear atmosphere at sea level (blue), fog with a vision range of 50 m (red) and rain with a rain rate of 50 mm/h (black)

If it is assumed that the maximum allowed attenuation for a given application amounts to 100 dB/km, 5 different transmission windows (shaded regions in Figure 6) can be allocated in the frequency range between 300 and 900 GHz. The center frequencies and bandwidths of these transmission windows are given in Table 1 [22, 23].

Table 1	Bandwidths and center frequencies of the transmission windows in the frequency range of 330 GHz and 900 GHz with an overall attenuation below 100 dB/km in the worst case

## Indoor Fixed Wireless Frequencies

## Nano Cell Frequencies

## WLAN Frequencies

## WPAN Frequencies

## Connecting devices on short range Frequencies

## Kiosk Download Frequencies

## Board-to-Board Communications Frequencies

## Data Center/Server Farm Wireless Data Distribution Frequencies

Millimeter wavelength and shorter.

# Coexistence/protection existing services and other use cases

The THz bands are currently used for earth science research (mainly passive) and coexistence/protection of these users is necessary to enable the broad acceptance of the technology.  This section discusses coexistence and protection issues [20], [21].

Also, each use case outlined in clause 4.0 must coexist with all other clause 4.0 use cases.  In some instances this coexistence will be explicit (i.e. different operation bands), but in other instances the coexistence mechanism might be more implicit.

## Potentially Critical Interference Scenarios

This is an ideal estimation of maximum occurring interference powers using worst case assumptions throughout all scenarios with no additional attenuation due to the impact of weather.

### Nomadic devices operated outdoors in rural environment

The assumptions are that the nomadic TX is accidentally pointed in immediate skyward direction as a satellite is passing overhead, directly looking at the ground (0° zenith angle, shortest path).  Also it is assumed that a ground reflection may superimpose constructively with the direct path towards the satellite

### Nomadic devices operated outdoors in urban environment

In this scenario we assume that no objects shadow the direct ray path and that additionally the reflections from buildings around the TX coherently combine at the satellite.

Indoor-operated devices are not relevant due to transmission losses; therefore, indoor setups are implicitly covered by the outdoor scenario.

### Fixed links with scattering objects close to direct ray path

In this scenario, objects close to the direct ray path may scatter/reflect power in a skyward direction towards the satellite.  This scenario is possible despite highly directive antennas.

### Relevant emission from antenna sidelobes

High sidelobe levels may cause non-negligible radiation in a skyward direction with fixed links become especially critical because of potentially high output powers.

### Airborne operated transmitters

In this scenario, radiation through the fuselage or windows is possible, and the transmitted rays may not be attenuated due to the use of composite fuselages.  At higher altitudes we can hardly expect any atmospheric absorption.

### Satellite in limb scanning mode

THz waves may be radiated in the immediate direction of the limb scanner.  Ground-based transmitters do not become relevant due to the long horizontal path.

## Spectral Masks between Active Services and Earth Exploration Services (EESS)

### Maximum equivalent isotropically radiated power (EIRP)

The clause shows how to calculate the maximum equivalent isotropically radiated power (EIRP), which is scenario path loss specific, and includes the satellite antenna gain.  If the TX radiated signal does not exceed the maximum allowed interference power mask, then there will be no interference.

### Definition of transmit power spectrum masks

This clause provides the definition of transmit power spectrum masks and constraints for the isotropically radiated power spectral density.  A simple approximation with line segments  (line parameter sets given in [1]) is used to calculate the effective limitation to several 10 dBm EIRP for bandwidths of several 10 GHz.

#### Nomadic devices in rural and urban areas

#### Fixed links and sidelobes

Fixed links are far less critical than nomadic transmitters and can support higher output powers and longer distances with no significant sidelobe limitations.  These relevant constraints are only applicable below 500 GHz.

#### Airborne TX and limb scanning

Airborne transmitters are most critical with constraints in almost the entire THz range resulting in restrictions requiring lower power and shorter ranges.

# Regulatory requirements

Devices will need to comply with the regulatory requirements specified for THz devices.  Unique to THz is that these regulatory requirements will be evolving over the next few years.  In this section these issues can be discussed.  All regulatory assumptions should be clearly stated.

Active THz systems will have to comply with the ITU Radio Regulations footnote 5.565.

# Transmission range

The transmission range is driven by use cases in conjunction with the operational frequency plan and channel model.  This section discusses the transmission range, which often is the justification for different PHY types.

## Outdoor Fixed Wireless Range

For antenna gains of 70 dBi and an adaptive antenna alignment the range for fixed wireless links with carrier frequencies in the first THz window can be up to 1 km even under worst environmental conditions of a rain rate of 50 mm/h. Even for higher carrier frequencies such link distances are possible, but than the maximum transmissible data rates decrease (see clause 12.1).

## Indoor Fixed Wireless Range

## Nano Cell Range

## WLAN Range

## WPAN Range

## Connecting devices on short range Range

## Kiosk Download Range

## Board-to-Board Communications Range

## Data Center/Server Farm Wireless Data Distribution Range

# Antenna gain and required alignment accuracy

The antenna gain and required alignment accuracy is driven by use cases.  This section discusses the antenna gain and required alignment accuracy, which may be part of the justification for different PHY types.

## Outdoor Fixed Wireless Gain and Accuracy

Due to the very high path loss accompanied with THz transmission, for outdoor applications high antenna gains are required. The antenna gain depends on the distance, transmitted data rate, carrier frequency and application. However, an example for a fixed wireless link with a distance of 1 km under worst environmental conditions of a rain rate of 50 mm/h is given in Figure 7. For shorter distances, or better atmospheric conditions, antennas with lower gain can be used. If a transmit power of 10 dBm, a noise figure of 10 dB, and an ambient temperature of 300 K is assumed, the maximum transmittable data rates per GHz bandwidth in a 1 km link are shown in Figure 7 [23].

From Figure 7, very high data rates can only be transmitted if the antenna gains are respectively high. For an antenna gain of 50 dBi for the transmitter and receiver antennas a maximum data rate of 25 Gbps can be transmitted in the 76 GHz bandwidth available in the first window. However, if the antenna gain is increased to 70 dBi, the maximum data rate can be increased to about 860 Gbps in the first transmission window.

Figure 6: Ideal antenna diameter (left and blue) and diameter of the radiated area (right and red) in dependence of the antenna gain for the first THz window and a distance of 1 km

For 70 dBi antenna gain the angle for loss of connectivity due to fluctuation of the pole and the pole twist is just 0.13°. The requirement for adaptive antenna alignments or control mechanisms to compensate for pole sway/twist depends on the grade of sway/twist impairments, given by the antenna installations, e.g. type of pole or building.

## Indoor Fixed Wireless Gain and Accuracy

## Nano Cell Gain and Accuracy

## WLAN Gain and Accuracy

Simple Link Budget based on realistic components for a direct path only in a wireless lan setup:

| Transmitter power (baseband) | 10 dBm |
| --- | --- |
| Conversion loss | -7.4 dB |
| Antenna gain (transmitter) | + Gtx [dBi] |
| Free space loss | - Lfsl [dB] |
| Antenna gain (receiver) | + Grx [dBi] |
| Noise figure | - 7.6 dB |
| Thermal noise | Pn [dBm] |
| Signal to Noise Ratio | = SNR [dB] |

Using a bandwidth of 50 GHz at a center frequency of 325 GHz and a direct path length of aprox. 6m, this leads to a free space loss (Lfsl) of 98.2dB. In order to achieve a reasonable SNR of 10dB a total antenna gain of 46.4 dBi is required.

The following simulations were performed for a very simple empty room (6m x 4m x 2.5m) wireless LAN scenario, c.f. [30]. Assuming antennas with a Gaussian beam shape in the transmitter and the receiver with identical Half Power Beam Widths (HPBWs in azimuth and elevation), a gain suitable gain of 51.2 dBi can be realized.

As shown in the above figure, a directive antenna can also cancel out the Inter Symbol Interference (ISI) form multipath propagation. A Bit Error Rate (BER) of 10^-4 is defined as Quasi Error Free (QEF) for simulations without Forward Error Correction. Please note that the channel impulse response was normalized, therefore a HPBW of 20° is sufficient to supress ISI for a 16 QAM. Still a HPBW of only 10° is required taking the absolute gain into account.

The perfect alignment of directive antennas is practically impossible. If the HPBW is assumed to be 15° then the next figure shows the impact of an increasing misalignment of the receiver’s antenna. Significant ISI only occurs for a a misaglinment of more than 30° with a 16 QAM. Again the absolute antenna gain is not represented in the simulations. A misalignment of 7.5° (half of the HPBW) means that the SNR will decrease by 3 dB. In a future system design, this will have to be considered in a safety margin for misaglinment in the link budget.

## WPAN Gain and Accuracy

## Connecting devices on short range Gain and Accuracy

## Kiosk Download Gain and Accuracy

## Board-to-Board Communications Gain and Accuracy

## Data Center/Server Farm Wireless Data Distribution Gain and Accuracy

# Channel models

Usually the channel model per sec is a stand-alone document.  This clause contains relevant highlights from the channel model along with a link to the more comprehensive document.  All models must be broadband inasmuch as the required operational bandwidth must be taken into consideration.

## Outdoor Fixed Wireless

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

#### Angle of arrival/departure

#### Time of arrival

#### Delay spread

#### Coherence time/bandwidth

## Indoor Fixed Wireless

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

#### Angle of arrival/departure

#### Time of arrival

#### Delay spread

#### Coherence time/bandwidth

## Nano Cells

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

#### Angle of arrival/departure

#### Time of arrival

#### Delay spread

#### Coherence time/bandwidth

## WLAN Types of Applications

In [36, 37] a stochastic channel model for WLAN/WPAN applications in indoor scenarios is proposed based on a deterministic ray tracing propagation simulator building upon the experimental understanding of the THz indoor radio channel. The model, based upon measurements and calibration described in [38], features broadband channel realizations with 50 GHz bandwidth and  is fully polarimetric, includes spatial channel information and allows for the fast generation of channel realizations. The software for generation of channel model realizations is available on request form the authors, see [36].

## WPAN Types of Applications

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

See section 13.4.3

## Connecting devices on short ranges

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

#### Angle of arrival/departure

#### Time of arrival

#### Delay spread

#### Coherence time/bandwidth

## Kiosk Downloading

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

#### Angle of arrival/departure

#### Time of arrival

#### Delay spread

#### Coherence time/bandwidth

## Board-to-Board Communications

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

#### Angle of arrival/departure

#### Time of arrival

#### Delay spread

#### Coherence time/bandwidth

## Data Center/Server Farm Wireless Data Distribution Gain and Accuracy

### Co-channel interference

### Fading mechanisms

#### Scintillation

#### Molecular attenuation

#### Doppler

### Spatial/temporal properties

#### Angle of arrival/departure

#### Time of arrival

#### Delay spread

#### Coherence time/bandwidth

# Link budget and SNR analysis

It is likely that each use case will have a unique link budget analysis due to differences in required performance and complexity of modulation formats.  Reference [22] contains useful information in regards to link budget calculations.

## Example Link Budget

## Outdoor Fixed Wireless Link Budget

## Indoor Fixed Wireless Link Budget

## Nano Cell Link Budget

## WLAN Link Budget

## WPAN Link Budget

## Connecting devices on short range Link Budget

## Kiosk Download Link Budget

| Simple AWGN Link Budget (Kiosk Downloading) |  |  |
| --- | --- | --- |
| Bit Rate (Gbps) | 20 |  |
| Avg TX Power (dBm) | 10 |  |
| TX Antenna Gain (dBi) | 30 |  |
| EIRP dBm (+40 dBm max) | 40 |  |
| Center Freq (GHz) | 300 |  |
| Aperature Loss @ 1 meter | 81.98419728 |  |
| Standoff Distance (meters) | 1 |  |
| Path Loss at Distance (dB) | 0 |  |
| Obstacle Loss (dB) | 0 |  |
| RX Antenna Gain (dBi) | 15 |  |
| RX Ingested Power (dBm) | -26.98419728 |  |
| Eb (energy / bit) | -129.9944972 |  |
| RX Noise Figure (dB) | 15 |  |
| No (noise power / Hz) | -159 |  |
| Required Eb/No (dB) | 16 |  |
| Implementation Loss (dB) | 10 |  |
| At Distance Link Margin (dB) | 3.005502763 |  |
| Zero Margin Distance (meters) | 1.413432712 |  |

## Board-to-Board Communications Link Budget

## Data Center/Server Farm Wireless Data Distribution Gain and Accuracy

# Size, Weight and Power

Size, weight and power (SWaP) concerns arise from use case expectations and influences range and data rate. The antenna is included in the SWaP estimate.   Of primary concern is mobile operation battery life.  This section discusses these issues.

## Outdoor Fixed Wireless SWaP

## Indoor Fixed Wireless SWaP

## Nano Cell SWaP

## WLAN SWaP

## WPAN SWaP

## Connecting devices on short range SWaP

## Kiosk Download SWaP

## Board-to-Board Communications SWaP

## Data Center/Server Farm Wireless Data Distribution SWaP

# Mobility and Link switching

There may be a need to support link switching due to physical movement or interference.  This section discusses these issues.  Obviously this is use case dependent since for some use cases link switching may not be appropriate.

## Outdoor Fixed Wireless Link Switching

## Indoor Fixed Wireless Link Switching

## Nano Cell Link Switching

## WLAN Link Switching

## WPAN Link Switching

## Connecting devices on short range Link Switching

## Kiosk Download Link Switching

## Board-to-Board Communications Link Switching

## Data Center/Server Farm Wireless Data Distribution Link Switching

# Beam steered or control channel assisted device discovery

There may be a need to support device discovery, which includes aligning the antenna to close the link.  This section discusses these issues. These parameters may be use case dependent.

## Outdoor Fixed Wireless Device Discovery

## Indoor Fixed Wireless Device Discovery

## Nano Cell Device Discovery

## WLAN Device Discovery

## WPAN Device Discovery

## Connecting devices on short range Device Discovery

## Kiosk Download Device Discovery

## Board-to-Board Communications Device Discovery

## Data Center/Server Farm Wireless Data Distribution Device Discovery

# Media access mechanism and superframe expectations

The following two figures are from [28] and are the basis for much of the MAC section material.

## Multiband operation

- Antenna alignment

- Fast session transfer, for example from 60 GHz to THz

# Security mechanisms

# I/O Interfaces and Memory Buffer Considerations

# References

[1] S. Priebe, C. Jastrow, M. Jacob, T. Kleine-Ostmann, T. Schrader, T. Kürner: Channel and Propagation Measurements at 300 GHz, IEEE Transactions on Antennas and Propagation, vol. 59, no. 5, pp. 1688–1698, 2011.

[2] C. Jansen, S. Priebe, C. Möller, M. Jacob, H. Dierke, M. Koch, T. Kürner: Diffuse Scattering from Rough Surfaces in THz Communication Channels, IEEE Transactions on Terahertz Science and Technology, vol.1, no. 2, pp.1688–1698, 2011.

[3] M. Jacob, S. Priebe, R. Dickhoff, T. Kleine-Ostmann, T. Schrader, T. Kürner: Diffraction in mm and Sub-mm Wave Indoor Propagation Channels, IEEE Transactions on Microwave Theory and Techniques, vol. 60, no. 3, pp. 833–844, 2011.

[4] S. Priebe, M. Jacob, C. Jastrow, T. Kleine-Ostmann, T. Schrader, T. Kürner: A Comparison of Indoor Channel Measurements and Ray Tracing Simulations at 300 GHz, Proc. 35th International Conference on Infrared, Millimeter and THz Waves (IRMMW-THz), 2 pages (electronic), Rome, September 2010.

[5] S. Priebe, M. Jacob, T. Kürner: Angular and RMS Delay Spread Modeling in View off THz Indoor Communication Systems, Accepted for publication in Proc. 12th URSI Commission F Triennial Open Symposium on Radio Wave Propagation and Remote Sensing, 8 pages, Garmisch Partenkirchen, March 2011.

[6] S. Priebe, M. Jacob, C. Jansen, T. Kürner: Non-Specular Scattering Modeling for THz Propagation Simulations, Proc. 5th European Conference on Antennas and Propagation (EuCAP), 5 pages (electronic), Rome, April 2011.

[7] S. Priebe, M. Jacob, T. Kürner: Polarization Investigation of Rough Surface Scattering for THz Propagation Modeling, Proc. 5th European Conference on Antennas and Propagation (EuCAP), 5 pages (electronic), Rome, April 2011.

[8] S. Priebe, M. Jacob, T. Kürner: AoA, AoD and ToA Characteristics of Scattered Multipath Clusters for THz Indoor Channel Modeling, Proc. 17th European Wireless Conference (EW), 9 pages (electronic), Vienna, April 2011.

[9] S. Priebe, M. Jacob, T. Kürner: The Impact of Antenna Directivities on THz Indoor Channel Characteristics, Accepted for publication in Proc. European Conference on Antennas and Propagation (EuCAP), Prague, 5 pages (electronic), March 2012.

[10] S. Priebe, M. Jacob, T. Kürner: Affection of THz Indoor Communication Links by Antenna Misalignment, Accepted for publication in Proc. European Conference on Antennas and Propagation (EuCAP), Prague, 5 pages (electronic), March 2012.

[11] T. Kürner, S. Priebe, M. Jacob, C. Jastrow, T. Kleine-Ostmann, T. Schrader: Measuring the Channel Characteristics at 300 GHz - Preliminary Results, IEEE 802.15-09-0496-01-0thz, San Francisco, July 2009.

[12] T. Kürner, S. Priebe, M. Jacob, C. Jastrow, T. Kleine-Ostmann, T. Schrader: Measurements of the Channel Characteristics at 300 GHz, IEEE 802.15-09-0756-00-0thz, Atlanta, November 2009.

[13] S. Priebe, M. Jacob, C. Jastrow, T. Kleine-Ostmann, T. Schrader, T. Kürner: Towards a 300 GHz Channel Model, IEEE 802.15-10-0436-01-0thz, San Diego, July 2010.

[14] S. Priebe, M. Jacob, C. Jastrow, T. Kleine-Ostmann, T. Schrader, T. Kürner: Digital Data Transmission at 300 GHz, IEEE 802.15-10-0437-01-0thz, San Diego, July 2010.

[15] S. Priebe, M. Jacob, T. Kürner: Diffuse Rough Surface Scattering Analysis for THz Communication Systems, IEEE 802.15-11-0146-01-0thz, Singapore, March 2011.

[16] S. Priebe, M. Jacob, T. Kürner: Spatial and Temporal Dispersion in THz Propagation Channels, IEEE 802.15-11-0180-02-0thz, Singapore, March 2011.

[17] S. Priebe, M. Jacob, T. Kürner: Stochastic Modeling of Scattered Multipath Clusters in THz Indoor Communication Channels, IEEE 802.15-11-0474-01-0thz, San Francisco, July 2011.

[18] S. Priebe, M. Jacob, T. Kürner: Will THz Communication Interfere with Passive Remote Sensing?, IEEE 802.15-12-0101-00-0thz, Hawaii, March 2012.

[19] S. Priebe, M. Jacob, T. Kürner: Performance of Antennas in THz Indoor Communication Channels, IEEE 802.15-12-0102-01-0thz, Hawaii, March 2012.

[20] S. Priebe, D. M. Britz, M. Jacob, S. Sarkozy, K. M. K. H. Leong, J. E. Logan, B. S. Gorospe, T. Kürner: Interference Investigations of Active Communications and Passive Earth Exploration Services in the THz Frequency Range“, accepted for publication in IEEE Transactions on THz Science and Technology, 14 pages, 2012

[21] S. Priebe: Interference between THz Communications and Spaceborne Earth Exploration Services, IEEE 802.15-12-0324-00-0thz, San Diego, July 2012

[22] M. Grigat, T. Schneider, S. Preußler, R. P. Braun: Link Budget Considerations for THz Fixed Wireless Links, IEEE 802.15-12-0582-01-0thz, San Antonio, November 2012

[23] T. Schneider, A. Wiatrek, S. Preußler, M. Grigat, R. P. Braun: Link Budget Analysis for Terahertz Fixed Wireless Links, IEEE Transactions on THz Science and Technology 2, 250 – 256 (2012)

[24] Attenuation by Atmospheric Gases, ITU Rec. ITU-R P.676-8, ITU, Oct. 2009

[25] “The am atmospheric model, submillimeter array,” Tech. Memo #152 https://www.cfa.harvard.edu/~spaine/am/

[26] “Attenuation due to clouds and fog”, ITU Rec. ITU-R P.840-4, ITU, Oct. 2009

[27] “Specific attenuation model for rain, for use in prediction methods”, ITU Rec. ITU-R P.838-3, ITU, 2005

[28] S. Priebe: MAC Layer Concepts for THz Communications, IEEE 802.15-13-0119-00-0thz, Orlando, March 2013

[29] Cai Yunlong: Data Center Traffic Characteristics and 100Gb/s Demand, IEEE 802.15-13-0519-00-0thz, Nanjing, September 2013

[30] S. Rey, S. Priebe, Thomas Kürner: Link Level Simulations of THz-Communications, IEEE 802.15-13-0406-00thz, Geneva, July 2013

[31] T. Kürner, Literature Review on Requirements for Wireless Data Centers, IEEE 802.15-13-0411-00-0thz, Geneva, July 2013

[32] H. Vardhan, Wireless Data Center with Millimeter Wave Network, Proc. IEEE Globecom 2010

[33] Zhang W et. al, “3D beamforming for wireless data centers”, in  Proceedings of the 10th ACM Workshop on Hot Topics in Networks. 2011

[34] K. Ramchadran, “60 GHz Data-Center Networking: Wireless Worryless?“, 2008

[35] “On the feasibility of Completely Wireless Data Centers“, http://www.cs.cornell.edu/courses/cs6452/2012sp/papers/cayley.pdf

[36] S. Priebe, Thomas Kürner: A Stochastic Indoor Radio Channel Model for THz WPANs/WLANs, IEEE 802.15-13-0358-00thz, Geneva, July 2013

[37] S. Priebe, T. Kürner: “Stochastic Modeling of THz Indoor Radio Channels“, accepted for publication in IEEE Transactions on Wireless Communications, 12 pages, 2013.

[38] Sebastian Priebe, Martin Jacob, Marius Kannicht, Thomas Kürner: Ultra Broadband Indoor Channel Measurements and Calibrated Ray Tracing Propagation Modeling at THz Frequencies, IEEE 802.15-13-0637-00thz, Dallas, Texas, November 2013