IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4g Sponsor Ballot Recirc. Resolutions for CIDs 23, 24 |  |
| Date Submitted | 7 November 2011 |  |
| Source | [Monique Brown] [M.B. Brown Consulting] | E-mail: [monique.brown@ieee.org] |
| Re: |  |  |
| Abstract | Proposed resolutions for CIDs 23 and 24 given in document 15-11-0753-02 |  |
| Purpose | Proposed resolutions for CIDs 23 and 24 given in document 15-11-0753-02 |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Instructions to the editors:

Modify Table 71 in subclause 9.3 as shown:

| Attribute | Type | Valid range | Description |
| --- | --- | --- | --- |
| phySUNPageEntriesSupported | List of SUN PHY modes | As shown in Figure 64a | Each entry in the list contains the description of a frequency band, modulation scheme, and particular PHY mode implemented by the device. In the case of the channel page 7 entries, there may only be one entry for any given frequency band/modulation scheme pair. If the Generic PHY mechanism is supported, there is only one channel page 8 entry. |
| phyCurrentSUNPageEntry | SUN PHY mode | As shown in Figure 64a | Defines the current frequency band, modulation scheme, and particular PHY mode when phyCurrentPage = 7 or 8. |

Replace Figure I.1 with the following figure:

| Entry | Channel page | Frequency band | Modulation scheme | PHY mode |
| --- | --- | --- | --- | --- |
| 1 | Page 7 | 915 MHz | Filtered FSK | operating modes 1 and 3 supported |
| 2 | Page 7 | 950 MHz | Filtered FSK | operating modes 1,2 and 3 supported |
| 3 | Page 7 | 915 MHz | O-QPSK | all four standard-defined modes supported |
| 4 | Page 8 | reserved | reserved | One Generic PHY mode supported |

Figure I.1 SUN page entries supported

Replace Figure I.2 with the following figure:

| Page 7 | 915 MHz | Filtered FSK | operating mode 3 (200 kb/s) supported |
| --- | --- | --- | --- |

Figure I.2 Current SUN page entry