IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4j Amendment Draft |  |
| Date Submitted | [2nd March 2012] |  |
| Source | [Dave Evans] [Philips, 101 Cambridge Science Park, Cambridge, UK] | Voice: [+44 1293 886490] Fax: [ ] E-mail: [david.evans@philips.com] |
| Re: | [] |  |
| Abstract | [This is the draft of the TG4j Amendment] |  |
| Purpose | [To enable editing of the draft TG4j MBAN amendment.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

###### TG4j Draft Amendment rev 00, 10th November 2011

###### Changes in rev 01, 2nd March 2012

Line numbering added

Normative reference added for 802.1d

Association Proxy text added to clauses 5.1, 5.3 and 6.2

Annex I added describing the Channel Bitmap

Amendment title added

TOC inserted

Insert text in 5.9.3.2 removed

macPeriodicGTSPermit added to 6.4.2 MAC PIB attributes in Table 52.

In Table 44i, references to Table 46 changed to Table 48

In Table 44m, references to Table 46 changed to Table 48

IEEE Standard for

Local and metropolitan area networks—

Part 15.4: Low-Rate Wireless Personal Area Networks (WPANs)

Amendment: Alternative Physical Layer Extension to support Medical Body Area Network (MBAN) services operating in the 2360-2400 MHz band

NOTE—The editing instructions contained in this amendment define how to merge the material contained therein into

the existing base standard and its amendments to form the comprehensive standard.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace.

Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change

and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material).

Delete removes existing material. Insert adds new material without disturbing the existing material. Deletions and

insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used

to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one.

Editing instructions, change markings, and this NOTE will not be carried over into future editions because the changes

will be incorporated into the base standard.

Contents

1. Overview	6

1.2 Scope	6

1.3 Purpose	6

2. Normative references	7

3. Definitions, acronyms and abbreviations	7

3.2 Acronyms and abbreviations	7

5.	MAC protocol	8

5.1 MAC functional description	8

5.1.2.3.5 Channel switch notification	8

5.1.3.3 Association Proxy	9

5.1.7.2 GTS allocation	10

5.1.7.4 GTS deallocation	11

5.1.7.6 GTS expiration	11

5.2 MAC frame format	12

5.2.2.1.3 GTS Specification field	12

5.2.2.1.5 GTS list field	12

5.3 MAC command frames	12

5.3.2.3 Association Status field	13

5.3.9 GTS request command	13

5.3.9.3 Periodic GTS Characteristic field	14

5.3.10 Channel switch notification command	14

5.3.10.1 MHR fields	15

5.3.10.2 Coordinator Short Address field	15

5.3.10.3 Timestamp field	15

5.3.10.4 Channel Number field	15

5.3.10.5 Channel Page field	15

5.3.11 Grant association proxy request	15

5.3.11.1 MHR fields	16

5.3.11.2 Device Number field	16

5.3.12 Grant association proxy response	16

5.3.12.1 MHR fields	17

5.3.12.2 Number of Allocated Short Addresses field	17

5.3.12.3 Short Address fields	17

5.3.12.4 Association Status field	17

5.3.13 Association proxy request	17

5.3.13.1 MHR fields	18

5.3.13.2 Device short address field	18

5.3.13.3 Device extended address field	18

5.3.13.4 Capability Information field	18

5.3.14 Association proxy response	18

5.3.14.1 MHR fields	19

5.3.14.2 Short Address field	19

5.3.14.3 Association Status field	19

6.	MAC services	20

6.2 MAC management service	20

6.2.18 Channel switch notification primitives	20

6.2.18.1 MLME-CHANNELSWITCH.request	20

6.2.18.2 MLME-CHANNELSWITCH.confirm	21

6.2.18.3 MLME-CHANNELSWITCH.indication	22

6.2.19 Periodic GTS management primitives	22

6.2.19.2 MLME-PERIODIC-GTS.confirm	23

6.2.19.3 MLME-PERIODIC-GTS.indication	24

6.2.20 Grant association proxy notification primitives	26

6.2.20.1 MLME-GRANTASSOCIATIONPROXY.request	26

6.2.20.2 MLME-GRANTASSOCIATIONPROXY.indication	27

6.2.20.3 MLME-GRANTASSOCIATIONPROXY.response	28

6.2.20.4 MLME-GRANTASSOCIATIONPROXY.confirm	29

6.2.21 Grant association proxy notification primitives	30

6.2.21.1 MLME-ASSOCIATIONPROXY.request	30

6.2.21.2 MLME-ASSOCIATIONPROXY.indication	31

6.2.21.3 MLME-ASSOCIATIONPROXY.confirm	32

6.4 MAC constants and PIB attributes	33

6.4.2 MAC PIB attributes	33

8. General PHY requirements	34

8. 1 General requirements and definitions	34

8.1.1 Operating frequency range	34

8.1.2 Channel assignments	34

8.1.2.6 Channel numbering for 2380 MHz band	34

9.  PHY services	34

9.3 PHY PIB attributes	34

10. O-QPSK PHY	35

10.2 Modulation and spreading	35

10.2.1 Data rate	35

10.2.4 Symbol-to-chip mapping	35

10.2.5 O-QPSK modulation	35

10.2.6 Pulse shape	35

10.2.7 Chip transmission order	35

10.3 O-QPSK PHY RF requirements	35

10.3.1 Operating frequency range	35

10.3.2 Transmit power spectral density (PSD) mask	36

10.3.3 Symbol rate	36

10.3.5 Receiver interference rejection	36

Annex I	37

Channel bitmap	37

I.1 General	37

I.2 Channel bitmap	37

# 1. Overview

## 1.2 Scope

Insert the paragraph from the PAR scope:

This amendment defines a physical layer for IEEE 802.15.4 in the 2360 to 2400 MHz band which complies with Federal Communications Commission (FCC) MBAN rules. This amendment defines modifications to the MAC needed to support this new physical layer.

## 1.3 Purpose

Insert:

─ 	2360-2400 MHz

# 2. Normative references

Insert the following new reference alphabetically into Clause 2:

IEEE Std 802.1D-2004, IEEE Standard for Local and Metropolitan Area Networks: Media Access Control (MAC) Bridges.

# 3. Definitions, acronyms and abbreviations

## 3.2 Acronyms and abbreviations

Insert the following acronym alphabetically into 3.2:

MBANS		medical body area network services

# MAC protocol

## 5.1 MAC functional description

Insert:

### 5.1.2.3.5 Channel switch notification

The channel switch notification procedure is initiated by the next higher layer by issuing the MLME-CHANNELSWITCH.request primitive, as described in 6.2.18.1, to the MLME.

When a coordinator wants one of its associated devices to switch its operating channel at specific time, the MLME of the coordinator shall send the channel switch notification command in the manner specified by the TxIndirect parameter of the MLME-CHANNELSWITCH.request primitive previously sent by the next higher layer. If TxIndirect is TRUE, the MLME of the coordinator shall send the channel switch notification command to the device using indirect transmission;  i.e. the channel switch notification command frame shall be added to the list of pending transactions stored on the coordinator and extracted at the discretion of the device concerned using the method described in 5.1.6.3. If the command frame is not successfully extracted by the device, the coordinator should consider the device disassociated. Otherwise, the MLME shall send the channel switch notification command to the device directly. In this case, if the channel switch notification command cannot be sent due to a channel access failure, the MAC sublayer shall notify the next higher layer.

Upon successful transmission of the channel switch command, the MAC sublayer shall issue the MLME-CHANNELSWITCH.confirm primitive with a status of SUCCESS.

If a device has received the channel switch command, as defined in 5.3.10, from the coordinator, the MLME shall issue the MLME-CHANNELSWITCH.indication primitive with the ChannelSwitchInfo parameter set to the respective field in the channel switch command.

Figure 16a illustrates the sequence necessary for a coordinator in a beacon-enabled PAN to successfully transmit channel switch notification command to a device using indirect transmission.

##### Figure 16a—Message sequence chart for channel switch notification initiated by a coordinator, using indirect transmission, in a beacon-enabled PAN

Insert:

### 5.1.3.3 Association Proxy

A device shall attempt a request for grant of association proxy after having first associated with a suitable PAN. A coordinator shall allow a grant of association proxy only if macAssociationPermit is set to TRUE. Similarly, a device shall attempt a request for grant of association proxy only if the coordinator is currently allowing association, as indicated in the results of the scanning procedure. If a coordinator with macAssociationPermit set to FALSE receives a grant association proxy request command from a device, the coordinator shall ignore the command.

The MAC sublayer shall initiate the grant association proxy procedure by sending a grant association proxy request command, as described in 5.3.11, to the coordinator of the PAN; if the grant association proxy request command cannot be sent due to a channel access failure, the MAC sublayer shall notify the next higher layer.

The acknowledgment to a grant association proxy request command does not mean that the device has associated. The coordinator needs time to determine whether the current resources available on the PAN are sufficient to allow other devices to associate. The coordinator shall make this decision within macResponseWaitTime of receiving this command. If the coordinator has sufficient resources, it shall allocate a set of short addresses to the devices, and its MAC sublayer shall generate a grant association proxy response command, as described in 5.3.12. The command shall specify the number of devices that have been allocated a short address, the actual short addresses that the devices may use and a status indicating a successful grant of proxy association. If the coordinator has insufficient resources its MLME shall generate a grant association proxy response command containing a status indicating a failure, as defined in Table 6. The grant association proxy response command shall be sent to the device requesting the grant of association proxy using indirect transmission; i.e., the grant association proxy response command frame shall be added to the list of pending transactions stored on the coordinator and extracted at the discretion of the device concerned using the method described in 5.1.6.3.

On receipt of the acknowledgment to the grant association proxy request command, the device shall wait for at most macResponseWaitTime from when the command was received for the coordinator to make its grant of association proxy decision; the PIB attribute macResponseWaitTime is a network-topology-dependent parameter and shall be set to match the specific requirements of the network that a device is trying to join. If the device is tracking the beacon, it shall attempt to extract the grant association proxy response command from the coordinator whenever it is indicated in the beacon frame. If the device is not tracking the beacon, it shall attempt to extract the grant association proxy response command from the coordinator after macResponseWaitTime. If the device does not extract a grant association proxy response command frame from the coordinator within macResponseWaitTime, the MLME shall issue the MLME-GRANTASSOCIATIONPROXY.confirm primitive, as described in 6.2.20.4, with a status of NO_DATA, and the request for grant of association proxy attempt shall be deemed a failure. In this case, the device shall terminate any tracking of the beacon,   by issuing an MLME-SYNC.request primitive, as described in 6.2.13.1, with the TrackBeacon parameter set to FALSE.

A device shall attempt a request for an association proxy after obtaining the extended address and capability information for each of the devices for which it has requested a grant of association proxy. The method for obtaining this information from the devices is outside the scope of this standard.

The MAC sublayer shall initiate the association proxy procedure by sending an association proxy request command, as described in 5.3.13, to the coordinator of the PAN.

On receipt of an association proxy request command, the coordinator shall update the device-specific information for the allocated short address with the device extended address and the device capability information. If the coordinator finds that the device was previously associated with its PAN, it shall replace all previously obtained device-specific information. If the coordinator has successfully updated the device information, it shall respond with an association proxy response command indicating success.

A message sequence chart for grant of an association proxy is illustrated in Figure 19a.

Figure 19a—Message sequence chart for grant of an association proxy

A message sequence chart for an association proxy is illustrated in Figure 19b.

Figure 19b—Message sequence chart for an association proxy

### 5.1.7.2 GTS allocation

Insert after the second paragraph the following text:

To request the allocation of a new periodic GTS, the MLME shall send the periodic GTS request command, as described in 5.3.9, to the PAN coordinator. The Characteristics Type field of the request shall be set to one, and the length, direction, start frame and period exponent fields shall be set according to the desired characteristics of the required periodic GTS.

Insert after the fifth paragraph the following text:

When the PAN coordinator determines whether capacity is available for the requested periodic GTS, it shall generate a periodic GTS descriptor with the requested specifications and the short address of the requesting device. If the periodic GTS was allocated successfully, the PAN coordinator shall set the start slot in the periodic GTS descriptor to the superframe slot at which the periodic GTS begins and the GTS BSN in the periodic GTS descriptor to the number of remained superframe for first allocated periodic GTS slot. In addition, the PAN coordinator shall notify the next higher layer of the new periodic GTS. This notification is achieved when the MLME of the PAN coordinator issues the MLME-PERIODIC-GTS.indication primitive, as described in 6.2.19.3 with the characteristics of the allocated periodic GTS.  If there was not sufficient capacity to allocate the requested GTS, the start slot and GTS BSN shall be set to zero. The PAN coordinator shall then include this periodic GTS descriptor in its beacon and update the GTS Specification field of the beacon frame accordingly. The PAN coordinator shall also update the Final CAP Slot field of the Superframe Specification field of the beacon frame, indicating the final superframe slot utilized by the decreased CAP. The periodic GTS descriptor shall remain in the beacon frame for aGTSDescPersistenceTime superframes, after which it shall be removed automatically. The PAN coordinator shall be allowed to reduce its CAP below aMinCAPLength to accommodate the temporary increase in the beacon frame length due to the inclusion of the periodic GTS descriptor.

### 5.1.7.4 GTS deallocation

Insert after the first paragraph the following text:

A device is instructed to request the deallocation of an existing periodic GTS through the MLME-PERIODIC-GTS.request primitive, as described in 6.2.19.1, using the characteristics of the periodic GTS it wishes to deallocate. From this point onward, the periodic GTS to be deallocated shall not be used by the device, and its stored characteristics shall be reset.

Change:

To request the deallocation of an existing GTS, the MLME shall send the GTS request command, and to request the deallocation of an existing periodic GTS, the MLME shall send the periodic GTS request command, as described in 5.3.9, to the PAN coordinator. The Characteristics Type field of either the GTS Characteristics field or the Periodic GTS Characteristics field of the request shall be set to zero (i.e., GTS deallocation), and the length and direction fields shall be set according to the characteristics of the GTS to deallocate. On receipt of the acknowledgment to either the GTS request command or the periodic GTS request command, the MLME shall notify the next higher layer of the deallocation. This notification is achieved when either the MLME issues the MLME-GTS.confirm primitive, as described in 6.2.6.2, with a status of SUCCESS and a GTSCharacteristics parameter with its Characteristics Type field set to zero or when the MLME issues the MLME-PERIODIC-GTS.confirm primitive, as described in 6.2.19.2, with a status of SUCCESS and a PeriodicGTSCharacteristics parameter with its Characteristics Type field set to zero. If either the GTS request command or the periodic GTS request command is not received correctly by the PAN coordinator, it shall determine that the device has stopped using either its GTS or its periodic GTS by the procedure described in 5.1.7.6.

### 5.1.7.6 GTS expiration

Insert at the end of 5.1.7.6 the following text:

The MLME of the PAN coordinator shall attempt to detect when a device has stopped using a periodic GTS using the following rules:

- For a transmit periodic GTS, the MLME of a PAN coordinator shall assume that a device is no longer using its periodic GTS if a data frame is not received from the device in the GTS at least every 2 × m superframes, where m is defined below.

- For a receive periodic GTS, the MLME of the PAN coordinator shall assume that a device is no longer using its periodic GTS if an acknowledgment frame is not received from the device at least every 2 × m superframes, where P is defined below. If the data frames sent in the GTS do not require acknowledgment frames, the MLME of the PAN coordinator will not be able to detect whether a device is using its receive periodic GTS. However, the PAN coordinator is capable of deallocating the periodic GTS at any time.

The value of m is defined as follows:

m = P * 2(8-macBeaconOrder)		0 ≤ macBeaconOrder ≤ 8, P is defined in 5.3.9.3

m = P				9 ≤ macBeaconOrder ≤ 14, P is defined in 5.3.9.3

## 5.2 MAC frame format

### 5.2.2.1.3 GTS Specification field

Change Figure 42 as indicated:

| Bits: 0-2 | 3-5 | 6 | 7 |
| --- | --- | --- | --- |
| GTS Descriptor Count | Reserved | Periodic GTS Permit | GTS Permit |

Insert the following text at the end of the text in 5.2.2.1.3:

The Periodic GTS Permit field shall be set to one if macPeriodicGTSPermit is equal to TRUE (i.e., the PAN coordinator is accepting periodic GTS requests). Otherwise, the Periodic GTS Permit field shall be set to zero.

### 5.2.2.1.5 GTS list field

Insert the following text at the end of the text in 5.2.2.1.5:

If a GTS descriptor represents a periodic GTS (i.e. the GTS descriptor corresponds to a periodic GTS request), the GTS Length field shall contain the four least significant bits of the macBSN indicating the first superframe that the periodic GTS is available for the addressed device.

# 5.3 MAC command frames

Insert the following and change Table 5:

Table 5—MAC command frames

| Command frame identifier | Command frame | Tx | Rx | Subclause |
| --- | --- | --- | --- | --- |
| 0x0a | Channel switch notification |  |  | 5.3.10 |
| 0x0b | Grant association proxy request |  |  | 5.3.11 |
| 0x0c | Grant association proxy response |  |  | 5.3.12 |
| 0x0d | Association proxy request |  |  | 5.3.13 |
| 0x0e | Association proxy response |  |  | 5.3.14 |
| 0x0af-0xff | Reserved |  |  | - |

### 5.3.2.3 Association Status field

Insert the following and change Table 6:

Table 6—Valid values of the Association Status field

| Association status | Description |
| --- | --- |
| 0x00 | Association successful. |
| 0x01 | PAN at capacity. |
| 0x02 | PAN access denied. |
| 0x03–0x7f 0x0f | Reserved. |
| 0xa0-0xbf | Number of allocated device addresses where the number is offset by 0xa0. |
| 0xc0–0x7f | Reserved. |
| 0x80–0xff | Reserved for MAC primitive enumeration values. |

## 5.3.9 GTS request command

Change the text of 5.3.9 as follows:

The GTS request command is used by an associated device that is requesting the allocation of either a new GTS or a new periodic GTS or the deallocation of either an existing GTS or an existing periodic GTS from the PAN coordinator. Only devices that have a short address less than 0xfffe shall send this command.

Change and insert in Figure 58 as indicated:

| Octets: 7 | 1 | 1/2 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | GTS Characteristics/ Periodic GTS Character |

Insert:

### 5.3.9.3 Periodic GTS Characteristic field

When the GTS request command is used by an associated device that is requesting the allocation of a new periodic GTS or the deallocation of an existing periodic GTS from the PAN coordinator, the Periodic GTS Characteristics field should be included.

The Periodic GTS Characteristics field shall be formatted as illustrated in Figure 59a.

| Bits: 0-3 | 4 | 5 | 6-7 | 8-11 | 12-14 | 15 |
| --- | --- | --- | --- | --- | --- | --- |
| GTS Length | GTS Direction | Characteristic Type | Reserved | Start Frame | GTS Period Exponent | Reserved |

Figure 59a – Periodic GTS Characteristics field format

The GTS Length field shall contain the number of superframe slots being requested for the periodic GTS.

The GTS Direction field shall be set to one if the periodic GTS is to be a receive-only periodic GTS. Conversely, this field shall be set to zero if the GTS is to be a transmit-only periodic GTS. GTS direction is defined relative to the direction of data frame transmissions by the device.

The Characteristics Type field shall be set to one if the characteristics refer to a periodic GTS allocation or zero if the characteristics refer to a periodic GTS deallocation.

The Start Frame field shall contain the value S, 0 ≤ S ≤ 7, which indicates the first periodic GTS for the requesting device is being requested to occur in a superframe no later than (S+1) superframes after the current superframe.

The GTS Period Exponent field shall specify the value, N, and this shall be used to define the period P, in superframes that is being requested for the periodic GTS.

The value of P is defined as follows:

P = 2(N+1)		for 0 ≤ N ≤ 7

Insert:

### 5.3.10 Channel switch notification command

The channel switch notification command shall only be sent by a PAN coordinator.

All devices shall be capable of receiving this command, although an RFD is not required to be capable of transmitting it.

This command is optional.

The channel switch notification command shall be formatted as illustrated in Figure 59b.

| Octets: variable | 1 | 2 | 2 | 6 | 1 | 0/1 |
| --- | --- | --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier | PAN Identifier | Coordinator Short Address | Timestamp | Channel Number | Channel Page |

Figure 59b–Channel switch notification command format

### 5.3.10.1 MHR fields

The Destination Addressing Mode field shall be set to indicate long addressing. The Source Addressing Mode field shall be set to indicate extended addressing.

The Frame Pending field shall be set to zero and ignored upon reception.

The AR field shall be set to one.

The Frame Version field shall be set to 0x01 if the Channel Page field is present. Otherwise it shall be set as specified in 5.2.3.

The Destination PAN Identifier field shall contain the broadcast PAN identifier. The Destination Address field shall contain the extended address of the destination device. The Source PAN Identifier field shall contain the value of macPANId, and the Source Address field shall contain the value of macExtendedAddress.

### 5.3.10.2 Coordinator Short Address field

The Coordinator Short Address field shall contain the value of macShortAddress.

### 5.3.10.3 Timestamp field

The Timestamp subfield is 6 octets in length and contains the time at which the channel switching will be occurred, in symbols. This is a 48-bit value, and the precision of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant.

### 5.3.10.4 Channel Number field

The Channel Number field shall contain the channel number that the coordinator intends to use for all future communications.

### 5.3.10.5 Channel Page field

The Channel Page field, if present, shall contain the channel page that the coordinator intends to use for all future communications. This field may be omitted if the new channel page is the same as the previous channel page.

Insert:

### 5.3.11 Grant association proxy request

The grant association proxy request command allows a FFD to associate a number of RFDs to a PAN coordinator.

Only FFDs shall be capable of transmitting and receiving this command. The FFD shall be associated with the PAN coordinator before using the grant association proxy request command

This command is optional.

The grant association proxy request command shall be formatted as illustrated in Figure 59c.

| Octets: variable | 1 | 1 |
| --- | --- | --- |
| MHR fields | Command Frame Identifier | Device Number |

Figure 59c – Grant association proxy request command format

### 5.3.11.1 MHR fields

The Destination Addressing Mode field shall be set to indicate long addressing. The Source Addressing Mode field shall be set to indicate long addressing.

The Frame Pending field shall be set to zero and ignored upon reception and the AR field shall be set to one.

The Destination PAN Identifier field shall contain the identifier of the PAN to which the grant association proxy is requested. The Destination Address field shall contain the address from the beacon frame that was transmitted by the coordinator to which the grant associated proxy request command is being sent. The Source PAN Identifier field shall contain the broadcast PAN identifier. The Source Address field shall contain the value of macExtendedAddress.

### 5.3.11.2 Device Number field

The Device Number field shall be formatted as illustrated in Figure 59d.

| Bits: 0-4 | Bits: 5-7 |
| --- | --- |
| Number of Devices | Reserved |

Figure 59d – Device Number field format

The Number of Devices shall be set to indicate the total number of devices for which device short addresses are being requested.

### 5.3.12 Grant association proxy response

The grant association proxy response command allows a PAN coordinator to communicate the results of a grant association proxy attempt back to the device requesting a grant association proxy.

This command shall only be sent by the PAN coordinator to a FFD that is currently trying to request a grant association proxy.

Only FFDs shall be capable of receiving this command.

The grant association proxy response command shall be formatted as illustrated in Figure 59e.

| Octets: variable | 1 | 1 | 2*A |  |  | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier | Number of Allocated Short Addresses (A) | Short Address 1 | … | Short Address A | Association Status |

Figure 59e – Grant association proxy response command format

### 5.3.12.1 MHR fields

The Destination Addressing Mode and the Source Addressing Mode fields shall each be set to indicate extended addressing.

The Frame Pending field shall be set to zero and ignored upon reception and the AR field shall be set to one.

The PAN ID Compression field shall be set to one. In accordance with this value of the PAN ID Compression field, the Destination PAN ID field shall contain the value of macPANId, while the Source PAN ID field shall be omitted.

The Destination Address field shall contain the extended address of the device requesting association. The Source Address field shall contain the value of macExtendedAddress.

### 5.3.12.2 Number of Allocated Short Addresses field

If the coordinator was able to associate a number of devices to its PAN, this field shall contain the number A, 1 ≤ A ≤ 32, of short addresses that the coordinator was able to allocate.

### 5.3.12.3 Short Address fields

The number of short address fields shall be equal to A, the number of short addresses that the coordinator was able to allocate. These fields shall contain the short addresses that the devices may use in their communications on the PAN until they are disassociated.

### 5.3.12.4 Association Status field

Valid values of the Association Status field are defined in 5.3.2.3 and Table 6.

### 5.3.13 Association proxy request

The association proxy request command allows a FFD to update a PAN coordinator with the extended addresses and the Capability Information of the RFDs that have been associated using the grant association proxy request and response commands.

Only FFDs shall be capable of transmitting and receiving this command.

This command is optional.

The association proxy request command shall be formatted as illustrated in Figure 59f.

| Octets: variable | 1 | 2 | 8 | 1 |
| --- | --- | --- | --- | --- |
| MHR fields | Command Frame Identifier | Device short address | Device extended address | Capability Information |

Figure 59f – Association proxy request command format

### 5.3.13.1 MHR fields

The Destination Addressing Mode field shall be set to indicate long addressing. The Source Addressing Mode field shall be set to indicate long addressing.

The Frame Pending field shall be set to zero and ignored upon reception and the AR field shall be set to one.

The Destination PAN Identifier field shall contain the identifier of the PAN to which the association proxy is requested.

### 5.3.13.2 Device short address field

The Device short address field shall contain the short address of the device whose extended address and Capability Information is being transferred to the PAN coordinator.

### 5.3.13.3 Device extended address field

The Device extended address field shall contain the extended address of the device whose information is being updated in the PAN coordinator.

### 5.3.13.4 Capability Information field

The Capability Information field shall contain the information of the device whose information is being updated in the PAN coordinator, and shall be formatted as illustrated in figure 50.

### 5.3.14 Association proxy response

The association proxy response command allows a PAN coordinator to communicate the results of an association proxy attempt to the device requesting an association proxy.

This command shall only be sent by the PAN coordinator to a FFD that is currently trying to request an association proxy.

Only FFDs shall be capable of transmitting and receiving this command.

The association proxy response command shall be formatted as illustrated in Figure 59g.

| Octets: variable | 1 | 2 | 1 |
| --- | --- | --- | --- |
| MHR fields | Command Frame Identifier | Short Address | Association Status |

Figure 59g – Association proxy response command format

### 5.3.14.1 MHR fields

The Destination Addressing Mode field shall be set to indicate long addressing. The Source Addressing Mode field shall be set to indicate long addressing.

The Frame Pending field shall be set to zero and ignored upon reception and the AR field shall be set to one.

The PAN ID Compression field shall be set to one. In accordance with this value of the PAN ID Compression field, the Destination PAN ID field shall contain the value of macPANId, while the Source PAN ID field shall be omitted.

The Destination Address field shall contain the extended address of the device requesting association. The Source Address field shall contain the value of macExtendedAddress.

### 5.3.14.2 Short Address field

If the coordinator was not able to associate the device to its PAN, the Short Address field shall be set to 0xffff, and the Association Status field shall contain the reason for the failure.

If the coordinator was able to update the extended address and the Capability Information of the device that had been associated using the association proxy request, this field shall contain the short address of that device.

### 5.3.14.3 Association Status field

Valid values of the Association Status field are defined in 5.3.2.3 and in Table 6.

# MAC services

## 6.2 MAC management service

Insert in Table 8 as indicated:

Table 8—Summary of the primitives accessed through the MLME-SAP

| Name | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| MLME-CHANNELSWITCH | 6.2.18.1* | 6.2.18.3* |  | 6.2.18.2* |
| MLME-PERIODIC-GTS | 6.2.19.1* | 6.2.19.3* |  | 6.2.19.2* |
| MLME-GRANTASSOCIATIONPROXY | 6.2.20.1* | 6.2.20.2* | 6.2.20.3* | 6.2.20.4* |
| MLME-ASSOCIATIONPROXY | 6.2.21.1* | 6.2.21.2* |  | 6.2.21.3* |

Insert:

### 6.2.18 Channel switch notification primitives

These primitives are used by a device to switch operating channel at specific time or by the PAN coordinator to order an associated device to switch operating channel at specific time.

### 6.2.18.1 MLME-CHANNELSWITCH.request

The MLME-CHANNELSWITCH.request primitive is used by the coordinator to instruct an associated device to switch operating channel at specific time.

The semantics of this primitive are:

MLME-CHANNELSWITCH.request	(

DeviceAddrMode,

DeviceAddress,

ChannelSwitchInfo,

TxIndirect,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44a.

Table 44a— MLME-CHANNELSWITCH.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the device which should change its operating channel at specific time |
| DeviceAddress | Device address | As specified by the DeviceAddrMode parameter | The address of the device which should change its operating channel at specific time |
| ChannelSwitchInfo | ChannelSwitch Information value | As defined in 5.3.10 | The information for the channel switch. |
| TxIndirect | Boolean | TRUE, FALSE | TRUE if the channel switch notification command is to be sent indirectly. |
| SecurityLevel | Integer | As defined in Table 46 | As defined in Table 46. |
| KeyIdMode | Integer | As defined in Table 46 | As defined in Table 46. |
| KeySource | Set of octets | As defined in Table 46 | As defined in Table 46. |
| KeyIndex | Integer | As defined in Table 46 | As defined in Table 46. |

If this primitive was received by the MLME of a coordinator with the TxIndirect parameter set to TRUE, the channel switch notification command will be sent using indirect transmission as described in 5.1.5.

If this primitive was received by the MLME of a coordinator with the TxIndirect parameter set to FALSE, the MLME sends a channel switch notification command to the device in the CAP for beacon-enabled PAN.

### 6.2.18.2 MLME-CHANNELSWITCH.confirm

The MLME-CHANNELSWITCH.confirm primitive reports the results of an MLME-CHANNELSWITCH.request primitive.

The semantics of this primitive are as follows:

MLME-CHANNELSWITCH.confirm 	(

status,

DeviceAddrMode,

DeviceAddress

)

The primitive parameters are defined in Table 44b.

Table 44b— MLME-CHANNELSWITCH.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| status | Enumeration | SUCCESS, TRANSACTION_OVERFLOW, TRANSACTION_EXPIRED, NO_ACK, CHANNEL_ACCESS_FAILURE, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY, INVALID_PARAMETER | The status of the channel switch notification attempt. |
| DeviceAddrMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the device which has been instructed to change its operating channel at specific time by its PAN coordinator |
| DeviceAddress | Device address | As specified by the DeviceAddrMode parameter | The address of the device which has been instructed to change its operating channel at specific time by its PAN coordinator |

This primitive returns a status of either SUCCESS, indicating that the channel switch notification was successful, or the appropriate status parameter value indicating the reason for failure.

### 6.2.18.3 MLME-CHANNELSWITCH.indication

The MLME-CHANNELSWITCH.indication primitive is used to indicate the reception of a channel switch notification command.

The semantics of this primitive are as follows:

MLME-CHANNELSWITCH.indication  	(

ChannelSwitchInfo,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44c.

Table 44c-MLME-CHANNELSWITCH.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelSwitchInfo | ChannelSwitch Information value | As defined in 5.3.10. | The information for the channel switch. |
| SecurityLevel | Integer | As defined in Table 48. | As defined in Table 48. |
| KeyIdMode | Integer | As defined in Table 48. | As defined in Table 48. |
| KeySource | Set of octets | As defined in Table 48. | As defined in Table 48. |
| KeyIndex | Integer | As defined in Table 48. | As defined in Table 48. |

Insert the text as follows:

### 6.2.19 Periodic GTS management primitives

These primitives are used to request and maintain periodic GTSs.

##### 6.2.19.1 MLME-PERIODIC-GTS.request

The MLME-PERIODIC-GTS.request primitive allows a device to send a request to the PAN coordinator to allocate a new periodic GTS or to deallocate an existing periodic GTS. This primitive is also used by the PAN coordinator to initiate a periodic GTS deallocation.

The semantics of this primitive are:

MLME-PERIODIC-GTS.request 	(

PeriodicGTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44d.

Table 44d— MLME-PERIODIC-GTS.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeriodicGTSCharacteristics | Periodic GTS characteristics | As defined in 5.3.9.3. | The characteristics of periodic GTS request. |
| SecurityLevel | Integer | As defined in Table 46. | As defined in Table 46. |
| KeyIdMode | Integer | As defined in Table 46. | As defined in Table 46. |
| KeySource | Set of octets | As defined in Table 46. | As defined in Table 46. |
| KeyIndex | Integer | As defined in Table 46. | As defined in Table 46. |

On receipt of the MLME-PERIODIC-GTS.request primitive by a device, the MLME of a device performs either the periodic GTS request procedure, as described in 5.1.7.2, or the periodic GTS deallocation procedure, as described in 5.1.7.4, depending on the value of the PeriodicGTSCharacteristics field.

### 6.2.19.2 MLME-PERIODIC-GTS.confirm

The MLME-PERIODIC-GTS.confirm primitive reports the results of a request to allocate a new periodic GTS or to deallocate an existing periodic GTS.

The semantics of this primitive are:

MLME-GTS.confirm 			(

PeriodicGTSCharacteristics,

status

)

The primitive parameters are defined in Table 44e.

Table 44e— MLME-PERIODIC-GTS.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeriodicGTSCharacteristics | Periodic GTS characteristics | As defined in 5.3.9.3. | The characteristics of periodic GTS request. |
| status | Enumeration | SUCCESS, DENIED, NO_SHORT_ADDRESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY, INVALID_PARAMETER. | The status of the periodic GTS request. |

If the request to allocate or deallocate a periodic GTS was successful, this primitive will return a status of SUCCESS and the Characteristics Type field of the PeriodicGTSCharacteristics parameter will have the value of one or zero, respectively. Otherwise, the status parameter will indicate the appropriate error code, as defined in 5.1.7.2 or 5.1.7.4.

If macShortAddress is equal to 0xfffe or 0xffff, the device is not permitted to request a periodic GTS and the status parameter will be set to NO_SHORT_ADDRESS.

### 6.2.19.3 MLME-PERIODIC-GTS.indication

The MLME-PERIODIC-GTS.indication primitive indicates that a periodic GTS has been allocated or that a previously allocated periodic GTS has been deallocated

The semantics of this primitive are:

MLME-PERIODIC-GTS.indication		(

DeviceAddress,

PeriodicGTSCharacteristics,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44f.

Table 44f— MLME-PERIODIC-GTS.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | 0x0000–0xfffd | The short address of the device that has been allocated or deallocated a periodic GTS. |
| PeriodicGTSCharacteristics | Periodic GTS characteristics | As defined in 5.3.9.3 | The characteristics of periodic GTS request. |
| SecurityLevel | Integer | As defined in Table 48. | If the primitive were generated when a periodic GTS deallocation is initiated by the PAN coordinator itself, the security level to be used is set to 0x00. If the primitive were generated whenever a periodic GTS is allocated or deallocated following the reception of a periodic GTS request command, then it is as defined in Table 48. |
| KeyIdMode | Integer | As defined in Table 48. | If the primitive were generated when a periodic GTS deallocation is initiated by the PAN coordinator itself, the security level to be used is set to 0x00. If the primitive were generated whenever a periodic GTS is allocated or deallocated following the reception of a periodic GTS request command, then it is as defined in Table 48. |
| KeySource | Set of octets | As defined in Table 48. | If the primitive were generated when a periodic GTS deallocation is initiated by the PAN coordinator itself, the security level to be used is set to 0x00. If the primitive were generated whenever a periodic GTS is allocated or deallocated following the reception of a periodic GTS request command, then it is as defined in Table 48. |
| KeyIndex | Integer | As defined in Table 48. | If the primitive were generated when a periodic GTS deallocation is initiated by the PAN coordinator itself, the security level to be used is set to 0x00. If the primitive were generated whenever a periodic GTS is allocated or deallocated following the reception of a periodic GTS request command, then it is as defined in Table 48. |

The value of the Characteristics Type field, as defined in 5.3.9.3, in the PeriodicGTSCharacteristics parameter indicates if the periodic GTS has been allocated or if a periodic GTS has been deallocated.

Insert the text as follows:

### 6.2.20 Grant association proxy notification primitives

These primitives are used when a device requests a grant of association proxy.

### 6.2.20.1 MLME-GRANTASSOCIATIONPROXY.request

The MLME-GRANTASSOCIATIONPROXY.request primitive is used by a FFD to request the grant of an association proxy with a coordinator.

The semantics of this primitive are:

MLME-GRANTASSOCIATIONPROXY.request	(

ChannelNumber,

ChannelPage,

CoordAddressMode,

CoordPANId,

CoordAddress,

NumberOfDevices,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44g.

Table 44g— MLME-GRANTASSOCIATIONPROXY.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelNumber | Integer | Any valid channel number | The channel number on which to attempt association proxy. |
| ChannelPage | Integer | Any valid channel page | The channel page on which to attempt association proxy. |
| CoordAddressMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The coordinator addressing mode for this primitive and subsequent MPDU. |
| CoordPANId | Integer | 0x0000–0xffff | The identifier of the PAN with which to associate. |
| CoordAddress | Device address | As specified by the CoordAddrMode parameter | The address of the coordinator with which to associate. |
| NumberOfDevices | Integer | 0xa0-0xbf | Specifies the number of devices for which a grant association proxy is requested. The number is offset by 0xa0. |
| SecurityLevel | Integer | As defined in Table 46 | As defined in Table 46. |
| KeyIdMode | Integer | As defined in Table 46 | As defined in Table 46. |
| KeySource | Set of octets | As defined in Table 46 | As defined in Table 46. |
| KeyIndex | Integer | As defined in Table 46 | As defined in Table 46. |

### 6.2.20.2 MLME-GRANTASSOCIATIONPROXY.indication

The MLME-GRANTASSOCIATIONPROXY.indication primitive is used to indicate the reception of a grant association proxy request command.

The semantics of this primitive are as follows:

MLME-GRANTASSOCIATIONPROXY.indication	(

DeviceAddress,

NumberOfDevices,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44i

Table 44i—MLME-GRANTASSOCIATIONPROXY.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended IEEE address | The address of the device requesting a grant of association proxy |
| NumberOfDevices | Integer | 0xa0-0xbf | Specifies the number of devices for which an association proxy is requested. The number of devices is offset by 0xa0. |
| SecurityLevel | Integer | As defined in Table 48 | As defined in Table 48. |
| KeyIdMode | Integer | As defined in Table 48 | As defined in Table 48. |
| KeySource | Set of octets | As defined in Table 48 | As defined in Table 48. |
| KeyIndex | Integer | As defined in Table 48 | As defined in Table 48. |

When the next higher layer of a coordinator receives the MLME-GRANTASSOCIATIONPROXY.indication primitive, the coordinator determines whether to accept or reject the unassociated devices using an algorithm outside the scope of this standard.

### 6.2.20.3 MLME-GRANTASSOCIATIONPROXY.response

The MLME-GRANTASSOCIATIONPROXY.response primitive reports the results of an MLME-GRANTASSOCIATIONPROXY.request primitive.

The semantics of this primitive are as follows:

MLME-GRANTASSOCIATIONPROXY.response 	(

DeviceAddress,

NumberAllocatedShortAddresses,

AssocShortAddress,

status,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44j.

Table 44j—MLME-GRANTASSOCIATIONPROXY.response parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DeviceAddress | Device address | An extended 64 bit IEEE address | The address of the device requesting association. |
| NumberAllocatedShortAddresses | Integer | 0xa0-0xbf | The number of allocated short addresses. |
| AssocShortAddress | Integer | 0x0000–0xfffd | The short device address allocated by the coordinator on successful grant association proxy. This parameter is repeated for all of the allocated device short addresses. |
| status | Enumeration | As defined in 5.3.2.3 | The status of the grant association proxy attempt. |
| SecurityLevel | Integer | As defined in Table 46 | As defined in Table 46. |
| KeyIdMode | Integer | As defined in Table 46 | As defined in Table 46. |
| KeySource | Set of octets | As defined in Table 46 | As defined in Table 46. |
| KeyIndex | Integer | As defined in Table 46 | As defined in Table 46. |

When the MLME of a coordinator receives the MLME-GRANTASSOCIATIONPROXY.response primitive, it generates a grant association proxy response command, as described in 5.3.12 and attempts to send it to the device requesting grant association proxy, as described in 5.1.3.3.

### 6.2.20.4 MLME-GRANTASSOCIATIONPROXY.confirm

The MLME-GRANTASSOCIATIONPROXY.confirm primitive is used to inform the next higher layer of the initiating device whether its request to grant an association proxy was successful or unsuccessful.

The semantics of this primitive are as follows:

MLME-GRANTASSOCIATIONPROXY.confirm	(

NumberAllocatedShortAddresses,

AssocShortAddress,

status,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44k.

Table 44k-MLME-GRANTASSOCIATIONPROXY.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| NumberAllocatedShortAddresses | Integer | 0xa0-0xbf | The number of allocated short addresses. |
| AssocShortAddress | Integer | 0x0000-0xfffd | The short device address allocated by the coordinator on successful grant association proxy. This parameter is repeated for all of the allocated device short addresses. |
| status | Enumeration | The value of the Status field of the association response command, as defined in 5.3.2.3, SUCCESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, IMPROPER_KEY_TYPE, IMPROPER_SECURITY_LEVEL, SECURITY_ERROR, UNAVAILABLE_KEY, UNSUPPORTED_LEGACY, UNSUPPORTED_SECURITY, INVALID_PARAMETER | The status of the association attempt. |
| SecurityLevel | Integer | As defined in Table 48 | As defined in Table 48. |
| KeyIdMode | Integer | As defined in Table 48 | As defined in Table 48. |
| KeySource | Set of octets | As defined in Table 48 | As defined in Table 48. |
| KeyIndex | Integer | As defined in Table 48 | As defined in Table 48. |

If the grant association proxy request was successful, then the status parameter shall be set to SUCCESS. Otherwise, the status parameter shall be set to indicate the type of failure.

### 6.2.21 Grant association proxy notification primitives

These primitives are used when a device requests a grant of association proxy.

### 6.2.21.1 MLME-ASSOCIATIONPROXY.request

The MLME-ASSOCIATIONPROXY.request primitive is used by a FFD to update the coordinator with the extended address and Capability Information of a device that has been associated using the grant associated proxy command 5.3.11.

The semantics of this primitive are:

MLME-ASSOCIATIONPROXY.request	(

CoordAddressMode,

CoordPANId,

CoordAddress,

AssocShortAddress,

DeviceAddress,

CapabilityInformation,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44l.

Table 44l—MLME-ASSOCIATIONPROXY.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddressMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the coordinator to which the association proxy is intended. |
| CoordPANId | Integer | 0x0000–0xffff | The PAN identifier of the coordinator to which the association proxy is intended. |
| CoordAddress | Device address | As specified by the CoordAddrMode parameter | The address of the coordinator to which the association proxy in intended. |
| AssocShortAddress | Integer | 0x0000–0xfffd | The short device address allocated by the coordinator on successful association. |
| DeviceAddress | Device address | An extended 64 bit IEEE address | The address of the device requesting an association proxy. |
| CapabilityInformation | Bitmap | As defined in 5.3.1.2 | The operational capabilities of the device requesting an association proxy. |
| SecurityLevel | Integer | As defined in Table 46 | As defined in Table 46. |
| KeyIdMode | Integer | As defined in Table 46 | As defined in Table 46. |
| KeySource | Set of octets | As defined in Table 46 | As defined in Table 46. |
| KeyIndex | Integer | As defined in Table 46 | As defined in Table 46. |

The AssocShortAddress is the address that was allocated to a device during the grant association proxy request. The DeviceAddress and Capability Information are for this device.

### 6.2.21.2 MLME-ASSOCIATIONPROXY.indication

The MLME-ASSOCIATIONPROXY.indication primitive is used to inform the next higher layer of the coordinator of the extended address and Capability Information for a device that was allocated a short address during the grant association proxy request.

The semantics of this primitive are:

MLME-ASSOCIATIONPROXY.indication		(

CoordAddressMode,

CoordPANId,

CoordAddress,

AssocShortAddress,

DeviceAddress,

CapabilityInformation,

SecurityLevel,

KeyIdMode,

KeySource,

KeyIndex

)

The primitive parameters are defined in Table 44m.

Table 44m—MLME-ASSOCIATIONPROXY.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| CoordAddressMode | Enumeration | SHORT_ADDRESS, EXTENDED_ADDRESS | The addressing mode of the coordinator to which the association proxy is intended. |
| CoordPANId | Integer | 0x0000–0xffff | The PAN identifier of the coordinator to which the association proxy is intended. |
| CoordAddress | Device address | As specified by the CoordAddrMode parameter | The address of the coordinator to which the association proxy in intended. |
| AssocShortAddress | Integer | 0x0000–0xfffd | The short device address allocated by the coordinator on successful association. |
| DeviceAddress | Device address | An extended 64 bit IEEE address | The address of the device requesting an association proxy. |
| CapabilityInformation | Bitmap | As defined in 5.3.1.2 | The operational capabilities of the device requesting an association proxy. |
| SecurityLevel | Integer | As defined in Table 48 | As defined in Table 48. |
| KeyIdMode | Integer | As defined in Table 48 | As defined in Table 48. |
| KeySource | Set of octets | As defined in Table 48 | As defined in Table 48. |
| KeyIndex | Integer | As defined in Table 48 | As defined in Table 48. |

### 6.2.21.3 MLME-ASSOCIATIONPROXY.confirm

The MLME-ASSOCIATIONPROXY.confirm primitive reports the results of an association proxy command to update the coordinator with the associated device-specific information.

The semantics of this primitive are:

MLME-ASSOCIATIONPROXY.confirm	(

AssocShortAddress,

DeviceAddress,

status

)

The primitive parameters are defined in Table 44n.

Table 44n—MLME.ASSOCIATIONPROXY.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| AssocShortAddress | Integer | 0x0000–0xfffd | The short device address allocated by the coordinator on successful association. |
| DeviceAddress | Device address | An extended 64 bit IEEE address | The address of the device requesting an association proxy. |
| status | Enumeration | The value of the Status field of the association response command, as defined in 5.3.2.3, SUCCESS, DENIED, NO_SHORT_ADDRESS, CHANNEL_ACCESS_FAILURE, NO_ACK, NO_DATA, COUNTER_ERROR, FRAME_TOO_LONG, UNAVAILABLE_KEY, UNSUPPORTED_SECURITY, INVALID_PARAMETER. | The status of the association proxy request. |

If the association proxy request was successful, then the status parameter shall be set to SUCCESS. Otherwise, the status parameter shall be set to indicate the type of failure.

## 6.4 MAC constants and PIB attributes

### 6.4.2 MAC PIB attributes

Insert in Table 52:

| Attribute | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| macPeriodicGTSPermit* | Boolean | TRUE, FALSE | TRUE if the PAN coordinator is to accept Periodic GTS requests. FALSE otherwise. | TRUE |

# 8. General PHY requirements

## 8. 1 General requirements and definitions

Insert in 8.1 as follows:

- O-QPSK PHY: direct sequence spread spectrum (DSSS) PHY employing offset quadrature phaseshift keying (O-QPSK) modulation, operating in the 780 MHz bands, 868 MHz, 915 MHz, 2380 MHz, and 2450 MHz, as defined in clause 10.

### 8.1.1 Operating frequency range

Insert in Table 66:

| PHY (MHz) | Frequency band (MHz) | Spreading parameters |  | Data parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chip rate (kchip/s) | Modulation | Bit rate (kb/s) | Symbol rate (ksymbol/s) | Symbols |
| 2380 DSSS | 2360-2400 | 2000 | O-QPSK | 250 | 62.5 | 16-ary Orthogonal |

### 8.1.2 Channel assignments

Insert in 8.1.2:

### 8.1.2.6 Channel numbering for 2380 MHz band

For channel page 7, 15 channels numbered zero to fourteen are available across the 2380 MHz band. The centre frequencies of these channels are defined as follows:

Fc = 2363 + 5 k in MHz, for k = 0, 1, …, 6

Fc = 2367 + 5 (k – 7) in MHz, for k = 7, 8, …, 13

and Fc = 2395 in MHz, for k = 14

where

k is the channel number.

# 9.  PHY services

## 9.3 PHY PIB attributes

# 10. O-QPSK PHY

## 10.2 Modulation and spreading

## 10.2.1 Data rate

Change and insert:

The data rate of the O-QPSK PHY shall be 250 kb/s when operating in the 2450 MHz, 915 MHz, or 780 MHz, or 2380 MHz bands and shall be 100 kb/s when operating in the 868 MHz band.

### 10.2.4 Symbol-to-chip mapping

Insert:

In the 2450 MHz and 2380 MHz bands, each data symbol shall be mapped into a 32-chip PN sequence as specified in Table 73. The PN sequences are related to each other through cyclic shifts and/or conjugation (i.e., inversion of odd-indexed chip values).

Table 73—Symbol-to-chip mapping for the 2450 MHz and 2380 MHz bands

### 10.2.5 O-QPSK modulation

Insert:

The chip sequences representing each data symbol are modulated onto the carrier using O-QPSK with halfsine pulse shaping. Even-indexed chips are modulated onto the in-phase (I) carrier, and odd-indexed chips are modulated onto the quadrature-phase (Q) carrier. In the 2450 MHz and 2380 MHz bands, each data symbol is represented by a 32-chip sequence, and so the chip rate is 32 times the symbol rate. In the 915 MHz, 868 MHz, and 780 MHz bands, each data symbol is represented by a 16-chip sequence, and so the chip rate is 16 times the symbol rate. To form the offset between I-phase and Q-phase chip modulation, the Q-phase chips shall be delayed by Tc with respect to the I-phase chips, as illustrated in Figure 70, where Tc is the inverse of the chip rate.

### 10.2.6 Pulse shape

Change and insert:

In the 2450 MHz, 915 MHz, and 868 MHz, and 2380 MHz bands, the half-sine pulse shape is used to represent each baseband chip and is given by:

### 10.2.7 Chip transmission order

Insert:

During each symbol period, the least significant chip, c0, is transmitted first and the most significant chip, either c31, for the 2450 MHz and 2380 MHz bands, or c15, for the 915 MHz, 868 MHz, and 780 MHz bands, is transmitted last.

## 10.3 O-QPSK PHY RF requirements

### 10.3.1 Operating frequency range

Insert as followss:

The O-QPSK PHY operates in the following bands:

— 779–787 MHz

— 868.0–868.6 MHz

— 902–928 MHz

— 2360–2400 MHz

— 2400.0–2483.5 MHz

### 10.3.2 Transmit power spectral density (PSD) mask

Insert:

When operating in the 2380 MHz and 2450 MHz bands, the transmitted spectral products shall be less than the limits specified in Table 76. For both relative and absolute limits, average spectral power shall be measured using a 100 kHz resolution bandwidth. For the relative limit, the reference level shall be the highest average spectral power measured within ±1 MHz of the carrier frequency.

Table 76—2380 MHz and 2450 MHz bands O-QPSK transmit PSD limits

### 10.3.3 Symbol rate

Insert:

The O-QPSK PHY symbol rate shall be 25 ksymbol/s when operating in the 868 MHz band and 62.5 ksymbol/s when operating in the 780 MHz, 915 MHz, 2380 MHz, or 2450 MHz bands with an accuracy of ±40 ppm.

### 10.3.5 Receiver interference rejection

Insert:

This subclause applies only to the 780 MHz, 915 MHz, 2380 MHz, and 2450 MHz bands as there is only one channel available in the 868 MHz band.

Table 77—Minimum receiver interference rejection requirements for the 780 MHz, 915 MHz, 2380 MHz, and 2450 MHz bands

Insert:

# Annex I

(informative)

## Channel bitmap

## I.1 General

To assist with the coexistence to the primary users of the 2360-2400 MHz band, some parts of this band may be excluded from use by MBANS devices. Those parts of the band that are excluded from use may change from time to time. Regulation requires that the operation of MBANS hub devices have access to a mechanism that provides information on which parts of the band cannot be used. The description of this mechanism is beyond the scope of this annex. To assist other MBANS devices such as sensors that do not have direct access to this mechanism, a bitmap may be used by a hub device to indicate those channels that are allowed to be used by MBANS devices. The bitmap only applies to those channels that are in the 2360-2390 MHz band (channels 0-5 and 7-12). The channels in the 2390-2400 MHz band (channels 6 ,13 and 14) are always available for use.

## I.2 Channel bitmap

A bitmap of those channels that are available for use may be inserted into beacon payload. The bitmap shall be formatted as shown in Table I.1.

Table I.1–Channel bitmap

| Bits: 0-11 | Bit 12-22 | Bit 23 |
| --- | --- | --- |
| Channel bitmap | Valid time | Reserved |

The bit to channel numbering of the channel bitmap field is shown in Table I.2. The channel numbering is described in 8.1.2.6.

Table I.2–Bit to channel numbering

| Bit | Channel, k |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 7 |
| 7 | 8 |
| 8 | 9 |
| 9 | 10 |
| 10 | 11 |
| 11 | 12 |

The bits of the channel bitmap field shall be set to one when the corresponding channels are available for use and the bits of the channel bitmap field shall be set to zero when the corresponding channels are not available for use.

The Valid time field indicates the duration in minutes of the availability of the channel bitmap.

IEEE publications are available from the Institute of Electrical and Electronics Engineers, 445 Hoes Lane, P.O. Box 1331, Piscataway, NJ 08855-1331, USA (http://standards.ieee.org).

MBANS hub devices are 802.15.4 PAN coordinators.