IEEE P802.15.9™/D0.01

Draft Recommended Practice for Information Technology—

Telecommunications and information exchange between systems—

Local and metropolitan area networks—

Specific requirements

Part 15.9: Transport of key management protocol (KMP) for IEEE 802.15 WPANs

- Overview

This document defines a recommended practice for the transport of Key Management Protocols (KMP) for WPANs.

  - Scope

This Recommended Practice defines a message exchange framework based on Information Elements as a transport method for key management protocol (KMP) datagrams and guidelines for the use of some existing KMPs with the IEEE Std 802.15.4 and IEEE Std 802.15.7. This Recommended Practice does not create a new KMP.

  - Purpose

This Recommended Practice describes support for transporting KMP datagrams to support the security functionality present in IEEE Std 802.15.4 IEEE Std 802.15.7.

- References

- Definitions

- Acronyms and abbreviations

HIP		Host Identity Protocol

IKEv2		Internet Key Exchange version 2

KMP		key management protocol

WPAN		wireless personal area network

- Introduction

IEEE Std 802.15.4 and IEEE Std 802.15.7 have always supported datagram security, but have not provided a mechanism for establishing the keys used by this feature. Lack of key management support in IEEE Std 802.15.4 and IEEE Std 802.15.7 results in weak keys which is a common avenue for attacking the security system. Adding KMP support is critical to a proper security framework

This Recommended Practice specifies a  transport of KMP datagrams within these standards. It will also provide guidelines for KMPs like IETF's HIP, IKEv2, PANA, and IEEE Std 802.1X.

  - Security Associations

- The security information between two devices is maintained in a Security Association (SA). The role of a KMP is to set up and maintain an SA between two devices.  SAs may be unicast or broadcast.

  - Process flow

- SAs can be established either when a device joins a PAN, or in a non-beaconing PAN just prior to the first data transmission.  In a beaconing PAN, either device configured to require security will start the KMP.  In a non-beaconing PAN, if the sender of the initial data transmission is configured to require security it MUST first establish the SA by starting the KMP.  If the receiver is configured to require security and receives a non-secured data transmission, it MUST start the KMP to establish the SA.

  - State Machine

  - Address formats

- Long addresses SHOULD be used when the KMP is performed to establish an SA.  Short address MAY be used when the KMP updates an existing SA.

  - KMP payload size

- The maximum KMP payload size supported by this recommended practice is 6KB.

- Key Management Transport

  - Description

- The Key Management Transport is encapsulated in payload Information Elements.  These Information Elements are the only content in the MAC frame.  As Key Management payloads may exceed the MPDU, a simple frame chaining method using Forced ACKs will provide the needed fragmentation support.  The use of the Forced ACKs allow the sending device to be assured the receiving device has all the frames to reassemble the Key Management payload.

  - MLME calls

    - KMP start

  - KMP Information Element format

- ID = Varies by standard (i.e. 802.15.4 or 802.15.7)

- Length = KMP fragment + 1; maximum value varies by standard

- IE Content

- Control Field – 1 byte

- Chaining Flag – 1 bit

- 0 = last/only one

- 1 = yes chaining

- Multipurpose ID/Chaining count – 7 bits

- First packet provides Multipurpose ID

- ID range 98 – 126, 97 & 127 reserved

- ID = 98 for KMP

- Chain count (1 – 96 to disambiguate count from ID)

- 1 = 1st fragment

- 2 = 2nd fragment

- 96 = last possible fragment

- 0,98 means only one frame for KMP payload

- 1,98 means 1st frame for KMP payload. 1,(n-1) are additional payload fragments and 0,n is the final, nth, payload fragment.

- KMP fragment

  - Fragmentation state machine

- MAC specifics

  - 802.15.4

    - Payload IE specifics

- ID = 0xa

- Maximum length is 2047 or limited by MPDU

    - Security Association content

  - 802.15.7

    - Payload IE specifics

- IEs limited to COMMAND frames

- ID =

- Maximum length is 255

- Support list of IEs up to number possible in a frame

    - Security Association content

- KMP specifics

  - 802.1X

  - IKEv2

  - HIP

    - Description

    - Use cases

    - 802.15 specifics

      - HIP BEX profile

      - HIP DEX profile

      - Deployment recommendations

      - HIT authentication

        - ACL based

        - RADIUS based

    - SA Definitions

  - PANA

  - SAE

- Annex