IEEE P802.15.9™/D0.01

Draft Recommended Practice for Information Technology—

Telecommunications and information exchange between systems—

Local and metropolitan area networks—

Specific requirements

Part 15.9: Transport of key management protocol (KMP) for IEEE 802.15 WPANs

- Overview

This document defines a recommended practice for the transport of Key Management Protocols (KMP) for WPANs.

  - Scope

This Recommended Practice defines a message exchange framework based on Information Elements as a transport method for key management protocol (KMP) datagrams and guidelines for the use of some existing KMPs with the IEEE Std 802.15.4 and IEEE Std 802.15.7. This Recommended Practice does not create a new KMP.

  - Purpose

This Recommended Practice describes support for transporting KMP datagrams to support the security functionality present in IEEE Std 802.15.4 IEEE Std 802.15.7.

- Normative References

The following referenced documents are indispensable for the application of this document (i.e., they must

be understood and used, so each referenced document is cited in text and its relationship to this document is

explained). For dated references, only the edition cited applies. For undated references, the latest edition of

the referenced document (including any amendments or corrigenda) applies.

IEEE Std 802.15.4™-2011, IEEE Standard for Information technology—Telecommunications and

information exchange between systems—Local and metropolitan area networks—Specific requirements

Part 15.4: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Low Rate

Wireless Personal Area Networks (WPANs).

IEEE Std 802.15.4e™, IEEE Standard for Information technology—Telecommunications and

information exchange between systems—Local and metropolitan area networks—Specific requirements

Part 15.4: Low Rate Wireless Personal Area Networks (LR-WPANs) Amendment to the MAC sub-layer.

IEEE Std 802.15.7™-2011, IEEE Standard for Information technology—Telecommunications and

information exchange between systems—Local and metropolitan area networks—Specific requirements

Part 15.7: Short-Range Wireless Optical Communication Using Visible Light.

- Definitions

Key Management Protocol (KMP): A collection of data transactions that provide the mechanism to manage cryptographic keys. This includes dealing with the generation, exchange, storage, use, and replacement of keys.

Security Association (SA): A simplex (one-way channel) and logical connection which endorses and provides a secure data connection between the network devices.  An SA may include attributes such as: cryptographic algorithm and mode; traffic encryption key(s); and parameters for the network data to be passed over the connection.

- Acronyms and abbreviations

HIP		Host Identity Protocol

IE		Information Element

IKEv2		Internet Key Exchange version 2

KMP		key management protocol

SA		Security Association

WPAN		wireless personal area network

- Introduction

IEEE Std 802.15.4 and IEEE Std 802.15.7 have always supported datagram security, but have not provided a mechanism for establishing the keys used by this feature. Lack of key management support in IEEE Std 802.15.4 and IEEE Std 802.15.7 results in weak keys which is a common avenue for attacking the security system. Adding KMP support is critical to a proper security framework

This Recommended Practice specifies a  transport of KMP datagrams within these standards. It will also provide guidelines for KMPs like IETF's HIP, IKEv2, PANA, and IEEE Std 802.1X.

  - Security Associations

The security information between two devices is maintained in a Security Association (SA). The role of a KMP is to set up and maintain an SA between two devices.  SAs may be unicast or broadcast.

  - Process flow

SAs can be established either when a device joins a PAN, or in a non-beaconing PAN just prior to the first data transmission.  In a beaconing PAN, either device configured to require security will start the KMP.  In a non-beaconing PAN, if the sender of the initial data transmission is configured to require security it MUST first establish the SA by starting the KMP.  If the receiver is configured to require security and receives a non-secured data transmission, it MUST start the KMP to establish the SA.

  - State Machine

  - Address formats

Long addresses SHOULD be used when the KMP is performed to establish an SA.  Short address MAY be used when the KMP updates an existing SA.

  - KMP payload size

The maximum KMP payload size supported by this recommended practice is 9KB.  This is based on a maximum of 96 KMP fragments and 96 bytes per fragment.  Note that IEEE 802.15.4-2011 PSDU maximum is 127 bytes, typical unsecured MHR+MFR = 23 bytes and the KMP IE header is 3 bytes.

- Key Management Transport

  - Description

The Key Management Transport is encapsulated in payload Information Elements.  These Information Elements are the only content in the MAC frame.  As Key Management payloads may exceed the MPDU, a simple frame chaining method using Forced ACKs will provide the needed fragmentation support.  The use of the Forced ACKs allow the sending device to be assured the receiving device has all the frames to reassemble the Key Management payload.

  - MLME calls

    - KMP start

  - KMP Information Element format

ID = Varies by standard (i.e. 802.15.4 or 802.15.7)

Length = KMP fragment + 1; maximum value varies by standard

IE Content

Control Field – 1 byte

Chaining Flag – 1 bit

0 = last/only one

1 = yes chaining

Multipurpose ID/Chaining count – 7 bits

First packet provides Multipurpose ID

ID range 98 – 126, 97 & 127 reserved

ID = 98 for KMP

Chain count (1 – 96 to disambiguate count from ID)

1 = 1st fragment

2 = 2nd fragment

96 = last possible fragment

0,98 means only one frame for KMP payload

1,98 means 1st frame for KMP payload.

1,(n-1) are additional payload fragments

and 0,n is the final, nth, payload fragment.

KMP fragment

First KMP fragment

KMP type – 1 byte

KMP payload fragment

Additional KMP fragments

KMP payload fragment

KMP type

1	802.1X

2	HIP

3	IKEv2

4	PANA

5	SAE

  - Fragmentation state machine

- MAC specifics

  - 802.15.4

    - Payload IE specifics

ID = 0xa

Maximum length is 2047 or limited by MPDU

    - Support via 6lowpan in devices prior to IE

6lowpan (IETF RFC 4944) provides payload differentiation via Dispatch Type.  A Dispatch Type of nnnn indicates the payload is for KMP and the IE format in sec 6.3 will be the dispatch payload.  KMP transport processing within 6lowpan will be as described here.

A PAN controller that supports the IE method and 6lowpan SHOULD support this method as well and be able to support a mix of communicating devices.

    - Security Association content

  - 802.15.7

    - Payload IE specifics

IEs limited to COMMAND frames

ID =

Maximum length is 255

Support list of IEs up to number possible in a frame

    - Security Association content

- KMP specifics

  - 802.1X

  - IKEv2

  - HIP

    - Description

    - Use cases

    - 802.15 specifics

      - HIP BEX profile

      - HIP DEX profile

      - Deployment recommendations

      - HIT authentication

        - ACL based

        - RADIUS based

    - SA Definitions

  - PANA

  - SAE

- Annex