IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |
| --- | --- |
| Title | TG4j Coexistence Assurance Document |
| Date Submitted | April 2012 |
| Source | David Evans (Philips), Raymond Krasinski (Philips), Anuj Batra (TI), Mark Dawkins (Toumaz), Srinath Hosur (TI), Dong Wang (Philips) |
| Re: | IEEE 802.15.4j |
| Abstract | Analysis on coexistence of IEEE 802.15.4j with other IEEE 802 systems within the same frequency band |
| Purpose | To address the coexistence capability of IEEE 802.15.4j to satisfy requirements of the IEEE 802.19 Task Group and IEEE 802 Executive Committee to determine if a proposed IEEE 802 standard has made a reasonable effort to be able to coexist with devices compliant to other IEEE 802 standards in their operating band. |
| Notice | This document has been prepared to assist IEEE P802.15 coexistence. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |

Table of Contents

1	Scope	1

2	References	1

3	Acronyms and abbreviations	2

4	IEEE 802.15.4j amendment overview	2

4.1	Operating frequency bands	2

4.2	Modulation parameters	2

4.3	Coexistence mechanisms	2

5	Other IEEE 802 standards occupying same frequency bands as IEEE 802.15.4j MBANS band	3

5.1	2360-2400 MHz band (US only)	3

6	Coexistence scenarios and analysis	3

6.1	Methodology	3

6.1.1	Path loss model	4

6.1.2	PER for IEEE 802.15.4j	4

6.2	Coexistence with IEEE 802.15.6	5

6.2.1	IEEE 802.15.6 narrow-band PHY parameters	5

6.2.2	Assumptions in coexistence performance	6

6.2.3	Coexistence performance	8

6.3	Coexistence with devices above 2400 MHz	9

7	Discussions and conclusion	9

# Scope

The IEEE 802.19 TAG has mandated that new wireless standards developed under IEEE 802 be accompanied by a Coexistence Assurance document. In [1], guidelines are provided for how coexistence can be quantified based on predicted packet error rates among IEEE 802 wireless devices. A detailed discussion of coexistence and coexistence methods can be found in IEEE Std 802.15.2-2003 [2]. Hence, this coexistence assurance document is provided by the IEEE 802.15.4j Task Group to satisfy the requirements of the IEEE 802.19 Task Group and IEEE 802 Executive Committee.

The IEEE 802.15 Task Group 4j defines an amendment to the IEEE Std 802.15.4 [3], including changes to the Medium Access Control (MAC) layer and additions to the PHY layer to support the 2360-2400 MHz FCC Medical Body Area Network Systems (MBANS) band. This amendment will enable devices to be used on or near a body in compliance with the FCC MBANS Rules [4]. The IEEE 802.15.4j Amendment is only valid for the United States (US) and therefore this document will only address coexistence within the US. At the time of writing of this document, this band cannot be used by MBANS devices outside of the US.

The lower part of the MBANS band from 2360-2390 MHz can only be used in healthcare facilities and the use of the band will be controlled by an MBANS coordinator [4]. The upper part of the MBANS band from 2390-2400 MHz can be used both within healthcare facilities and outside those facilities such as in ambulances and for home healthcare applications. Devices using this band will be under the supervision of healthcare professionals.

This document addresses the coexistence of the IEEE 802.15.4j MBANS band systems with other IEEE 802 standards operating in the same frequency bands. It addresses the interference caused by IEEE 802.15.4j devices to these existing systems, and the interference from these existing standards with IEEE 802.15.4j.

# References

[1]	S. Shellhammer, “Writing a Coexistence Assurance Document,” IEEE 802.19-09/0001r0, 2009.

[2] 	“Coexistence of Wireless Personal Area Networks with Other Wireless Devices Operating in Unlicensed Frequency Band,” IEEE Std 802.15.2TM-2003.

[3] 	IEEE Std 802.15.4TM-2011.

[4] 	FCC MBANS Rules, xxx

[5]	S. Shellhammer, “Estimating of Packet Error Rate Caused by Interference – A Coexistence Assurance Methodology,” IEEE 802.19-05/0028r2, 2005.

[6] 	IEEE Std. 802.15.4TM-2006, Annex E.4.1.

[7] 	IEEE Std. 802.15.6TM-2012.

[8] 	J .G. Proakis, Digital Communications 4th edition, 2000.

[9] 	A. Batra et al, “TG6 Coexistence Assurance Document,” IEEE 802.15-11-0296-00-006, March 2011.

[10]	FCC CFR47 Part 95—Personal Radio Services.

# Acronyms and abbreviations

AWN 			Affected Wireless Network

BAN			Body Area Network

BCH			Bose, Ray-Chaudhuri, Hocquenghem Code

BER			Bit Error Rate

CCA			Clear Channel Assessment

D-MPSK		Differential M-ary Phase Shift Keying

DSSS			Direct Sequence Spread Spectrum

ED			Energy Detection

IWN 			Interfering Wireless Network

LQI			Link Quality Indicator

MBANS			Medical Body Area Network Service

OOB			Out-of-band

O-QPSK		Offset Quadrature Phase Shift Keying

PER			Packet Error Rate

SINR			Signal to Interference plus Noise Ratio

SIR			Signal to Interference Ratio

# IEEE 802.15.4j amendment overview

## Operating frequency bands

The allocated frequency band for the IEEE 802.15.4j Amendment is:

- 2360-2400MHz  (MBANS)

## Modulation parameters

The IEEE 802.15.4j MBANS device modulation parameters in the frequency band as shown in Table 1.

| Frequency band | Data Rate | Symbol rate | Modulation | Spreading factor | Channel spacing |
| --- | --- | --- | --- | --- | --- |
| 2360-2400 MHz | 250 kbps | 62.5 ksps | DSSS O-QPSK | 8 | 5 MHz |

Table 1 – IEEE 802.15.4j modulation parameters

It should be noted that MBANS devices use the same modulation scheme as used by IEEE 802.15.4 devices in the 2400-2483 MHz band.

## Coexistence mechanisms

The coexistence mechanisms for the MBANS band specified in the IEEE 802.15.4j standard are:

- Clear Channel Assessment (CCA)

  - Energy detection and/or carrier sense

- Dynamic channel selection

- Low Duty cycle

- Limited transmission power

- Modulation

  - Direct sequence spread spectrum (DSSS) with half-sine pulse shaping

- ED and LQI

- Channelization Scheme

  - Channel alignment between IEEE 802.15.6 and IEEE 802.15.4j

  - Configurable channelization scheme to minimize co-channel operations with IEEE 802.15.6

# Other IEEE 802 standards occupying same frequency bands as IEEE 802.15.4j MBANS band

This clause lists the IEEE 802 standards that occupy the same frequency band as the IEEE 802.15.4j MBANS band.  The common frequency band is 2360-2400 MHz. There are no other frequency bands used by the IEEE 802.15.4j Amendment.

## 2360-2400 MHz band (US only)

Table 2 shows the list of other IEEE 802 standards that share the 2360-2400 MHz band with the IEEE 802.15.4j Amendment.

| Standard | PHY specification |
| --- | --- |
| IEEE 802.15.6 | D-MPSK |

Table 2 – Other IEEE 802 standards in the 2360-2400 MHz band

# Coexistence scenarios and analysis

This clause presents the analysis of the interference caused by IEEE 802.15.4j MBANS devices to the IEEE 802 standard devices identified in section 5, and vice versa. Note that the interferer parameters chosen in the following sub-clauses are chosen for the purpose of coexistence analysis in this CA document and are a sub-set of the available modes.

## Methodology

In order to quantify the coexistence performance, the techniques described in Shellhammer [1] and [2] have been adopted. The coexistence assurance methodology predicts the PER of an affected wireless network (AWN), in the presence of an interfering wireless network (IWN). In its simplest form, the methodology assumes an AWN and an IWN, each composed of a single transmitter and a receiver. The methodology takes as input a path loss model, a BER function for the AWN, and in some case predicted temporal models for packets generated by the AWN and for packets generated by the IWN. Based on these inputs, the methodology predicts the PER of the AWN as a function of the physical separation between the IWN transmitter and the AWN receiver.

The appeal of the coexistence assurance methodology is that multiple networking standards can be characterized and compared with just a few parameters, notably:

- Bandwidth of AWN and IWN devices

- Path loss model for the networks

- PER as a function of SIR of AWN devices

### Path loss model

The coexistence methodology uses the path loss model described in IEEE 802.15.2-2003 [2], which stipulates a two-segment function with a path loss exponent of 2.0 for the first 8 meters and then a path loss exponent of 3.3 thereafter. The equations for determining the path loss are given in Table 3. The model does not apply below about 0.5 meters due to near-field and implementation effects [2].

| Equation | Condition |
| --- | --- |
| Path loss = 40.2 + 20 log10(d) | 0.5 m ≤ d ≤ 8 m |
| Path loss = 58.2 + 33log10(d/8) | d > 8 m |

Table 3 – Equations for path loss (dB) at 2.4 GHz versus distance (m)

The path loss versus distance is shown in Figure 1 [5].

Figure 1 – Path loss curve for 2.4 GHz band

### PER for IEEE 802.15.4j

The PER for the 2450 MHz O-QPSK PHY in IEEE 802.15.4 is calculated from a BER model. The BER (Pb) is given as a function of SIR as follows [6]:

Pb=815116j=216−1j16jexp20SIR1j−1    (1)

From the computed BER, the PER is obtained as shown in the Equation (2).

PER=1−1−PbL                               (2)

where L is the number of bits in a packet and the typical length of an IEEE 802.15.4j packet is 22 octets [6].

The PER curve as a function of SIR for the IEEE 802.15.4j O-QPSK PHY is plotted in Figure 2.

Figure 2 – PER as a function of SIR for IEEE 802.15.4j PHYs

## Coexistence with IEEE 802.15.6

This section considers issues regarding coexistence between IEEE 802.15.4j devices with IEEE 802.15.6 devices.

### IEEE 802.15.6 narrow-band PHY parameters

The IEEE 802.15.6 narrow-band (NB) PHY has modulation parameters are shown in Table 3 [7].

| Frequency Band | Bit rate | Symbol Rate | Modulation | Spreading factor | Channel bandwidth |
| --- | --- | --- | --- | --- | --- |
| 2360-2400 MHz | 121.4 kbps | 600 ksps | /2 DBPSK | 4 | 1000 kHz |

Table 3 – Specifications of IEEE 802.15.6 NB PHY ((63, 51)-BCH coded π/2-shift D-BPSK)

### Assumptions in coexistence performance

#### Bandwidth

In the 2360-2400 MHz band the IEEE 802.15.6 channel is 1 MHz wide. The IEEE 802.15.4j has a receiver bandwidth that is approximately 2 MHz wide [6].

#### Path loss model

The path loss model used in this section is given in section 6.1.1

#### PER model

To calculate the PER for the IEEE 802.15.6 NB PHY, first of all the error probability of one BCH codeword is derived. According to Proakis [8], the codeword error probability of a perfect code for hard-decision decoding is given by a simple closed form, i.e.

Pw=1−m=0tNmPbm1−PbN−m                    (3)

Where N is the number of symbols in a codeword, Pb is the bit error probability, and t = 2 is the number of symbol error symbols that can be corrected. Although the (63, 51)-BCH code itself is not a perfect code, as shown in Figure 3, the error probability shows good agreement with BER obtained by simulations. This closed form given in Equation (3) is used to compute the PER. The BER for the IEEE 802.15.6 PHY, /2-DBPSK, in the 2400-2483.5 MHz band is given by

Pb=0.5exp−2.42SIR                           (4)

These equations are based on the bit error probability of BPSK with differentially coherent detection [8]. The difference of the coefficients multiplied to SIR comes from the difference of the spreading factors. Then, PER for IEEE 802.15.6 NB-PHY is computed as shown in the Equation (5).

PER=1−1−PwNc                              (5)

where Nc is the number of BCH codewords in one IEEE 802.15.6 payload. For example, if the number of payload bits is 255*8 bits, the number of codewords Nc becomes ceil(255*8/51) = 40. The PER curve for the IEEE 802.15.6 NB-PHY in the 2400-2483.5 MHz band is displayed in Figure 3.

Figure 3 – PER as a function of SINR for IEEE 802.15.6 PHY (2400-2483.5 MHz)

The PER curve for IEEE 802.15.4j is given in 6.1.2

#### Computation of SIR

In this coexistence test, the receive signal power at a receiver in an AWN is assumed to be 10 dB higher than the receiver sensitivity SAWN. The physical separation d, between the AWN’s receiver and a transmitter in an IWN, determines the path loss pl(d) as given in Table 3. Assuming antenna gains for both devices are 0 dBi, SIR at the AWN’s receiver is simply given by

SIR=(SAWN+10)−(Ptx,IWN−pld)                       (6)

Where Ptx,IWN is the transmit signal power from the IWN’s transmitter. The Ptx,IWN is assumed to be 0 dBm for the IEEE 802.15.4j transmitters, and -10 dBm for the IEEE 802.15.6 transmitters. According to [2], if its receiver bandwidth of an AWN’s device, BAWN, is narrower than the IWN’s signal bandwidth BIWN, the SIR is scaled by the ratio between these bandwidths:

SIR=(SAWN+10)−(Ptx,IWN−pld)+BAWNB           (7)

#### Temporal model

Neither the IEEE 802.15.4j devices nor the IEEE 802.15.6 NB-PHY devices operate with 100% duty cycle. By taking into account the duty cycle, a temporal model given in [2] is introduced. In this temporal model, interference packets are uniformly generated in a given duty cycle. The time duration and idle duration is given by the bit rate, length of the payload, and duty ratio. Typical numbers for these parameters are listed in Table 5. The length of typical payload is set to 22 octets for IEEE 802.15.4 [6]. Assuming that IEEE 802.15.6 device has 255-octet payload, which is the maximum length of the payload. On the duty ratio, since IEEE 802.15.6 covers wide variety of applications, it is reasonable to assume the duty cycle at 10 % worst case.

| Modulation | Data rate | Length | Duration | Duty cycle |
| --- | --- | --- | --- | --- |
| π/2-shift D-BPSK (802.15.6) | 121.4 kbps | 255 octets | 16.80 ms | 10 % |
| O-QPSK (802.15.4) | 250 kbps | 22 octets | 0.70 ms | 10 % |

Table 5 – Specifications related to set a temporal model

### Coexistence performance

This clause shows the results of the coexistence assurance methodology [9]. Figure 4-(a) shows the PER of the IEEE 802.15.4 O-QPSK receiver in the presence of interference from an IEEE 802.15.6 signal. The results show that the required physical separation between the IEEE 802.15.4 receiver and an IEEE 802.15.6 transmitter is 12 meters to maintain PER of 10-2. On the other hand, the PER of the IEEE 802.15.6 receiver is displayed in Figure 4-(b) when interfered by IEEE 802.15.4 O-QPSK signal. The required separation is 40 meters to obtain PER of 10-2. Note that the PER that is shown in the figures is for the 2400-2483.5 MHz band. It is assumed that the same PER will be observed in the 2360-2400 MHz band.

- AWN: IEEE 802.15.4, IWN: IEEE 802.15.6

(b) AWN: IEEE 802.15.6, IWN: IEEE 802.15.4

Figure 4 – PER for coexistence assurance results for 2400-2483.5 MHz

## Coexistence with devices above 2400 MHz

The top edge of the MBANS band at 2400 MHz lies adjacent to the 2400-2483.5 MHz band. This band is used by several IEEE standards such as IEEE 802.11, IEEE 802.15.1 and IEEE 802.15.4. It is therefore important to consider the likely impact of MBANS devices that are operating adjacent to other IEEE 802 wireless systems.

The FCC rules [4] for MBANS devices specify that transmitter out-of-band (OOB) emissions are attenuated in accordance with § 95.635 [10]. This requires that the emissions above 2400 MHz are to be less than 500 µV/m measured at 3m and this is equivalent to a power of -41.3 dBm/MHz. Note that this limit is lower than the OOB emissions limits for devices that operate in the 2400-2483.5 MHz band.

To ensure that MBANS devices will meet this OOB emission limit a guard band is placed between the upper MBANS channel and the edge of the band at 2400 MHz. Furthermore, to assist MBANS devices to be able to use the full 20 mW (13 dBm) transmit power that is allowed in the 2390-2400 MHz band, the IEEE 802.15.4j Amendment has specified a channel that is centered on 2395 MHz for this purpose.

# Discussions and conclusion

This document has presented an analysis of the coexistence between the IEEE 802.15.4j Amendment and IEEE 802.15.6 systems. The results of the analysis as presented in figure 4 indicate that distances of over 10m and 30m are required to protect IEEE 802.15.4j and IEEE 802.15.6 systems respectively. This is somewhat alarming since it is expected that these two systems may be used together in close proximity around the human body.

The analysis has focused on the worst case “on-channel” situation where both systems are operating on the same center frequency with 100% duty cycle. The dynamic channel selection feature of IEEE 802.15.4j would minimize the probability of such worst case in practice. When devices are not operated on the same center frequency it is the adjacent and alternate channel performance of the receiver of the device that becomes important.

For IEEE 802.15.6 devices the adjacent channel rejection is 17 dB at a 121.4 kbps data rate [7]. No figure is defined for the alternate channel. This would suggest that once IEEE 802.15.4j devices are operated in the adjacent channel of IEEE 802.15.6 and beyond, safe operating distances of a few meters and below may be possible. It should be noted that the channel plan for the IEEE 802.15.4j Amendment was developed to be in alignment with the channel raster of IEEE 802.15.6. There is therefore the potential for an IEEE 802.15.6 device to operate between two IEEE 802.15.4j channels.

For IEEE 802.15.4j devices the adjacent channel rejection is 0dB and for the alternate channel the rejection figure is 30 dB [3]. The alternate channel figure indicates with suitable channel separation, IEEE 802.15.6 devices can be operated at distances well below 1m.

Concerning the operation of MBANS devices on the band edge at 2400 MHz and adjacent to other IEEE 802 wireless systems, good protection of the IEEE 802 wireless systems operating in the 2400-2483.5 MHz band is ensured by both the FCC MBANS rules and IEEE 802.15.4j Amendment specification.