IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4p Technical Guidance Document |  |
| Date Submitted | 30 July 2026 |  |
| Source | [Jon Adams] [ Lilee Systems] | Voice: [ +1.480.628.6686 ] Fax: [ ] E-mail: [ jonadams @ ieee.org] |
| Re: | Task Group 15.4p |  |
| Abstract | TG4p work in progress: technical guidance for PHY proposals. (Work in Progress) |  |
| Purpose | Discussion within task group |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

802.15.4p PHY System Parameters

Abbreviations

The following abbreviations are used in this document.

AAR		Association of American Railroads

dB		Decibel

EIRP		Equivalent Isotropically Radiated Power

ERP		Effective Radiated Power

FCC		US Federal Communications Commission

FD		Fixed Device

FRA		US DOT Federal Railroad Administration

FTA		US DOT Federal Transit Administration

GSM		Global System for Mobile Communications, originally Groupe Spécial Mobile

GHz 		Gigahertz (1 GHz = 109 Hertz or a frequency of one billion cycles per second)

ITU		International Telecommunication Union

LOS		Line-Of-Sight

M2M		Machine To Machine

MAC		Media Access Control Layer

MHz 		Megahertz (1 MHz = 106 Hertz or a frequency of one million cycles per second)

NLOS		Non-Line-Of-Sight

PAR		Project Authorization Request

PDP		Power Delay Profile

PHY		Physical Layer

PICS		Protocol Implementation Conformance Statement

PTC		Positive Train Control

TG4p		IEEE 802 Working Group 15 Task Group p

TGD		Technical Guidance Document

UHF		Ultra High Frequency

VHF		Very High Frequency

WPAN		Wireless Personal Area Network

# Introduction

## Purpose

This document provides technical parameters and criteria to guide the selection of PHY proposals for task group 15.4p. This document defines the PHY characteristics that satisfy train control wireless application requirements, and provides a framework for evaluating proposals.  This document outlines the key characteristics that are specified as a set of PHY layer parameters, with some performance criteria and constraints for the parameter set.

The intent of the task group is to use a flexible and efficient process which provides sufficient descriptions of the technical requirements to enable relevant responses, with efficiency of effort while meeting the critical need for a timely standard.

## Methodology

The methodology provides a consensus approach to defining a minimal subset of features, characteristics, performance and constraints. This document will provide

- A functional view of the PHY characteristics, in the form of specific parameters which define externally verifiable performance and interoperability characteristics;

- A dependency matrix, which considers the interdependency of tradeoffs which are made in the selection of PHY features;

- An application/performance matrix which characterizes by classes of rail control applications the priority of performance characteristics; this shall include a link budget, with at least transmitter parameters, receiver parameters, and link margin in AWGN (see Link Budget section).

The parameters table provides guidance on developing complete technical proposals. This represents a subset of parameters, and the absence of a parameter should not be seen as a constraint. The parameter column consists of two sub-columns. The first identifies the parameter, which should be addressed in the proposal; the second provides some examples of how this may be addressed in a proposal; there may be alternatives appropriate to specifying the characteristic.  The performance criteria column includes constraints (derived form the PAR) or explanations.  The regulator column is intended to identify where regional differences in regulations (present and anticipated) may affect the PHY characteristics.

## High Level Requirements

The PAR establishes general requirements for the amendment in the “scope”, which can be summarized as follows:

- PHY amendment

  - MAC additions only where needed to support PHY

- Licensed or license exempt frequency bands

  - In general, operation up to 6 GHz

  - Specific bands of interest or bands in use today for similar systems: 216 – 222 MHz, 452/457 MHz, 896 - 902 MHz, 902 – 928 MHz, 928 – 952 MHz, 5850 – 5925 MHz

- Low data rate: 4800 baud to 1000 kbps

- Optimal energy efficient link margin given typical rail and rail transit environments

  - Principally outdoors, but also in subway, viaduct, tunnel, urban canyon, or other environments where propagation may be impacted

- Moderately dense deployments: at least 100 - 200 direct neighbors

- Good coexistence mechanisms (both ways) and potentially the ability for the PHY to adapt to unintentional interference caused by anomalous propagation

The PAR and 5C documents can be found on the 802.15 document page (https://mentor.ieee.org/802.15/documents):

15-11-0821-09-0ptc_PTC_PAR.doc

15-11-0876-07-0ptc-Positive_Train_Control_Five_Criteria.doc

Major points expressed in the Scope paragraph of the PAR:

- This amendment specifies a physical layer (PHY) for IEEE 802.15.4, and any medium access control (MAC) changes needed to support this PHY

  - For use in equipment intended to address industry needs and to meet United States (US) Positive Train Control regulatory requirements and similar regulatory requirements in other parts of the world

- Operation in licensed or license-free radio bands

- Operation up to 6 GHz

- Depending on frequency band and operating rules, TX output power can  >+30dBm

- Meets requirements at speeds up to 600 km/h

- Range up to 70 km

- Allows operation in contiguous or non-contiguous channel bandwidths as narrow as 5 kHz

- supports data rates up to 1 Mbps with flexible and robust quality of service

- Provides modulation methods and spectral characteristics consistent with local regulatory requirements

- Accommodates rapidly changing network membership.

While the PAR is focused currently on rail and rail transit systems due to regulatory requirements, it may

# PHY Parameters

| PHY Parameter |  | Performance Criteria, Constraints, Comments | Regional Regulatory |
| --- | --- | --- | --- |
| Parameter: | Example: |  |  |
| Operating band (band/channel plan) | - Channel change time min/max - Channel bandwidth - Channel spacing - Number of channels | License and License exempt Specified use (dedicated, application constrained) license exempt | Band availability varies greatly. Proposer should specify what regions they intend to target. |
| Modulation and Coding Scheme(s) | - Modulation method - Modulation accuracy (e.g. EVM) - Whitening - Chipping | Suitable for low cost implementation Meet regulator domain |  |
| PHY frame structure | - Pre-amble - Sync Header, SFD length - codes and/or patterns - as appropriate to proposal |  |  |
| Synchronization and Timing | - might come from specific sync mechanisms or may be dependent on other PHY features - clock accuracy / stability required |  |  |
| Data rate(s) |  | 4800 baud to 1000 kbps in the PAR Specify intended supported rate(s) |  |
| Symbol / chip rate | - As appropriate to the proposed technique |  |  |
| Transmit Power | - MAX - MIN - Peak to Average - Management |  | Very definitely regionally dependent and upon the operating rules |
| PSD | - In band - Out of band |  | US FCC Part 90 specifies a variety of transmit PSD masks, and measurement methods. Most of the operation of rail and rail transit systems in the US is contemplated to be under Part 90 rules. There are systems that may use the Intelligent Transportation Band at |
| Receiver Sensitivity | Specify dBm and conditions, for example: -85dBm PSDU length = 20 octets. PER < 1%. Power measured at antenna terminals. Interference not present. - (see section 6.1.7 of 15.4-2006 for example) |  | Note that the parameter set shown is for the original 250kbps DSSS coding |
| Chan availability (interference detection) | - Spectrum scanning - CCA |  |  |
| Link Quality Indication | - Technique used - Frequency of assessment - Accuracy and resolution - Bi-directional (cooperative)? |  |  |
| Reliability enhancing features/methods | - Detection (check sequence) - Prevention (FEC) - Interference mitigation/avoidance - Collision avoidance |  |  |
| Co-existence features | - Adjacent channel rejection - Channel sensing, scanning, assessment - Coordination mechanisms for different radios in close proximity | (Key parameters which impact coexistence should be described) | Latest copy of doc # TBD is good guidance on coexistence scenarios. |
| MAC dependencies / support required | - Channel sequence control - Error control - Fragmentation support - Channel access - Scanning | (MAC features required to support the PHY) | Identify two categories: “Must have” MAC features “nice to have” MAC features |

| MAC Parameter (Additional MAC features within the scope of the PAR) |  | Performance Criteria, Constraints, Comments | Regional Regulatory |
| --- | --- | --- | --- |
| Duty cycle support |  |  |  |
| Channel dwell time |  |  |  |
| Channel access |  |  |  |
| Sequence control |  |  |  |
| … |  |  |  |

# PHY Parameter Dependency Matrix

| PHY Parameter |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Freq Band | Channel Bandwidth | Number of Channels | Modulation | TX power | PSD | Data Rate | Range | Duty Cycle | Co-ex footprint |  |
| Frequency Band | - | x | x | x | x | x |  | x | x | x |  |
| Channel Bandwidth |  | - | x | x | x | x | x | x |  | x |  |
| Number of Channels |  |  | - |  | x | x |  |  |  | x |  |
| Modulation |  |  |  | - |  | x | x | x |  |  |  |
| TX power |  |  |  |  | - | x |  |  | x | x |  |
| PSD |  |  |  |  |  | - |  |  | x | x |  |
| Data Rate |  |  |  |  |  |  | - |  | x | x |  |
| Range |  |  |  |  |  |  |  | - |  | x |  |
| Duty cycle |  |  |  |  |  |  |  |  | - | x |  |
| Co-ex footprint |  |  |  |  |  |  |  |  |  | - |  |
|  |  |  |  |  |  |  |  |  |  |  | - |

# Performance

This section can identify performance considerations, constraints and requirements.

## Link Budget

| Parameter | Value |
| --- | --- |
| Transmit power |  |
| Frequency of operation |  |
| Transmit antenna gain |  |
| Noise figure |  |
| Eb/No |  |
| Implementation losses |  |
| Sensitivity |  |
| Link margin |  |
| Distance |  |

## Channel Characteristics

The 15.4p PAR allows for PHYs that are well behaved in mobility environments up to 600 km/h. This mobility requirement has not been a strong component of previous 15.4 amendments and this work needs to be done in order to understand the impact of that mobility as a function of frequency and path. However, previous work in channel modeling has provided some baselines. The 802.15.4g channel characteristics sub-group was presented measurement data from several real-world deployments which demonstrated a wide range of actual channel conditions.  The conclusions of that group are summarized in 802.15-09-0279-00-004g.

Regarding Multipath Mitigation, the subgroup recommends that the TG4p proposers state the following:

- For proposed PHY in widespread deployment, state how multipath has been mitigated,

Otherwise, state planned multipath mitigation techniques.

# Frequency Bands to Be Considered

[include table based on the preliminary proposals and the regulatory information presented]

# Definitions

The following provides definition of specific terms in the context of discussion with respect to TG4p applications and PHY proposals.

| Term: | Definition |
| --- | --- |