IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG4j Amendment text for channel switch IE |  |
| Date Submitted | [16th July 2012] |  |
| Source | [Dave Evans] [Philips] [101 Cambridge Science Park, Cambridge, UK] | Voice: [+44 1293 886490] Fax: [ ] E-mail: [david.evans@philips.com] |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [Defines the two IE that are used for channel switching when using the O-QPSK PHY. This is the text for the TG4j amendment draft 2.] |  |
| Purpose | [For consideration by TG4j] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

## 5.2.4.23 PHY Parameter Change IE

The PHY Parameter Change IE is used by a device to notify a peer device or devices to switch operating band, channel, or other PHY specific operational parameters.   The IE may be used in a directed frame to initiate a change between specific peers, or may be used in periodic beacons to effect a coordinated change among members of a PAN.  The specific procedures for affecting a change are out of scope of this standard.

The IE shall be formatted as illustrated in Figure 48uu.

| Octets: 4 | 2 |
| --- | --- |
| Effective Time of Change | Notification time |

Figure 48uu—PHY Parameter Change IE

The Effective Time of Change field shall contain a time in the future, in microseconds, when the change should occur.  This time shall be decremented accordingly every time the PHY Parameter Change IE is transmitted.

The Notification time field shall contain the local time value in the generating device at the time the frame containing the IE is generated.

The PHY Parameter Change IE shall always be followed in the frame by either a valid Operating Mode Description IE or a valid Specific Parameter Change IE describing the desired change.

## 5.2.4.24 O-QPSK PHY Specific Parameter Change IE

An encoding for representing an operating mode for the O-QPSK PHY defined in Clause 10 is shown in Figure 48vv.

| Octets: 1 | 1 |
| --- | --- |
| Band Selector | Channel Index |

Figure 48vv—O-QPSK PHY Specific Parameter Change IE

The Band Selector field identifies the band for the operating mode and shall take one of the values in Table 4s.

Table 4s—Band Selector frequency bands

| Band selector value | Corresponding Band |
| --- | --- |
| 1 | 779–787 MHz |
| 2 | 868.0–868.6 MHz |
| 3 | 902–928 MHz |
| 4 | 2360–2400 MHz |
| 5 | 2400.0–2483.5 MHz |

The Channel Index field shall contain a valid channel number as defined in 8.1.2.1, 8.1.2.2, or 8.1.2.11 as appropriate to the band selector value.